3GPP TSG-RAN WG2 Meeting #127		     R2-2406645

Maastricht , NL: 19th August – 23rd August 2024

Source:	Qualcomm Incorporated

Title:	On UE Side Data Collection

Agenda Item:	8.1.4 UE side data collection

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, at RAN #102, a new Work Item on artificial intelligence/machine learning for NR air interface has been approved in [3].

The Rel-19 WI “a new Work Item on artificial intelligence/machine learning for NR air interface” [3] describes the study objectives for the UE-side data collection as follows:

- CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods

In the post RAN2#126 email discussion [POST126][034][AIML PHY] TP for data collection (Ericsson) [4], the table below is proposed to be captured in TS 38.843 [2] based on the agreement in the RAN2#126 meeting [5].

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Controllability can be achieved. FFS: level of controllability | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. |  |  |  |  |

This paper discusses FFS items for solutions 1b), 2, and 3. This paper discusses controllability and visibility aspects of solutions 1b), 2, and 3. Without the lack of generality, we consider the Event Exposure (EVEX) framework as a reference for discussing how controllability and visibility can be achieved in solution 1b). However, exact solution details for solution 1b) can be left to relevant working groups.

# On the UE side Data Collection

As discussed above, this paper discusses FFS items for solutions 1b), 2, and 3. This paper discusses controllability and visibility aspects of solutions 1b), 2, and 3. Without the lack of generality, we consider the Event Exposure (EVEX) framework as a reference for discussing how controllability and visibility can be achieved in solution 1b). However, exact solution details for solution 1b) can be left to relevant working groups.

## 2.1 On UE side data collection using solution 1b)

This section describes how controllability and visibility can be achieved in solution 1b).

### 2.1.1 Controllability of Data Collection for Solution 1b)

As discussed in TS 26.531 [6], in the collaboration/deployment scenario presented below, the Data Collection Application Function (DCAF), Application Service Provider (ASP) as well as Application Server (AS) are internal to the 5G system. In solution 1b) the DCAF can be considered as “the server for data collection for UE-side model training”, providing full control to the network on the data collection procedure.

Observation 1: TS 26.531 [6] elaborates collaboration/deployment scenario, where “the server for data collection for UE-side model training”, for example, Data Collection Application Function (DCAF) is internal to the 5G system, and is under direct control of MNO.

Although TS 26.531 [6] describes other deployment/collaboration scenarios, the relevant working groups can restrict the deployment/collaboration scenarios for UE-side radio-related data collection for model training to achieve full network controllability and visibility.

Proposal 1: Full network controllability on UE-side data collection is achieved with a deployment/collaboration scenario where “the server for data collection for UE-side model training” is internal to the 5G system and under the direct control of MNO.

### 2.1.2 Visibility of collected data in Solution 1b)

In TS 26.532 [7], the different 3GPP standards are referred to for describing the format of data reports submitted to the data collection AF, as shown below

Table A.1-1: Externally defined data types used by Ndcaf_DataReporting_Report operation

| Data type | Comments | Reference |
| --- | --- | --- |
| BitRate |  | 3GPP TS 29.571 [12] |
| PacketDelBudget |  |  |
| PacketLossRate |  |  |
| DateTime |  |  |
| DurationSec |  |  |
| SvcExperience |  | 3GPP TS 29.517 [5] |
| AddrFqdn |  |  |
| TimeWindow |  | 3GPP TS 29.122 [14] |
| Volume |  |  |
| FlowInfo |  |  |
| LocationArea5G |  |  |
| LocationData |  | 3GPP TS 29.572 [15] |
| HorizontalSpeed |  |  |

Considering ‘HorizontalSpeed’ as an example, several parameters and their values are described as follows:

6.1.6.2.18 Type: HorizontalVelocity

Table 6.1.6.2.18-1: Definition of type HorizontalVelocity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Horizontal speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angle in degrees, measured clockwise from North. |

6.1.6.2.19 Type: HorizontalWithVerticalVelocity

Table 6.1.6.2.19-1: Definition of type HorizontalWithVerticalVelocity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Horizontal speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| vSpeed | VerticalSpeed | M | 1 | Vertical Seed in kilometres per hour. |
| vDirection | VerticalDirection | M | 1 | Vertical Direction: upward or downward. |

6.1.6.2.20 Type: HorizontalVelocityWithUncertainty

Table 6.1.6.2.20-1: Definition of type HorizontalVelocityWithUncertainty

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| uncertainty | SpeedUncertainty | M | 1 | Uncertainty of horizontal speed in kilometres per hour. |

6.1.6.2.21 Type: HorizontalWithVerticalVelocityAndUncertainty

Table 6.1.6.2.21-1: Definition of type HorizontalWithVerticalVelocityAndUncertainty

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hspeed | HorizontalSpeed | M | 1 | Speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| vSpeed | VerticalSpeed | M | 1 | Vertical Seed in kilometres per hour. |
| vDirection | VerticalDirection | M | 1 | Vertical Direction: upwards or downwards. |
| hUncertainty | SpeedUncertainty | M | 1 | Uncertainty of horizontal speed in kilometres per hour. |
| vUncertainty | SpeedUncertainty | M | 1 | Uncertainty of vertical speed in kilometres per hour. |

Considering the example above, the standardized data collection in solution 1b) is performed as described by relevant working groups. The table in TS 26.532 [7], can be extended as below for radio-related parameters.

Table A.1-1: Externally defined data types used by Ndcaf_DataReporting_Report operation

| Data type | Comments | Reference |
| --- | --- | --- |
| BitRate |  | 3GPP TS 29.571 [12] |
| PacketDelBudget |  |  |
| PacketLossRate |  |  |
| DateTime |  |  |
| DurationSec |  |  |
| SvcExperience |  | 3GPP TS 29.517 [5] |
| AddrFqdn |  |  |
| TimeWindow |  | 3GPP TS 29.122 [14] |
| Volume |  |  |
| FlowInfo |  |  |
| LocationArea5G |  |  |
| LocationData |  | 3GPP TS 29.572 [15] |
| HorizontalSpeed |  |  |
| (RAN AI/ML data types) |  | e.g., 3GPP TS 38.331 |

As discussed above, with deployment/collaboration restriction by MNO where “the server for data collection for UE-side model training” is internal to the 5G system and under the direct control of MNO, full visibility for standardized data content can be ensured by the server for data collection for UE-side model training. The server for data collection for UE-side model training can ensure that the data is collected as described in the relevant standard document.

Observation 2: In TS 26.532 [7], the different 3GPP standards are referred to for describing the format of data reports submitted to the data collection AF.

Proposal 2: In the deployment/collaboration scenario where “the server for data collection for UE-side model training” is internal to the 5G system and under the direct control of MNO, full visibility for standardized data content can be ensured by the server for data collection for UE-side model training. The server for data collection for UE-side model training ensures that the data is collected as described in the relevant standard document.

## 2.2 On Partial Visibility for Partially Standardized Data Contents

In the RAN2#126 meeting [4], partial visibility for partially standardized data content was discussed for solutions 1b), 2, and 3. While some companies believe that partial visibility for partially standardized data content can be supported for solutions 2 and 3, there is no clarity on how partial visibility for partially standardized data content can be supported, for example,

- How are partial standardized data contents defined?

- Which working group is responsible for defining/standardizing partial standardized data contents?

Observation 3: There is no clarity on how partial visibility for partially standardized data content can be supported, for example,

- How are partial standardized data contents defined?

- Which working group is responsible for defining/standardizing partial standardized data contents?

Proposal 3: RAN2 should discuss how partial visibility for partially standardized data content can be supported, for example,

- How are partial standardized data contents defined?

- Which working group is responsible for defining/standardizing partial standardized data contents?

### 2.2.1 Partial Visibility for Partially Standardized Data Contents in Solution 1b)

In our understanding, the operator may achieve partial visibility of the data content being collected based on

- Service level agreement between the operators and vendors,

- Standard specification capturing partially standardized data contents.

Observation 4: The operator may achieve partial visibility of the data content being collected based on

- Service level agreement between the operator and UE vendor,

- Standard specification capturing partially standardized data contents.

In option 1b) operators can have visibility based on SLA or standardized of partially standardized data types. However, we are unsure how partial standardized data contents are defined and which WG is responsible for defining them.

Proposal 4: The server for data collection for UE-side model training (within the 5G system and direct control of MNO) can provide operators with partial visibility of the collected data content based on SLA or standardization of partially standardized data contents.

### 2.2.1 Partial Visibility for Partially Standardized Data Contents in Solutions 2 and 3

Note that it is unclear how partially standardized data collection and visibility can be supported in solutions 2 and 3. Note that the SLA-based data collection is currently not supported in solutions 2 and 3. In solutions 2 and 3, no network entity is provisioned that can configure UE with data types based on SLA. Therefore, without the standardization activity for defining the partially standardized data types, partial visibility for partially standardized data contents in Solutions 2 and 3 cannot be supported.

Observation 5: SLA-based data collection is currently not supported in solutions 2 and 3, as no network entity is provisioned that can configure UE with data types based on SLA.

Proposal 5: Without the standardization activity for defining the partially standardized data types, partial visibility for partially standardized data contents in Solutions 2 and 3 cannot be supported.

## 2.3 No standardized visibility in Solution 1b), 2, and 3

### 2.3.1 No standardized visibility in Solution 1b)

TS 26.532 [7] defines the data contents that are not visible to the operators and third parties. In our understanding, if MNO visibility is not required for certain data types/contents, for example, proprietary information, then containers can be used for configuration and reporting in solution 1b).

Observation 6: TS 26.532 [7] defines the data contents that are not visible to the operators and third parties.

Proposal 6: If MNO visibility is not required for certain data types/contents, for example, proprietary information, then containers can be used for configuration and reporting in solution 1b).

### 2.3.2 No standardized visibility in Solutions 2 and 3

In 3GPP, apart from TS 26.532 [7], data collection containers are defined for QoE measurement configuration and reporting. However, the QoE configuration container is provided by the OAM and the measurement (container) is reported to the OAM. Therefore, the QoE container is transparent to the RAN but visible to the 3GPP system (e.g., OAM). Currently, containers not visible to the 3GPP system are not defined.

Observation 7: The QoE container is transparent to the RAN but visible to the 3GPP system (e.g., OAM). Currently, containers transparent to the 3GPP system are not defined.

Our understanding is that companies that argue for the container for data collection and reporting, prefer to have a container that is transparent to the 3GPP system. However, such containers are not defined for solutions 2 and 3. Furthermore, the procedure/service/signaling for providing the container from a non-3GPP entity to an OAM or 3GPP entity is not supported for UE data collection.

Proposal 7: Unless the procedure for providing the container from a non-3GPP entity to an OAM or 3GPP entity is supported, “no standard visibility” is not supported in solutions 2 and 3.

## 2.4 Updates to the table in TS 38.843

In sections 2.1, 2.3, and 2.3, we discussed different aspects associated with controllability and visibility on data collection in solutions 1b), 2, and 3. We believe that the table can be updated considering the above discussion below.

Proposal 8: Update the table on Analysis of different data collection options for UE-side model training as below.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Full controllability (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 5) (Note 6) FFS Opt C) No standardized visibility. | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 6) Opt C) No standardized visibility. (Note 7) | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 6) FFS Opt C) No standardized visibility. (Note 7) |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: Partial visibility of the collected data content can be achieved based on SLA or standardization of partially standardized data contents. - Note 6: To achieve partial visibility for partially standardized data, standardization activity may be required for defining the partially standardized data contents. - Note 7: Procedures may be required for providing the data collection configuration container from a non-3GPP entity to an OAM or 3GPP entity to support no standardized visibility, and vice versa. |  |  |  |  |

## 2.5 Recommendation to RAN Plenary

In our understanding, solutions under discussion will require standard activities from SA groups. We further believe that RAN2 / RAN plenary may not be able to accurately estimate the standardization work required by different working groups for supporting solutions under discussion. Therefore, the Table should be discussed in the joined RAN and SA plenary session for further action.

Observation 8: The solutions under discussion will require standard activities from SA groups.

Observation 9: RAN2 / RAN plenary may not be able to accurately estimate the standardization work required by different working groups for supporting solutions under discussion.

Proposal 9: The table captured in TS 38.843 [2] should be discussed in the joint RAN and SA plenary session for further action.

# Conclusion

Observation 1: TS 26.531 [6] elaborates collaboration/deployment scenario, where “the server for data collection for UE-side model training”, for example, Data Collection Application Function (DCAF) is internal to the 5G system, and is under direct control of MNO.

Proposal 1: Full network controllability on UE-side data collection is achieved with a deployment/collaboration scenario where “the server for data collection for UE-side model training” is internal to the 5G system and under the direct control of MNO.

Observation 2: In TS 26.532 [7], the different 3GPP standards are referred to for describing the format of data reports submitted to the data collection AF.

Proposal 2: In the deployment/collaboration scenario where “the server for data collection for UE-side model training” is internal to the 5G system and under the direct control of MNO, full visibility for standardized data content can be ensured by the server for data collection for UE-side model training. The server for data collection for UE-side model training ensures that the data is collected as described in the relevant standard document.

Observation 3: There is no clarity on how partial visibility for partially standardized data content can be supported, for example,

- How are partial standardized data contents defined?

- Which working group is responsible for defining/standardizing partial standardized data contents?

Proposal 3: RAN2 should discuss how partial visibility for partially standardized data content can be supported, for example,

- How are partial standardized data contents defined?

- Which working group is responsible for defining/standardizing partial standardized data contents?

Observation 4: The operator may achieve partial visibility of the data content being collected based on

- Service level agreement between the operator and UE vendor,

- Standard specification capturing partially standardized data contents.

Proposal 4: The server for data collection for UE-side model training (within the 5G system and direct control of MNO) can provide operators with partial visibility of the collected data content based on SLA or standardization of partially standardized data contents.

Observation 5: SLA-based data collection is currently not supported in solutions 2 and 3, as no network entity is provisioned that can configure UE with data types based on SLA.

Proposal 5: Without the standardization activity for defining the partially standardized data types, partial visibility for partially standardized data contents in Solutions 2 and 3 cannot be supported.

Observation 6: TS 26.532 [7] defines the data contents that are not visible to the operators and third parties.

Proposal 6: If MNO visibility is not required for certain data types/contents, for example, proprietary information, then containers can be used for configuration and reporting in solution 1b).

Observation 7: The QoE container is transparent to the RAN but visible to the 3GPP system (e.g., OAM). Currently, containers transparent to the 3GPP system are not defined.

Proposal 7: Unless the procedure for providing the container from a non-3GPP entity to an OAM or 3GPP entity is supported, “no standard visibility” is not supported in solutions 2 and 3.

Proposal 8: Update the table on Analysis of different data collection options for UE-side model training as below.

Table 7.2.1.3.2-1. Analysis of different data collection options for UE-side model training.

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| First termination entity | Training entity (e.g., Over-The-Top (OTT) server) | Server for data collection for UE-side model training | Inside the CN | Inside OAM domain |
| AI/ML-specific Data Transfer Path | UE to OTT server via either 3GPP or non-3GPP network | UE ->Server for data collection for UE-side model training/OTT server (Note 4) | UE-> CN -> Server for data collection for UE-side model training/OTT server (Note 4) | UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server (Note 4) |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |
| Controllability of MNO on data transfer | No AI/ML specific controllability | Full controllability (Note 1) | Full controllability (Note 1) | Full controllability (Note 1) |
| Solution for network controllability | N/A (the OTT server can directly request data from the UE) | Example: per PDU sessions | Via NAS procedure FFS: impact to other layers | Via RRC procedure |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 5) (Note 6) FFS Opt C) No standardized visibility. | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 6) Opt C) No standardized visibility. (Note 7) | Opt A) Full visibility for standardized data content. Opt B) Partial visibility for partially standardized data content. (Note 6) FFS Opt C) No standardized visibility. (Note 7) |
| Involved WGs | N/A | SA2, SA3, RAN2, RAN3, CT1 | SA2, SA3, RAN3, RAN2, CT1 and CT3 | RAN2, RAN3, SA3, SA5, FFS SA2 |
| - Note 1: Full controllability: The MNO has the capability to manage data transfer to the server for UE-side data collection. This includes initiating, terminating, and fully managing data transfer. FFS if further refinements/modifications to this definition are needed. - Note 2: Visibility of data content signifies the capability of the MNO to, at least, be aware of, access, and comprehend the data during transfer. FFS if further refinements/modifications to this definition are needed (e.g. on the capability of the MNO to modify the collected data). - Note 3: The following options are identified to realize the different levels of data content visibility to the MNO: - Full visibility for standardized data content. - Partial visibility for partially standardized data content. - No standardized visibility. - Note 4: The potential involvement of NF or other higher layers entities/functionalities should be discussed in other WGs. Impact on the OTT server is not in the scope of RAN2 discussion. - Note 5: Partial visibility of the collected data content can be achieved based on SLA or standardization of partially standardized data contents. - Note 6: To achieve partial visibility for partially standardized data, standardization activity may be required for defining the partially standardized data contents. - Note 7: Procedures may be required for providing the data collection configuration container from a non-3GPP entity to an OAM or 3GPP entity to support no standardized visibility, and vice versa. |  |  |  |  |

Observation 8: The solutions under discussion will require standard activities from SA groups.

Observation 9: RAN2 / RAN plenary may not be able to accurately estimate the standardization work required by different working groups for supporting solutions under discussion.

Proposal 9: The table captured in TS 38.843 [2] should be discussed in the joint RAN and SA plenary session for further action.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- [POST126][034][AIML PHY] TP for data collection (Ericsson) report

- RAN2#126 meeting report

- TS 26.531, “Data Collection and Reporting; General Description and Architecture”

- TS 26.532, “Data Collection and Reporting; Protocols and Formats”