# 3GPP TSG-RAN2 Meeting #127	R2-2406646

# Maastricht, Netherlands, August 19-23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5042 | rev | - | Current version: | 14.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

8

| Title: | Correction to MIB-MBMS systemFrameNumber field description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung, ABS, SJTU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MBMS_LTE_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently the field description of systemFrameNumber in MIB-MBMS is as follows: MasterInformationBlock-MBMS field descriptions systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21, 6.6.1], the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the fourth radio frame: 01, the eighth radio frame: 10, the last radio frame: 11). The problem is with the second part (yellow highlight above). It looks like this was due to copy-paste from non-MBMS MIB, shown below: MasterInformationBlock field descriptions systemFrameNumber Defines the 8 most significant bits of the SFN. As indicated in TS 36.211 [21], 6.6.1, the 2 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 40ms P-BCH TTI indicates 2 least significant bits (within 40ms P-BCH TTI, the first radio frame: 00, the second radio frame: 01, the third radio frame: 10, the last radio frame: 11). For non-MBMS MIB, Section 5.2.1.2 captures the following: The MIB uses a fixed schedule with a periodicity of 40 ms and repetitions made within 40 ms. The first transmission of the MIB is scheduled in subframe #0 of radio frames for which the SFN mod 4 = 0, and repetitions are scheduled in subframe #0 of all other radio frames. So, it is clear that the 2 LSBs of SFN can be inferred from the MIB transmission as described in the field description. In contrast, one should note that the periodicity (and repetition) of MIB and MIB-MBMS are not the same. The MIB-MBMS uses a fixed schedule with a periodicity of 160 ms and repetitions made within 160 ms. The first transmission of the MIB-MBMS is scheduled in subframe #0 of radio frames for which the SFN mod 16 = 0, and repetitions are scheduled in subframe #0 of all other radio frames for which the SFN mod 4 = 0. This means, unlike MIB, although 4 LSBs are indicated by P-BCH decoding, in reality the 2 LSBs in the systemFrameNumber in MBMS-MIB are always ‘00’ and the remaining 2 out of the 4 LSBs are indicated implicitly based on the P-BCH decoding as specified in TS 36.211. So, the correct intent of the above field description is as follows: For an MBMS-dedicated cell, within the 160ms P-BCH periodicity, the 2 LSBs are always ‘00’ and the 3rd/4th LSBs indicated implicitly by the P-BCH decoding. I.e., the P-BCH transmitted in the 1st RF indicates 0000 (i.e. first radio frame within 160ms, not within 40ms), the P-BCH transmitted in the second 40ms duration indicates 0100 (i.e. the fifth radio frame within 160ms, not the fourth radio frame nor within the 40ms), the P-BCH transmitted in the 3rd 40ms duration indicates 1000 (i.e. the ninth radio frame within 160ms, not the 8th, nor within 40ms), the P-BCH transmitted in the 4th 40ms duration indicates 1100 (i.e. the 13th radio frame within 160ms, not the last, nor within 40ms). Additionally, regarding “within 40ms P-BCH TTI”: - For MBMS, the P-BCH TTI is 160ms, not 40ms. - there cannot be ‘eighth’ radio frame within 40ms P-BCH TTI. So, the existing text is clearly wrong at multiple places. In summary, the field description is not only unclear, but also incorrect as well, needing an essential correction. Instead of trying to come up with clear wording, given that TS 36.211 is already referred in the description, the proposal is to simply remove the incorrect text. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove incorrect text from the field description of systemFrameNumber in MasterInformationBlock-MBMS, as TS 36.211 is already referenced. Impact Analysis: Impacted feature: LTE MBMS Interoperability Analysis: - Implementations done according to TS 36.211 are not impacted. Since the incorrect text in the concerned field description is impossible to implement (e.g. there is no 8th radio frame in a 40ms period), no interoperability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RRC specification for MBMS remains ambiguous, incorrect, and misaligned with TS 36.211. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.2.2	Message definitions

<<unchanged text skipped>>

#### –	MasterInformationBlock-MBMS

The MasterInformationBlock-MBMS includes the system information transmitted on BCH.

Signalling radio bearer: N/A

RLC-SAP: TM

Logical channel: BCCH

Direction: E-UTRAN to UE

MasterInformationBlock-MBMS

-- ASN1START

MasterInformationBlock-MBMS-r14 ::=			SEQUENCE {

dl-Bandwidth-MBMS-r14						ENUMERATED {

n6, n15, n25, n50, n75, n100},

systemFrameNumber-r14					BIT STRING (SIZE (6)),

additionalNonMBSFNSubframes-r14		INTEGER (0..3),

spare									BIT STRING (SIZE (13))

}

-- ASN1STOP

MasterInformationBlock-MBMS field descriptions

additionalNonMBSFNSubframes Configures additional non-MBSFN subframes where SystemInformationBlockType1-MBMS and SystemInformation-MBMS may be transmitted. Value 0, 1, 2, 3 mean zero, one, two, three additional non-MBSFN subframes are configured after each subframe which has PBCH.

dl-Bandwidth-MBMS Parameter: transmission bandwidth configuration, NRB in downlink, see TS 36.101 [42, table 5.6-1]. n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

systemFrameNumber Defines the 6 most significant bits of the SFN of the MBMS-dedicated cell. As indicated in TS 36.211 [21, 6.6.1], the 4 least significant bits of the SFN are acquired implicitly in the P-BCH decoding, i.e. timing of 160ms P-BCH TTI indicates 4 least significant bits.

End of Changes