3GPP TSG-RAN2 Meeting #127	R2-2406652

Maastricht, Netherlands, August 19-23, 2024

Agenda Item:	8.2.3

Study Item:	FS_Ambient_IoT_Solutions

Source: 	Qualcomm Incorporated

Title: 	Topology 2 related aspects

Document for:	Discussion/Decision

# Introduction

RAN#103 approved SI for Ambient IoT in [1]. TR 38.848 [2] refers to the two topologies under consideration as shown below.

Figure 4.2.1.1-1: Topology 1

In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a base station. The communication between the base station and the ambient IoT device includes Ambient IoT data and/or signalling.

Figure 4.2.1.2-1: Topology 2

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and base station. In this topology, typically the intermediate node would be a UE capable of functioning as Ambient IoT Reader. The Reader transfers Ambient IoT data and/or signalling between the base station and the Ambient IoT device.

This document discusses about topology 2 aspects for Ambient IoT.

# Resources for AIoT operation between the Reader and Device in Topology 2

The WID indicates that “FR1 licensed spectrum in FDD” is to be assumed for the R2D and D2R link.

WID assumption indicates “FR1 licensed spectrum in FDD” for the R2D and D2R link regardless of the topology.

This means that, for topology 2, the scenarios discussion for radio resource allocation is somewhat similar to that for the NR sidelink discussions, e.g., whether the UE Reader needs to be always in-coverage or can it be out-of-coverage, how are the resources provided to the Reader/UE to use it in D2R/R2D (like mode1/mode2), can the UE be in RRC_IDLE/INACTIVE or does it have to be in CONNECTED, etc.

## UE Reader in-coverage/out-of-coverage

While one can argue that the UE reader should always be in coverage of the network e.g. to get the commands from AIoT Controller, there can be valid scenarios where the UE Reader operates out of coverage. E.g. the commands from the Controller arrive to the UE Reader at time t1 while it is inside a control room in the warehouse; to perform the actual inventory or command with the AIoT devices (tags) at t2, the UE Reader is moved to the area where the devices actually are deployed. Then, after performing the inventory or command, the Reader comes back to the coverage at t3 and transmits the collected AIoT data to the AIoT Controller.

UE Reader performing AIoT operations with the AIoT devices while being out of NR coverage area is a valid scenario.

Therefore, we propose:

Both in-coverage and out-of-coverage scenarios are studied.

## UE Reader’s RRC state

Similar to the discussion above, while one can argue that the UE reader should always be in RRC_CONNECTED state e.g. to get the commands from AIoT Controller, there can be valid scenarios where the UE Reader operates while being in other RRC states. E.g. the commands from the Controller arrive to the UE Reader at time t1 while it is in RRC_CONNECTED (or RRC_INACTIVE using MT SDT), and the UE Reader performs the actual inventory or command with the AIoT devices (tags) while being in RRC_IDLE or RRC_INACTIVE state, then UE Reader transitions to RRC_CONNECTED (or uses SDT from RRC_INACTIVE) after performing the inventory or command, to transmit the collected AIoT data to the AIoT Controller.

UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities.

Therefore, we propose:

The UE Reader can be in any RRC State while performing R2D/D2R activities.

## Resource allocation for the Reader

Note that, in the following discussion, resource allocation refers to the allocation of the radio resources for the UE/Reader that can be used in R2D/D2R link, but not the allocation of resources by the reader for the R2D/D2R link.

We recognize that since licensed spectrum is to be used in D2R/R2D link, there needs to be coordination between the network and the Reader on which radio resources are to be used for D2R/R2D communication.

Since licensed spectrum is to be used in D2R/R2D link, there needs to be coordination between the network and the Reader on which radio resources are to be used for D2R/R2D communication.

Borrowing the terminologies/discussion from NR Sidelink/V2X, there can be two modes of resource allocation [3, section 5.7.2]: mode 1 and mode 2. In mode 1, the resource allocation is provided by the network. In mode 2, UE decides the transmission resources from the (pre)configured resource pool(s).

While mode 1 needs the device to be in-coverage, mode 2 can be suitable for both in-coverage and out-of-coverage scenarios. Mode 2 is also more suitable for scenarios where the resource scheduling for D2R and R2D link needs to be quick and made locally by the Reader.

Furthermore, mode 1-like resource allocation can add more complexity to the Reader as well as the network.

For the UE Reader out-of-coverage scenario and also for quick local decisions on D2R/R2D communication resources, mode-2-like resource allocation is more suitable.

Therefore, we propose:

SL-mode-2-like resource allocation is further studied for resource allocation for the UE Reader. SL-mode-1-like resource allocation is not considered.

## AIoT Data transport from Reader to the network

It is our understanding that companies have proposed various options for AIoT data communication between the UE Reader and the AIoT CN. At least the following 3 options seem to be on the table.

Option 1 (RRC-based): In this option, UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly using RRC. gNB then forwards the AIoT messages to the AIoT CN node explicitly over N2. No end-to-end protocol between the UE reader and the AIoT CN node (see R-AP in options 2 and 3).

Option 2 (NAS-based): In this option, UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node (via SBI interfaces in the core).

In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container.

Option 3 (PDU session-based): In this option, UE Reader uses user plane signalling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node (via SBI interfaces in the core).

In this solution, the AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane.

In the table below, we compare the three options by discussing the protocol stack, interface impacts and their pros vs. cons.

| RRC-based (option 1) | NAS-based (option 2) | PDU session-based (option 3) |
| --- | --- | --- |
| UE Reader uses its RRC connection with the gNB to send/receive AIoT messages explicitly in RRC. gNB then decodes the AIoT RRC messages and forwards the AIoT messages to the AIoT CN node explicitly over N2. | UE Reader uses its NAS association with the AMF to send/receive AIoT messages as NAS PDUs. AMF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. | UE Reader uses user plane signaling (establishes a PDU session till UPF) to communicate with AIoT CN node. UPF can then forward the AIoT messages to the AIoT CN node via SBI interfaces in the core. |
| AIoT enabled gNB is aware of the contents of the AIoT messages as it is received explicitly over RRC. However, it adds unnecessary RRC ASN.1 complexity. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is received as a NAS container AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information. | AIoT enabled gNB is unaware of the contents of the AIoT messages as it is sent over user plane AIoT enabled gNB can be made aware explicitly by the AIoT CN node providing assistance information. |
| No end-to-end protocol between the UE reader and the AIoT CN node | There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node. | There is an end-to-end protocol (R-AP) between the UE reader and the AIoT CN node |
| RRM like solution This is like how gNB can explicitly request RRM measurements to UE explicitly via RRC | LPP-like solution This is like how LPP is defined between UE and LMF and carried over NAS/NGAP/RRC | UPL like solution This is like how UPL (User Plane Location Protocol) is used to carry location between a UE and location server |
| UE Reader selection can be performed by gNB or AIoT CN node | UE Reader selection performed by AIoT CN node | UE Reader selection performed by AIoT CN node |
| Overload on AMF if there are lot of AIoT devices | Overload on AMF if there are lot of AIoT devices | UPF can handle IP traffic better. |
| RAN2 impact: High RAN2 would need to define new RRC signaling to support the reception/transmission of AIoT related messages from gNB to UE Reader RAN2 would need to define procedures when the UE Reader’s serving gNB changes (e.g., due to mobility) or RRC state changes | RAN2 impact: No/minimal | RAN2 impacts: No/minimal |
| RAN3 impact: High RAN3 would need to define explicit signaling over N2 for the AIoT messages In case the serving gNB of the UE Reader changes (e.g., due to mobility) or the RRC state of the UE reader changes, Xn coordination is needed to provide UE Reader’s context from source gNB to target gNB | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state | RAN3 impact: No/minimal No new procedures needed when the UE Reader’s serving gNB changes or the UE Reader changes its RRC state |
| SA2 impact: Medium SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) | SA2 impact: Medium SA2 needs to define the interface between AMF and AIoT CN node (Nac interface) and support the end-to-end R-AP protocol | SA2 impact: Medium SA2 needs to define the interface between UPF and AIoT CN node (Nuc interface) and support the end-to-end R-AP protocol |
| Non-homogeneous control plane transport - RRC between UE Reader and gNB - N2 between gNB and AMF - API between AMF and AIoT CN node | Non-homogeneous control plane transport - NAS between UE Reader and AMF - API between AMF and AIoT CN node | Homogenous transport: End to-end IP - IP connectivity (PDU session) between UE Reader and UPF - IP connectivity between UPF and AIoT CN node |
| Needs support of RRC at UE Reader (already supported by UEs today) | Needs support of NAS at UE Reader (already supported by UEs today) | Needs support of HTTP client at UE Reader (already supported by UEs today) |
| Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will receive signaling over RRC | Fragmented protocol stacks for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over NAS | Can support common SBI protocol stack for Reader in T1 and T2 - AIoT RAN node in T1 will have R-AP over IP - UE Reader in T2 will have R-AP over IP |

Considering there are high impacts to the UE reader, the AIoT enabled gNB, as well as the specification efforts for RAN2/RAN3 with Option 1 (RRC-based), it is our view that RAN2 should not prefer this option.

Comparing the pros and cons between the Option 2 (NAS-based) and Option 3 (PDU-based), it is clear that option 3 has less impacts to RAN2/3 work, can provide homogenous transport between different nodes (from Reader to the CN), and can also support common protocol stack for the Reader in both topologies. So, while our preference is clear that option 3 should be the way to go, we invite companies to study further the options 2 and 3.

Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

For AIoT data transfer between the reader and the AIoT CN, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Therefore, we propose:

PDU-session based AIoT data transfer between the reader and AIoT CN is studied. FFS NAS-based data transfer between Reader and AIoT CN.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	WID assumption indicates “FR1 licensed spectrum in FDD” for the R2D and D2R link regardless of the topology.

Observation 2.	UE Reader performing AIoT operations with the AIoT devices while being out of NR coverage area is a valid scenario.

Observation 3.	UE Reader performing AIoT operations with the AIoT devices is not limited to RRC_CONNECTED state, i.e. the UE Reader can be in RRC_IDLE or RRC_INACTIVE state while performing R2D/D2R activities.

Observation 4.	Since licensed spectrum is to be used in D2R/R2D link, there needs to be coordination between the network and the Reader on which radio resources are to be used for D2R/R2D communication.

Observation 5.	For the UE Reader out-of-coverage scenario and also for quick local decisions on D2R/R2D communication resources, mode-2-like resource allocation is more suitable.

Observation 6.	Given the high impacts to RAN2/RAN3, RRC-based AIoT data transport between the Reader and the gNB is not considered.

Observation 7.	For AIoT data transfer between the reader and the AIoT CN, PDU-session based AIoT data transfer has clear advantages over NAS-based AIoT data transfer.

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Both in-coverage and out-of-coverage scenarios are studied.

Proposal 2:	The UE Reader can be in any RRC State while performing R2D/D2R activities.

Proposal 3:	SL-mode-2-like resource allocation is further studied for resource allocation for the UE Reader. SL-mode-1-like resource allocation is not considered.

Proposal 4:	PDU-session based AIoT data transfer between the reader and AIoT CN is studied. FFS NAS-based data transfer between Reader and AIoT CN.

# References

[1] RP-240826, Study on solutions for Ambient IoT (Internet of Things) in NR, RAN#103, March 2024

[2] TR 38.848, Study on Ambient IoT (Internet of Things) in RAN, Release 18

[3] TS 38.300, NR and NG-RAN Overall Description; Stage 2