3GPP TSG-RAN WG2 Meeting #127	                                          R2-2406704

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	Xiaomi

Title:  	Consideration on cell and beam RRM measurement prediction

Agenda Item:	8.3.2.2

Document for:	Discussion and Decision

Introduction

According to the SID [1] of the Rel-19 AI/ML for mobility in NR as quoted below, the RRM measurement prediction include at least two parts (i.e. Cell-level measurement prediction and Beam-level measurement prediction).

- AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - NOTE 1: RAN1/3 work can be triggered via LS - NOTE 2: RAN4 scope/work can be defined and confirmed by RAN#105 after some RAN2 discussions (within the RAN4 pre-allocated TUs) NOTE 3: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 4: Two-sided model is not included

In this contribution, we provide our understandings on RRM measurement prediction.

Discussion

  - Cell-level measurement prediction for L3 Mobility

In measurement reduction simulation, temporal prediction case B is assumed. In case B, UE can skip the measurement. Actual measured result is used as model input to derive the skipped result by AI. In RAN1, study the accuracy is calculated based on the predicted measurement result and actual measured result. It’s assumed there is no difference between the actual measured result in AI and legacy solution. However, in RRM measurement prediction, L3 filtering is applied. The measurement result is derived based on current and previous measurement result. This means the actual measured result in AI and legacy may be different and shall be considered when evaluating the accuracy. One example is depicted in the following Fig,

Fig 1. Measurement reduction

This fig shows a 50% measurement reduction scenario. In legacy, the actual result in time instance 3 in legacy is derived based on the result of instance 2 and 3. The measurement in time instance 2 is derived from AI, which may not be the same as legacy. Therefore, if same L3 filtering parameter is used in AI solution, the L3 filtered measurement result in time instance 3 may be different as legacy. This difference shall be considered in AI accuracy evaluation as well.

Proposal 1: The RSRP difference of actual measured result between AI solution and legacy shall be considered in temporal prediction case B.

In last meeting, cluster-based RRM measurement prediction is proposed by several companies. However, it’s not clear what is the definition of cluster-based prediction. In our understanding, the cells in one cluster share similar characteristic and can be used for model input or output. The characteristic can be deployed at close location or deployed at same frequency. Such characteristic may be correlated with the radio channel and RRM measurement result. The correlation may be learned by AI model and AI model can predict cell measurement result of one cell based on other cells’ measurement result in the same cluster. Theoretically, AI may be able to predict multiple cells’ measurement result. But the accuracy and complexity may also increase. We can first focus on the simple design, i.e. prediction output only provides one cell’s measurement result. Based on above observation, the input of cluster-based prediction can be measurement results from multiple cells. The output of cluster-based prediction can be one cell’s measurement result. The mapping between cell and cluster can be configured by NW or determined by UE.

Proposal 2: In cluster-based RRM prediction, AI predicts cell measurement result of one cell based on other cells’ measurement result in the same cluster. The cells in the same cluster share similar characteristic, e.g. deployed location or frequency.

  - Beam-level measurement prediction for L3 Mobility

RAN2 has considered 3 sub-use cases for cell-level RRM measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then cell-level measurement result is generated

- Sub-use case 2: Cell-level measurement result(s) is predicted based on actual cell-level measurement result(s)

- Sub-use case 3: Cell-level measurement result(s) is predicted based on L1 beam-level measurement result(s)

To support the sub-use case 1 of the cell-level RRM measurement prediction, L1 beam-level measurement prediction is needed.

Observation 1：L1 beam-level measurement prediction has been studied by the sub-use case 1 of the cell-level RRM measurement prediction.

Whether to support L3 beam-level prediction

TS 38.300 [2]: The corresponding high-level measurement model is described below: Figure 9.2.4-1: Measurement Model NOTE 1: K beams correspond to the measurements on SSB or CSI-RS resources configured for L3 mobility by gNB and detected by UE at L1.

One open issue is whether to support the L3 beam-level measurement prediction.

In the legacy measurement model as illustrated in TS 38.300 Figure 9.2.4-1, the UE also needs to measure and report L3 beam-level measurement results, if it is configured by the network. Power saving gain still applies if L3 beam measurement can be predicted by reduced measurements. Furthermore, L3 beam level measurements can be used for beam and RA resource selection of the target cell. If the selected beam becomes poor during handover, handover failure may occur.

Hence, to reduce the measurement and improve the handover performance, the L3 beam-level measurement prediction shall be considered.

Proposal 3：The L3 beam-level measurement prediction shall be considered.

Similar to the cell-level RRM measurement prediction, the input can be the L1 or L3 level results and the direct and indirect L3 beam-level measurement prediction can be considered

Proposal 4: 3 sub-use cases are considered for the L3 beam-level measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 2: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 3: L3 beam-level measurement result(s) is predicted based on L1 beam-level measurement result(s)

L3 beam is mainly used in FR2. We can focus on FR2 for L3 beam prediction. FR1 can be deprioritized.

Proposal 5: Focus on FR2 for L3 beam prediction.

For cell-level measurement prediction, the following agreements have been reached in previous meetings [3]:

Agreements-RAN2#125bis: - We will consider intra-frequency intra and inter-cell spatial domain measurement predictions, for beam and cell level measurements. - For temporal domain measurement prediction, we will consider the AI-PHY beam management Case A and Case B from the RAN1 AI/ML PHY TR and it applies to both beam level and cell level. As baseline we will focus on pure temporal predicition. Agreements to start evaluations: - FR1-to-FR1 - Focus on intra-frequency in time domain prediction for the purpose of measurement reduction - Study inter-frequency scenario in terms of which scenarios can be studied without requiring new channel model and also resolving any simulation assumptions (if possible). - FR2-to-FR2 - Focus on intra-frequency - Perform evaluation both in time and spatial domain

According to the above agreements, for FR2-to-FR2, spatial domain and temporal domain prediction have been studied for the cell-level prediction. To follow the scenarios of the cell-level measurement prediction, intra-frequency spatial domain and temporal domain prediction can be considered for L3 beam-level measurements.

In the current cell-level prediction, spatial domain is considered for inter-cell case. For the L3 beam-level measurement prediction, spatial domain prediction is also beneficial, since UE can reduce the measurement on beams in such case to save power. Both intra and inter cell prediction can be considered.

Proposal 6: Intra-frequency intra and inter-cell spatial domain measurement predictions can be considered for L3 beam-level measurements.

For temporal domain cell-level measurement prediction, we will focus on pure temporal prediction. As discussed above, temporal prediction can assist NW to select optimal beam for RACH during handover. Handover performance may be improved. Also, UE can save power due to reduced measurement. For L3 beam-level measurements, to follow the cell-level measurement prediction, we can focus on the intra-cell case and the intra-cell intra-beam and inter-beam temporal domain prediction can be considered.

Proposal 7: Intra-cell intra and inter-beam temporal domain measurement predictions can be considered for L3 beam-level measurements.

KPI metrics

For KPI metrics of the cell-level prediction, RAN2 will start the evaluation with measurements prediction accuracy and model complexity and discuss system performance after we see which scenarios have good measurements prediction accuracy.

RAN2 clarified the prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases and average RSRP difference is taken as prediction accuracy metric for RRM measurement prediction. Note the RSRP difference values should be an absolute value before they are averaged [4].

The KPI metrics of the L3 beam-level prediction shall follow the cell-level prediction.

Proposal 8: The KPI metrics of the L3 beam-level prediction shall follow the cell-level prediction. And the following items can be considered.

- The evaluation is started with measurements prediction accuracy and model complexity.

- The prediction accuracy metric for the L3 beam-level prediction is defined as “RSRP difference between predicted L3 beam level measurement result and actual L3 beam level measurement result of the same beam” for all sub cases and the average RSRP difference is taken as prediction accuracy metric for the L3 beam-level prediction.

  - LCM for RRM prediction

According to the SID [1], some potential AI mobility specific enhancements should be based on the Rel-19 AI/ML-air interface WID general framework (e.g., LCM, performance monitoring etc). So, LCM and performance monitoring for RRM measurement prediction can be designed based on the general framework of AI/ML Air.

Proposal 9: For LCM and performance monitoring in AI/ML Mobility, the general framework of AI/ML-air interface WID can be reused.

In addition, the training data collection has also been studied in the Rel-19 AI/ML-air WID, we can also wait the progress of AI/ML Air.

Proposal 10: The basic procedure and framework defined in AI/ML-air interface WID can be reused for the training data collection of AI/ML Mobility.

For AI/ML mobility, the RRM measurement prediction is performed to assist mobility optimization. Therefore, the specified trigger of the data collection can be discussed for AI/ML based mobility. The mobility events triggered training data collection can be considered. When the UE performs L3 Mobility, i.e., L3 handover, the UE and/or network side shall perform the training date collection for RRM measurement prediction. The network may decide to perform L3 handover based on the L3 measurement reporting, thus, the measurement events can also be used to trigger the training data collection. For example, when the configured measurement event is fulfilled, the UE can perform the training date collection.

Proposal 11: The training data collection can be performed when one of the following events occurs:

- Alt1: When the UE performs L3 Mobility, i.e., L3 handover.

- Alt2: When the UE fulfils the measurement events, e.g., event A2, event A3, event A4, event A5.

  - Potential AI mobility solution

Both NW sided and UE sided model are considered for RRM measurement prediction. Hence, we can discuss the potential solutions for NW sided and UE sided model separately.

Network sided model:

For network sided model, predicted measurement results can be used for the mobility optimization, e.g., based on the prediction, the network can select the best target cell and/or beam to trigger L3 handover and how to use the prediction is up to network implementation.

Observation 2: Predicted measurement results can be used for the mobility optimization. And how to use the prediction is up to network implementation.

To support the inference of the network side model, some UE assistance information for the network sided model may be needed. For example, the UE can report the measurement results as the input of the network sided model.

Proposal 12: The UE may need to report data as the input for the inference of network sided model. The report can be included in the measurement report as a baseline.

UE sided model:

For UE sided model, the predicted beam-level and cell-level measurement results can be reported to the network to assist the L3 mobility optimization and these predicted measurement results can be included in the measurement report.

Proposal 13: For UE sided model of RRM measurement prediction, including both beam-level and cell-level prediction, the predicted measurement results can be reported to network. And the predicted beam-level and cell-level measurement results can be included in the measurement report as a baseline.

In the legacy procedure, the UE evaluates the measurement events (e.g. event A3) based on the actual cell-level measurements. To reduce measurement, cell-level measurement prediction is supported and the UE may not perform the actual measurement for some cells but perform cell-level measurement prediction to acquire the measurement results of these cells. For some measurement events associated with the measurement object covered these cells, the UE shall use these predicted cell-level results to evaluate the associated events and trigger the measurement report.

Proposal 14: The UE can use the predicted cell-level measurement results to evaluate measurement report events and trigger measurement report.

In addition, the beam-level measurement prediction can also be used for the beam selection when the UE performs the L3 mobility. For example, the UE can select the beam for the random access to the target cell based on the prediction.

Proposal 15：The beam-level measurement prediction can be used to assist the beam selection for the target cell, e.g., the UE can perform beam selection in the random access procedure to the target cell based on the predicted beam-level measurement results.

Conclusions

According to the analysis given above, we have the following observations and proposals:

Cell-level measurement prediction

Proposal 1: The RSRP difference of actual measured result between AI solution and legacy shall be considered in temporal prediction case B.

Proposal 2: In cluster-based RRM prediction, AI predicts cell measurement result of one cell based on other cells’ measurement result in the same cluster. The cells in the same cluster share similar characteristic, e.g. deployed location or frequency.

Beam-level measurement prediction

Observation 1：L1 beam-level measurement prediction has been studied by the sub-use case 1 of the cell-level RRM measurement prediction.

Proposal 3：The L3 beam-level measurement prediction shall be considered.

Proposal 4: 3 sub-use cases are considered for the L3 beam-level measurement prediction:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 2: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 3: L3 beam-level measurement result(s) is predicted based on L1 beam-level measurement result(s)

Proposal 5: Focus on FR2 for L3 beam prediction.

Proposal 6: Intra-frequency intra and inter-cell spatial domain measurement predictions can be considered for L3 beam-level measurements.

Proposal 7: Intra-cell intra and inter-beam temporal domain measurement predictions can be considered for L3 beam-level measurements.

Proposal 8: The KPI metrics of the L3 beam-level prediction shall follow the cell-level prediction. And the following items can be considered.

- The evaluation is started with measurements prediction accuracy and model complexity.

- The prediction accuracy metric for the L3 beam-level prediction is defined as “RSRP difference between predicted L3 beam level measurement result and actual L3 beam level measurement result of the same beam” for all sub cases and the average RSRP difference is taken as prediction accuracy metric for the L3 beam-level prediction.

LCM for RRM prediction

Proposal 9: For LCM and performance monitoring in AI/ML Mobility, the general framework of AI/ML-air interface WID can be reused.

Proposal 10: The basic procedure and framework defined in AI/ML-air interface WID can be reused for the training data collection of AI/ML Mobility.

Proposal 11: The training data collection can be performed when one of the following events occurs:

- Alt1: When the UE performs L3 Mobility, i.e., L3 handover.

- Alt2: When the UE fulfils the measurement events, e.g., event A2, event A3, event A4, event A5.

Potential AI mobility solution

Observation 2: Predicted measurement results can be used for the mobility optimization. And how to use the prediction is up to network implementation.

Proposal 12: The UE may need to report data as the input for the inference of network sided model. The report can be included in the measurement report as a baseline.

Proposal 13: For UE sided model of RRM measurement prediction, including both beam-level and cell-level prediction, the predicted measurement results can be reported to network. And the predicted beam-level and cell-level measurement results can be included in the measurement report as a baseline.

Proposal 14: The UE can use the predicted cell-level measurement results to evaluate measurement report events and trigger measurement report.

Proposal 15：The beam-level measurement prediction can be used to assist the beam selection for the target cell, e.g., the UE can perform beam selection in the random access procedure to the target cell based on the predicted beam-level measurement results.

Reference

[1] RP-240082, “Revised SID on AIML for mobility in NR”, OPPO

[2] 3GPP TS 38.300, “NR; NR and NG-RAN Overall Description”

[3] Report of 3GPP TSG RAN WG2 meeting #125bis, Changsha, China

[4] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan