3GPP TSG-RAN WG2 #127 meeting	R2-2406798

# Maastricht, The Netherlands, 19th Aug – 23rd Aug, 2024

Source:		NEC

Agenda item:		8.7.4	Scheduling enhancements

Title:		Considerations on delay-sensitive scheduling for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#126 meeting, agreements [1] on scheduling enhancements for XR were made as below:

RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. FFS whether DSR triggering is impacted FFS whether PDU set importance needs to be included

In this contribution, we will provide our views on the enhancement of delay-sensitive scheduling.

# 2. Discussion

2.1 LCP enhancements for delay-critical data.

LCP procedure enhancements

In the RAN2#126 meeting, it was agreed that an additional priority will be configured to LCHs with delay-critical data, but it hasn’t been decided whether the priority only applies to delay-critical data within the LCH or for the whole LCH.

From our understanding, the intention of delay-awareness LCP is to prioritize the delay-critical data over non-delay-critical data to avoid blockage of delay-critical data. If the additional priority applies to the whole LCH, non-delay-critical data of the LCH shall be prioritized as well. For example, if additional priority of LCH A is higher than that of LCH B, then the non-delay-critical data of LCH A shall be multiplexed in a MAC PDU before the delay-critical data of LCH B, which may lead to insufficient resources for the delay-critical data of LCH B. Hence, we suggest that the additional priority only applies to delay-critical data within an LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Proposal 1: RAN2 to agree the additional priority only applies to delay-critical data within a LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Furthermore, the concept of PDU Set has been introduced in Rel-18, and all PDUs belonging to a PDU Set may be needed for the decoding of the corresponding information unit. In this situation, all PDUs belonging to a PDU Set shall be discarded as a whole if at least one PDU is discarded. Similarly, as all PDUs belonging to a PDU Set share the same PSDB, to avoid only part of a PDU Set is multiplexed in a MAC PDU but all PDUs are needed for decoding, which still brings extra delay for a PDU Set, RAN2 should consider maximizing the transmission of a PDU Set during delay-awareness LCP procedure.

Proposal 2: RAN2 considers maximizing the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

In the RAN2#125bis meeting, it was agreed to consider potential impact of delay-awareness LCP on traffic from SRBs. From our understanding, we should discuss SRB 0 and SRBs other than SRB 0 separately.

RAN2 will study whether/how to resolve the issue of data with low remaining time being delayed due to other data from LCHs with higher LCH priority when using the existing LCP procedure. At least the following alternatives will be studied: - Alternative 1: Enhance LCP restrictions/LCH selection. - Alternative 2: Enhance LCH prioritization. RAN2 should consider potential impact on traffic from SRBs.

For SRB0 (i.e., data from UL-CCCH), it always have a higher priority than delay-critical data (i.e., data from any Logical Channel, except data from UL-CCCH), which has already been specified by the priority order of logical channels in current specification [2]. There is no need to break the rules.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; … - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; … NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

For SRBs other than SRB 0, a priority value is associated with the corresponding logical channel, either by applying the default configuration or an explicit configuration. From our understanding, by configuring a proper priority value for the logical channel with delay-critical data, the network can control the impact and the priority order between logical channels with delay-critical data and logical channels for SRBs other than SRB 0. So it can left to the network’s implementation and there should be no specification impact.

Proposal 3: It can be left to network’s implementation to configure the priority for delay-critical data of a LCH and logical channels for SRBs other than SRB 0.

Delay-awareness LCP was ever discussed during Rel-18 XR, and it was agreed in the RAN2#120 meeting that delay-awareness LCP should be able to be turned on/off.

If delay-aware LCP is introduced, need the ability to turn it off. SRBs not impacted. Not considered further unless fundamental issues are identified.

Based on it, to reduce the impacts on the legacy LCP procedure and avoid RAN1 impact, we propose RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

Proposal 4: RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

LCP restrictions

In the RAN2#126 meeting, it was agreed that no dynamic indications are needed for triggering the delay-aware LCP mechanism and RAN1 impacts should be avoided. From our understanding, the intention of the agreements is mainly to avoid introducing dynamic indications through DCI. However, we still need to consider whether to apply delay-awareness LCP for the configured UL grant.

We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial

Regarding the configured UL grant originally for XR traffic, delay-awareness LCP mechanism can be applied to multiplex the delay-critical data with a higher priority. However, there exist other types of periodic traffic (e.g., voice or URLLC service) that are configured with configured UL grant as well. For these services, if delay-awareness LCP is applied to the configured UL grants, delay-critical data of XR traffic may occupy the resources originally reserved for the periodic voice or URLLC traffic, which shall bring extra delay for these services and cause a bad user experience. For this, we propose that different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism. For example, the network can configure UE to do so via RRC signalling, and there is no impact on RAN1.

Proposal 5: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

SR enhancements

In the previous sections, we discuss the delay-awareness LCP mechanism for UL data with UL-SCH resources. However, delay-critical data without UL-SCH resources may also suffer extra delay because of the current SR/DSR mechanism.

When a UE has a UL packet whose remaining delay time is almost exhausted, it should quickly notify the gNB that it has delay-critical data to be scheduled. However, in the current BSR/DSR mechanism, if there is no UL-SCH resource, UE needs to “send SR -> receive UL grant 1 and then send BSR/DSR -> receive UL grant 2 and then send the delay-critical data”. The delay of such a mechanism is illustrated in Figure 1.

Figure 1: delay of UL data caused by current SR/DSR mechanism

Since the size of a PDU or PDU set for XR service is usually large while the UL grant 1 triggered by SR is small, it is highly likely that UL grant 1 cannot accommodate all the delay-critical data. As a result, the UE still needs to wait for UL grant 2 for the transmission of delay-critical data. In this case, the round-trip time of the whole procedure may last long and cause the remaining delay budget of delay-critical data to be exhausted before it can be transmitted.

Observation 1: Current SR mechanism may bring extra delay for the transmission of delay-critical data without UL-SCH resources.

Figure 2: delay of UL data if priori-knowledge of delayed buffer size is associated with an SR

As illustrated in Figure 2, if prior knowledge of delayed buffer size is associated with an SR transmission, the gNB will get the expected buffer size information after the reception of the SR and then perform scheduling accordingly. For example, if an SR is associated with a large delayed buffer size, a bigger UL grant shall be provided for the delay-critical data. Since there is no need to wait for further BSR/DSR and UL grant 2, the latency of delay-critical data shall be reduced.

Proposal 6: Priori knowledge of delayed buffer size can be associated with SR transmission to help gNB schedule an appropriate size of UL grant for delay-critical data without waiting for further DSR/UL grant.

2.2 DSR enhancements.

Enhanced DSR

As the delay information reported by the current DSR MAC CE is coarse, it was agreed in the RAN2#126 meeting to enhance DSR to report multiple pairs of remaining time and buffer size for an LCG. To report delay information with a finer granularity, multiple remaining time levels for an LCG can be introduced, and the following approaches can be considered to achieve the goal.

- Option 1: The network configures multiple remaining time thresholds to UE.

- Option 2: Introducing a remaining time table with each entry corresponding to a predefined range of remaining time of PDCP discard timer.

- Option 3: A reference remaining time is reported for an LCG, and differential encoding is reported for other remaining time values.

Proposal 7: To enhance DSR with multi-pair delay information, multiple remaining time levels for an LCG can be introduced, and the following approaches can be considered:

  - Option 1: The network configures multiple remaining time thresholds to UE.

  - Option 2: Introducing a remaining time table with each entry corresponding to a predefined range of remaining time of PDCP discard timer.

  - Option 3: A reference remaining time is reported for an LCG, and differential encoding is reported for other remaining time values.

The legacy DSR in Rel-18 does not provide the status of non-delay-critical data to the network, as a result, the network cannot schedule it accordingly. For the resources scheduled based on the current DSR, if non-delay-critical data of LCH1 has a higher priority than delay-critical data of LCH2, non-delay-critical data of LCH1 shall be multiplexed first and the remaining resources may not be sufficient for the transmission of delay-critical data. Although BSR can provide the total buffer size information, there may be a gap between the reporting timing of DSR and BSR, which causes the information is not instant and accurate when delay-critical data is scheduled. To avoid this issue, we suggest RAN2 consider that the information of non-delay-critical data is also reported in the enhanced DSR.

Proposal 8: The information of non-delay-critical data is reported in the enhanced DSR.

Currently, PDU Set Importance (i.e., PSI) is only used during PDCP discard operation when congestion happens. If the congestion does happen, PDU Sets with low importance shall be discarded to guarantee the transmission of PDU Sets with high importance. If no congestion happens, there should be sufficient resources for the uplink transimssion. Based on the analysis, we don't see how low-importance data will adversely affect high-importance data. Since the benefit of reporting PSI-related information is not clear, we suggest PSI-related information is not carried in the enhanced DSR.

Proposal 9: PSI-related information is not reported in the enhanced DSR.

Trigger condition of enhanced DSR

The legacy DSR in Rel-18 is triggered when a remaining time of the running PDCP discardTimer is below a configured threshold (i.e., remainingTimeThreshold).

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Since the delay information reported by legacy DSR and enhanced DSR is mainly different in granularity, we think the trigger condition of the legacy DSR can be reused by enhanced DSR, that is, enhanced DSR is triggered when a remaining time of the running PDCP discardTimer is below remainingTimeThreshold.

Proposal 10: The trigger condition of the legacy DSR is reused by enhanced DSR.

Impacts on BSR/legacy DSR

If an enhanced DSR includes both delay-critical and non-delay-critical information for an LCG or LCH, we should avoid reporting redundant information in BSR when the total size of delay-critical and non-delay-critical data reported in enhanced DSR is equal to the total buffer size reported in BSR. For example, the BSR triggered for an LCG or LCH should be cancelled if the total buffer size can be derived from the enhanced DSR.

Proposal 11: The BSR triggered for an LCG or LCH should be cancelled if the total buffer size can be derived from the enhanced DSR.

A MAC PDU should contain at most one type of DSR MAC CE, either legacy DSR MAC CE or enhanced DSR MAC CE. Furthermore, enhanced DSR with finer granularity should be prioritized over legacy DSR. For example, if an enhanced DSR is triggered or configured while a legacy DSR is pending for an LCH, an enhanced DSR should be reported and the legacy DSR should be cancelled.

Proposal 12: If an enhanced DSR is triggered or configured while a legacy DSR is pending for an LCH, the legacy DSR should be cancelled.

# Summary

This contribution provides our analysis on the enhancement of UL delay-sensitive scheduling, and has the following proposals:

LCP procedure enhancements

Proposal 1: RAN2 to agree the additional priority only applies to delay-critical data within a LCH, that is, delay-critical and non-delay-critical data of an LCH shall be associated with different priority values when performing LCP.

Proposal 2: RAN2 considers maximizing the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

Proposal 3: It can be left to network’s implementation to configure the priority for delay-critical data of a LCH and logical channels for SRBs other than SRB 0.

Proposal 4: RAN2 to introduce a MAC CE-based mechanism for the activation/deactivation of delay-awareness LCP.

LCP restrictions

Proposal 5: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

SR enhancements

Observation 1: Current SR mechanism may bring extra delay for the transmission of delay-critical data without UL-SCH resources.

Proposal 6: Priori knowledge of delayed buffer size can be associated with SR transmission to help gNB schedule an appropriate size of UL grant for delay-critical data without waiting for further DSR/UL grant.

Enhanced DSR

Proposal 7: To enhance DSR with multi-pair delay information, multiple remaining time levels for an LCG can be introduced, and the following approaches can be considered:

  - Option 1: The network configures multiple remaining time thresholds to UE.

  - Option 2: Introducing a remaining time table with each entry corresponding to a predefined range of remaining time of PDCP discard timer.

  - Option 3: A reference remaining time is reported for an LCG, and differential encoding is reported for other remaining time values.

Proposal 8: The information of non-delay-critical data is reported in the enhanced DSR.

Proposal 9: PSI-related information is not reported in the enhanced DSR.

Trigger condition of enhanced DSR

Proposal 10: The trigger condition of the legacy DSR is reused by enhanced DSR.

Impacts on BSR/legacy DSR

Proposal 11: The BSR triggered for an LCG or LCH should be cancelled if the total buffer size can be derived from the enhanced DSR.

Proposal 12: If an enhanced DSR is triggered or configured while a legacy DSR is pending for an LCH, the legacy DSR should be cancelled.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.2.0