3GPP TSG RAN WG2 #127                                                                                  R2-2406828

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on NW side data collection

Agenda Item:	8.1.3

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting [1], some general agreements have been achieved for NW side data collection in the aspect of BM use case:

Agreements for beam management

1. 	For gNB centric and OAM centric (for RRC signaling between UE and gNB), reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature.  FFS how to handle case when single RRC message is not sufficient.   FFS if there will be any further enhancement needed pending RAN1 agreement.

2.	Immediate MDT is the baseline framework for OAM-centric data collection for the training of a network-sided model

3.	Enhance the immediate MDT framework to support periodical reporting.   FFS whether and what event-based reporting is supported and FFS on network request reporting

In this contribution, we further discuss the data collection for NW-side model training for BM and Pos use cases.

- Beam management

  - Mechanism of NW side data collection

It is agreed that immediate MDT is the baseline framework for OAM-centric training data collection. In legacy, immediate MDT reuse the RRC message of MeasurementReport in the air-interface. To continue and enhance this message for AI/ML purpose, new mechanism is necessary:

- Under NW controlled data logging and reporting;

For immediate MDT, the UE measurement and report mechanism is entirely under NW control. For gNB centric and OAM centric, reporting multiple instances of logged L1 measurement result from UE to gNB via a RRC message as configured by gNB is an optional feature. The gNB should configure UE to perform data logging and reporting based on the potential AI/ML training requirements, such as:

- Measurement result recording trigger conditions;

- Measurement result reporting trigger conditions;

- Measurement result storage space capacity.

Proposal 1: For AI/ML specific UE data collection, network configures data logging trigger conditions, reporting trigger conditions, and storage limitation.

- Enhanced data logging in UE;

The measurement result recording trigger conditions may include the periodically, event-based, event-based periodically.

For periodically recording, it is more like combination of immediate MDT and logged MDT. The UE can record one or multiple AL/ML related items of UE measurement results in one reporting period;

For event-based recording, UE could only record one item if the pre-configured event is triggered, e.g. the HOF occurred, the UE measurement has fulfill the event A3 threshold. etc.

For event-based periodically recording, the UE periodically records the measurement results on the UE side according to the network-configured recording trigger conditions and interval parameters after the recording trigger conditions are met.

To compare among these recording mechanism, different triggering condition may apply to different data collection content requirement, and periodically recording could reduce the frequency of reporting.

Proposal 2: RAN2 to discuss whether to support periodic or event-based data logging.

- Enhanced data reporting by UE;

The measurement result reporting trigger conditions may include the periodically, event-based, or on-demand.

For periodically reporting, it is more like immediate MDT. The UE can report the AL/ML related UE measurement results based on reporting interval configured by the network. Since the frequent reporting may lead to waste of resources, the reporting interval can be lengthened and the UE could record multiple AL/ML related items of UE measurement results in one reporting period which seems a combination of immediate MDT and logged MDT;

For event-based reporting, UE could only report one item if the pre-configured event is triggered, e.g. the HOF occurred, the UE measurement has fulfill the event A3 threshold, the UE recorded AI/ML measurement results reached the storage space threshold, and UE power reached the threshold. etc. Some of the events will not be considered in this WI but belong to another AI mobility SI for UE-sided model training. Therefore we think event-based reporting could be further studied.

For on-demand reporting, the similar available indicator in logged MDT can be used. Upon receiving the request from the network, the UE sends the measurement results to the network. The available indicator of legacy logged MDT will be sent to the network in the message of RRCSetupComplete / RRCReconfigurationComplete / RRCReestablishmentComplete / RRCResumeComplete if the UE stores available measurement result. Since the BM related measurement will be collected only in CONNECTED state, to avoid frequent reporting in the air-interface, the AI/ML specific available indicator may be sent to the network base on some trigger condition, e.g. the allocated storage space has reached a configured threshold, or the battery power is low.

Proposal 3: On-demand reporting of training data is supported, and an availability indicator in uplink RRC message can be used to indicate training data is available.

  - Large size of single RRC message for AI/ML data report

Current spec supports uplink RRC segmentation of UECapabilityInformation and MeasurementReportAppLayer. If the RRC message segmentation is enabled, UE could use UL message segments to send the UL message to the NW. The current max value of UL message segments is 16. To send the AI/ML related data, MeasurementReport used for AI/ML data reporting could also be allowed to be transmitted in segments. And since it is based on immediate MDT, the logging size in the UE could be controlled, e.g. no more than 16 * PDCP SDU size limit. Both the periodic reporting and on-demand reporting with a UE storage-triggered available indicator could implicitly control the size of the stored data, ensuring that it does not exceed the threshold, e,g, 16 * PDCP SDU size limit.

Proposal 4: The RRC segmentation mechanism can be reused for MeasurementReport to handle the case when the payload of single RRC message is not sufficient.

- Positioning accuracy enhancements

  - Mechanism of NW side data collection

Immediate MDT involves UE, gNB, and TCE node. Some CN node may also be utilized for MDT configuration transmission without information change, i.e. for signaling based MDT. Since LMF is not a related node of MDT, immediate MDT could not be directly reused for Pos use case. The training data collection for Pos use case could be performed between UE/gNB and LMF, and the mechanism could be designed similar as the collection mechanism of BM:

1) Current LPP message can support periodic information transfer procedures, i.e. location information reports, periodic assistance data delivery. Take location information reports as an example, in the commonIEsRequestLocationInformation of RequestLocationInformation message, periodicalReporting field could be included to indicate the reportingAmount and reportingInterval by the location server’s request. Therefore for periodic reporting of AI/ML related measurement, current periodic procedure could be utilized. If other reporting mechanism such as on-demand reporting could be introduced for BM, similar new procedure could also be considered in TS37.355.

2) Current NRPPa message can support periodic and on-demand information transfer related procedures:

If the Report Characteristics IE is set to "OnDemand", the NG-RAN node shall return the corresponding measurement results in the MEASUREMENT RESPONSE message, and the LMF shall consider that this reporting has been terminated by the NG-RAN node. If the Report Characteristics IE is set to "Periodic", the NG-RAN node shall initiate the corresponding measurements, and it shall reply with the MEASUREMENT RESPONSE message without including any measurement results in the message. The NG-RAN node shall then periodically initiate the Measurement Report procedure for the corresponding measurements, with the requested reporting periodicity.

But this on-demand report is triggered by the LMF, and no available indicator is sent from the measurement collection node to trigger network-side retrieval. Therefore for the AI/ML related measurement result collection from gNB to LMF, current periodic procedure could be utilized. If other reporting mechanism such as on-demand reporting could be introduced for BM, similar new procedure could also be considered in TS38.455.

Proposal 5: The similar training data collection mechanism agreed for BM can be applied for Pos use case between UE/gNB and LMF.

  - Data collection per sub-case

Among all 5 Pos cases, NW side model cases include:

- Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

- Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

- Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

For training data collection of AI/ML based positioning, the collected data sample can include two parts defined by RAN1:

| Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement | Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label |
| --- | --- |

And based on agreements in RAN1#116bis and RAN1#117, for the 3 NW side model cases, the information for training could be generated by different nodes:

Table 1 Training data generate node per case

|  | Part A (measurement and its related data) | Part B (labels related) |
| --- | --- | --- |
| Case 3a | TRP/gNB | LMF |
| Case 3b | TRP/gNB | PRU/UE/LMF |
| Case 2b | PRU/UE | PRU/UE/LMF |

If we assumed LMF may be used for gNB-side model training as noted in TR38.843, all data collected for NW sided model training should be transmitted from UE/gNB to LMF.

Collection of Part A

For Part A defined by RAN1, channel measurements include time (DP) and time+power (PDP) at least for case 3a/3b, but whether to support time+power+phase (CIR) is FFS and can be further discussed by RAN1. These channel measurements are considered as model input in inference and should be collected by the training node for all cases. These channel measurements are new introduced for AI/ML and cannot be covered by traditional UE positioning procedures.

Observation 1: For RAN1 defined Part A (measurement and its related data), DP, PDP, or CIR (FFS) should be collected by the training node for NW sided model training of case 3a/3b/2b.

Since the transmission format of the DP/PDP/CIR(FFS) measurement should be provided by RAN1, RAN2 could first discuss in which message(s) such channel measurement could be carried.

- For case 2b, the AI/ML related channel measurement should be transferred from UE/PRU to LMF, the LPP message could be used to carry them. Such AI/ML related channel measurement could be included into the legacy measurement related message, e.g. ProvideLocationInformation message, and new fields may be introduced for AI/ML usage;

- For case 3a/3b, the AI/ML related channel measurement should be transferred from TRP/gNB to LMF, the NRPPa message could be used to carry them. Such AI/ML related channel measurement could be included into the legacy measurement related message, e.g. MEASUREMENT RESPONSE or MEASUREMENT REPORT, and new fields may be introduced for AI/ML usage.

Proposal 6: Introduce new fields in LPP/NRPPa messages for transmission of the AL/ML related channel measurement, and wait for more RAN1 progress on detailed channel measurement content and format, e.g. DP/PDP/CIR(FFS).

Collection of Part B

For Part B (labels related), the label and the related information could be generated at LMF (for case 3a/3b/2b) or PRU/UE (for case 3b/2b):

- Case 3a: The label includes LOS/NLOS indicator and/or timing information (UL RTOA or gNB Rx-Tx time difference). The label and its related data (e.g. quality indicator, time stamp) can be generated by at least in LMF based on UE’s location and gNB/TRP coordinates, where the UE’s location may be:

  - provided by UE/PRU;

  - estimated by LMF using intermediate positioning measurement provided by gNB/TRP(s).

- Case 3b: The label is the UE location coordinates. The label and its related data (e.g. quality indicator, time stamp) can be generated by UE/PRU and transmitted to LMF, or the LMF can generate ground truth label based on measurement transmitted by gNB/TRP.

- Case 2b: The label is the UE location coordinates. The label and its related data (e.g. quality indicator, time stamp) can be generated by UE/PRU and transmitted to LMF, or the LMF can generate ground truth label based on measurement transmitted by UE/PRU.

If the ground truth label (UE location) is provided by UE/PRU, traditional UE positioning method via LPP procedures could be used; if the UE location is estimated by LMF using intermediate positioning measurement provided by UE/PRU, the UE/PRU should provide the intermediate positioning measurement based on the traditional positioning method via LPP procedures; if the UE location is estimated by LMF using intermediate positioning measurement provided by gNB/TRP, the gNB/TRP should provide the intermediate positioning measurement based on the traditional positioning method via NRPPa procedures.

Proposal 7: Reuse legacy LPP/NRPPa procedures and UE positioning method for ground truth label (UE location) collection for NW sided model training of case 3a/3b/2b.

- Conclusion

In this contribution, we provide our views on the training data collection for NW side model, and the observation and proposals are summarized as follows:

For beam management

Proposal 1: For AI/ML specific UE data collection, network configures data logging trigger conditions, reporting trigger conditions, and storage limitation.

Proposal 2: RAN2 to discuss whether to support periodic or event-based data logging.

Proposal 3: On-demand reporting of training data is supported, and an availability indicator in uplink RRC message can be used to indicate training data is available.

Proposal 4: The RRC segmentation mechanism can be reused for MeasurementReport to handle the case when the payload of single RRC message is not sufficient.

For positioning accuracy enhancements

Proposal 5: The similar training data collection mechanism agreed for BM can be applied for Pos use case between UE/gNB and LMF.

Observation 1: For RAN1 defined Part A (measurement and its related data), DP, PDP, or CIR (FFS) should be collected by the training node for NW sided model training of case 3a/3b/2b.

Proposal 6: Introduce new fields in LPP/NRPPa messages for transmission of the AL/ML related channel measurement, and wait for more RAN1 progress on detailed channel measurement content and format, e.g. DP/PDP/CIR(FFS).

Proposal 7: Reuse legacy LPP/NRPPa procedures and UE positioning method for ground truth label (UE location) collection for NW sided model training of case 3a/3b/2b.

- References

- Chairman’s Notes RAN2#126, Fukuoka, Japan, May 20th – 24th, 2024