3GPP TSG-RAN WG2 Meeting #127	R2-2406829

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:	CATT

Title:	Consideration on UE side data collection

Agenda Item:	8.1.4

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, some progress has been made for UE side data collection [1]. And the TR was also updated in Post meeting email discussion [2]. But there are still some FFSs left for further clarification. In this document, we continue focusing on the remaining FFSs and our proposals are provided.

Discussion

2.1	FFS on UP tunnel for option 2/3

FFS in the TR

| Option Aspect | Option 1a) | Option 1b) | Option 2 | Option 3 |
| --- | --- | --- | --- | --- |
| UP/CP tunnel | UP tunnel (for the case of data transfer from UE to OTT server via 3GPP network) | UP tunnel | CP tunnel (provided that the data volume remains within the NAS signalling capacity) FFS: UP tunnel FFS: LPP | CP tunnel (provided that the data volume remains within the RRC signalling capacity) FFS: UP tunnel |

It has been captured in the TR 38.843 that CP tunnel (provided that the data volume remains within the NAS signalling capacity) for option 2/3. But it is FFS on whether UP tunnel can be applied to option 2/3. One of the concerned issues is whether UE side data exceeds RRC/NAS signalling capacity. For UL RRC signalling, segmentation has been supported for UECapabilityInformation or MeasurementReportAppLayer message. Based on the current specification, the maximum number for signalling segmentation is 16. Hence, the maximum data size can be as large as 9000bytes*16 = 144Kbytes. While for NAS signaling capacity, taking UL NAS TRANSPORT as one example, the data size can approximately be 65K (65537-3) bytes [3]. Therefore, CP signaling capacity may be the bottleneck for data collection. But MNO can collect data from multiple UEs. Hence, the data volume per UE can be controlled to be under the maximum CP signaling capacity. Therefore, the data volume for CP tunnel can be controlled to within the CP signaling capacity. Furthermore, both the periodic reporting and on-demand reporting with a UE storage-triggered available indicator could implicitly control the size of the stored data, ensuring that it does not exceed the UL signaling capacity.

Observation 1: The data volume can be controlled in order not to exceed the UL signaling capacity for data collection, e.g., collecting data from multiple UEs, periodic reporting or on-demand reporting mechanism.

Another advantage of UP tunnel is that it shows its good characteristics in terms of privacy. And companies provide the following solutions for UP tunnels in option 2/3 in the Post email [5]:

- Option 2：existing frameworks like UP-based LPP procedures and suggest potentially mimicking these established connections for data collection mechanisms； - Option 3：a UP tunnel between UE and OAM with RAN control

For option 2, UP-based LPP procedure is introduced for positioning. But, for BM, this procedure is not applicable. Thus, these solutions highly depend on other groups and new network structures may be required, which brings more spec work. From our perspective, the privacy protection can be achieved considering partial/no visibility for CP tunnel in option 2/3 has been assumed.

Observation 2: Privacy protection can be achieved for CP tunnel in option 2/3.

In light of Observation 1/2, we can start from CP tunnel for option 2/3 for its simplicity and remove UP tunnel for option 2/3.

Proposal 1: Remove “FFS: UP tunnel” in the aspect of UP/CP tunnel for option 2/3.

Regarding positioning, data can be transferred to the network via LPP signaling. I.e. the LMF collects data from UE/PRU. One option for LPP transmission is to transfer LPP via NAS signaling. This has been included in option CP tunnel. Therefore, “FFS: LPP” should be removed from the TR.

Proposal 2: Remove “FFS: LPP” in the aspect of UP/CP tunnel for option 2.

FFS on the protocol layer for UP tunnel

| Option Aspect | Option 1a) | Option 1b) | Option 2 |  | Option 3 |
| --- | --- | --- | --- | --- | --- |
| Protocol layer for data transfer | Application layer | Application layer | NAS layer for CP tunnel FFS: the protocol layer for UP tunnel | RRC layer for CP tunnel FFS: the protocol layer for UP tunnel |  |

If Proposal 1 is agreed, we think RAN2 does not need to discuss the protocol layer for UP tunnel in option 2/3.

Proposal 3: If Proposal 1 is agreed, remove FFS: the protocol layer for UP tunnel in option 2/3.

2.2	FFS on Possible Options for Visibility of data content in MNO and Data format

For visibility of data content in MNO and Data format, it has been agreed that:

Agreements For the options identified to realize the different levels of data content visibility 1. Full visibility for standardized data content. 2. Partial visibility for partially standardized data content. 3. No standardized visibility NOTE in the TR that RAN2 discussed that visibility can be achieved as per SLA only but is outside of the scope of our discussions.

But there are some FFSs left. We discuss the remaining FFSs in the following.

FFS on the level of visibility in option 1b

| Option Aspect | Option 1a) | Option 1b) | Option 2 |  | Option 3 |
| --- | --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |  |  |

It has been agreed that visibility can be achieved as per SLA only but is outside of the scope of our discussions. For option 1b), we see at least no standardized visibility can be applied for visibility in option 1b). Hence, no standardized visibility in MNO and data format can be achieved in option 1b). And we propose that:

Proposal 4: Capture No standardized visibility for Visibility of data content in MNO and Data format for option 1b.

FFS on visibility in option 2

| Option Aspect | Option 1a) | Option 1b) | Option 2 |  | Option 3 |
| --- | --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |  |  |

For option 2 and 3, it is still FFS on meaning of ‘partial/partially’ visibility. From our perspective, considering NAS signaling is used for data collection in option 2, it is possible that the data can be carried in the container. In this case, the MNO is aware of the procedure/PDU for data collection, e.g., via signaling container, but has limited access/comprehension to some elements of the data content as defined in [5]. Thus, it is proposed that:

Proposal 5: Partial/partially visibility can be defined as: the MNO is aware of the procedure/PDUs data collection procedure, but has limited access/comprehension to some elements of the data content.

Regarding how to achieve partial/partially visibility in option 2, the collected data can be multiplexed into the NAS signaling via container. In this way, the MNO figures out that the collected data is transferred via NAS signaling but is not aware of the data content as data format is not open to the MNO.

Hence, we are wondering how no visibility works in option 2 considering the data is carried using NAS signaling and the data needs to be embedded in the signaling. Hence, it is proposed that:

Proposal 6: For option 2, Partial visibility for partially standardized data content in MNO and Data format is captured into TR 38.843.

Proposal 7: For option 2, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

FFS on visibility in option 3

| Option Aspect | Option 1a) | Option 1b) | Option 2 |  | Option 3 |
| --- | --- | --- | --- | --- | --- |
| Possible Options for Visibility of data content in MNO and Data format (Note 2, Note 3) | No standardized visibility | FFS | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility. | Opt A) Full visibility for standardized data content. FFS: Opt B) Partial visibility for partially standardized data content. FFS Opt C) No standardized visibility. FFS: meaning of ‘partial/partially’ and how to achieve different levels of visibility |  |

In option 3, the similar FFSs were left. In option 3, the AI/ML-specific Data Transfer Path is UE-> gNB->OAM -> Server for data collection for UE-side model training/OTT server. For option 3, like option 2, the data can be carried in the OAM signaling, e.g. container. Hence, it is proposed that:

Proposal 8: For option 3, Partial visibility for partially standardized data content in MNO and Data format can be captured into TR 38.843.

Proposal 9: For option 3, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

Conclusion

In this document, we analyse the FFSs for data collection, and we have the following observations and proposls:

Observation 1: The data volume can be controlled in order not to exceed the UL signaling capacity for data collection, e.g., collecting data from multiple UEs, periodic reporting or on-demand reporting mechanism.

Observation 2: Privacy protection can be achieved for CP tunnel in option 2/3.

Proposal 1: Remove “FFS: UP tunnel” in the aspect of UP/CP tunnel for option 2/3.

Proposal 2: Remove “FFS: LPP” in the aspect of UP/CP tunnel for option 2.

Proposal 3: If Proposal 1 is agreed, remove FFS: the protocol layer for UP tunnel in option 2/3.

Proposal 4: Capture No standardized visibility for Visibility of data content in MNO and Data format for option 1b.

Proposal 5: Partial/partially visibility can be defined as: the MNO is aware of the procedure/PDUs data collection procedure, but has limited access/comprehension to some elements of the data content.

Proposal 6: For option 2, Partial visibility for partially standardized data content in MNO and Data format is captured into TR 38.843.

Proposal 7: For option 2, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

Proposal 8: For option 3, Partial visibility for partially standardized data content in MNO and Data format can be captured into TR 38.843.

Proposal 9: For option 3, FFS Opt C) No standardized visibility in the TR 38.843 is removed.

Reference

- RAN2#126 chairman notes;

- [POST126][034][AIML PHY] TP for data collection (Ericsson);

- TS 24.501, V18.7.0, Non-Access-Stratum (NAS) protocol for 5G System (5GS), Release 18;

- TS 38.305, V18.2.0, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, Release 18;

- R2-2405931, Report of [POST125bis][020][AI/ML PHY] UE side data collection, MediaTek Inc.