3GPP TSG-RAN WG2 Meeting #127                                                                    R2-2407002

Maastricht, Netherlands, 19th - 23th August, 2024

Source:	CATT

Title:	Consideration on on-demand SSB SCell operation

Agenda Item:	8.5.2

Document for:	Discussion and Decision

Introduction

For on-demand SSB SCell operation, the objective of the WID [1] includes:

Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

- Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

- Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Based on the WID, it can be observed that on-demand SSB transmission may affect the time/frequency synchronization, L1/L3 measurements and SCell activation. In this contribution, we will discuss the RAN2 specification impact of on-demand SSB SCell operation.

Discussion

Scenarios and cases of on-demand SSB SCell operation

During RAN1#116 bis meeting [2], the scenarios/cases for on-demand SSB SCell operation were discussed, with the following agreements:

Agreement

For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases:

- Scenario #2 and Case #1

- Scenario #2 and Case #2

- Scenario #2A and Case #1

- Scenario #2A and Case #2

- FFS: Scenario #3A and Case #1

- FFS: Scenario #3A and Case #2

- FFS: Scenario #3B and Case #1

- FFS: Scenario #3B and Case #2

- For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner.

o   Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered.

- Notes:

o	Scenario #2A refers to

	“When UE receives SCell activation command (e.g., as defined in TS 38.321)”

o	Scenario #3A refers to

	“After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed”

o	Scenario #3B refers to

	“When SCell activation is completed and SCell is activated” or

	“After SCell activation is completed and SCell is activated”

o	For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications.

o	Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed.

Based on RAN1 agreement, on-demand SSB can be triggered in Scenario #2 and Scenario #2A with Case #1 and Case #2, so RAN2 can begin discussing the RAN2 spec impact under Scenario #2 and Scenario #2A first, and wait for RAN1's conclusion on Scenario #3A and Scenario #3B.

Proposal 1: RAN2 begin discussing the RAN2 spec impact under Scenario #2 and Scenario #2A with case #1 and Case#2 first, and wait for RAN1's conclusion on Scenario #3A and Scenario #3B.

RAN2 specification impact under Scenario #2 and Scenario #2A

- L3 measurement based on on-demand SSB

L1 measurement based on on-demand SSB was discussed in RAN1#117 meeting [3], with the following agreements.

Agreement

At least support L1 measurement based on on-demand SSB

- For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported.

o	FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB

As usual, L3 measurement is in the scope of RAN2. Hence, in the following we will give brief analyses on the L3 measurement impact with on-demand SSB.

- Scenario 2

For Case #1, UE cannot have measurements on the SCell until the on-demand SSB is transmitted, but NW need to decide whether to activate the SCell or release the SCell based on measurement results. Therefore, triggering on-demand SSB transmission is beneficial for UE to obtain and report the measurement results, so that the network can make correct decisions for SCell.

Observation 1: For Scenario #2 with Case #1, L3 RRM measurement results can be used to assist the network to decide whether to activate the SCell or release the SCell.

For Case #2, UE can perform measurement, but the measurement result report may not trigger or trigger too early. According to RAN4 [4], there is a limit to the time between the measurement report and UE receiving the SCell activation command.

8.3.2	SCell Activation Delay Requirement for Deactivated Scell

SCell in FR1 is known if it has been meeting the following conditions:

- During the period equal to max(5*measCycleSCell,  5*DRX cycles) for FR1 before the reception of the SCell activation command:

-	the UE has sent a valid measurement report for the SCell being activated and

-	the SSB measured remains detectable according to the cell identification conditions specified

in clause 9.2 and 9.3.

- the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Otherwise SCell in FR1 is unknown.

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions:

- During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable):

-	the UE has sent a valid L3-RSRP measurement report with SSB index, and

-	SCell activation command is received after L3-RSRP reporting and no later than the time

when UE receives MAC-CE command for TCI activation

- During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

…

Based on the above spec, the SCell is an unknown cell if the time between the measurement report and UE receiving the SCell activation command exceeds the limit. The activation delay of unknown cell is greater than that of known cell. Therefore, triggering on-demand SSB transmission is beneficial for UE to obtain and report the measurement results to change the SCell from unknown cell to known cell, accelerating the activation of SCell.

Observation 2: For Scenario #2 with Case #2, L3 RRM measurement report can change the SCell from unknown cell to known cell.

- Scenario 2A

According to RAN4 [4], if the SCell is an unknown SCell, after UE receives SCell activation command, UE has to perform AGC, synchronization and L1 measurement/report based on SSB, and then gNB actives TCI state for data transmission according to UE’s report.

Enhancement for multiple unknown SCells’ activation was discussed at RAN4. The LS [5] received from RAN4 at the RAN2 #123 bis meeting is shown below.

Enhancement for multiple FR2 unknown SCells’ activation

Agreements:

- Define requirements for multiple SCell activation (with and without PUCCH SCell) considering at least enhancement of L3 measurement reporting triggered by SCell activation command when all to-be-activated SCells are in the same band.

o	Note: If requirements are not finalized in RAN4 #109 then they will be deprioritized

TCI activation command transmitted is based on L3 measurement results or L1 measurement results?

Agreements:

- If the L3 report is triggered after SCell activation, the TCI activation command is assumed based on:

o	the L3 report, if no L1-RSRP report is received before transmitting the TCI activation command.

o	the L3 report or L1-RSRP report (i.e., up to network implementation), if L3 report and L1-RSRP report is received before transmitting the TCI activation command.

Based on the LS, after receiving SCell activation command, L3 RRM measurement report can be used to assistant the TCI activation command and accelerate the activation of SCell.

Observation 3: During SCell activation procedure, L3 RRM measurement report can be used to assistant the TCI activation command and accelerate the activation of SCell.

The similar function for L3 RRM measurement report can be reused for Scenario #2A with Case #1 and Case #2.

The existing L3 RRM measurement is based on always on reference signal. And during Scenario #2 and Scenario #2A, on-demand SSB can be triggered by gNB. According to the above observations, we propose RAN2 to support L3 RRM measurement based on on-demand SSB and study the possible RAN2 impact. For example, if there is specification impact when the UE starts to measure based on on-demand SSB, or if it is necessary to trigger a new measurement report after on-demand SSB is transmitted.

Proposal 2: Support L3 RRM measurement based on on-demand SSB.

- Signaling to indicate on-demand SSB transmission

On-demand SSB transmission indication signaling was discussed in RAN1, following agreements have been achieved in RAN1#117 meeting [3]:

Agreement

For a cell supporting on-demand SSB SCell operation,

o Support RRC based signaling to indicate on-demand SSB transmission on the cell.

o Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

o FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell.

- This DCI signaling does not provide SCell activation/deactivation.

- If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc.

o FFS: Scenarios where the above signalings are applicable

Based on the above agreement, it has been agreed to use RRC and MAC CE to indicate on-demand SSB transmission, but the applicable scenarios for RRC and MAC CE signaling have not yet been concluded. Considering the common signaling design in different scenarios, if MAC CE can be applied to both Scenario #2 and Scenario #2A, introducing a new MAC CE signaling is more appropriate. Considering RAN1 will continue to discuss the detail of signaling to indicate on-demand SSB transmission, RAN2 can wait for RAN1 further conclusions.

Proposal 3: RAN2 wait RAN1 further conclusions on MAC CE based signaling and RRC based signaling for on-demand SSB transmission.

Conclusion

According to the analysis in section 2, it is observed and proposed that:

Observation 1: For Scenario #2 with Case #1, L3 RRM measurement results can be used to assist the network to decide whether to activate the SCell or release the SCell.

Observation 2: For Scenario #2 with Case #2, L3 RRM measurement report can change the SCell from unknown cell to known cell.

Observation 3: During SCell activation procedure, L3 RRM measurement report can be used to assistant the TCI activation command and accelerate the activation of SCell.

Proposal 1: RAN2 begin discussing the RAN2 spec impact under Scenario #2 and Scenario #2A with case #1 and Case#2 first, and wait for RAN1's conclusion on Scenario #3A and Scenario #3B.

Proposal 2: Support L3 RRM measurement based on on-demand SSB.

Proposal 3: RAN2 wait RAN1 further conclusions on MAC CE based signaling and RRC based signaling for on-demand SSB transmission.

Reference

- RP-234065 New WID: Enhancements of network energy savings for NR

- Draft Report of 3GPP TSG RAN WG1 meeting #RAN1 116 bis

- Draft Report of 3GPP TSG RAN WG1 meeting #RAN1 117

- 3GPP TS 38.133, “Requirements for support of radio resource management (Release 18)”, V18.4.0 (2023-12).

- R2-2309450, LS on FR2 SCell activation enhancements, RAN2#123bis