3GPP TSG-RAN WG2#127												    R2-2407018

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.9.2 Support of Store & Forward

Source: 	NEC

Title:	        Support of Store and Forward

Document for:	 Discussion

# Introduction

Objective of support store and forward satellite operation is as following:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

It was agreed that S&F implies that at least the full eNB will be onboard, S&F feature is common for NB-IOT and eMTC, and applicable to both CP and UP solutions. Furthermore, we have made following agreements regarding barring and general procedures:

Agreements: 1. For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available) 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again 2. S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed. 3. RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN.

In this contribution, we share our view on scenarios and procedure aspects.

# Discussion

## Scenarios

Even though SA2 is discussing one and multiple satellite scenarios, it sounds too restrict to limit to one satellite scenario.  At least from network perspective, there is likely more than one eNB providing the Store&Forward service together. This means a UE could see one satellite/eNB at this time but see another satellite/eNB at another time. An example scenario is shown in below, there are two eNBs flying around with only one NTN gateway infrastructure on ground to support store and forward service, the same UE could see/connect to eNB1 and eNB2 sequentially:

Figure 2.1-1: Example of two satellites/eNBs circulate around for “store and forward” service

On the other hand, considering there is no continuous full CN connection, a NAS signaling or MO data would need to be sent and stored at satellite when UE firstly connects to an eNB, the responding NAS signaling or data would need to be transmitted back to UE when UE secondly connects to an eNB. It would be easier if a UE connect to the same eNB at the first and second time. More consideration e.g., UE context fetching would be needed if at the first time and second time, the UE connects to different satellites.

RAN2 discuss multiple satellites scenario, where a given UE’s camping cell/serving cell could be different at different time window

RAN2 discuss if UE should be encouraged to reconnect to the same satellite/satellite sets at least when there is unfinished store and forward data/signalling transmission.

In WID justification, it is mentioned the benefits associated with the support of Store&Forward (S&F) Satellite operation:

- S&F operation allows for delay-tolerant, non-real-time IoT NTN services to be offered in areas visited by the satellites but with no need to have NTN gateway infrastructure (e.g.mid-sea, remote areas)

- Along with support for discontinuous coverage, support for S&F operation is key to facilitate cost-effective deployment of IoT NTN services and enable an immediate operational service with (1) sparse LEO constellations and (2) reduced ground segment infrastructure.

- The support of S&F puts the 3GPP IoT NTN solution on par with other non-3GPP solutions intended for massive satellite IoT, which natively already supports S&F services

From above justification, a common scenario would be

- From gateway and core network perspective, there is no continuous feeder link availability.

- from UE /service link perspective, There is likely no enough satellite/cells to provide a continuous coverage, which means UE/NW supporting store and forward will experience discontinuous coverage .

Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Considering the cases when there is no neighbour cell, for UE in RRC_IDLE, the network may provide information for operating during discontinuous coverage (e.g., ephemeris parameters, the start-time and/or carrier frequency of upcoming NTN payload coverage). And UE shall use these discontinuous coverage information to predict its in and out of coverage time window. If UE is out of coverage, it could suspend all AS idle procedure for power saving.

For UE in RRC_CONNECTED, we expect NW will release the UE connection before it is out of coverage. Otherwise, upon declaring RLF, UE may determine not to reestablishment the RRC connection by implementation (e.g., based on ephemeris parameters).

UE(s) who support Store&Forward feature, support discontinuous coverage feature upon service link becoming unavailable and there is no neighbor cell:

- For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations.

- For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

## 2.2 Basic procedure

We agreed the basic procedure for MO as follows:

“For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): 1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it; 2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN; 3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available); 4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again”

In general, both transmission for MO and MT will experience two stages:

- service link is available while feeder link is unavailable

- feeder link is available while service link is unavailable

In one hand, since feeder link and service link are not available at the same time, it is inevitable/clear to split the procedure into two parts: procedure when service link is available and procedure when feeder link is available. In another hand when service link or feeder link is available, there might be either uplink data/signalling or downlink data/signalling waiting for transmission, it is also likely with both uplink and downlink data after the long disconnection. Fortunately, MT can reuse MO procedure apart from additional paging step.

We propose that the following steps are taken as baseline for S&F satellite operation when service link is available and there is stored data either in UE or eNB for forwarding：

- the eNB send paging to UE if there is stored data/signalling at the eNB for the UE.

- the UE establish RRC connection with the eNB

- The UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

Please note that that 2), 3) maybe part of 1) in case e.g., EDT is enabled. 2) and 3) can take in any order.

Figure 2.2-1: Example of procedure when service link is available

From RAN2 perspective, the following steps are taken as baseline for S&F satellite operation when feeder link is available and there is stored data/signalling for forwarding either in eNB or CN, however, this is obviously out of RAN2 scope and should be discussed in other WG:

Figure 2.2-2: Example of procedure when feeder link is available

from RAN2 perspective S&F procedure when feeder in is available is out of RAN2 scope; The following steps (shown in figure 2.2-1) are taken as baseline for S&F satellite operation when service link is available and there is stored data either in UE or eNB for forwarding：

- the eNB send paging to UE if there is stored data/signalling at the eNB for the UE.

- the UE establish RRC connection with the eNB

- the UE sends uplink data/signalling to the eNB for storing and forwarding

- the eNB forward the stored downlink data/signalling to the UE

- the eNB release the RRC connection

- Please note that that 2) 3) maybe part of 1) in case e.g. EDT is enabled. 2) and 3) can take in any order

## 2.3 CP and UP CIOT specific aspects

Following figure shows our understanding on the store and forward procedure based on CP CIOT optimisation feature as example:

Figure 2.3-1 Data transport in NAS PDU

Data encapsulated in (initial and following) NAS messages can be sent to eNB via RRC signalling without a UE context established. Because the data transmission can be processed before the UE context establishment, this will less dependent on feeder link availability. And the procedure over the Uu interface could be reused as baseline, with the awareness that data will be stored first and forward later .

In another hand, lack of UE context and full CN connection, there is a risk of misusing. RAN2 may discuss the limitations/restrictions to use this feature, e.g., only a group of UE could be allowed to use a particular satellite/eNB for data and forward, only limit amount data could be stored and forward via CP-CIOT procedure:

Store and forward based-on CP-CIOT procedure, RAN2 discuss how to avoid that a UE misuses/overuses CP-CIOT based store and forward

Following figure shows our understanding on the store and forward procedure based on UP CIOT optimisation procedure:

Figure 2.3-2 Data transport via resumed DRB

A suspended UE context with configured DRB(s) can be resumed for data transmission. Opposite to CP-CIOT, the data transmission will rely on stored UE context at UE and network side. this requests UE context establishment and maintenance, which might become very complicated when feeder link and service link are not available at the same time. moreover, if the UE connects to an eNB different from the anchor eNB, context retrieving is needed, without available feeder link, this will cause further complication and delay. To be simpler, it would be better that the UE connect to the same eNB where its UE context was suspended.

With this option, same as CP solution, potentially the procedure over Uu interface could be reused as baseline without big change. And it can support larger data transmission than CP-CIOT.

Store and forward based-on UP-CIOT procedure, RAN2 discuss if UE prioritizes the eNB where its UE context was suspended for RRC connection resume.

# Conclusion

In summary, we would like to share our view on scenarios and procedure aspects as follows:

Observation 1:	Service link becoming unavailable may lead to either cell change (if there is neighbour cell) or discontinuous coverage behaviour (if there is no neighbour cell).

Proposal 1:	RAN2 discuss multiple satellites scenario, where a given UE’s camping cell/serving cell could be different at different time window

Proposal 2:	RAN2 discuss if UE should be encouraged to reconnect to the same satellite/satellite sets at least when there is unfinished store and forward data/signalling transmission.

Proposal 3:	UE(s) who support Store&Forward feature, support discontinuous coverage feature upon service link becoming unavailable and there is no neighbor cell:

-	For UE in RRC_IDLE, RAN2 assume the UE suspend all AS idle mode operations.

-	For UE in RRC_CONNECTED mode, either NW will release the UE or the UE will declare RLF and enter idle mode directly.

Proposal 4:	from RAN2 perspective S&F procedure when feeder in is available is out of RAN2 scope; The following steps (shown in figure 2.2-1) are taken as baseline for S&F satellite operation when service link is available and there is stored data either in UE or eNB for forwarding：

1)	the eNB send paging to UE if there is stored data/signalling at the eNB for the UE.

2)	the UE establish RRC connection with the eNB

3)	the UE sends uplink data/signalling to the eNB for storing and forwarding

4)	the eNB forward the stored downlink data/signalling to the UE

5)	the eNB release the RRC connection

Please note that that 2) 3) maybe part of 1) in case e.g. EDT is enabled. 2) and 3) can take in any order

Proposal 5:	Store and forward based-on CP-CIOT procedure, RAN2 discuss how to avoid that a UE misuses/overuses CP-CIOT based store and forward

Proposal 6:	Store and forward based-on UP-CIOT procedure, RAN2 discuss if UE prioritizes the eNB where its UE context was suspended for RRC connection resume.

# References

[1] R2-24xxxxx, Report of 3GPP TSG RAN WG2 meeting #126, ETSI MCC