3GPP TSG-RAN WG2 Meeting #127                        R2-2407022

Maastricht, Netherlands, Aug.19th – Aug. 23rd, 2024

Agenda Item:	8.2.4

Source: 	Transsion Holdings

Title:  	Discussion on Random Access for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#126 meeting, the A-IoT Random Access was discussed and the following agreements were made[1]:

Agreements on “4 step” RA

- A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader echos the ID received in Msg1. Further information may be included in mgs2 based on RAN1 agreements

- A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

- The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

- “Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed.

- RAN2 will not use “Msg4” term for further discussion of the random access.

- Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- 2A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.  “Msg2” usage/presence can be further discussed

Agreement

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we continue to discuss the Random Access for A-IoT.

- Discussion

2.1 A-IoT contention-free access procedure

In last meeting the procedure of CFRA for AIoT device was discussed with which the device sends Device ID and/or any other upper layer data in MSG1, the reader may echo some information from MSG1 in MSG2. For the contention-free access procedure RAN2 will study single and multi-device case. For single and multi-device case, as the number of devices is already known by the reader, when trigger the device to access, the corresponding access configuration can also be assigned to each device to increase efficiency, the access configuration can include time and frequency configuration.

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

When the device sends msg1, it will monitor the ACK of the reader. As the AIoT system is a asynchronous system, the timer-based monitoring may can’t work well. As the device can receive as long as there is enough energy, the monitoring can based on specific message. If the device has monitored specific message and the message has echo some information from MSG1, the device can consider access succeed. If the device hasn’t received the ACK from the reader after N times receiving specific message, the device can consider access failed.

Proposal 2: The ACK/NACK of MSG1 can be based on the device whether received specific message with specific information or not.

2.2 A-IoT Random access failure handling

If the triggered device access failed, the reader may meed to triggered the device access again which may not efficient. In legacy RACH procedure, the UE can re-try access using the RACH resources provided in SIB if RACH failed. For AIoT system the re-try access mechanism can be reused. However as the periodical system information is not supported by AIoT devices, the retry access configuration can be provided in the trigger message. If the devices being triggered access failed, it can re-try access using the retry access configuration without the reader re-initiated trigger. And the reader can limited the re-try times and provides the corresponding amount of retry access configuration.To save resources, the retry access configuration can be shared by devices.

Proposal 3: The retry access configuration can be provided in the trigger message for the failed access device to retry access without the reader re-initiated trigger.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT Random Access and made the following proposals:

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

Proposal 2: The ACK/NACK of MSG1 can be based on the device whether received specific message with specific information or not.

Proposal 3: The retry access configuration can be provided in the trigger message for the failed access device to retry access without the reader re-initiated trigger.

- 4 References

- “Draft_R2_126_Meeting_Report_v1”