3GPP TSG-RAN WG2 Meeting #127	R2-2407097

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source:                Qualcomm Incorporated

Title:	LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 RAN2 impact on LP-WUS options

In RAN1, there are several options about how UE performs LP-WUS in CONNECTED state. Since these options impacts RAN2 specification, it is worthy to evaluate these options from RAN2 perspective.

- For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring:

  - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- Combination of options in Case 1 should be considered.

- RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2

- Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

### 2.1.1    RAN2 aspects for Option 1-1

In the last RAN2 meeting, it is agreed to take the existing DCP mechanism as baseline. Based on this agreement.

For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline.

RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE.

Referring to the existing DCP configuration, the following configuration can be considered for Option 1-1 configuration:

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- The indications similar with the following DCP indications.

  - LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

  - LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

  - LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

- For Option 1-1, the following configuration can be provided to the UE.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- The indications similar with the following DCP indications.

  - LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

  - LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

  - LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

In existing DCP mechanism, how to monitor PDCCH after receiving DCP indication is specified in TS 38.331 as follows. Since Option 1-1 LP-WUS just replaces existing DCP using LR, then the procedure can be reused for how to use LP-WUS to trigger PDCCH monitoring, i.e. all

2>	if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3:

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

3>	if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap, or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or

3>	if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers:

4>	start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe.

- For Option 1-1, all the existing conditions to start drx-onDurationTimer defined in clause 5.7 of TS38.321 for DCP can be reused to Option 1-1 LP-WUS.

### 2.1.2    RAN2 aspects for Option 1-2

Option 1-2 introduces additional enhancements that UE can monitor PDCCH after successfully detecting LP-WUS, irrespective of the PDCCH occasion being outside or inside of the legacy PDCCH active time window, as shown in Figure 2. There are two options  on whether UE needs to monitor legacy PDCCH occasions, as shown in the figure below.

While it could be argued that this approach could reduce the DL transmission latency due to legacy C-DRX cycle, there could be a significant specification impact and  the benefits have not been evaluated.

The newly introduced PDCCH occasions do not follow existing C-DRX configurations such as drx-LongCycleStartOffset, drx-SlotOffset, shortDRX. This would require new mechanisms and a new RRC configurations. Also, a new MAC procedure and PDCCH monitoring procedure (e.g. UE behavior on starting or stopping monitor PDCCH occasions) need to be introduced.

The relationship of the new PDCCH occasions and legacy PDCCH occasions needs to be further discussed, e.g. whether UE additionally monitors the legacy PDCCH occasions, as Option 1-2-1 and Option 1-2-2. The legacy C-DRX normally is configured according to DL QoS requirements and traffic characteristics, and if UE does not monitor legacy PDCCH occasion, whether the QoS requirement may be deviated. Also, interaction with HARQ operation has to be specified.

For Option 1-2-1, gNB has to send PDCCH in both of the new PDCCH occasions and legacy PDCCH occasions, which will consume PDCCH signalling overhead, and UE also needs to monitor both of the newly introduced PDCCH active time window and legacy active time window defined by drx-onDurationTimer. Then from RAN2 perspective, Option 1-2-1 is not preferred.

- From RAN2 perspective, Option 1-2-1 is not preferred.

For Option 1-2-2, LP-WUS related configuration needs to be provided to the UE. A drx-onDurationTimer-LPWUS which is similar to drx-OnDurationTimer in the legacy C-DRX operation. The UE can stop PDCCH monitoring when the timer expires. Besides, the LP-WUS monitoring occasions need to be configured to the UE using LP-WUS cycle and offset.

- For Option 1-2-2, the following LP-WUS configuration needs to be provided to the UE

-  drx-onDurationTimer-LPWUS: newly introduced PDCCH monitoring on-duration timer for LP-WUS, which is similar with drx-onDurationTimer.

- LPWUSCycleStartOffset: LP-WUS DRX cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

-  SlotOffset-LPWUS: the delay before starting the drx-onDurationTimer-LPWUS in the subframe

- LPWUS-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

For other timer, something very similar to drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, in the legacy C-DRX configuration are also necessary. A simple approach is to re-use all these parameters for Option 1-2. When a UE monitors LP-WUS, the UE applies Option 1-2 with all these C-DRX parameters being re-used. When the UE does not monitor LP-WUS (e.g., switch to PDCCH monitoring from LP-WUS monitoring due to coverage issue), the UE applies legacy C-DRX operation with all these C-DRX parameters.

- The existing timers of drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, in the legacy C-DRX configuration are reused for Option 1-2.

### 2.1.3    RAN2 aspects for Option 1-3

Option 1-3 is further enhanced of the Option 1-1, where LP-WUS is monitored inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring, as shown in Figure 3.

This option has potential benefit on UE power consumption as UE does not need to monitor all PDCCH occasions during the active time duration. Only when the UE detects LP-WUS successfully, it will monitor PDCCH occasion during the active time duration. This option enables more flexible triggering for PDCCH monitoring that is not restricted by C-DRX configuration. However, unlike Option 1-2, the start of PDCCH monitoring in Option 1-3 is not fully flexible – it is anytime but within ON-duration identified by C-DRX configuration. In other words, PDCCH monitoring windows still follows existing C-DRX configurations such as drx-LongCycleStartOffset, drx-SlotOffset, shortDRX. From MAC layer point of view, a PDCCH monitoring, start at a middle of a DRX active time, is not different from the PDCCH monitoring in the DRX active time that starts at the beginning of the DRX active time. Therefore, there is no reason not to re-use all the other legacy MAC timers, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, for Option 1-3.

- For Option 1-3, all the legacy RRC parameters for C-DRX configuration can be re-used

  - drx-OnDurationTimer, drx-LongCycleStartOffset, drx-SlotOffset provides the understanding of legacy C-DRX active time

  - drx-OnDurationTimer, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, enables PDCCH monitoring window extension same as in the legacy C-DRX configuration

## 2.2 RRM/RLM measurement for LP-WUS

- RRM/RLM measurement

For all options, it is assumed that RRM/RLM measurement are performed by MR, then no impact introduced by LP-WUS, and existing MR measurements mechanisms in CONNECTED state is reused.

- RAN2 assumes that RRM/RLM measurements are performed by MR, so no impact is introduced by LP-WUS to RRM/RLM measurements.

## 2.3 LP-WUS in DC/CA

First of all, it must be common understanding that for a UE configured with dual connectivity (DC), support LP-WUS monitoring on a serving cell per cell-group. It does not make sense to enable LP-WUS that triggers PDCCH monitoring for serving cells in different cell-groups, since different cell-groups are supposed to be associated with different gNB schedulers.

For CA, from Rel-16, dual DRX groups has been supported. A UE can be configured with different values of drx-onDurationTimer and drx-InactivityTimer for one DRX group and for the other DRX group. Long DRX cycle is common for both DRX groups.

Dual DRX groups

Rel-16 DCP was enabled only on PCell or PSCell and indicates to start drx-onDurationTimer of all the DRX groups. In other words, it was not possible to trigger PDCCH monitoring of different DRX groups independently. This is not beneficial for UE power saving. For Rel-19 LP-WUS in connected mode, per DRX group PDCCH monitoring triggering should be considered. There are two options to enable this:

- Opt.1: LP-WUS is configured to be monitored per DRX group

- Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

Option 1 is enabled if LP-WUS is monitored in two different serving cells in different DRX groups. The LP-WUS in each serving cell is nothing to do with PDCCH monitoring for the cell(s) in the other DRX group. Option 2 is enabled without LP-WUS monitoring on multiple serving cells. However, it has to reserve a couple of bits in the LP-WUS to indicate DRX group. Further study is necessary on which option to support.

- For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

  - Opt.1: LP-WUS is configured to be monitored per DRX group

    - For dual DRX groups, LP-WUS monitoring is configured per DRX group

    - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

  - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

## 2.3 LP-WUS configuration and activation/deactivation

There was some discussion in RAN1 on how to configure and activate/deactivate LP-WUS as follows:

----------------------------------------------------------------------------------------------------------------------------------

For RRC CONNECTED mode, from RAN1 perspective,

- PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling

  - FFS whether to support UE assistance.

- LP-WUS monitoring by UE is known to gNB.

  - FFS whether implicit/explicit indication from UE is necessary

- In case LP-WUS monitoring is enabled, following options are further studied

  - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring

  - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance.

  - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer.

  - Option4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

----------------------------------------------------------------------------------------------------------------------------------

Since RAN1 is discussing this issue and what should be configured to the UE depends on how LP-WUS works, as discussed in clause 2.2. It is proposed to postpone the RAN2 discussion on LP-WUS activation/deactivation.

- Postpone RAN2 discussion on activation/deactivation until the LP-WUS operation is determined.

# Conclusion

RAN2 impact on LP-WUS options,

- For Option 1-1, the following configuration can be provided to the UE.

- LPWUS-Offset: The start of LPWUS monitoring relative to the start of drx-onDurationTimer of Long DRX.

- The indications similar with the following DCP indications.

  - LPWUS-Wakeup (optional): the configuration to start associated drx-onDurationTimer in case DCP is monitored but not detected;

  - LPWUS-TransmitOtherPeriodicCSI (optional): the configuration to report periodic CSI that is not L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

  - LPWUS-TransmitPeriodicL1-RSRP (optional): the configuration to transmit periodic CSI that is L1-RSRP on PUCCH during the time duration indicated by drx-onDurationTimer in case DCP is configured but associated drx-onDurationTimer is not started;

- For Option 1-1, all the existing conditions to start drx-onDurationTimer defined in clause 5.7 of TS38.321 for DCP can be reused to Option 1-1 LP-WUS.

- From RAN2 perspective, Option 1-2-1 is not preferred.

- For Option 1-2-2, the following LP-WUS configuration needs to be provided to the UE

-  drx-onDurationTimer-LPWUS: newly introduced PDCCH monitoring on-duration timer for LP-WUS, which is similar with drx-onDurationTimer.

- LPWUSCycleStartOffset: LP-WUS DRX cycle and drx-StartOffset which defines the subframe where LP-WUS DRX cycle starts;

-  SlotOffset-LPWUS: the delay before starting the drx-onDurationTimer-LPWUS in the subframe

- LPWUS-offset: The start of drx-onDurationTimer-LPWUS relative to the start of LPWUS monitoring.

- For Option 1-2, the existing timers of drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, in the legacy C-DRX configuration are reused.

- For Option 1-3, all the legacy RRC parameters for C-DRX configuration can be re-used

  - drx-OnDurationTimer, drx-LongCycleStartOffset, drx-SlotOffset provides the understanding of legacy C-DRX active time

  - drx-OnDurationTimer, drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL, etc, enables PDCCH monitoring window extension same as in the legacy C-DRX configuration

- RAN2 assumes that RRM/RLM measurements are performed by MR, so no impact is introduced by LP-WUS to RRM/RLM measurements.

- For a UE configured with DC, support LP-WUS monitoring on a serving cell per cell-group

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

  - Opt.1: LP-WUS is configured to be monitored per DRX group

    - For dual DRX groups, LP-WUS monitoring is configured per DRX group

    - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

  - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

- Postpone RAN2 discussion on activation/deactivation until the LP-WUS operation is determined.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)