3GPP RAN WG2 Meeting #127 	R2-2407142

# Masstricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	Event triggered L1 measurement reporting for LTM.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement Wis. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

In the previous meeting, the following progress was made:

- Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. - FFS for the cell level measurement. - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. - FFS on the need of Event LTM1. - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. - Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - RAN2 assumes filtering of the L1 measure results is needed. - It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. - FFS on the need of measurement reporting once leaving condition is met.

# Discussion

In addition to improving the latency of event trigger, we should improve the latency of report transmission. Options include using L1 signalling (E.g. in a SR, or other UCI on PUCCH) and using MAC CE. Use of a PUCCH resource may provide better latency while MAC CE, on the other hand, can be transmitted using existing scheduling, has the benefit of more flexible payload size, and re-using HARQ for transmission .

Proposal 1: Support at least MAC CE for transmission of the event-triggered beam quality measurement report. FFS if UCI-based reporting is additionally supported.

As an example, the MAC CE report could be encoded in a similar way as below.

Figure 1: Potential MAC CE report encoding.

In the example, the MAC CE contains the following information.

- Event ID – assuming multiple events can be configured, the report need to include as a minimum the event ID.

- Beam indication(s) – the report may need to include a bitmap indicating which of the configured candidate beams meet the event criteria (Assuming that one event might consider multiple candidate beams – then the report needs to indicate which beams are above a threshold or other criteria). The beam bitmap might also be used to indicate which beams are included in the list of reported power values (e.g. in the illustration above, candidate beam 1 in octet 7 could correspond to the lowest bit which is set to “1”). Another way would be to explicitly indicate beam indexes. The most efficient way in terms of signalling overhead depends on how many beams may need to be indicated, and what the information reported will be. At a high level it should be clear we need a way to identify the reported beams, and the details can be discussed later.

- Power value(s) – the power value of the best N beams, or an explicitly configured N beams, may need to be included. It’s likely that this power value would take the form of a table index, similar to e.g. power headroom report, in order to minimise signalling overhead.

Figure 2: Possible power value mapping.

Proposal 2: At least the following information needs to be contained in an event-triggered L1 measurement report for LTM:

- Event ID.

- Beam indication(s) (identifying the beams reported/meeting event criteria).

- Power value(s).

With the existing mechanism, and assuming MAC CE for LTM reporting has higher logical channel priority for transmission, most of the time there will be sufficient uplink grant to transmit the report straight away. However, in case insufficient grant is available to transmit all of the information, a truncated or reduced report content could be considered. For example, if only enough grant is available to transmit the event ID, in some cases the event ID may be sufficient (for some events, at least). If only enough grant is available to transmit event ID and beam bitmap then the power values may be omitted.

Proposal 3: A truncated MAC CE report should be defined, and transmitted in case there is not enough grant to transmit a full report.

In some cases there may not be sufficient grant to transmit a report at all. In this case, the legacy procedure would be to transmit a SR And BSR, which would delay the LTM report transmission. Some consideration should be given to what the procedure should be in this case – for example, special SR value to indicate an LTM report.

For UCI-based event-triggered beam management reporting, RAN1 agreed to support two modes. In the first mode (Mode A), the UE transmits a first PUCCH to request a resource for a second UL channel (at least PUSCH) to carry the beam report, and subsequently provides the report in the resource indicated by DCI. In the second mode (Mode B), the UE transmits a first PUCCH to notify that it will transmit in a second pre-configured UL channel. From the perspective of reliability, Mode A seems attractive provided that a retransmission mechanism for the first PUCCH is supported in case the UE does not get the second resource.

In case event-triggered beam management reporting is configured, an early indication for L1 event triggered LTM report could be sent as part of the UCI resources used for beam management reports – in other words, re-use the beam management mechanism at least in some specific cases to ensure latency. E.g. in case there is not enough grant to transmit MAC CE, or for specific events, an indication can be sent using the first PUCCH resource in beam management reporting mode A to notify the gNB that an LTM report is available for transmission.

Proposal 4: Support early indication of LTM report using UCI, at least for the case of insufficient/no grant for MAC CE transmission.

To have the option of enabling CSI reporting or a subset of SSB/CSI-RS resources based on the event triggered L1 report, it makes sense to have a low latency method of doing so, rather than requiring RRC reconfiguration. Since LTM cell switch comes in a downlink MAC CE, and MAC CE looks like the most suitable way to report the triggered event, it makes sense to use a MAC CE for controlling the CSI reporting configurations. For example, RRC can pre-configure CSI reporting configurations and MAC CE can enable/disable. If combined with early TA acquisition, then a PDDCH order could be considered, however MAC CE is more robust as it is transmitted on PDSCH with HARQ.

Proposal 5: Introduce DL MAC CE to enable/disable CSI reporting configurations.

Event A1 relates to serving cell measurement. It needs to be discussed whether or not any LTM related functions may occur based on serving beam going above a threshold (for example, perhaps there is no need to have CSI reporting of neighbour cells active as long as the serving cell quality is good). Event A1 is normally used for disabling measurements (e.g. disable particular reporting events, or disable inter-freq or inter-RAT). Since this is currently done at RRC, there seems no need to introduce event LTM1, unless a corresponding mechanism is introduced for LTM to disable certain measurements, as per proposal 5.

Proposal 6: If it is agreed to introduce a DL MAC CE to enable/disable CSI reporting, then introduce event LTM1 (Beam of serving cell becomes better than absolute threshold)

# Conclusion

In this paper we make some initial suggestions on event-triggered L1 reporting for LTM as follows:

Proposal 1: Support at least MAC CE for transmission of the event-triggered beam quality measurement report. FFS if UCI-based reporting is additionally supported.

Proposal 2: At least the following information needs to be contained in an event-triggered L1 measurement report for LTM:

- Event ID.

- Beam indication(s) (identifying the beams reported/meeting event criteria).

- Power value(s).

Proposal 3: A truncated MAC CE report should be defined, and transmitted in case there is not enough grant to transmit a full report.

Proposal 4: Support early indication of LTM report using UCI, at least for the case of insufficient/no grant for MAC CE transmission.

Proposal 5: Introduce DL MAC CE to enable/disable CSI reporting configurations.

Proposal 6: If it is agreed to introduce a DL MAC CE to enable/disable CSI reporting, then introduce event LTM1 (Beam of serving cell becomes better than absolute threshold)

# References

- RP-240299,   “Revised Work Item: NR mobility enhancements Phase 4”