3GPP TSG-RAN WG2 Meeting #127	R2-2407166

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	7.6.2

Source:	Nokia, Nokia Shanghai Bell

Title:	Further discussion on T390 stop and GNSS validity report during C-DRX inactive time

WID/SID:	IoT_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-126 meeting, it was agreed that T390 is stopped after successful GNSS position fix during C-DRX inactive time while how to capture the stop condition should be discussed in the CR review. However, during the post-meeting email discussion [1] for CR review, companies have different view on the exact time point for when the timer should be stopped. After a successful GNSS position fix, it was controversial the UE should stop the timer upon successful GNSS measurement, or the timer should be stopped upon sending the GNSS validity duration report.

- Option1： UE stops T390 upon successful GNSS measurement.

- Option2： UE stops T390 upon sending the GNSS validity duration report.

RAN2-126 meeting agreement: Proposal 4: T390 is stopped after successful GNSS position fix during C-DRX inactive time. Agreed (can further discuss how to capture this in the CR review) In the CR review we focus on how to capture T390 stopping condition for GNSS position fix during C-DRX inactive time

In this contribution, we elaborate more on the possible options and propose to go with Option2 to avoid the RRC state misalignment between UE and NW. Furthermore, we want to further clarify that it is mandatory for the UE to report the remaining GNSS validity duration when the UE completes GNSS acquisition using C-DRX inactive time.

# 2	Discussion

## 2.1 T390 stop during C-DRX inactive time

The T390 timer is defined to control the uplink transmission extension, which may be configured to start upon the GNSS position becoming out-of-date. At the expiry of T390 the UE will start an autonomous GNSS measurement, if configured, and otherwise leave RRC Connected. Due to the potential switch to RRC Idle it is important that network and UE have a common understanding of the T390 state. Otherwise, if the network releases the UE to RRC idle while UE keeps in RRC Connected, there will have an error case if the network receives a request from a “released” UE.

Observation 1: UE and network must have a common understanding of the T390 state because the UE will leave RRC Connected upon T390 expiry unless the autonomous GNSS measurement is configured.

When the UE performs a GNSS measurement during the C-DRX inactive time the network is not aware of the ongoing measurement nor of the outcome of the measurement. There is a risk of T390 status mismatch between UE and network because the UE is no longer in need of uplink transmission extension (i.e. a running T390) if the GNSS position is reacquired successfully.

Therefore, RAN2 has discussed the following options on how the UE may stop the T390 timer:

- Option 1: UE stops the T390 timer upon successful GNSS measurement

  - Option 1a: It is up to UE implementation on whether UE triggers the validity report

  - Option 1b: UE must trigger a validity report after a GNSS measurement in C-DRX but it is up to UE implementation to decide when to trigger the report

- Option 2: UE stops the T390 timer upon sending the GNSS validity report

Option 1 proposes the UE stops the T390 timer, when the GNSS position is successfully reacquired. This will result in a state mismatch between UE and network if the UE considers the timer to be stopped since UE has a new, valid GNSS position, while the network considers the T390 to have expired and thus UE leaving RRC Connected.

Observation 2: UE autonomously stopping T390 will cause a state mismatch where network considers the UE to leave RRC Connected, while UE considers itself to be RRC Connected with a new, valid GNSS position.

The state mismatch issue can be addressed by Option 2, where the UE is mandated to report the new GNSS validity duration to the network. This will make the network aware that uplink transmission extension is no longer applicable.

Observation 3: The T390 state mismatch can be avoided by the UE reporting the new GNSS validity duration before the T390 expiry.

One may argue the option 1b can address the T390 state mismatch issue since UE will report the new GNSS validity duration to network. However, the challenges for option 1b arises in the case where the UE completes the GNSS measurement successfully at time T1 (and stops T390) and reports the new validity duration, such that it is received by the network at T2. If the T2 occurs after the T390 expires on network side, the network may still consider the UE to have left RRC Connected, despite the UE stopped T390 at T1.

Observation 4: The T390 timer may expire in between the UE completing the GNSS measurement and the network receiving the new GNSS validity duration.

Furthermore, for option 1b, the UE may attempt to report the new GNSS validity duration prior to T390 expiry in the network but depending on the Random Access Occasion (RO) availability this may not be possible and therefore it is not possible to rely on UE implementation to report the new GNSS validity duration timely.

Specifically, TS 36.331 defines the following parameters:

- GNSS-PositionFixDuration-r18: {s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31},

- Drx-Cycle-r13:{sf256, sf512, sf1024, sf1536, sf2048, sf3072, sf4096, sf4608, sf6144, sf7680, sf8192, sf9216} with additional drx-Cycle-v1430 {sf1280, sf2560, sf5120, sf10240},

- nprach-Periodicity-r13:{ms40, ms80, ms160, ms240, ms320, ms640, ms1280, ms2560},

- ul-TransmissionExtensionValue-r18:	{sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240}

Based on the parameters, the UE may require 1-31 seconds to perform the GNSS measurement. The DRX cycle is up to 10.24 seconds and thus the UE having a GNSS position fix time duration in the 1-7 seconds range could potentially perform the GNSS measurement autonomously in the C-DRX inactive time. At the same time the T390 (ul-TransmissionExtensionValue-r18) is in the range 0.5-10.24 seconds, and it may therefore expire during the C-DRX inactive time. Finally, the RO periodicity (nprach-Periodicity) is in the range 0.04-2.56 seconds, which means a UE may have to wait up to 2.56 seconds before it can transmit the Random Access preamble to initiate the reporting of the new GNSS validity duration. This could, depending on network configuration, result in the UE not being able to report the new GNSS validity duration before the T390 has expired on network side. One scenario is illustrated in Figure 1.

Observation 5: Depending on Random Access Occasion availability the UE may not be able to report the new GNSS validity duration before the T390 expiry on network side.

Figure 1 Illustration of T390 expiry before UE can report the GNSS validity duration which causes the RRC state mismatch (Option 1).

Option 2 does not face this challenge, because the UE postpones the stopping of T390 until the time of reporting the new GNSS validity duration to the network (i.e. only the UE-eNB propagation delay of some ms exist between UE stopping T390 and network stopping T390). Thus, if there are no Random Access Occasions available both UE and network will be aware the T390 expired and UE has left RRC Connected. For option 2 it may happen that the T390 timer expires after a successful GNSS measurement, but before the GNSS validity duration report is sent. Then the UE will somewhat unnecessarily (since it has a valid GNSS position) go to RRC Idle, but at least the eNB will share the same understanding and also consider the UE to be in RRC Idle and thus the RRC state mismatch is avoided.

Observation 6: Stopping T390 timer in the UE at time of reporting the new GNSS validity duration avoids the T390 state mismatch.

Table 1 provides an overview of the discussed options. As can be observed only option 2 can avoid RRC state mismatch.

Proposal 1: RAN2 to specify the UE stops timer T390 upon sending the GNSS validity duration report. A text proposal is provided below.

Table 1 T390 state analysis for the discussed options.

| Option for T390 stopping in the UE | T390 stopping on network side | T390 expiry on network side |
| --- | --- | --- |
| Option 1a:UE stops the T390 timer upon successful GNSS measurement. UE may not trigger the validity report | Network will not know UE has a valid GNSS and cannot stop T390 | Can occur despite UE has valid GNSS. Will cause RRC state mismatch |
| Option 1b: UE stops the T390 timer upon successful GNSS measurement. UE must trigger a validity report after a GNSS measurement in C-DRX but it is up to UE implementation to decide when to report. | Network stops the T390 when receiving the GNSS validity duration report. | T390 may expire on network side if UE cannot report the GNSS validity duration timely. Will cause RRC state mismatch, because UE stops T390 upon successful GNSS measurement. |
| Option 2: UE stops the T390 timer upon sending the GNSS validity report | Network stops the T390 when receiving the GNSS validity report. | Will be aligned with UE T390 expiry and thus there is no RRC state mismatch. |

Text Proposal for Proposal 1:

### TP for TS36.331: ### 5.5.9 GNSS measurement triggering and reporting For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods. The UE shall: 1> if an indication to perform GNSS measurement is received from lower layers: 2> perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23]; 2> stop timer T390, if running; 1> if gnss-AutonomousEnabled is configured: 2> perform GNSS measurement using an autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry, with a gap length indicated by lower layers or equal to the latest reported time duration required for the UE to acquire a GNSS position if not indicated by lower layers; NOTE: UE can also autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid and stop T390 upon transmitting the report of the new remaining GNSS validity duration, and the exact time of starting GNSS measurements during available idle periods is left to UE implementation. 1> upon starting GNSS measurement: 2> stop timer T318, if running; 1> upon indication that a new GNSS position becomes valid: 2> instruct lower layers to report the remaining GNSS measurement validity duration (see TS 36.321 [6]); 1> upon indication that GNSS measurement has failed: 2> if GNSS position is out-of-date; and 2> if ul-TransmissionExtensionEnabled is not configured or T390 is not running: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'. ### 7.3.1 Timers (Informative) Timer; Start; Stop; At expiry. T390 NOTE1; Upon GNSS validity duration expiry if ul-TransmissionExtensionEnabled is configured.; Upon leaving RRC_CONNECTED, or upon reception of network triggered GNSS measurement, or upon initiating the connection re-establishment procedure, or upon indication that a new GNSS position becomes valid or upon transmitting the report of remaining GNSS validity duration for GNSS acquired during available idle periods in RRC_CONNECTED.; Perform the actions as specified in 5.3.3.21..

## 2.2 GNSS validity duration report during C-DRX inactive time

In the previous RAN2 meeting the NOTE in clause 5.4.10 was updated according to TS36.321 CR [3].

### 5.4.10 GNSS validity duration reporting For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS measurement validity duration. If the MAC entity receives an indication from upper layers to report the remaining GNSS measurement validity duration: - stop the timeAlignmentTimer associated with the pTAG, if running. - initiate a Random Access procedure (see clause 5.1). NOTE: When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

In our view, further clarification of the note is needed because it is not evident that it is mandatory for the UE to report the remaining GNSS validity duration.

Observation 7: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period does not clearly state reporting of remaining GNSS validity duration is mandatory.

Furthermore, it is our understanding that after performing the GNSS measurement a UE shall initiate CBRA to report the validity duration to network, because N_TA shall be reset to 0 by CBRA as the agreement reached in the discussion [AT126][303][IoT NTN Enh] MAC issues (R2-2405761). Therefore any available uplink grant cannot be used for the reporting. The note currently implies it is up to UE implementation whether to perform the Random Access procedure, but if the UE does not perform the procedure the N_TA will be incorrect, because the RAR with new TA has not been received by the UE. If it is not addressed it seems the UE can skip the CBRA procedure for the particular scenario where the UE is performing the GNSS measurement in the C-DRX idle period, but not for any other type of GNSS measurements.

Observation 8: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period implies it is up to UE implementation whether to perform the CBRA, which is mandatory for other types of GNSS measurements.

Proposal 2: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period is updated to reflect the reporting of the remaining GNSS validity duration is mandatory. A text proposal is provided below.

### TP for TS36.321: ### 5.4.10 GNSS validity duration reporting For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS measurement validity duration. If the MAC entity receives an indication from upper layers to report the remaining GNSS measurement validity duration: - stop the timeAlignmentTimer associated with the pTAG, if running. - initiate a Random Access procedure (see clause 5.1). NOTE: When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, it is up to UE implementation whether when to stop timeAlignmentTimer and initiate a Random Access procedure for reporting the remaining GNSS validity duration.

# 3	Conclusion

This document has made the following observations:

Observation 1: UE and network must have a common understanding of the T390 state because the UE will leave RRC Connected upon T390 expiry unless the autonomous GNSS measurement is configured.

Observation 2: UE autonomously stopping T390 will cause a state mismatch where network considers the UE to leave RRC Connected, while UE considers itself to be RRC Connected with a new, valid GNSS position.

Observation 3: The T390 state mismatch can be avoided by the UE reporting the new GNSS validity duration before the T390 expiry.

Observation 4: The T390 timer may expire in between the UE completing the GNSS measurement and the network receiving the new GNSS validity duration.

Observation 5: Depending on Random Access Occasion availability the UE may not be able to report the new GNSS validity duration before the T390 expiry on network side.

Observation 6: Stopping T390 timer in the UE at time of reporting the new GNSS validity duration avoids the T390 state mismatch.

Observation 7: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period does not clearly state reporting of remaining GNSS validity duration is mandatory.

Observation 8: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period implies it is up to UE implementation whether to perform the CBRA, which is mandatory for other types of GNSS measurements.

And proposed the following:

Proposal 1: RAN2 to specify the UE stops timer T390 upon sending the GNSS validity duration report. A text proposal is provided below.

Proposal 2: The note on UE behavior for GNSS acquisition in an idle/C-DRX inactive period is updated to reflect the reporting of the remaining GNSS validity duration is mandatory. A text proposal is provided below.

# Reference

[1] R2-2406105 Report of [Post126][305][IoT-NTN Enh] 36.331 CR (Huawei)

[2] R2-2405758 Corrections to IoT NTN

[3] R2-2405759 Corrections on UE behaviour on DRX for IoT NTN