3GPP TSG-RAN WG2 Meeting #127	R2-2407235

Maastricht, Netherlands, August 19th – 23rd, 2024

Agenda Item:	8.8.6

Source:	Ericsson

Title:	E-UTRAN TN to NR-NTN mobility

Document for:	Discussion, Decision

# 1	Introduction

RAN plenary #103 approved a work item [1] on mobility between LTE (EUTRAN) terrestrial networks (TN) and NR non-terrestrial networks (NTN). The work item aims at specifying support for the following objectives:

- RRC_IDLE mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbour cells in a System Information Block. [RAN2]

In the previous two meetings, RAN2 has made good progress on this topic. In this contribution, we address some remaining open issues.

# 2	Discussion

Two types of information are essential for idle-mode mobility from LTE Terrestrial Networks (TN) to NR Non-Terrestrial Networks (NR NTN):

Cell reselection information concerning NR neighbour cells, e.g., frequency information, information about the SSBs to measure (SMTC configuration).

Satellite assistance information to help the UE adjust the SMTC for measuring NR NTN neighbour cells, compensate propagation delay and doppler shift.

As discussed in our previous contribution [2] and in RAN2#126, the existing LTE SIB24 (providing information about NR neighbouring cells for inter-RAT cell re-selection) and SIB33 (providing neighbouring cell/satellite information for IoT NTN) can be largely reused for the LTE TN to NR NTN idle-mode mobility. However, some details are to be completed and will be discussed next.

### 2.1	NR neighbour cell information for cell reselection

Related RAN2#126 agreement:

- SIB24 is reused to provide the NR NTN cell reselection related information (e.g. frequency information, SMTC config, etc.), introducing a satellite ID list in per frequency. The EUTRA cell provides the satellite assistance information for NR neighbor cell per satellite, as identified by the satellite ID.

LTE SIB24 contains essential information for NR neighbour cell measurements, especially the SMTC, which is encoded in the IE MTC-SSB-NR below.

[36.331-V18.1.0] SystemInformationBlockType24 information element -- ASN1START […] CarrierFreqNR-r15 ::= SEQUENCE { carrierFreq-r15 ARFCN-ValueNR-r15, multiBandInfoList-r15 MultiFrequencyBandListNR-r15 OPTIONAL, -- Need OR multiBandInfoListSUL-r15 MultiFrequencyBandListNR-r15 OPTIONAL, -- Need OR measTimingConfig-r15 MTC-SSB-NR-r15 OPTIONAL, -- Need OR […] } […] -- ASN1STOP #### MTC-SSB-NR The IE MTC-SSB-NR specifies the SS/PBCH block measurement timing configuration (SMTC) applicable for SSB based NR measurements i.e. the time occasions for performing these measurements, see 5.5.2.13. MTC-SSB-NR information elements -- ASN1START MTC-SSB-NR-r15 ::= SEQUENCE { periodicityAndOffset-r15 CHOICE { sf5-r15 INTEGER (0..4), sf10-r15 INTEGER (0..9), sf20-r15 INTEGER (0..19), sf40-r15 INTEGER (0..39), sf80-r15 INTEGER (0..79), sf160-r15 INTEGER (0..159) }, ssb-Duration-r15 ENUMERATED {sf1, sf2, sf3, sf4, sf5 } } MTC-SSB2-LP-NR-r16::= SEQUENCE { pci-List-r16 SEQUENCE (SIZE (1..maxNrofPCI-PerSMTC-r16)) OF PhysCellIdNR-r15 OPTIONAL, -- Need OR periodicity-r16 ENUMERATED {sf10, sf20, sf40, sf80, sf160, spare3, spare2, spare1} } -- ASN1STOP

For the current WI, a relevant question is whether multiple SMTCs are needed. Currently, LTE SIB24 contains a single SMTC configuration for NR neighbour cell. However, in NR NTN, the basic SMTC of NR was extended by the field smtc4list (present in SIB2 and SIB4) which contains a list of NTN cells that may have different offsets. RAN2#125bis discussed this aspect and agreed on the following working assumption:

Working Assumption: We don’t introduce multiple SMTCs in LTE

In our view, given a TN cell is considerably smaller than an NTN cell, the chance for the UEs in a TN cell to observe several satellites that require separate SMTCs is much smaller than that for UEs in an NTN cell. Therefore, we do not see a need to introduce multiple SMTCs in LTE.

Confirm RAN2#125bis WA: do not introduce multiple SMTCs in LTE.

### 2.2	Satellite assistance information

#### 2.2.1	Downlink Polarization

Related RAN2#126 agreements:

4.	To support the idle mode mobility from EUTRA TN to NR NTN, the satellite assistance information for NR NTN neighbor cells is needed and should include the following parameters:

-	Satellite ephemeris information

-	TA common information

-	k-Mac

-	epoch time

-	validity duration

-	ntn-PolarizationDL (FFS if mandatory or optional)

5.	The Ephemeris information/epoch time/k-mac/validity duration IEs defined in SIB33 specified in TS36.331 should be reused for NR satellite assistance information.

6.	The signalling format for ntn-PolarizationDL and TA common related configurations within NTN-Config specified in TS38.331 should be introduced in TS36.331 for NR satellite assistance information.

One open issue is to decide whether the NTN DL polarization information is mandatory or optional. This information is currently supported for NR NTN as an optional field, but not supported for IoT NTN. Furthermore, in the NTN SI, we note that S-band UEs were assumed to have linear polarization and a 3 dB loss due to satellite-UE polarization mismatch in link budget calculations (see TR 38.811). Hence, we think it should not be mandatory for the network to signal the parameter ntn-PolarizationDL.

For satellite assistance information, the field ntn-PolarizationDL is optional.

#### 2.2.2	Choice of SIB

The next question is whether to define a new SIB or to enhance SIB33 for providing satellite assistance information for the present WI. RAN2#126 agreed on the following:

7.	RAN2 will decide in the next meeting which of the following options to adopt for the provision of the NR satellite assistance information (based on TPs provided by the WI RRC Rapporteur):

Option 1: Introduce a new SIB to include the NR satellite assistance information.

Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Option 3: Extend the NeighSatelliteInfo defined for IoT NTN to include the parameters needed for NR satellite, and reuse the neighSatelliteInfoList defined in SIB33 to provide either NR or IoT NTN information.

##### 2.2.2.1	Background: system information broadcast in IoT NTN

LTE-M UEs can only handle 6 RBs, the system BW is partitioned into multiple non-overlapping narrow bands, each occupies 6 RBs, and all LTE-M transmissions must take place in one of the narrow bands. SIB1-RB (reduced bandwidth) message is scheduled by the MIB, instead of by a PDCCH as in the case for SIB1 for normal LTE UEs. The SIB1-RB message then schedules subsequent SI messages that carry other SIBs for LTE-M UEs, including the SIB33. SIB33 was introduced solely in the context of IoT NTN. Therefore, in the case RAN2 reuse the SIB33 for the current WI, SIB33 will be transmitted separately for normal LTE UEs and for LTE-M UEs.

LTE and LTE-M SIBs are transmitted separately. Thus, it is possible to re-use SIB33.

##### 2.2.2.2	Comparison of the selected options

Back to the three listed options in the RAN2 agreement above, Option 3 is not feasible since there is no extension marker for the NeighSatelliteInfo IE in SIB33, see below (the extension marker was removed in a recent ASN.1 review). A non-critical extension would imply adding another extension IE with the same number of items as the original list. We think this option is not as signalling efficient as the other being considered.

SystemInformationBlockType33 information element

-- ASN1START

SystemInformationBlockType33-r18 ::= SEQUENCE {

neighSatelliteInfoList-r18	NeighSatelliteInfoList-r18			OPTIONAL,	-- Need OR

neighValidityDuration-r18		ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40,

s45, s50, s55, s60, s120, s180, s240, s900}

OPTIONAL,	-- Need OP

lateNonCriticalExtension		OCTET STRING							OPTIONAL,

...

}

NeighSatelliteInfoList-r18 ::=	SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18

NeighSatelliteInfo-r18 ::=	SEQUENCE {

satelliteId-r18				SatelliteId-r18,

ephemerisInfo-r18				CHOICE {

stateVectors-r18				EphemerisStateVectors-r17,

orbitalParameters-r18			EphemerisOrbitalParameters-r17

},

nta-CommonParameters-r18		SEQUENCE {

nta-Common-r18					INTEGER (0..8316827)		OPTIONAL,	-- Need OP

nta-CommonDrift-r18			INTEGER (-261935..261935)	OPTIONAL,	-- Need OP

nta-CommonDriftVariation-r18	INTEGER (0..29479)			OPTIONAL	-- Need OP

},

epochTime-r18					SEQUENCE {

startSFN-r18					INTEGER (0..1023),

startSubFrame-r18				INTEGER (0..9)

}																OPTIONAL,	-- Need OP

k-Mac-r18						INTEGER (1..512)				OPTIONAL,	-- Need OP

t-ServiceStartNeigh-r18		TimeOffsetUTC-r17				OPTIONAL	-- Need OR

}

-- ASN1STOP

Option 3 for satellite assistance information is not feasible since the IE NeighSatelliteInfo is not extendable.

Option 1 (new SIB) and Option 2 (new IE in SIB33) are both viable. In case of Option 2, despite the SIB is the same, network will set different contents depending on whether it is sent in an LTE cell or a IoT NTN cell (narrowband). In conclusion, we prefer Option 2 which has the least specification impact.

Adopt Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

## 2.3	Cell reselection evaluation

There has been some interest in discussing other aspects of cell re-selection, e.g., the cell re-selection criteria (specified in TS 36.304) and RAN2#126 reached a working assumption:

Working Assumption:

- NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell (can come back in the next meeting to see if the WA can be confirmed)

We understand that revisiting the cell reselection evaluation for LTE TN to NR NTN would result in an extension of the WI scope. In our view, RAN2 should focus solely on specifying SIB signalling to facilitate RRC_IDLE mode mobility. Therefore, we propose to confirm the working assumption.

Confirm the WA of RAN2#126: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	LTE and LTE-M SIBs are transmitted separately. Thus, it is possible to re-use SIB33.

Observation 2	Option 3 for satellite assistance information is not feasible since the IE NeighSatelliteInfo is not extendable.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Confirm RAN2#125bis WA: do not introduce multiple SMTCs in LTE.

Proposal 2	For satellite assistance information, the field ntn-PolarizationDL is optional.

Proposal 3	Adopt Option 2: Define new IE for NR satellite assistance information and define separate neighbour satellite information list to provide the NR satellite information in SIB33.

Proposal 4	Confirm the WA of RAN2#126: NR NTN cell reselection evaluation is based on RRM measurements as legacy; no spec impact foreseen for EUTRA TN to NR NTN cell.

# 4	References

RP-240846, Inter RAT mobility support from E-UTRAN TN to NR-NTN, RAN#103, Mar. 2024

R2-2404840, E-UTRAN TN to NR-NTN mobility, Ericsson, RAN2#126, May 2024