3GPP TSG-RAN WG2 #127	R2-2407249

Maastricht, Netherlands, 19th – 23th August 2024

Agenda Item:	8.10.5

Source:	Ericsson

Title:	On Rel.18 leftovers

Document for:	Discussion

# 1	Introduction

Rel-19 WID supports the following leftover topics in RAN2.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

•	RACH optimization for SDT

•	MHI Enhancement for SCG Deactivation/Activation

•	MRO for MR-DC SCG failure

In this paper, we discuss these leftovers.

# 2	Discussion

## 2.1	MHI Enhancement for SCG Deactivation/Activation

In Rel-16, Mobility History Information (MHI) was introduced, logging up to 16 PCells visited by the UE. In Rel-17, PSCell MHI was introduced, enabling the UE to log up to 16 visited PSCells, or time with no PSCell, across all reported PCells. In Rel-17, SCG activation and deactivation was also introduced, which enables power saving while still allowing fast access to the SCG when needed.

In Rel-18, the impact of SCG activation and deactivation on the Mobility History Information was brought up by some companies, see for example [1], [2], but the topic was postponed due to lack of time. In the Rel-19 SON/MDT Work Item [3], MHI enhancement for SCG deactivation/activation is listed as one of the objectives for the Work Item.

The logging of Mobility History Information is performed when the stay in the cell to be logged is ending, typically upon handover or cell change. The cell the UE is leaving is then logged together with information on the time spent in the cell. Similarly, the logging of PSCell Mobility History Information is done upon addition of a PSCell, release of a PSCell, or upon change of PCell (since the PSCell entries are stored in a substructure to the PCell entries). Just as the PCell history, the PSCell history includes the cell ID and the time spent in the cell, or the time spent with no PSCell. The logged information will, after reporting to the network, help the network to understand the movement of the UE and identifying ping-pong or coverage issues in the network.

From the current MHI framework, there is no possibility to see from the Mobility History Information whether the SCG state has been set to activated or deactivated during the time the UE spent in the PSCell.

SCG activation and deactivation was introduced in Rel-17.

There is no possibility to see from the Mobility History Information whether the SCG state was set to activated or deactivated.

With the introduction of SCG activation and deactivation, a PSCell addition or change could take place with the SCG state set to either activated or deactivated. Information indicating the SCG state during the stay in the PSCell could give additional valuable insights and enable optimization of the configuration of dual connectivity.

Understanding the usage of secondary cells is of importance for understanding in which situations there will be a gain from setting up a secondary cell.

To increase the understanding of the secondary cell usage, information on the accumulated time with SCG state set to deactivated for each PSCell entry could be included in the MHI. Since the time spent with the PSCell configured is already logged by the UE since Rel.17, this new information will give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state.

For each PSCell entry of the MHI, log the accumulated time with SCG state set to deactivated during the stay in the PSCell.

## 2.2	RA report logging in case of SDT

In the last RAN2#126 meeting, it was agreed that the UE includes the buffered uplink data volume information and measured downlink RSRP information in the RA report.

- For the purpose of SON enhancements for SDT, include RSRP/data volume related information

- Downlink RSRP value and buffered uplink data volume at the time when the UE evaluates if it should perform SDT.

From the agreement, it should be noted that the UE includes RSRP and data volume information when it evaluates if it should perform MO-SDT irrespective of whether SDT was initiated or not.

The UE performs the evaluation by comparing the buffered uplink data volume pending transmission on radio bearers with SDT enabled and downlink RSRP with the network configured threshold for data volume and RSRP for MO-SDT, as mentioned in TS-38.321-

RRC configures the following parameters for SDT procedure:

-	sdt-DataVolumeThreshold: data volume threshold for the UE to determine whether to perform SDT procedure initiated for MO-SDT;

-	sdt-RSRP-Threshold: RSRP threshold for UE to determine whether to perform SDT procedure initiated for MO-SDT;

Hence from the above agreements, even if SDT was not eventually initiated by the UE, i.e. the UE instead falls back to common RACH resources, it is clear that if the UE includes downlink RSRP value and buffered uplink data volume as RA information, then the gNB can deduce that the UE at least attempted SDT.

From the RAN2#126 agreements, if the UE includes buffered uplink data volume and downlink RSRP information in the RA information, then the network can implicitly deduce that the UE tried to perform SDT, irrespective of whether the UE eventually used RA-SDT or common RACH resources.

However, MO-SDT can be of two types, RA-SDT and CG-SDT. Thus, for both CG-SDT and RA-SDT evaluation, the UE utilizes the same data volume threshold and RSRP threshold as reference. Hence, in case the UE fails to initiate SDT procedure and accesses the network using common RACH resources, it remains unclear if the problem was on CG-SDT or RA-SDT or both. In particular, the following scenarios can be envisaged according to the MAC spec (see TS 38.321, Section 5.27.1):

- The UE evaluated only RA-SDT (e.g. not configured with CG-SDT), but either the RSRP-based condition or the data volume-based condition was not fulfilled. The UE performs RA on common RACH resources.

- The UE evaluated only CG-SDT (i.e. RA-SDT not configured), but either the RSRP-based condition or the data volume-based condition or the CG-based condition (e.g. TA for CG-SDT not valid, next CG occasion too far away in time, etc) was not fulfilled. The UE performs RA on common RACH resources.

- Both the RSRP-based condition and the data volume-based condition are fulfilled. The UE first evaluated CG-SDT (according to the MAC spec procedure, the UE shall first evaluate CG-SDT), but a CG-based condition (e.g. TA for CG-SDT not valid, next CG occasion too far away in time, etc) was not fulfilled. The UE performs RA-SDT.

The following remains unclear to network:

  - The UE evaluates RA-SDT and/or CG-SDT, but if falls back to common RACH resources due to the RSRP-based condition or the data volume-based condition or CG-based conditions not being fulfilled.

  - The RSRP-based condition and the data volume-based conditions are fulfilled, the UE evaluates CG-SDT, but it falls back to RA-SDT, due to CG-based conditions not being fulfilled.

Knowing whether the UE initially tried to perform RA-SDT or CG-SDT is useful information that the network can use to optimize RA resources. For example, if many UEs fail to initiate CG-SDT and fall back to RA-SDT, the RA-SDT resources dimensioned for the cell may become overloaded. Additionally, in this case both the RSRP-based condition and the data volume-based condition is fulfilled (otherwise the UE would not select RA-SDT resources). Hence, the network will not be able to know that the selection of the RA-SDT was because issues with the CG-SDT, and not because only RA-SDT was configured/evaluated.
Similar problem arises, if many UEs fail to initiate CG-SDT or RA-SDT and fallback to common RACH resources.

A UE may fail to initiate CG-SDT and perform RA-SDT causing overload of RA-SDT resource utilization, or it may fail to initiate CG-SDT or RA-SDT and perform RA in common RACH resources.

Hence, we propose the following.

If the UE evaluated to perform SDT, and common RACH resources were used (i.e. conditions for initiating RA-SDT or CG-SDT were not fulfilled), the UE indicates in the RA-Report whether RA-SDT and/or CG-SDT initiation was evaluated.

If the UE evaluated to perform CG-SDT, and RA-SDT resources were used (i.e. conditions for initiating CG-SDT not fulfilled), the UE indicates in the RA-Report that CG-SDT initiation was evaluated.

Related to CG-SDT, we believe that it is also beneficial to include the reason related to why the CG-SDT failed, which according to TS 38.321 are TA for CG-SDT not valid, SSB RSRP measured level smaller than the CG-SDT SSB RSRP threshold, i.e., smaller than cg-SDT-RSRP-ThresholdSSB, or the next occasion for CG-SDT is too far away in time. This would help the gNB to improve CG-SDT configuration, thereby limiting the overload issues in common RA or RA-SDT resources.

RAN2 to further discuss whether to include in the RA-Report information related to the cause of CG-SDT evaluation failure prior to the random access procedure.

From RAN2#126 agreement there is also the following FFS:

- When SDT failure happens, the UE can indicate the failure cause of SDT to the network, e.g. T319a expiration. Details are TBD, e.g. if RSRP and data volume can also be included in such report.

According to TS 38.300, an initiated SDT can fail in the following cases:

- From TS 38.300:

Once initiated, the SDT procedure is either:

-	successfully completed after the UE is directed to RRC_IDLE (via RRCRelease) or to continue in RRC_INACTIVE (via RRCRelease or RRCReject) or to RRC_CONNECTED (via RRCResume or RRCSetup); or

-	unsuccessfully completed upon cell re-selection, expiry of the SDT failure detection timer, a MAC entity reaching a configured maximum PRACH preamble transmission threshold, an RLC entity reaching a configured maximum retransmission threshold, or expiry of SDT-specific timing alignment timer while SDT procedure is ongoing over CG and the UE has not received a response from the network after the initial PUSCH transmission.

Since the UE does not declare RLF upon SDT failure, then it may be beneficial if the UE could include the cause of failures, as listed in 38.300. Including the value of the RSRP when the SDT failure happens was left as TBD during last meeting. In the case of the RLF, the last radio measurements performed by the UE prior the RLF are included in the RLF-Report because they are considered important information for the troubleshooting of the problem. With the same logic, since the SDT failure is not captured in the RLF-Report, we believe that is beneficial if the UE could include RSRP-related information in the RA-Report. It could be for example just an indication on whether the RSRP is above or below the sdt-RSRP-Threshold.
We see also benefits in the inclusion of the data volume information at SDT failure. Especially if the failure occurs due to T319 expiry, then it would be very interesting to know the remaining data volume so that the link adaptation can potentially be optimized, as well as the T319 value configuration.

The UE includes in the RA-Report the SDT failure causes, i.e. the ones specified in TS 38.300 Section 18.0.

The UE includes in the RA-Report RSRP-related information at the time of failure, e.g. information on whether the last measured RSRP is above or below the sdt-RSRP-Threshold.

The UE includes in the RA-Report the remaining SDT data volume in the UE buffer at time of SDT failure, at least for the case of T319 expiry.

## 2.3	Enhancement of SCGFailureInformation

In RAN2#126 meeting, it was agreed to reply to RAN3 LS that RAN2 would work on EN-DC scenario for enhancement of SCGFailureInformation.

- Reply to RAN3 that we will only do EN-DC. RAN2 understands that whether also supporting (NG)EN-DC has no additional RAN2 impact hence RAN3 can decide. If later we get time we can consider other options.

In this section we analyse on the necessary requirements for the agreement and propose some additional enhancements of SCGFailureInformationNR.

The SCGFailureInformaitonNR message is used in EN-DC to inform LTE MCG regarding failure at the NR SCG. The message is copied below-

SCGFailureInformationNR message

-- ASN1START

SCGFailureInformationNR-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

scgFailureInformationNR-r15			SCGFailureInformationNR-r15-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

SCGFailureInformationNR-r15-IEs ::=	SEQUENCE {

failureReportSCG-NR-r15				FailureReportSCG-NR-r15				OPTIONAL,

nonCriticalExtension					SCGFailureInformationNR-v1590-IEs	OPTIONAL

}

SCGFailureInformationNR-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension					OCTET STRING					OPTIONAL,

nonCriticalExtension						SEQUENCE {}					OPTIONAL

}

FailureReportSCG-NR-r15 ::=		SEQUENCE {

failureType-r15						ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx,

synchReconfigFailureSCG, scg-reconfigFailure,

srb3-IntegrityFailure, dummy},

measResultFreqListNR-r15				MeasResultFreqListFailNR-r15		OPTIONAL,

measResultSCG-r15						OCTET STRING						OPTIONAL,

...,

[[	locationInfo-r16				LocationInfo-r10						OPTIONAL,

logMeasResultListBT-r16			LogMeasResultListBT-r15					OPTIONAL,

logMeasResultListWLAN-r16		LogMeasResultListWLAN-r15				OPTIONAL,

failureType-v1610				ENUMERATED {t312-Expiry, scg-lbtFailure,

beamFailureRecoveryFailure, bh-RLF-r16,

beamFailure-r17,

spare3, spare2, spare1}	OPTIONAL

]]

}

MeasResultFreqListFailNR-r15 ::=	SEQUENCE (SIZE (1..maxFreqNR-r15)) OF MeasResultFreqFailNR-r15

MeasResultFreqFailNR-r15 ::=		SEQUENCE {

carrierFreq-r15						ARFCN-ValueNR-r15,

measResultCellList-r15				MeasResultCellListNR-r15			OPTIONAL,

...

}

-- ASN1STOP

In NR SCGFailureInformation message, UE includes both failed PSCell ID and previous PSCell ID information. The purpose of which is to assist MN to perform fast analysis of the problem and perform any recovery. However, such information is not present in SCGFailureInformationNR message. Hence, we propose to include this information.

To enhance SCG failure information sent in EN-DC, UE includes failed PSCell ID and previous PSCell ID in the SCGFailureInformationNR message.

# Conclusion

In the previous sections we made the following observations:

Observation 1	SCG activation and deactivation was introduced in Rel-17.

Observation 2	There is no possibility to see from the Mobility History Information whether the SCG state was set to activated or deactivated.

Observation 3	Understanding the usage of secondary cells is of importance for understanding in which situations there will be a gain from setting up a secondary cell.

Observation 4	From the RAN2#126 agreements, if the UE includes buffered uplink data volume and downlink RSRP information in the RA information, then the network can implicitly deduce that the UE tried to perform SDT, irrespective of whether the UE eventually used RA-SDT or common RACH resources.

Observation 5	The following remains unclear to network:

a.	The UE evaluates RA-SDT and/or CG-SDT, but if falls back to common RACH resources due to the RSRP-based condition or the data volume-based condition or CG-based conditions not being fulfilled.

b.	The RSRP-based condition and the data volume-based conditions are fulfilled, the UE evaluates CG-SDT, but it falls back to RA-SDT, due to CG-based conditions not being fulfilled.

Observation 6	A UE may fail to initiate CG-SDT and perform RA-SDT causing overload of RA-SDT resource utilization, or it may fail to initiate CG-SDT or RA-SDT and perform RA in common RACH resources.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For each PSCell entry of the MHI, log the accumulated time with SCG state set to deactivated during the stay in the PSCell.

Proposal 2	If the UE evaluated to perform SDT, and common RACH resources were used (i.e. conditions for initiating RA-SDT or CG-SDT were not fulfilled), the UE indicates in the RA-Report whether RA-SDT and/or CG-SDT initiation was evaluated.

Proposal 3	If the UE evaluated to perform CG-SDT, and RA-SDT resources were used (i.e. conditions for initiating CG-SDT not fulfilled), the UE indicates in the RA-Report that CG-SDT initiation was evaluated.

Proposal 4	RAN2 to further discuss whether to include in the RA-Report information related to the cause of CG-SDT evaluation failure prior to the random access procedure.

Proposal 5	The UE includes in the RA-Report the SDT failure causes, i.e. the ones specified in TS 38.300 Section 18.0.

Proposal 6	The UE includes in the RA-Report RSRP-related information at the time of failure, e.g. information on whether the last measured RSRP is above or below the sdt-RSRP-Threshold.

Proposal 7	The UE includes in the RA-Report the remaining SDT data volume in the UE buffer at time of SDT failure, at least for the case of T319 expiry.

Proposal 8	To enhance SCG failure information sent in EN-DC, UE includes failed PSCell ID and previous PSCell ID in the SCGFailureInformationNR message.

# References

R2-2312678, MHI Enhancement for SCG Activation/Deactivation, CMCC, Ericsson, CATT, RAN2 #124, Nov. 13th - 17th, 2023.

R2-2312746, Discussion on MHI Enhancement for SCG Deactivation/Activation, CATT, RAN2 #124, Nov. 13th - 17th, 2023.

RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, CMCC, RAN #102, Dec. 11th – 15th, 2023.