3GPP TSG RAN2 Meeting #127                                                               R2-2407312

19th Aug. – 23rd Aug. 2024, Maastricht, Netherlands

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

Among the WID[1], objectives related with RAN2 RRC CONNECTED mode are as follows:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode.

And, RAN1 made the below related summary at the meeting #117 w.r.t. various LP-WUS procedure options [4]:

Since RAN2 starts the discussion on Procedures for LP-WUS in RRC_CONNECTED from May RAN2 meeting, it would be better to wait for RAN2 progress before further down-selection among the options. Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any In general, it can be said that, - Case 1 - Option 1-1: small power saving gain, no latency reduction, small spec impact - Option 1-2: tradeoff between large power saving gain and large latency reduction, large spec impact - Companies have different understanding which options 1-2-1 or 1-2-2 have larger spec impact, especially on C-DRX related timers aspect, which should be discussed in RAN2 according to the previous RAN1 agreement - Option 1-3: small power saving gain, useful for traffic with jitter, small/medium spec impact As some companies also pointed out, there would be a number of interpretations on whether to reuse drx-onDurationTimer or introduce new timer(s) for PDCCH monitoring especially for Options 1-2-1/1-2-2 and Option 1-3, while this aspect should be discussed in RAN2 according to previous RAN1 agreement.

During the previous meeting #126, with respect to procedures for LP-WUS in RRC connected mode agenda, RAN2 agreed as follows:

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'. In light of this use case, RAN1 is currently engaged in the thorough examination and evaluation of three different options, each of which holds its unique advantages and potential challenges. These three options are being considered to determine their respective impacts on system design and operation:

    - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

      - This option may replace DCP functionality

    - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

      - PDCCH monitoring possibly irrespective of drx-onDurationTimer

        - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

          - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX

        - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

    - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

For a quick summary, we capture the each options examples from CATT’s paper [5]:

Option 1-1: Replacing DCP following legacy configurations

Figure 1: LP-WUS monitor procedure in Option 1-1

Option 1-2-1: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer (configured together with Option 1-1)

Figure 2: LP-WUS monitor procedure in Option 1-2-1

Option 1-2-2: UE performs LP-WUS monitoring and PDCCH monitoring IS NOT triggered based on legacy C-DRX cycle and drx-onDurationTimer

Figure 3: LP-WUS monitor procedure in Option 1-2-2

Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time

Figure 4: LP-WUS monitor procedure in Option 1-3

And RAN1 discussed pros and cons of each options are as follows: (without company names) [4]

|  |  | Pros | Cons |
| --- | --- | --- | --- |
| Case 1 | Option 1-1 | - Can be enabled in the same way as for Rel-16 DCP - Most parameters can follow legacy drx-config that provides a wide range of parameters, e.g., short duty cycle, short inactivity timer - Simple fallback behaviour to legacy C-DRX with some new LP-WUS exit criteria/trigger - significant power saving gain for FR2 | - no power saving gain brought by WUS during DRX active time - no traffic latency reduction outside DRX active time - shorter DRX cycle increases the MR power consumption for RLM/BM/BFD/RRM measurements |
|  | Option 1-2-1 (configured together with Option 1-1) | - Can enable more flexible triggering for PDCCH monitoring - Simple fallback behaviour to legacy C-DRX with some new LP-WUS exit criteria/trigger - Lower latency than Option 1-1 - Relax Legacy cDRX setting to reduce power consumption for ongoing MR measurements | - no power saving gain brought by WUS during DRX active time - same as Option 1-1 with shorter DRX cycle length configuration - require new mechanisms to make the new ‘floating’ PDCCH monitoring windows work, e.g., PDCCH monitoring termination conditions such as timers/signalling, relevant MAC procedure and PDCCH monitoring procedure |
|  | Option 1-2-2 | - Can enable more flexible triggering for PDCCH monitoring and power saving - Simple fallback behaviour to legacy C-DRX with some new LP-WUS exit criteria/trigger - keep the configurations of DRX cycle and DRX related timers and thus does not have impacts on the measurement - The impact of legacy C-DRX can be minimized - works standalone, can obtain more power saving and reduce latency - Relax Legacy cDRX setting to reduce power consumption for ongoing MR measurements | - Cause significant complexity and impacts RAN2 aspects - require new mechanisms to make the new ‘floating’ PDCCH monitoring windows work, e.g., PDCCH monitoring termination conditions such as timers/signalling, relevant MAC procedure and PDCCH monitoring procedure - Unclear measurement behaviors within the new PDCCH monitoring timer triggered by LP-WUS - similar performance can be achieved with Option 1-1 by configuring a shorter C-DRX cycle-length - UEs monitoring LP-WUS have to completely depend on LP-WUS for DL scheduling without any fallback option |
|  | Option 1-3 | - Can enable more flexible triggering for PDCCH monitoring starting from the middle of DRX Active Time for power saving - Relax Legacy cDRX setting to reduce power consumption for ongoing MR measurements | - Cannot reduce latency in case the traffic arrives outside the C-DRX active time - use case is limited to traffic with jitter, e.g., XR traffic, where the power saving gain of WUS is minor - Legacy cDRX sets upper bound on MR power consumption for ongoing measurements - Need to clarify the relationship between LP-WUS and C-DRX related timers - similar performance can be achieved with Option 1-1 by configuring a shorter C-DRX cycle-length |

Firstly, as a baseline approach, we suggest RAN2 to support the Option 1-1, configuration of LP-WUS prior to C-DRX activation, akin to the current DCP mechanism. By doing so, it is possible to maintain compatibility with existing standards while harnessing the benefits of LP-WUS for PDCCH monitoring during C-DRX active periods.

Proposal 1. As a baseline scenario, RAN2 supports LP-WUS configuration before C-DRX on duration, similar as DCP (option 1-1).

Regarding option 1-2-1 and option 1-3, it is understood that without the introduction of additional behaviors and specification changes, option 1-1 can achieve a similar effect through modifications to the C-DRX cycle and on-duration timer.

For instance, implementing option 1-2-1 mandates designing an additional PDCCH monitoring window outside the DRX onduration, which imposes a substantial specification burden. Nevertheless, by leveraging a shorter DRX cycle in option 1-1, the UE can obtain comparable latency and power gains.

Proposal 2. Propose RAN2 to rule out Option 1-2-1 since the similar gain can be achieved with Option 1-1 having modified DRX cycle and on-duration.

In the case of option 1-3, if the on-duration is short, then the use of LP-WUR/WUS appears to be redundant. By employing a short on-duration and a longer drx-cycle, option 1-1 can effectively replicate this scenario without requiring significant modifications to the specifications.

Proposal 3. Propose RAN2 to rule out Option 1-3 since the similar gain can be achieved with Option 1-1 having modified DRX cycle and on-duration.

Now, let's consider the remaining option 1-2-2, which was not addressed through the modification of option 1-1. As far as we understand, option 1-2-2 provides greater flexibility and does not require alignment with the DRX cycle. To implement this option, we must disregard (or deactivate) the DRX mechanism when LP-WUS monitoring is enabled. This will require modifying the specifications to design the PDCCH monitoring duration, including timers and extensions. Additionally, both the gNB and UE must coordinate their behaviors regarding whether the LP-WUS monitoring of the UE is active or not. Nonetheless, we have questions about whether the power saving benefits of option 1-2-2 would truly be significant compared to options 1-1 or 1-2-1.

Proposal 4. Propose RAN2 to discuss the required spec change for Option 1-2-2 such as

- How to ignore DRX when LP-WUS monitoring is activated

- How to restart DRX when LP-WUS monitoring is deactivated

- How to terminate triggered MR PDCCH monitoring

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. As a baseline scenario, RAN2 supports LP-WUS configuration before C-DRX on duration, similar as DCP (option 1-1).

Proposal 2. Propose RAN2 to rule out Option 1-2-1 since the similar gain can be achieved with Option 1-1 having modified DRX cycle and on-duration.

Proposal 3. Propose RAN2 to rule out Option 1-3 since the similar gain can be achieved with Option 1-1 having modified DRX cycle and on-duration.

Proposal 4. Propose RAN2 to discuss the required spec change for Option 1-2-2 such as

- How to ignore DRX when LP-WUS monitoring is activated

- How to restart DRX when LP-WUS monitoring is deactivated

- How to terminate triggered MR PDCCH monitoring

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC

[2] 3GPP TR38.869 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on low-power wake up signal and receiver for NR (Release 18)

[3] Draft_Minutes_report_RAN1#116_v030, Draft Report of 3GPP TSG RAN WG1 #116 v0.3.0 (Athens, Greece, February 26th - March 1st, 2024), RAN1

[4] R1-2405721 FL summary #4 on LP-WUS operation in CONNECTED mode, NTT DOCOMO, RAN1

[5] R2-2404378 Analysis on LP-WUS in RRC_CONNECTED, CATT