3GPP RAN WG2 Meeting #127                                                     R2-2407351

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item:	8.5.3

Source:	HONOR

Title:	Further discussion on on-demand SIB1

Document for:	Discussion and Decision

1. Introduction

In RAN2#126, the following aspects were agreed to be specified in the on-demand SIB1 work item [1]:

Agreements on on-demand SIB1:

Further clarification

1.	Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

2.	RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Information of WUS configuration

3.	Can use the PCI and frequency of a NES Cell to associate the UL WUS configuration with a NES Cell.

4.	For Message 1 based on-demand SIB1 request, the on-demand SI request configuration that currently included in SIB1 may be used as the design baseline.

Transmission of WUS configuration

5.	Cell A’s SIB can be used to configure on-demand SIB1 related configuration for neighbour NES cells, e.g., via new SIB or the existing SIB.

Triggering condition of WUS transmission

6.	If the UE chooses the NES cell using legacy intra-F/inter-F cell re-selection procedure (as baseline), the UE triggers WUS transmission.

7.	After UE successfully receives OD-SIB1 for that NES Cell and if it is a suitable cell, UE camps in the NES Cell “similar” to a legacy Cell.

UE request SIB1 to perform initial access in NES cell

8.	RAN2 not to support on-demand SIB1 request that is combined with an initial access to perform RRC connection establishment/resume on the NES cell.

Legacy UE impact

9.	NW need to bar the legacy UE from accessing the on-demand SIB1 cell (e.g. based on the existing barring mechanism)

10.	How to avoid/deprioritize the legacy UE camping at the Cell A attempting to switch to the NES Cell (but allowing the R19 NES UE to do that).

Barring for R19 NES UE

11.	If the NES UE is unable to acquire the SIB1 of NES Cell before UE initiates OD-SIB1 procedure, it will not consider it as barred at that moment. R19 NES UE bars the cell if the UE fails to acquire SIB1 via on-demand SIB1 for NES cell.

Paging and SIB1 update in NES cell

12.	After Rel-19 NES UEs camp in NES cell, the UE behaviour is same as the one defined as legacy normal camped state, e.g. paging reception, SIB1 update, etc.

RAR monitoring for MSG1 based OD-SIB1 REQ

13.	RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for Msg1-based on-demand SIB1 procedure, as the baseline.

UE behaviour on RACH failure of OD-SIB1 REQ

14.	As baseline, upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation.

SIB1 acquisition upon SIB change notification in NES cell

15.	Once the NES UE camps on the NES cell, if the UE receives SIB change notification, the UE is expected to receive SIB1 from NES cell.

Scenario 3 (case 1 in RAN1)

16.	RAN2 to wait for RAN1’s progress whether to support scenario 3.

In this contribution, we will further discuss how to bar the legacy UE and the possible procedure to acquire SIB1 from NES cell in scenario 1a.

2 Discussion

2.1 Barring for Legacy UE

The UE will treat a cell as barred if it is so indicated in the MIB or SIB1 message as specified in TS 38.331[2] which includes:

- cellBarred (IE type: "barred"/"not barred") indicated in MIB message.

- cellBarredxxx (IE type: "barred" or "barred/not barred") indicated in SIB1 message, where the xxx could be NTN, ATG, Redcap, NES and so on.

Take the nes-CellDTX-DRX capable UE as an example, the UE will consider the cell as barred according to Section 5.2.2.4.2 of TS 38.331[2]:

Observation 1: "barred" in MIB could bar some legacy UEs.  The other legacy UEs will acquire SIB1 and then decide whether the cell is barred.

In NES cell, the SIB1 is not broadcasting periodically. Legacy UE will treat the cell as barred if it is unable to acquire the SIB1 as specified in section 5.3.1 of TS 38.304 [3] which is also highlighted in green in the captured text below.

Thus, the legacy UE will treat the NES cell as barred and reselect to the other cells if cellBarred in MIB setting to "barred" and SIB1 is not available.

Observation 2: If the UE is unable to acquire the SIB1, the cell will be treated as barred for 300 seconds and the UE will select another cell that could fulfil the selection criteria.

To bar the legacy UE from accessing NES cell, it is proposed to set cellBarred in MIB to "barred" in the NES cell.

Proposal 1: In the NES cell, cellBarred in MIB should be set to "barred" to avoid legacy UE camping.

There are still some legacy UEs, who need to check cell status through SIB1, could camp on the NES cell if SIB1 is broadcasting like usual (e.g. on receiving UL-WUS from NES capable UE) when they enter the NES cell.

Observation 3: It is possible for some legacy UEs, who need to acquire SIB1 to confirm the cell status, to camp on the NES cell in case of occasionally SIB1 acquisition, i.e. the on-demand SIB1 is broadcasting on receiving UL-WUS from NES capable UEs.

As specified, the UE shall not consider any exclude-listed cells as candidate for cell reselection. One possible solution to further bar the legacy UE is that the Cell A could include the NES Cell in the excluded list and the legacy UE will not re-select to the NES cell from Cell A. The camping for occasionally SIB1 acquisition will not happen.

Meanwhile, legacy UE will regularly search for a better cell according to the neighbor cell configurations and cell reselection criteria. If the NES cell is the exclude-listed cell, legacy UE will not treat it as the candidate cell for ranking which will avoid the rejection for re-selection.

Proposal 2:  RAN2 to discuss that Cell A includes the NES cell in the excluded list to further bar the legacy UE.

For initial cell selection, the legacy UE will still be able to select the NES cell as there is no Cell A assistance information. However, these UEs will perform uplink data transmission, e.g registration, and then perform reselection to the NES cell with SIB1 unavailable or non-NES cell.

2.2 Selection or Reselection from Cell A

As agreed, R19 NES UE will bar the cell if the UE fails to acquire SIB1 after sending UL-WUS to NES cell. Firstly, the NES UE should know whether the cell is a NES cell that could broadcast SIB1 according to UL-WUS to avoid unnecessary request.

If the UE performs reselection from Cell A, Cell A will indicate whether the cell is a NES cell, e.g. through UL-WUS configuration including ARFCN/PCI. Then, the NES UE could initiate RA procedure for SIB1 after reselection from Cell A.

Observation 4: In cell re-selection case, the NES UE could know whether the cell is a NES cell via UL-WUS configuration of Cell A if the UE performs cell reselection from Cell A to the NES cell.

For initial cell selection, the UE can’t identify the NES cell without assistance information from Cell A. It is confusion whether to send UL-WUS or try to acquire SIB1 in the position MIB indicated. The blind detection may introduce reselection delay and false judgement as the UE could not differentiate SIB1 miss-detection or SIB1 not broadcasting.

Observation 5: In cell selection case, the NES UE could not be able to differentiate SIB1 miss-detection or SIB1 not broadcasting, which will introduce selection delay.

It is quite necessary to indicate the NES cell capability. Generally, the cell capability will be indicated in SIB1 which can’t be applicable to on-demand SIB1 case. As MIB is broadcasting, it is possible to indicate the NES capability in MIB, e.g. indication in MIB or special configuration of MIB.

Proposal 3: The UE should know whether the cell is a NES cell to decide whether to initiate SIB1 request. And RAN2 to study how to indicate the cell capability via MIB for UEs perform the initial cell selection.

Before triggering SIB1 acquisition. it is possible that SIB1 is broadcasting on receiving request from previous UE and the UE could acquire SIB1 directly. Or SIB1 is not broadcasting and the UE needs to trigger UL-WUS to ask for SIB1. In both instances, the UE should detect SIB1 broadcasting status firstly which will introduce camping delay. If there are data packets waiting for transmission or reception, the user experience will be impacted.

So, it is proposed that the UE should not detect SIB1 broadcasting status before triggering UL-WUS for SIB1, especially there is UL/DL data for transmission.

Proposal 4: For selection/reselection scenario, the UE should trigger the UL-WUS for SIB1 immediately without determining SIB1 broadcasting status, especially there is UL/DL data for transmission.

As baseline, upon random access procedure failure of SIB1 request, UE regards SIB1 can’t be acquired in the NES cell.  For MSG1-based SIB1 request, the random access procedure failure includes:

- The NES UE doesn’t receive the RAR after sending preamble.

RAN2 assumes the UE is expected to receive the RAR responding to the preamble transmission for MSG1-based SIB1 request procedure, as the baseline.  The UE could detect the RAR after sending the preamble to decide whether the procedure is successful.

Observation 6: For MSG1-based SIB1 request, the UE will consider it as SIB1 acquisition failure/cell barred if the UE doesn’t receive the RAR responding corresponding to the SIB1 request preamble.

In OD-OSI, the UE is permitted to perform the preamble transmission for several times until achieve maximum number of random access Preamble transmission. In R18, preambleTransMax-Msg1-Repetition is introduced to further improve the request efficiency.  As SIB1 is essential for camping, the preambleTransMax and preambleTransMax-Msg1-Repetition should be used for SIB1 request to improve the successful rate.

Proposal 5: For MSG1-based SIB1 request, the UE should consider it as the SIB1 acquisition failure if there is no RAR responding received after several times preamble transmission with configurable repetition.

- The UE received the RAR and doesn’t acquire the SIB1 successfully.

After receiving RAR, the UE could acquire the SIB1 in the specified position. It is unclear how to identify the acquisition failure as the UE does not know the detect duration.

RAN1 agreed to consider monitoring occasions of SIB1 within a time window as depicted below, which could be helpful for SIB1 detection deterministically.

Observation 7: For MSG1-based SIB1 request, time window for SIB1 monitoring could help to decide the detection duration, which could reduce the impact to traffic or ETWS/CMAS reception.

RAN2 has taken RAR as the baseline, and the starting point of the time window could be receiving the RAR.

Proposal 6: For MSG1-based SIB1 request, RAN2 to discuss introducing time window for SIB1 transmission and the RAR responding for random access could be the starting point of the window.

For MSG3-based SIB1 request, there is no SI-RequestConfig assigned for SIB1 and a CCCH message is constructed to indicate the SIB1 request after receiving the RAR responding. Compared with MSG1-based solution, 4-step request will introduce additional delay and may occupy more space of Cell A to configure generic RACH configure information.

Proposal 7: MSG3-based SIB1 request should not be supported for UE selects NES cell through legacy intra-F/inter-F cell re-selection procedure, i.e. Cell A doesn’t need to configure UL-WUS configuration for MSG3-based SIB1 request.

2.3 Camping on NES Cell

UEs camping on NES cell may trigger UL-WUS to re-acquire SIB1 when the stored SIB1 is invalid, e.g. deleted after 3 hours. If the UL-WUS configuration has changed, the UE can’t request SIB1 using stored configuration in Cell A.

In this case, the UE needs to reselect to Cell A to get the latest UL-WUS configuration first and then go back to NES cell for SIB1. This will introduce unnecessary delay. Thus, the UL-WUS configuration should be sent in the NES cell as well to avoid NES UE camping on NES cell to maintain the UL-WUS configuration from Cell A.

Proposal 8: The UL-WUS configuration should be sent in the NES cell for NES capable UE to re-acquire SIB1 with updated UL-WUS configuration.

In NES cell, the UE needs to trigger lower layers to perform random access procedure if SIB(s) in SI message is invalid and the status is “notbroadcasting”. If the SIB1 is in invalid at the same time, the UE needs to perform two separate request procedures.

To improve the acquisition efficiency, one request procedure for both SIB1 and SI message(s) should be considered. As MSG1-based request can only request one SI message for one time or all SI messages, MSG3-based request is a more suitable option for request SIB1 and required SI message(s) via one random access procedure, where the required SIB1 and SI message(s) can be indicated in RRCSystemInfoRequest message.

Proposal 9: In NES cell, the MSG3-based request should be adopted to support requesting SIB1 and SI message(s) simultaneously.

For on-demand OSI, the UE should decide whether and how to initiate the SI message request procedure according to SchedulingInfo/si-BroadcastStatus and si-RequestConfig/si-RequestResources configured in SIB1. And the change of the si-BroadcastStatus and si-RequestResources will not result in system information change notification.

Therefore, the UE should re-acquire SIB1 before performing MSG1-based request to get PRACH resource(s) in si-RequestConfig corresponding to the SI message(s) that the UE requires to operate within the cell, and for which si-BroadcastStatus is set to notBroadcasting. That is two times random access procedure for SI message(s) request: one for SIB1 and one for the required SI message(s).

Observation8: In NES cell, the UE needs to acquire SIB1 through random access procedure at first and then initial random access for SI message(s) that the UE wants to acquire to avoid unnecessary OSI request.

Proposal 10: The UE should trigger the UL-WUS for SIB1 before initiate the SI request procedure.

For NES UE camping on the NES cell, SIB change notification will be sent in the NES cell for SIB1 or OSI modification. Considering the cell load is quite low and less UE will camp on the Cell, it is proposed to not broadcast the modified SIB1/OSI until there is request from the UE to enjoy more energy saving gains.

Proposal 11: RAN2 to evaluate whether modified SIB1/OSI broadcasted after change notification or waiting for the UL-WUS requesting the modified SIB1/OSI.

3 Conclusions

In this contribution, we discussed the detailed on-demand SIB1 procedure, and made the following observations and proposals:

Observation 1: "barred" in MIB could bar some legacy UEs.  The other legacy UEs will acquire SIB1 and then decide whether the cell is barred.

Observation 2: If the UE is unable to acquire the SIB1, the cell will be treated as barred for 300 seconds and the UE will select another cell that could fulfil the selection criteria.

Observation 3: It is possible for some legacy UEs, who need to acquire SIB1 to confirm the cell status, to camp on the NES cell in case of occasionally SIB1 acquisition, i.e. the on-demand SIB1 is broadcasting on receiving UL-WUS from NES capable UEs.

Observation 4: In cell re-selection case, the NES UE could know whether the cell is a NES cell via UL-WUS configuration of Cell A if the UE performs cell reselection from Cell A to the NES cell.

Observation 5: In cell selection case, the NES UE could not be able to differentiate SIB1 miss-detection or SIB1 not broadcasting, which will introduce selection delay.

Observation 6: For MSG1-based SIB1 request, the UE will consider it as SIB1 acquisition failure/cell barred if the UE doesn’t receive the RAR responding corresponding to the SIB1 request preamble.

Observation 7: For MSG1-based SIB1 request, time window for SIB1 monitoring could help to decide the detection duration, which could reduce the impact to traffic or ETWS/CMAS reception.

Observation8: In NES cell, the UE needs to acquire SIB1 through random access procedure at first and then initial random access for SI message(s) that the UE wants to acquire to avoid unnecessary OSI request.

Barring for Legacy UE

Proposal 1: In the NES cell, cellBarred in MIB should be set to "barred" to avoid legacy UE camping.

Proposal 2:  RAN2 to discuss that Cell A includes the NES cell in the excluded list to further bar the legacy UE.

Selection or Reselection from Cell A

Proposal 3: The UE should know whether the cell is a NES cell to decide whether to initiate SIB1 request. And RAN2 to study how to indicate the cell capability via MIB for UEs perform the initial cell selection.

Proposal 4: For selection/reselection scenario, the UE should trigger the UL-WUS for SIB1 immediately without determining SIB1 broadcasting status, especially there is UL/DL data for transmission.

Proposal 5: For MSG1-based SIB1 request, the UE should consider it as the SIB1 acquisition failure if there is no RAR responding received after several times preamble transmission with configurable repetition.

Proposal 6: For MSG1-based SIB1 request, RAN2 to discuss introducing time window for SIB1 transmission and the RAR responding for random access could be the starting point of the window.

Proposal 7: MSG3-based SIB1 request should not be supported for UE selects NES cell through legacy intra-F/inter-F cell re-selection procedure, i.e. Cell A doesn’t need to configure UL-WUS configuration for MSG3-based SIB1 request.

Camping on NES Cell

Proposal 8: The UL-WUS configuration should be sent in the NES cell for NES capable UE to re-acquire SIB1 with updated UL-WUS configuration.

Proposal 9: In NES cell, the MSG3-based request should be adopted to support requesting SIB1 and SI message(s) simultaneously.

Proposal 10: The UE should trigger the UL-WUS for SIB1 before initiate the SI request procedure.

Proposal 11: RAN2 to evaluate whether modified SIB1/OSI broadcasted after change notification or waiting for the UL-WUS requesting the modified SIB1/OSI.

4. References

[1]	R2-24xxxx    Report of 3GPP TSG RAN WG2 meeting #126

[2]	TS 38.331-v18.2.0, Radio Resource Control (RRC) protocol specification, 2024-07.

[3]	TS 38.304-v18.2.0, User Equipment (UE) procedures in Idle mode and RRC Inactive state, 2024-07.