3GPP TSG-RAN WG2 Meeting #127                                                                   R2-2407385

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda Item	:	8.2.4

Title	:	Discussion on random access for ambient IoT

Source	:	Google Inc.

Work Item	:	FS_Ambient_IoT_solutions

Document for	:	Discussion and Decision

# 1		Introduction

In this paper, we discuss the 3-step CBRA and the contents of the Msg1, Msg2, and Msg3.

# 2	Discussion on 3-step CBRA

In the RAN2 #126 meeting, it was agreed that the AIoT device sends a random ID to the reader in Msg1. The reader echoes the ID in Msg2. The Msg1 is used to contend for channel access with other devices so the size of Msg1 should not be large. Otherwise, the channel utilisation will be low due to longer collision time. If the device needs to transmit specific information to the reader, those information can be conveyed in Msg3.

Proposal 1: Msg1 contains only the random ID generated by the device.

In addition to the random ID that the reader echoes in Msg2, the reader can also allocate a RNTI to the device in Msg2. In the last meeting, RAN2 agreed to study several cases for paging message. One of the cases is that a paging message contains an ID of a single device.

The ID in the paging message can be the RNTI that is allocated in Msg2. With the RNTI, the reader can page the device afterward. For example, if the reader does not receive the Msg3, the reader can page the device with the RNTI to retransmit the Msg3.

Proposal 2: Msg2 contains a RNTI for the device.

The reader can also send a command to the device in Msg2. For example, the command can be an authentication command or a request to report data. It is FFS what commands should be supported.

If the device receives a command, the device may need to transmit a response or feedback to the reader. Therefore the Msg2 may also need to provide a resource allocation for the device to transmit Msg3.

Proposal 3: Msg2 contains a command for the device and a resource allocation for Msg3 transmission.

When the device receives the Msg2, the device transmits a Msg3 to the reader. The Msg3 contains a response to the command in Msg2. If Msg2 contains a command requesting the device to perform authentication procedure, the device should transmit authentication information to the reader in Msg3.

Proposal 4: Msg3 contains a response to the command received in Msg2.

In addition to the response to command, the device can report buffer status/size or power status in Msg3. Those information helps the reader to allocate resources to the device.

Moreover, since there are different types of ambient IoT are defined in R19 ambient IoT. Each type of the devices may have different capabilities. The device can report its device type in Msg3.

It may happen that the size of a transport block is larger than a MAC PDU. In such a case, the device can insert padding bits in the MAC PDU.

Proposal 5: Msg3 can contain a buffer status, power status, or device capability information.

# 3		Conclusions

In conclusion, we have the following proposals:

Proposal 1: Msg1 contains only the random ID generated by the device.

Proposal 2: Msg2 contains a RNTI for the device.

Proposal 3: Msg2 contains a command for the device and a resource allocation for Msg3 transmission.

Proposal 4: Msg3 contains a response to the command received in Msg2.

Proposal 5: Msg3 can contain a buffer status, power status, or device capability information.