# 3GPP TSG RAN2#127     	                                                   R2-2407431

# Maastricht, Netherlands, 19th – 23th Aug , 2024

Agenda item:	5.1.2.1

Title:	Clarification on Rel-16 BFR and Rel-17 BFR

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

In RAN2#126 meeting, one Rel-17 issue related to the BFD-RS set specific BFR in [1] is raised which is caused by the DL BWP switch and it is postponed to the next meeting for companies to further consider such issue

R2-2405188	Clarification to 38.321 On BFD-RS set specific BFR	 ZTE Corporation	  CR  	Rel-17  	38.321	17.8.0 	1856	   F 	 NR_FeMIMO-Core

-	Vivo, oppo, not needed.  LG supports the intention

=>	The CR is postponed

In addition, one Rel-16 issue regarding the BFR is also disclosed in the paper. In this contribution we will share our views on both Rel-16 and Rel-17 issue.

Discussion

Rel-16 issue

For BFR on SCell, one scheduling request can be triggered by the lack of the available UL-SCH resources to accommodate the BFR MAC CE. The related description in SR subclause is shown as below:

The MAC entity shall for each pending SR not triggered according to the BSR procedure (clause 5.4.5) for a Serving Cell: 1> if this SR was triggered by Pre-emptive BSR procedure (see clause 5.4.7) prior to the MAC PDU assembly and a MAC PDU containing the relevant Pre-emptive BSR MAC CE is transmitted; or 1> if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and a MAC PDU is transmitted and this PDU includes a BFR MAC CE or a Truncated BFR MAC CE which contains beam failure recovery information for this SCell; or 1> if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and this SCell is deactivated (see clause 5.9); or 1> if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and a MAC PDU is transmitted and the MAC PDU includes an LBT failure MAC CE that indicates consistent LBT failure for this SCell; or 1> if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and all the triggered consistent LBT failure(s) for this SCell are cancelled: 2> cancel the pending SR and stop the corresponding sr-ProhibitTimer, if running.

According to the green highlighted wording, it can be seen that the pending SR is triggered and canceled on a basis of cell level. However, the sr-ProhibitTimer is maintained per SR configuration according to the yellow highlighted wording, besides, the SR configuration for BFR and LBT is configured per MAC entity, which means the sr-ProhibitTimer is handled in a granularity of MAC entity.

RRC configures the following parameters for the scheduling request procedure: - sr-ProhibitTimer (per SR configuration); - sr-TransMax (per SR configuration).

As a result, the cancellation of one pending SR triggered by BFR for an SCell will stop sr-ProhibitTimer for a SR configuration. And hence, the pending SR of the same SR configuration for another serving cell where BFR is triggered still can be transmitted even though the sr-ProhibitTimer is pretended to be running and SR transmission should be prohibit, which violates the basic intention of introducing sr-ProhibitTimer.

According to the current MAC specification, the cancellation of one pending SR triggered by BFR for an SCell will stop sr-ProhibitTimer of an SR configuration, it results in that the pending SR for another SCell, if any, still can be instructed to the lower layer for the transmission which is pretended not to do like that.

We propose to fix the bug:

RAN2 clarifies that the sr-ProhibitTimer for the SR Configuration of BFR for SCell shall be stopped if all pending SRs for the SR configuration are canceled.

Rel-17 Issue

In Rel-17, the BFD-RS set specific BFR is introduced for recovering the beam of each TRP. In BFD-RS set specific BFR, for a DL BWP, two BFD-RS sets (i.e. failureDetectionSet1 , failureDetectionSet2) and two corresponding BFR candidate RS sets (i.e. candidateBeamRS-List-r16, candidateBeamRS-List2-r17) are configured, the beam failure triggered by BFD-RS set only can be recovered by the corresponding candidate beam RS set.

Two BFD-RS sets and two corresponding candidate Beam RS sets are configured per BWP, and beam failure triggered by a BFD-RS set only can be recovered by the corresponding candidate beam RS set according to the current MAC spec.

For a BFR triggered by BFD-RS set, the BFR will be cancelled by either receiving a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information of this BFD-RS set of the Serving Cell or sending the enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information of this BFD-RS set of the Serving Cell.

However, the yellow highlighted wording does not always work in some scenarios, for instance, for a serving cell, one BFR have been triggered by one BFD-RS set of DL BWP#1, and then the DL BWP have been switched into DL BWP#2,  but the BFR triggered by BFD-RS set of DL BWP#1 still keeps pending, the enhanced BFR MAC CE would be generated when there is valid UL grant is received from DL BWP#2, at this moment, there are two assumptions for UE to select the candidate beam RS for the enhanced BFR MAC CE:

- Assumption 1: the candidate beam RS from the beam RS list of DL BWP#1 would be selected.

- Assumption 2: the candidate beam RS from the beam RS list of DL BWP#2 would be selected.

In the assumption 1, UE select the candidate beam RS from the candidate beam RS list of DL BWP#1, the BFR for the BFD-RS set can be cancelled as long as the candidate beam RS information is sent out according to above yellow highlighted wording. However, it may provide the incorrect the beam information to the NW since NW always consider the received Beam RS in BFR MAC CE is from the candidate beam RS list of the current DL BWP but the actual candidate beam RS selected by UE is from the previous DL BWP, then the NW is misled by the beam information.

In the assumption 2, UE selects the candidate beam RS from the candidate beam RS list of DL BWP#2. , the BFR for the BFD-RS set cannot be cancelled since the candidate beam RS list is not corresponding to the BFD-RS set by which the BFR is triggered, then the BFR still keeps pending.

In the case where a BFR for a BFD-RS set is triggered before the DL BWP switch and the BFR MAC CE is generated after DL BWP switch, UE either provides the incorrect beam failure recovery information to mislead the NW (i.e. above assumption 1) or cannot cancel the pending BFR due to the candidate beam RS information is not from the corresponding candidate beam RS list (i.e. above assumption 2)

Solution for Rel-17 issue

For resolving above issues, the following options can be taken into account:

- Option 1: The BFR triggered by one BFD-RS set shall be cancelled if the corresponding DL BWP is deactivated.

- Option 2: As same as legacy UE behaviour for cell specific BFR, the BFR triggered by one BFD-RS set can be cancelled regardless of the candidate beam RS in the enhanced BFR MAC CE is from the corresponding candidate beam RS list or not.

In option 1, the BFR information for the BFD-RS set by which the BFR is triggered is no longer needed, it can totally resolve the issue raised in the first section.

In option 2, UE still provides the BFR information for a BFD-RS set even though the DL BWP of the BFD-RS set is deactivated. For avoiding misleading the NW due to the incorrect candidate beam RS information provided in the enhanced BFR MAC CE, it is suggested that the candidate beam RS selected by UE shall be from the candidate beam RS list of the current active DL BWP which can be guaranteed by UE implementation.

Compare between option 1 and option 2, option 1 may be a obvious NBC change, and option 2 is to reuse the legacy behaviour and it can be up to UE to guarantee to provide the correct candidate beam information for the triggered BFR (e.g. to provide the beam RS information, if suitable, from the candidate beam RS list of the current active BWP). In this sense, the option 2 does not cause either the NBC change or the NW will never be misled.

For a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, The BFR triggered by a BFD-RS set shall be cancelled if the candidate beam RS information for the BFD-RS set is contained in the enhanced BFR MAC CE.

For avoiding misleading NW with a incorrect candidate beam RS information, we propose to confirm this in RAN2 chairman note:

RAN2 confirms that, for a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, the candidate beam RS selected by UE, if any, in the enhanced BFR MAC CE is from the candidate beam RS list of the current active DL BWP.

For understanding easily, the TP for each option is provided in the 5th section Annex

Conclusion

In this contribution, we provide our views on BFD-RS set specific BFR. We have the following observations and proposals:

Observation 1: Two BFD-RS sets and two corresponding candidate Beam RS sets are configured per BWP, and beam failure triggered by a BFD-RS set only can be recovered by the corresponding candidate beam RS set according to the current MAC spec.

Observation 2: In the case where a BFR for a BFD-RS set is triggered before the DL BWP switch and the BFR MAC CE is generated after DL BWP switch, UE either provides the incorrect beam failure recovery information to mislead the NW (i.e. above assumption 1) or cannot cancel the pending BFR due to the candidate beam RS information is not from the corresponding candidate beam RS list (i.e. above assumption 2)

Proposal 1: RAN2 clarifies that the sr-ProhibitTimer for the SR Configuration of BFR shall be stopped if all pending SRs for the SR configuration are canceled.

Proposal 2: For a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, The BFR triggered by a BFD-RS set shall be cancelled if the candidate beam RS information for the BFD-RS set is contained in the enhanced BFR MAC CE.

Proposal 3: RAN2 confirms that, for a BFR triggered by a BFD-RS set of a DL BWP that is deactivated, the candidate beam RS selected by UE, if any, in the enhanced BFR MAC CE is from the candidate beam RS list of the current active DL BWP.

References

- R2-2405188 Clarification to 38.321 On BFD-RS set specific BFR

Annex

R16 Issue

5.4.4	  Scheduling Request

The Scheduling Request (SR) is used for requesting UL-SCH resources for new transmission.

The MAC entity may be configured with zero, one, or more SR configurations. An SR configuration consists of a set of PUCCH resources for SR across different BWPs and cells. For a logical channel or for SCell beam failure recovery (see clause 5.17) and for consistent LBT failure recovery (see clause 5.21), at most one PUCCH resource for SR is configured per BWP.

Each SR configuration corresponds to one or more logical channels and/or to SCell beam failure recovery and/or to consistent LBT failure recovery. Each logical channel, SCell beam failure recovery, and consistent LBT failure recovery, may be mapped to zero or one SR configuration, which is configured by RRC. The SR configuration of the logical channel that triggered a BSR (clause 5.4.5) or the SCell beam failure recovery or the consistent LBT failure recovery (clause 5.21) (if such a configuration exists) is considered as corresponding SR configuration for the triggered SR. Any SR configuration may be used for an SR triggered by Pre-emptive BSR (clause 5.4.7).

RRC configures the following parameters for the scheduling request procedure:

-	sr-ProhibitTimer (per SR configuration);

-	sr-TransMax (per SR configuration).

The following UE variables are used for the scheduling request procedure:

-	SR_COUNTER (per SR configuration).

If an SR is triggered and there are no other SRs pending corresponding to the same SR configuration, the MAC entity shall set the SR_COUNTER of the corresponding SR configuration to 0.

When an SR is triggered, it shall be considered as pending until it is cancelled.

All pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) prior to the MAC PDU assembly shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes a Long or Short BSR MAC CE which contains buffer status up to (and including) the last event that triggered a BSR (see clause 5.4.5) prior to the MAC PDU assembly. All pending SR(s) for BSR triggered according to the BSR procedure (clause 5.4.5) shall be cancelled and each respective sr-ProhibitTimer shall be stopped when the UL grant(s) can accommodate all pending data available for transmission.

The MAC entity shall for each pending SR not triggered according to the BSR procedure (clause 5.4.5) for a Serving Cell:

1>	if this SR was triggered by Pre-emptive BSR procedure (see clause 5.4.7) prior to the MAC PDU assembly and a MAC PDU containing the relevant Pre-emptive BSR MAC CE is transmitted; or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and a MAC PDU is transmitted and this PDU includes a BFR MAC CE or a Truncated BFR MAC CE which contains beam failure recovery information for this SCell; or

1>	if this SR was triggered by beam failure recovery (see clause 5.17) of an SCell and this SCell is deactivated (see clause 5.9); or

1>	if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and a MAC PDU is transmitted and the MAC PDU includes an LBT failure MAC CE that indicates consistent LBT failure for this SCell; or

1>	if this SR was triggered by consistent LBT failure recovery (see clause 5.21) of an SCell and all the triggered consistent LBT failure(s) for this SCell are cancelled:

2>	cancel the pending SR;

2>  if all pending SRs for an SR configuration are cancelled:

3> Stop the corresponding sr-ProhibitTimer, if running.

R17 issue

Beam Failure Detection and Recovery procedure

/*omit for short*/

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if the Serving Cell is configured with two BFD-RS sets:

2>	if beam failure instance indication for a BFD-RS set has been received from lower layers:

3>	start or restart the beamFailureDetectionTimer of the BFD-RS set;

3>	increment BFI_COUNTER of the BFD-RS set by 1;

3>	if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount:

4>	trigger a BFR for this BFD-RS set of the Serving Cell;

2>	if BFR is triggered for both BFD-RS sets of the SpCell and the Beam Failure Recovery procedure is not successfully completed for any of the BFD-RS sets:

3>	initiate a Random Access procedure (see clause 5.1) on the SpCell;

2>	if the Serving Cell is SpCell and the Random Access procedure initiated for beam failure recovery of both BFD-RS sets of SpCell is successfully completed (see clause 5.1):

3>	set BFI_COUNTER of each BFD-RS set of SpCell to 0.

3>	consider the Beam Failure Recovery procedure successfully completed.

2>	if the beamFailureDetectionTimer of this BFD-RS set expires; or

2>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers or by the BFD-RS Indication MAC CE associated with a BFD-RS set of the Serving Cell; or

2>	if the reference signal(s) associated with a BFD-RS set of the Serving Cell used for beam failure detection is changed:

3>	set BFI_COUNTER of the BFD-RS set to 0.

2>	if a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information for this BFD-RS set of the Serving Cell:

3>	set BFI_COUNTER of the BFD-RS set to 0;

3>	consider the Beam Failure Recovery procedure successfully completed for this BFD-RS set and cancel all the triggered BFRs of this BFD-RS set of the Serving Cell.

2>	if the Serving Cell is SCell and the SCell is deactivated as specified in clause 5.9:

3>	set BFI_COUNTER of each BFD-RS set of SCell to 0;

3>	consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs of all BFD-RS sets of the Serving Cell.

1>	else:

2>	if beam failure instance indication has been received from lower layers:

3>	start or restart the beamFailureDetectionTimer;

3>	increment BFI_COUNTER by 1;

3>	if BFI_COUNTER >= beamFailureInstanceMaxCount:

4>	if the Serving Cell is SCell:

5> 	trigger a BFR for this Serving Cell;

4>	else if the Serving Cell is PSCell and, the SCG is deactivated:

5> 	if beam failure of the PSCell has not been indicated to upper layers since the SCG was deactivated or since the deactivated SCG was last reconfigured with BFD-RS:

6> 	indicate beam failure of the PSCell to upper layers.

NOTE:	After beam failure is indicated to upper layers, the UE may stop the beamFailureDetectionTimer and lower layer beam failure indication while BFI_COUNTER >= beamFailureInstanceMaxCount for the deactivated SCG.

4>	else:

5>  	initiate a Random Access procedure (see clause 5.1) on the SpCell;

5>  	if beam failure is detected for an NCR-MT:

6>	  indicate to NCR-Fwd to cease forwarding.

2>	if the beamFailureDetectionTimer expires; or

2>	if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell; or

2>	if the reference signal(s) associated with this Serving Cell used for beam failure detection is changed:

3>	set BFI_COUNTER to 0.

2>	if the Serving Cell is SpCell and the Random Access procedure initiated for SpCell beam failure recovery is successfully completed (see clause 5.1):

3>	set BFI_COUNTER to 0;

3>	stop the beamFailureRecoveryTimer, if configured;

3>	if the Random Access procedure was triggered by beam failure recovery for NCR-MT:

4>	indicate to NCR-Fwd to resume forwarding using the last forwarding configuration received by NCR-MT as part of side control information before beam failure detection;

3>	consider the Beam Failure Recovery procedure successfully completed.

2>	else if the Serving Cell is SCell, and a PDCCH addressed to C-RNTI indicating uplink grant for a new transmission is received for the HARQ process used for the transmission of the MAC CE for BFR which contains beam failure recovery information of this Serving Cell; or

2>	if the SCell is deactivated as specified in clause 5.9:

3>	set BFI_COUNTER to 0;

3>	consider the Beam Failure Recovery procedure successfully completed and cancel all the triggered BFRs for this Serving Cell.

/*omit for short*/

All BFRs triggered for an SCell shall be cancelled when a MAC PDU is transmitted and this PDU includes a MAC CE for BFR which contains beam failure information of that SCell. All BFRs triggered for a BFD-RS set of a Serving Cell shall be cancelled when a MAC PDU is transmitted and this PDU includes an Enhanced BFR MAC CE or Truncated Enhanced BFR MAC CE which contains beam failure recovery information for that BFD-RS set of the Serving Cell.