3GPP TSG-RAN WG2 Meeting #127	R2-2407478

Maastricht, Netherlands, 19 – 23 August 2024

Agenda item:	8.6.2

Source:	Nokia

Title:	RRC Modelling for Inter-CU LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#126, following agreement was made:

RRC Configuration/structure 7. The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

Furthermore, there was a long discussion on whether to support single or multiple reference configuration. The discussion was summarized in the Chair Notes as below:

Single reference configuration as Rel-18 LTM (P3: R2-2404642: Apple, China Telecom) Proposal 3: A single reference config is used for inter-CU LTM, and this reference configuration is the same as Rel-18 reference configuration. RAN2 expects additions to the Rel-18 reference configuration structure for Rel-19 LTM. Multiple reference configuration, e.g. a reference configuration per CU (P5: R2-2404826: LG) Proposal 5. Multiple reference configurations are provided per CU for inter-CU LTM. [Ericsson]: Single reference configuration sounds simpler, but each CU may have different capabilities or default configurations. We need to see each solution’s pros and cons. [Rakuten]: Considering one CU covers many DUs, two reference configurations should be enough. [MediaTek]: Considering max 8 candidate cells, one reference configuration should be enough. [Qualcomm]: It is not easy to determine which one is better in the signalling overhead. It will be different case by case. [KDDI]: We don’t need coordinate with multiple CUs if we have multiple reference configurations, e.g. one reference per CU. [Huawei]: Don’t agree with KDDI. Even with single configuration, the source CU doesn’t need the coordination with others. [Nokia]: With the mixture of subsequent inter-CU and intra-CU LTM, a reference configuration per CU will reduce the signalling overhead in the end.

In this contribution, we discuss the above open issue on reference configuration and describe our proposal on RRC signaling structure and modeling for inter-CU LTM.

# 2	Discussion

In Rel. 18 for intra-CU LTM, we had an extensive discussion on RRC signaling structure. For example, whether to use RRC Reconfiguration or Cell Group Configuration for each candidate cell configuration. Furthermore, we had discussions on the placement of multiple IEs such as CSI Resource Configuration either outside LTM Candidate IE or inside LTM Candidate IE.  In Rel. 19 for inter-CU LTM, our preference is to minimize the discussions on these aspects and try to reuse the Rel. 18 RRC signaling structure as much as possible. This is also in accordance with the agreement below.

Agreement in RAN2#126: The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM.

The signaling structure and modelling for LTM designed in Rel. 18 allows for flexibility in the design of the reference and candidate configurations. As was highlighted in the discussions we had in RAN2#126, the usage of a single reference configuration sounds simpler, however it does not account for the fact that different CUs have different capabilities or default configurations. Such drawback becomes even more significant if we consider LTM deployments wherein different CUs belong to different vendors. In such cases, the resulting reference configuration would likely be very small, and the corresponding delta configurations would have large size, thus obfuscating the signaling gains that come from such structure. On the other hand, using multiple reference configurations pertaining to different CUs, can maximize the use of the Rel. 18 structure. At the same time, it also enables a modular RRC design which is both backward compatible and allows for adding more functionalities to the feature. Another advantage of this approach compared to the single reference configuration is that no inter-CU coordination and dependencies are required with separate reference configurations. The configurations of cells under a CU can be added/modified/released without impacting other configurations, i.e., excessive processing of RRC signaling is avoided.

Observation 1: From NW perspective, using multiple reference configurations pertaining to different CUs exhibits at least the following benefits.

- It maximizes the use of the already agreed Rel. 18 structure.

- It minimizes signaling overhead associated with sending the RRC configurations and the required inter-CU coordination.

- It enables modular RRC design which is not only backward compatible (reuses Rel. 18) but also forward compatible (i.e. enable more functionalities to be added later in a modular fashion)

- It mitigates potential inter-operability issues arising in multi-vendor deployments of LTM.

In addition to the benefits this brings for the NW-side, such an RRC signaling structure and modelling (incl. multiple reference configuration per CU) is also beneficial for the UE side. First, it allows for keeping the size of the delta configurations smaller than the case where there would be a single reference configuration, thus reducing the memory requirements associated with storing the LTM configurations. As an example, consider inter-vendor LTM deployments which have no IEs in common. If there would only be 1 reference configuration shared across candidate CUs, then the UE would need to store up to 8 (nearly) complete configurations, especially in case of multi-vendor scenarios. On the other hand, if each CU can have its own reference and delta configuration structure, the size of the candidate configurations can be reduced as the duplicated entries can be given as part of the reference configuration.

Observation 2: From UE perspective, using multiple reference configurations pertaining to different CUs exhibits at least the following benefits.

- Reduces the memory footprint of the LTM configurations.

- UE can do modular RRC processing which is more efficient and likely incurs lower RRC processing time.

Proposal 1: Multiple CU-specific reference configurations are supported for inter-CU LTM. The structure presented in Appendix A should be considered for Rel. 19 inter-CU LTM.

Based on the discussion that took place in RAN2#126, we note that the cases of single vs multiple reference configurations can both co-exist. As was discussed in [1], it is still possible to have 1 CU-specific reference configuration, and other LTM candidate configurations, which belong to other CUs, can be given as either complete configurations or delta over the source CU reference configuration. In our view, this would be the case where 1 CU-specific reference configuration is non-empty, and the others are empty.

Observation 3: Inter-CU LTM deployment with multiple CU-specific reference configurations does not preclude the case where there is only one reference configuration specific to one CU (i.e. source CU).

Therefore, in order to make progress on this topic, we propose that RAN2 agree on the support of multiple reference configuration which are generated by each CU and allow the possibility where each CU can decide whether to generate its own reference configuration or use source CU reference configuration.

Proposal 2: A flexible framework is designed which supports both single reference configuration and multiple reference configuration.

Furthermore, we believe that this should be each gNB decision (or each CU) decision whether to generate reference configuration or not.

Proposal 3: Decision on whether to generate reference configuration or not should be up to CU/gNB.

Based on this, we also propose the following:

Proposal 4: Send LS to RAN3 so that RAN3 can decide signaling to support the reference configuration per CU, including the possibility of using source CU reference configuration.

# 3	Conclusion

We have the following observations for the Inter-CU LTM RRC signaling structure:

Observation 1: From NW perspective, using multiple reference configurations pertaining to different CUs exhibits at least the following benefits.

- It maximizes the use of the already agreed Rel. 18 structure.

- It minimizes signaling overhead associated with sending the RRC configurations and the required inter-CU coordination.

- It enables modular RRC design which is not only backward compatible (reuses Rel. 18) but also forward compatible (i.e. enable more functionalities to be added later in a modular fashion)

- It mitigates potential inter-operability issues arising in multi-vendor deployments of LTM.

Observation 2: From UE perspective, using multiple reference configurations pertaining to different CUs exhibits at least the following benefits.

- Reduces the memory footprint of the LTM configurations.

- UE can do modular RRC processing which is more efficient and likely incurs lower RRC processing time.

Observation 3: Inter-CU LTM deployment with multiple CU-specific reference configurations does not preclude the case where there is only one reference configuration specific to one CU (i.e. source CU).

We have the following proposals:

Proposal 1: Multiple CU-specific reference configurations are supported for inter-CU LTM. The structure presented in Appendix A should be considered for Rel. 19 inter-CU LTM.

Proposal 2: A flexible framework is designed which supports both single reference configuration and multiple reference configuration.

Proposal 3: Decision on whether to generate reference configuration or not should be up to CU/gNB.

Proposal 4: Send LS to RAN3 so that RAN3 can decide signaling to support the reference configuration per CU, including the possibility of using source CU reference configuration.

# 4	References

[1] R2- 2404296 Further discussion on inter-CU LTM, MediaTek, RAN2#125bis, May 2024, Fukuoka, Japan

# 5	Appendix

The RRC signaling structure for multiple CU-specific reference configurations can be visualized as follows:

Figure 3: RRC signaling structure for Rel. 19 Inter-CU LTM

An implementation showing how the proposed Rel. 19 LTM RRC signaling structure encompasses the Rel. 18 LTM RRC signaling structure as baseline is shown below.

Note 1: New Rel. 19 IEs important for defining signaling structure are shown in yellow highlighted font.

Note 2: Additional Rel. 19 IEs can be added in the above structure. FFS.

Note 3: Need Codes are removed for easy reading.

------------------------------------------------------Start--------------------------------------------------------------

    - RRCReconfiguration

The RRCReconfiguration message is the command to modify an RRC connection. It may convey information for measurement configuration, mobility control, radio resource configuration (including RBs, MAC main configuration and physical channel configuration) and AS security configuration.

Signalling radio bearer: SRB1 or SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCReconfiguration message

-- ASN1START

-- TAG-RRCRECONFIGURATION-START

RRCReconfiguration ::=                  SEQUENCE {

rrc-TransactionIdentifier               RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfiguration                      RRCReconfiguration-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfiguration-v19xy-IEs ::=        SEQUENCE {

ltm-Config-r19                          SetupRelease {LTM-Config-r19}

nonCriticalExtension                    SEQUENCE {}

}

}

-- TAG-RRCRECONFIGURATION-STOP

-- ASN1STOP

    - LTM-Config

The IE LTM-Config is used to provide LTM candidate configurations groups.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r19 ::=   SEQUENCE {

ltm-ConfigGroupToAddModList-r19		SEQUENCE(SIZE (1…maxNrofLTM-Configs-r19)) of LTM-ConfigGroup-r19;

ltm-ConfigGroupToReleaseList-r19		SEQUENCE(SIZE (1…maxNrofLTM-Configs-r19)) of LTM-ConfigGroupId-r19;

...

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

    - LTM-ConfigGroup

The IE LTM-ConfigGroup is used to provide LTM candidate configurations and Reference configurations.

LTM-ConfigGroup information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-ConfigGroup-r19 ::=   SEQUENCE {

Ltm-ConfigGroupId-r19				  LTM-ConfigGroupId-r19;

ltm-ReferenceConfiguration-r19        SetupRelease {ReferenceConfiguration-r18}

ltm-CandidateToReleaseList-r19        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r19)) OF LTM-CandidateId-r18

ltm-CandidateToAddModList-r19         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r19)) OF LTM-Candidate-r18

ltm-CSI-ResourceConfigToAddModList-r19         SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r19)) OF LTM-CSI-ResourceConfig-r18

ltm-CSI-ResourceConfigToReleaseList-r19        SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r19)) OF LTM-CSI-ResourceConfigId-r18

...

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

    - LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId

ltm-SSB-Config-r18                             LTM-SSB-Config-r18

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)

ltm-ConfigComplete-r18                         ENUMERATED {true}

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)

ltm-TCI-Info-r18                               LTM-TCI-Info-r18

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)

...

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1}

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)

...

}

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

----------------------------------------------------------------------------Stop------------------------------------------------------------