3GPP TSG-RAN WG2 Meeting #127											R2-2407485

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 			CEWiT

Title: 	Discussion on Functionality-based LCM of UE-sided Model for Beam Management Use Cases

Agenda item:    	8.1.2.2

Document for: 		Discussion and Decision

# Introduction

In Rel 18 the application of AI/ML techniques to NR air interface has been studied and captured in TR 38.843 [1]. Based on this normative phase is agreed in Rel 19 with following objective.

Provide specification support for the following aspects:

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

- Beam management – DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]:

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”)

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”)

  - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases if any

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE

NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

    - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning

  - AI/ML assisted positioning

    - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning

    - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning

  - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any

  - Investigate and specify the necessary signalling of necessary measurement enhancements (if any)

  - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, LCM-related aspects of UE-sided models for beam management use cases are proposed to continue the discussion in RAN 2.

# Discussion

The following agreements were made during RAN2#125-bis and RAN2#126 meetings on LCM for the UE-sided model.

Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to.

Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability.

Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP 	message.

Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality.

FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed).

For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach).

“UE-autonomous, 	UE’s decision is not reported to the network” is not considered for Rel-19.

RAN2 will support functionality activation/deactivation after inference configuration. FFS initial state of configuration and how activation/deactivation is achieved. FFS what Deactivation refers to: examples discussed: 1) fallback to legacy 2) switching, etc

We will work offline on the definitions for functionality types and define what is availability.

The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable. For a functionality to be applicable at least there should at least one model available within it. FFS other details on what is applicability/non-applicability.

For NW-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning

For BM use case, as a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality. FFS reactive

## Functionality Definition and Identification

According to TR 38.843, the functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. The UE capability message shall indicate the list of supported functionalities by the UE. The supported functionalities can be indicated with a functionality identifier. Whether the network or the UE assigns the functionality ID can be discussed further.

Configured functionalities refer to the functionalities that the network has configured to the UE. However, getting configured by the network doesn’t imply that the corresponding functionalities are applicable. The network is expected to maintain the corresponding functionality identifiers while configuring the respective functionalities.

Applicable functionalities refer to the functionalities that the UE can use for inference. Applicable functionalities can be considered as candidates for functionality activation/switching/deactivation. Applicable functionality is determined based on the scenarios, locations, configuration, and deployments, among other factors, and the UE reports it to the network either reactively or proactively. In reactive reporting of UE-sided applicable functionality(es), the network may request for the applicable functionality(es) and the UE may provide the relevant information in response via UAI message. In proactive reporting, the UE may report its applicable AI/ML functionalities unsolicited via UAI message. While the UE reports the applicable functionalities, it is expected to use the corresponding functionality identifiers to maintain consistency.

Applicable functionalities are a subset of the configured functionalities. Suppose a UE is configured with functionalities that are not applicable under some conditions but may be applicable under some other conditions. In that case, the functionality activating/switching/deactivating feature can be seamlessly performed using the configured functionality list. The UE need not wait for the configurations from the network to activate the newly applicable functionality. However, it is up to the network whether the configuration shall be provided to the UE before determining the applicable functionalities. In such scenarios, the applicable functionalities and the configured functionalities would be the same.

Activated functionalities refer to the functionalities that the UE is using for inference. Activated functionalities are a subset of applicable functionalities. The UE shall indicate the currently activated functionalities to the network and if any switching/deactivation is performed, it shall be informed to the network. The corresponding functionality identifiers are used for any reporting or configurations.

The model availability is not mandated until the stage of inference. Once the applicable functionalities are decided at the UE end, the UE can opt for means to obtain the required model(s) from the OTT server, gNB, CN, or OAM, if not already available with the UE. If UE has multiple applicable models available or transferred from external entities, it can decide which one to use for inference.

Available functionalities refer to the functionalities for which the UE already has model(s) with it. A model available does not imply it is configured/applicable/activated. It shall just indicate the respective functionality is supported but not vice versa. If a functionality has multiple models, model identifiers are used to uniquely identify them. Similarly, if a model can be associated with multiple functionalities, functionality identifiers can be used.

Proposal 1: The UE capability message shall indicate the list of supported functionalities by the UE.

Proposal 2: The supported functionalities can be indicated with a functionality identifier. FFS who assigns/manages the functionality IDs.

Proposal 3: Configured functionalities refer to the functionalities that the gNB/LMF has configured to the UE.

Proposal 4: Applicable functionalities refer to the functionalities that the UE can use for inference.

Proposal 5: Applicable functionalities can be considered as candidates for functionality activation/switching/deactivation.

Proposal 6: Applicable functionalities are a subset of the configured functionalities.

Proposal 7: Activated functionalities refer to the functionalities that the UE is using for inference.

Proposal 8: Available functionalities refer to the functionalities for which the UE already has model(s) with it.

## Additional Conditions

Additional conditions are considered parameters that may impact a functionality's applicability. The additional conditions can be NW-side and UE-side. It is expected that the applicable functionality will be aligned with both NW- and UE-side additional conditions. For the UE-sided model, the network shall provide the NW-side additional conditions to the UE so that the UE can decide upon the applicable functionalities, aligning with UE-side additional conditions that are internally available with the UE. The network shall use RRCReconfiguration message to signal the NW-side additional conditions.

The additional conditions of the neighboring cells can be optionally included with the NW-side additional conditions of the serving cell. It is up to the UE implementation whether to consider the neighboring cell’s applicability while determining the applicable functionalities for itself.

Further details on the contents of the NW-side additional conditions and how to represent the additional conditions shall await RAN1’s progress.

Proposal 9: The additional conditions of the neighboring cells can be optionally included with the NW-side additional conditions of the serving cell.

Proposal 10: RAN2 shall wait for RAN1's progress in determining what the NW-side additional conditions are and how the additional conditions are represented.

## Reporting of Applicable Functionalities

For a UE-sided model, the UE determines the applicable functionality from the list of supported/configured functionalities, based on UE-side and NW-side additional conditions. The network may or may not configure a functionality before it is considered applicable. However, the network can configure the UE to report applicable functionality using the RRC reconfiguration message. The UE shall send the applicable functionalities to the network via UAI or RRC reconfiguration complete message.

The UE may also send the applicable functionality report if there is a change in the previous applicability condition. In such cases, the network may or may not update the configurations on receiving the applicability report from the UE. If the configurations are already provided to the UE (configured functionalities) and there is no change in NW-side additional conditions, the UE can switch to a newly applicable functionality from the configured functionalities list.

The model availability information shall be shared by the UE mandatorily in the applicability report, if not already indicated prior. If the model is available at the UE, it can indicate to the network with the corresponding model ID and the network shall configure the respective applicable functionality for inference. If the model is unavailable in the UE, the UE can look for means to obtain the model via OTT server or can request for model transfer to the network in the applicability report. The network can assist the UE in obtaining the model, if required.

The activated functionalities are decided at the UE after obtaining the inference configurations from the network based on the applicability report. If there are multiple models available for a particular applicable functionality, the UE can decide which model to use based on its requirements. Similarly, if there are multiple functionalities associated with one model, the UE can decide which functionality to activate. The UE may optionally report the current activated functionality to the network.

It should be considered that a particular supported/configured functionality that may be non-applicable at some point in time can be applicable when there is a change in the applicability conditions due to additional conditions or feedback from performance monitoring metrics. The applicability report from the UE can optionally include the non-applicable but supported/configured functionalities.

Proposal 11: The UE shall send the applicable functionalities to the network via UAI or RRC reconfiguration complete message. The UE may also send the applicable functionality report if there is a change in the previous applicability condition.

Proposal 12: The model availability information shall be shared by the UE mandatorily in the applicability report, if not already indicated prior.

Proposal 13: The activated functionalities are decided at the UE after obtaining the inference configurations from the network based on the applicability report.

Proposal 14: The UE may optionally report the current activated functionality to the network.

Proposal 15: The applicability report from the UE can optionally include the non-applicable but supported/configured functionalities.

# Conclusion

This contribution provides the following proposals for functionality-based LCM of UE-sided model for the beam management use case.

Proposal 1: The UE capability message shall indicate the list of supported functionalities by the UE.

Proposal 2: The supported functionalities can be indicated with a functionality identifier. FFS who assigns/manages the functionality IDs.

Proposal 3: Configured functionalities refer to the functionalities that the gNB/LMF has configured to the UE.

Proposal 4: Applicable functionalities refer to the functionalities that the UE can use for inference.

Proposal 5: Applicable functionalities can be considered as candidates for functionality activation/switching/deactivation.

Proposal 6: Applicable functionalities are a subset of the configured functionalities.

Proposal 7: Activated functionalities refer to the functionalities that the UE is using for inference.

Proposal 8: Available functionalities refer to the functionalities for which the UE already has model(s) with it.

Proposal 9: The additional conditions of the neighboring cells can be optionally included with the NW-side additional conditions of the serving cell.

Proposal 10: RAN2 shall wait for RAN1's progress in determining what the NW-side additional conditions are and how the additional conditions are represented.

Proposal 11: The UE shall send the applicable functionalities to the network via UAI or RRC reconfiguration complete message. The UE may also send the applicable functionality report if there is a change in the previous applicability condition.

Proposal 12: The model availability information shall be shared by the UE mandatorily in the applicability report, if not already indicated prior.

Proposal 13: The activated functionalities are decided at the UE after obtaining the inference configurations from the network based on the applicability report.

Proposal 14: The UE may optionally report the current activated functionality to the network.

Proposal 15: The applicability report from the UE can optionally include the non-applicable but supported/configured functionalities.

# Reference

[1] TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”

[2] RAN2#126 Chair Notes

[3] Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 1

[4] R2-2406381, “Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2”