3GPP TSG-RAN WG2 #127	R2-2407491

Maastricht, Nederland, 19th August - 23rd August 2024

Source:                    	DENSO CORPORATION

Title:  	Consideration on S&F operation

Document for:        	Discussion and decision

Agenda Item:         	8.9.2	Support of Store & Forward

- Introduction

In Rel-19, the objectives for Store & Forward operation of IoT-NTN are described in the WID of Rel-19 IoT-NTN [1] as follows:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

Based on the chair notes for last meeting RAN2#126[2], the following agreements are made for S&F operation:

Agreements:

1.	For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite):

1) The UE sends uplink data signalling to eNB when service link is available and the eNB stores it.

2) When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN.

3) The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available).

4) The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again

2.	S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed

3.	RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

This paper attempts to investigate about consideration on S&F operation based on the above objectives and agreements.

- Discussion

As mentioned above, it was agreed that eNB provides “S&F indication” by using SIB. Additionally, it was discussed to consider cellBarred (new or legacy) as the S&F indication, however, signaling details for S&F indication was not concluded. For the signaling details, we think S&F related parameters (e.g., estimated delivery time) could be included in SIB as the S&F indication, as mentioned in a paper in last meeting [3].

Proposal 1:  S&F related parameter(s) could be included in SIB as the “S&F indication”

Furthermore, we agree with the benefit of indication for the estimated delivery time, as some papers mentioned in the last meeting. It would be helpful for UE to judge if the expected delay can match to UE’s requirement, since the delay on S&F operation could be significantly longer than the delay expected on the legacy delay-tolerant communication.

Proposal 2:  Estimated delivery time could be indicated to UE

However, UE may attempt to send UL data even though the estimated delivery time is longer than UE’s tolerable delay, since the estimated delivery time could be the worst case and actual delivery time could be shorter. Of course, if the estimated delivery time is not introduced (or, not indicated in case it’s optional information), UE would attempt to send UL data anyway.

Based on the above assumption, at the step 2 of the agreement 1 in last meeting, storing & forwarding the UL data would be wasteful after the actual delay exceeds UE’s tolerable delay. Therefore, we would like to propose to introduce a mechanism to indicate UE’s requirement for UL data transmission (e.g., level of delay-tolerance, data expiration time) from UE to eNB, so that eNB can discard the data for storing & forwarding when the actual delay exceeds the requirement.

Proposal 3:  RAN2 to consider whether and how UE can indicate its requirement for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to eNB

Aside from S&F indication, introducing Early Data Transmission (EDT) is discussed on Uplink Capacity Enhancement. We think EDT is beneficial to reduce the number of signaling and the total delay even for S&F operation. Therefore, RAN2 may discuss about considerations for using EDT on S&F operation, however, we would like to clarify if EDT is applicable for S&F operation before the discussion.

Proposal 4:  RAN2 to clarify if EDT is applicable for S&F operation

- Summary and proposal

This paper discussed about consideration on S&F operation and proposed the following:

Proposal 1:  S&F related parameter(s) could be included in SIB as the “S&F indication”

Proposal 2:  Estimated delivery time could be indicated to UE

Proposal 3:  RAN2 to consider UE could indicate its requirements for the UL data transmission (e.g., level of delay-tolerance, data expiration time) to eNB

Proposal 4:  RAN2 to clarify if EDT can be applicable for S&F operation

- References

[1] RP-234007, “New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”

[2] R2-2405703, “Report from Break-out session on NR-NTN and IoT-NTN”

[3] R2-2404801, “Store and Forward support in IoT NTN”, Lenovo