3GPP TSG-RAN WG2 Meeting #127	R2-2407492

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:	8.3.4.1

Source:	Samsung

Title:	Discussion on simulation assumptions and evaluation methodology for RLF prediction

Document for:	Discussion & Decision

Introduction

This document discusses simulation assumptions and evaluation methodology for RLF failure prediction.

Discussion

RLF specific methodology and simulation assumptions

For RLF definition, the following agreement was made in the last meeting.

Agreements

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

According to TS 38.331[1], T310 begins when signal strength (i.e., SINR) falls below the Qout value for N310 occurrences. If signal strength subsequently exceeds the Qin value for N311 counts, T310 stops. Otherwise, if signal strength remains below the Qin value and T310 expires, this should be counted as an RLF event. Hence, Qout/Qin, N310/N311, T310 values should be decided for RLF modelling in simulation. Considering that TR36.839[2] already provides the reference values for these parameters, we can adopt them as baseline.

Proposal 1. For the parameters related to RLF modelling (e.g., T310, Qout, Qin, N310, N311), TR 36.839 is a baseline.

However, [2] is for 4G LTE frequency whereas this Rel-19 study item covers FR2 frequency. Considering FR2 channel characteristic, the signal fluctuation could be higher, fast decision is desirable in FR2 case. We see shorter T310 value than FR1 is needed, e.g., 500ms.

Observation 1. Considering FR2 channel characteristics, shorter value of T310 needs to be considered.

Proposal 2. For RLF modelling in FR2, a shorter value of T310 is considered.

Furthermore, the representative value of signal strength compared to Qout/Qin is not instantaneous SINR value but rather L1 processed SINR. The L1 processing means linear averaging instantaneous SINR values within the sliding window [2]. In our simulation, it is observed that the performance of RLF varies extremely depending on the sliding window size. Hence, for RLF evaluation, sliding window size should be determined for comparing Qout/Qin values. We can use these values as 200/100ms for Qout and Qin, respectively, based on [2].

Observation 2. The sliding window length for L1 processing of SINR affects RLF performance significantly.

Proposal 3. For sliding window length for L1 processing of SINR, TR 36.839 is a baseline.

For RLF prediction, serving cell selection and handover modelling are necessary. For mobility modelling part, RAN2 does not need to precisely model signalling of RRC messages.  [2] used simplified modelling of handover procedure. As shown in Figure 1, we can consider the handover procedure divided into 3 states as in [2].

State 1: Before the event A3 entering condition, is satisfied;

State 2: After the event A3 entering condition, is satisfied but before the handover command is successfully received by the UE;

State 3: After the handover command is received by the UE, but before the handover complete is successfully sent by the UE

Figure 1. A Definition of Handover States.

As shown in Figure 1, the transition from state 1 to state 2 is triggered by event entering condition, and the transition from state 2 to state 3 is triggered by receiving the HO Command. Also, in state 2, the measurement report is triggered once the TTT expiry, and then the handover preparation time elapsed. During the handover execution time, it remains in state 3. Since T310 stops when the UE receives a HO Command as described in [1], RLF event by T310 expiry will be only occurred in handover state 1 or state 2. Note that state 1 is the period before UE triggers handover procedure (i.e., the period during which UE is not involved with handover procedure). Thus, we propose, for RLF modelling in simulation, RLF is defined as T310 expiry in the handover state 1 and 2 in TR 36.839. Here, we also would like to clarify, although our proposal includes the text “state 1 and state 2” which is borrowed from TR 36.839 for common understanding, it does not mean they are distinguished in simulation.

Proposal 4. For RLF modelling in simulation, RLF is defined as T310 expiry in the handover state 1 and 2 in TR 36.839. There is no need to distinguish RLF in state 1 and RLF in state 2.

To evaluate RLF events, factors such as event entry conditions, TTT, handover preparation time, and handover execution time should be considered. From the perspective of the measurement report, A3 is agreed as a baseline for the event for HO and thus we need to determine reference values for the A3 event offset and TTT. Since it is uncertain which value can most efficiently enhance handover KPI performance through prediction in the future, we suggest to derive results for the 5parameter sets (TTT-480/160/160/80/ msec, A3 Offset-3/3/2/1/-1dB) used in [2].

For HO preparation time, since 50 ms is assumed for LTE environment in [2], we can also consider values within 50ms for our evaluation. Considering the agreed sampling period (i.e., 40msec for FR1, 20msec for FR2), we propose a multiple of 40 and the nearest value to 50ms, which is 40ms. For HO execution time, considering both air delay and UE's tuning time requirements, the value of 40 ms from [2] can be reused as before. Re-establishment procedure modelling or T311 settings are not taken into account as they do not impact RLF performance. Other parameters, including cell loading, remain consistent with [2].

Proposal 5. For A3 Offset/TTT parameter sets, TR 36.839 is a baseline.

Proposal 6. HO preparation/execution time can be set to 40/40ms considering the agreed sampling period (i.e., 40ms for FR2, 20ms for FR1).

Proposal 7. RAN2 to adopt the following parameter values for RLF modelling in simulation.

Table 1. The parameters for RLF performance evaluation.

| Items | Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Qout | -8 dB |  |  |  |  |
| Qin | -6 dB |  |  |  |  |
| T310 | 1000 ms (FR1), 500 ms (FR2) |  |  |  |  |
| N310 | 1 |  |  |  |  |
| N311 | 1 |  |  |  |  |
| HO preparation time | 40 ms |  |  |  |  |
| HO execution time | 40 ms |  |  |  |  |
| Sliding window for Qout/Qin | 200/100 ms |  |  |  |  |
| TTT | Set1 | Set2 | Set3 | Set4 | Set5 |
|  | 480 ms | 160 ms | 160 ms | 80 ms | 40 ms |
| A3 Offset | Set1 | Set2 | Set3 | Set4 | Set5 |
|  | 3 dB | 3 dB | 2 dB | 1 dB | -1 dB |
| Cell loading (i.e., traffic load) | 100 % |  |  |  |  |

Further discussion on direct/indirect RLF prediction

The following agreements were made in the last meeting for RLF prediction.

Agreements

1	Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR).

2	Study Direct: Directly RLF prediction by AI/ML models.

3	FR2 study will be prioritized for RLF prediction.

4	The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction.

5	HOF prediction is downprioritized in our study.  NO simulations/evaluations should be done/submitted

6	RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case.  FFS on expected RLF time and indirect case.

7	No evaluation/simulations are expected for August meeting for RLF

8	Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible).

Let us recap the indirect RLF prediction (see Figure 1) first.

Figure 1. Indirect RLF prediction

In the indirect RLF prediction, RRM prediction model is re-used for RLF prediction. The inputs for the RRM prediction model are cell-level or beam-level measurement results (depending on Case 1, 2, and 3 in RRM prediction) for the serving cell, but the outputs for the RRM prediction model are the predicted cell level measurement results (e.g., SINR) for serving cell, as agreed in the last meeting. As shown in the Figure 1, using this RRM prediction result, there could be a rule to derive RLF prediction results. For example, if the predicted SINR for the serving cell is worse than a threshold during the T310 period (i.e., to emulate T310 expiry which is agreed for representative RLF case), we can assume RLF is predicted. Although any further details for the rule are FFS, we assume this simple example as the baselined rule. As a result, for indirect RLF prediction, we can predict the time of RLF occurrence (i.e., the earliest time instance before which the predicted SINR for the serving cell has been worse than the threshold during the T310 period). We believe this understanding can clarify the FFS point in the last meeting.

Proposal 8. For indirect RLF prediction, the RLF prediction result is the expected time of RLF occurrence.

Figure 2. Direct RLF prediction

Next, in direct RLF prediction (see Figure 2), similar to indirect one, actual RRM measurement results (e.g., SINR) could be an input parameter. One of differences between direct and indirect RLF prediction is, RLF specific inputs for the model inference can be considered in direct RLF prediction. For example, RLF related RRC parameters like T310, N310, and N311 can be considered as inputs. Besides, random access related MAC parameters for handover or RLC retransmission parameters can be also used as input of the model. So, although RAN2 agreed T310 expiry for representative RLF case, we may expect more general RLF prediction considering RLF can be caused by not only T310 expiry but also other reasons (e.g., random access problem, excess of max. RLC retransmissions).

When most AI/ML models predict a certain event, a normal output of the AIML model inference is the probability of the event occurrence within a prediction window. The prediction window could be pre-configured or the input of the model. We would like to note probability of the event occurrence at a certain “time instance” is always 0, but the probability of the event occurrence during some “time period” could be larger than 0. Therefore, we would like to remove “at time instance” in the last agreement.

Proposal 9. For direct RLF prediction, the RLF prediction result is the probability of RLF occurrence within a time window (rather than time instance).

Conclusion

RAN2 is requested to discuss and agree the following proposals:

Proposal 1. For the parameters related to RLF modelling (e.g., T310, Qout, Qin, N310, N311), TR 36.839 is a baseline.

Observation 1. Considering FR2 channel characteristics, shorter value of T310 needs to be considered.

Proposal 2. For RLF modelling in FR2, a shorter value of T310 is considered.

Observation 2. The sliding window length for L1 processing of SINR affects RLF performance significantly.

Proposal 3. For sliding window length for L1 processing of SINR, TR 36.839 is a baseline.

Proposal 4. For RLF modelling in simulation, RLF is defined as T310 expiry in the handover state 1 and 2 in TR 36.839. There is no need to distinguish RLF in state 1 and RLF in state 2.

Proposal 5. For A3 Offset/TTT parameter sets, TR 36.839 is a baseline.

Proposal 6. HO preparation/execution time can be set to 40/40ms considering the agreed sampling period (i.e., 40ms for FR2, 20ms for FR1).

Proposal 7. RAN2 to adopt the following parameter values for RLF modelling in simulation.

Proposal 8. For indirect RLF prediction, the RLF prediction result is the expected time of RLF occurrence.

Proposal 9. For direct RLF prediction, the RLF prediction result is the probability of RLF occurrence within a time window (rather than time instance).

References

[1] 3GPP TS 38.331, Radio Resource Control (RRC) protocol specification, v18.2.0 (2024-06)

[2] 3GPP TR 36.839, E-UTRA Mobility enhancements in heterogeneous networks, v11.1.0 (2012-12)