3GPP TSG-RAN WG2 Meeting #127	R2-2407509

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Agenda item:		8.2.4

Source:		Futurewei

Title:		Discussions on AIoT random access

Document for:		Discussion and Decision

# Introduction

In meeting #126, RAN2 agreed on the following with respect to the random access of Ambient IoT (AIoT) [1]:

Agreements on “4 step” RA

1	A-IoT Msg1: the device sends an ID to the reader.  ID is a random ID generated by device (FFS how it is generated, e.g. randomly generated or generated based on Device ID).  FFS on ID size.  This doesn’t preclude any other RAN1 agreed information

2	A-IoT Msg2: the reader echoes the ID received in Msg1.   Further information may be included in mgs2 based on RAN1 agreements

3	A-IoT Msg3: device sends Device ID and/or any other upper layer data (depending on upper layer request)

4	The device considers the contention resolution as successful, if the Msg2 including the same random ID in Msg1 is received. RAN2 assumes the size of random ID in Msg1 should be sufficient for contention resolution purpose.

5	“Msg4” (i.e. the subsequent R2D transmission after D2R transmission) does not need to be always sent in random access. “Msg4” can be considered to handle the Msg3 transmission failure (due to various reasons). “Msg4” usage/presence can be further discussed. RAN2 will not use “Msg4” term for further discussion of the random access.

Agreements on 2 step CB RA

- A-IoT Msg1: The device sends Device ID and/or any other upper layer data (depending on upper layer request). FFS what device ID is and whether an additional random ID is needed.  This doesn’t preclude any other RAN1 agreed information

- A-IoT Msg2: the reader may echo some information from Msg1.  FFS what some information is.   “Msg2” usage/presence can be further discussed

Agreement on contention-free access

-	From reader perspective, contention-free access procedure we will study single and multi-device case (depending on RAN1 discussion).

In this contribution, we provide our views on some content required in Msg1 and Msg3.

# Discussions

## On the ID sent in Msg 1

A random ID generated by the device is an obvious way for identifying the device in Msg1 and for contention resolution when the device has never responded to an inventory request under (and hence unknown to) the reader before. We consider 16 bits being adequate for the random ID size.

Proposal 1. The random ID size is 16 bits.

However, it is unclear how long the device should keep using the same value as its random ID. For example, should the device change its random ID for each inventory round or keep using the same value? It is also unclear whether the device stores its random ID in a non-volatile memory before its enters the OFF or Sleep state for energy harvesting so that it may resume the use of the same random ID after it returns to the ON state or the device simple loses its random ID when entering the OFF or Sleep state and pick another random ID when it returns to the ON state.

Although the probability that different devices pick a same random ID (i.e., an ID collision) in their respective Msg1 is low, when it indeed occurs, one colliding device or each colliding device should pick another random ID; otherwise the reader may be unable to uniquely identify the colliding devices in the subsequent communications with them. For example, if the reader receives a Msg1 with a random ID from a first device, which is later identified by a device ID (e.g., a long ID or NAS ID) in Msg3, and later on, a Msg1 with the same random ID is received from a second device, which is later identified by a different device ID in Msg3, the reader may inform the second device to pick another random ID.

Proposal 2. RAN2 study the following conditions that may trigger a device to change its random ID:

- Change of inventory round

- Change of device state

- Instruction to change from the reader

Another way to ensure the uniqueness among IDs sent in Msg1 after the first time the device is inventoried under the reader is for the reader to assign a unique AS ID to each device when the device is inventories for the first time. To differentiate between the random ID and the reader-assigned AS ID, an indication of ID type may be included in Msg1 and Msg2.

Proposal 3. RAN2 study a short AS ID to be uniquely assigned to a device when the device is inventoried for the first time under the reader and to be used in the Msg1s sent by the device after it is inventoried for the first time under the reader.

Proposal 4. Instead of the random ID, if the reader-assigned AS ID of the device may be sent in Msg1, an indication of ID type may be included in Msg1 and Msg2 to differentiate between the random ID and the reader-assigned AS ID.

## On the device ID sent in Msg3

The device ID sent in Msg3 should be a long ID (longer than the random ID or the reader-assigned AS ID of the device) that can uniquely identify the device at the AIoT-NAS layer or above. For example, the long device ID may be a TMSI-like or S-TMSI-like CN-assigned temporary ID that uniquely identifies the device within a deployment or an EPC-like permanent ID of an equipment attached to the device. The long device ID may be assigned to the device during a registration procedure or an on-boarding procedure with the network or during manufacturing of the device.

Proposal 5. The device ID sent in Msg3 should be a long ID that can uniquely identify the device at the AIoT-NAS layer and/or above. The long device ID may be assigned to the device during a registration procedure or an on-boarding procedure with the network or during the manufacturing of the device.

# Conclusions

We propose the following:

Proposal 1. The random ID size is 16 bits.

Proposal 2. RAN2 study the following conditions that may trigger a device to change its random ID:

- Change of inventory round

- Change of device state

- Instruction to change from the reader

Proposal 3. RAN2 study a short AS ID to be uniquely assigned to a device when the device is inventoried for the first time under the reader and to be used in the Msg1s sent by the device after it is inventoried for the first time under the reader.

Proposal 4. Instead of the random ID, if the reader-assigned AS ID of the device may be sent in Msg1, an indication of ID type may be included in Msg1 and Msg2 to differentiate between the random ID and the reader-assigned AS ID.

Proposal 5. The device ID sent in Msg3 should be a long ID that can uniquely identify the device at the AIoT-NAS layer and/or above. The long device ID may be assigned to the device during a registration procedure or an on-boarding procedure with the network or during the manufacturing of the device.

# References

R2_126_ChairNotes_24-05-24_13-10_eom.