3GPP TSG-RAN WG2 #127	R2-2407512

Maastricht, Netherland, 19th – 23rd Aug, 2024

Source:	NTT DOCOMO, INC.

Title:	Discussion on LP-WUS in RRC_CONNECTED

Document for:	Discussion

Agenda Item:	8.4.4

- Introduction

A new WID on LP-WUS/WUR has been approved at the RAN#102 meeting [1] and revised at the RAN#103[2]. In the WID, following objectives are included.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2#126 meeting, following agreements has been made,

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In this contribution, we describe our view for LP-WUS in RRC_CONNECTED

- Discussion

  - LP-WUS monitoring procedure

The following solutions are candidates for the LP-WUS procedure in RRC_CONNECTED in RAN1.

Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Combination of options in Case 1 should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

Option 1-1 as shown in Figure 1 is a straightforward solution. As discussed in last RAN2 meeting, in case LP-WUS is considered to replace legacy DCP feature, a lot of Rel-16 DCP framework can be reused (e.g., PDCCH monitoring, RRM measurements) and we do not need to consider coexistence of LP-WUS and legacy signals (i.e., the current specifications support coexistence of DCP and other signals). In addition, LP-WUR to MR fallback mechanism needs to be discussed. On the other hand, if the enhancements focus only on the replacement of DCP feature, the power saving gain may be smaller than other options.

Options 1-2 allows PDCCH monitoring outside the legacy C-DRX active time, thus providing more flexible operation.

Since option 1-2-1 may cause more UE power consumption compared to legacy C-DRX cycle, it needs to be configured with option 1-1 as shown in Figure 2. In that case, option 1-2-2 can provide same function. Thus, option 1-2-1 should not be supported.

In option 1-2-2 as shown in Figure 3, since PDCCH monitoring is not restricted to on-duration, the latency performance may be improved compared to option 1-1 and option 1-3. RRM measurement in legacy C-DRX may be reused, however, the detail procedure of PDCCH monitoring in legacy C-DRX cannot be reused. Thus, new mechanisms related to PDCCH monitoring need to be discussed.

Option 1-3 as shown in Figure 4 would be beneficial for XR use case which traffic jitter may occur during C-DRX active time. We understand XR is one important use case of LP-WUS, but Rel-19 would be better to focus on the solutions applicable to various use cases. In order to extend applicable use case, it may be better to merge option 1-2-2 and option 1-3 as shown in Figure 5. There would be no additional specification impacts by the merge, i.e., LP-WUS/PDCCH monitoring irrespective of legacy C-DRX active time. We think that the unified option 1-2-2 + 1-3 would be beneficial for not only XR use case but also various use cases to achieve best tradeoff between large power saving gain and large latency reduction.

Based on the above discussion, Rel-19 LP-WUS in RRC_CONNECTED mode should focus on key enhancement such as option 1-1 and merged option 1-2-2 + 1-3. At this stage, the required specification impacts of option 1-1 and merged option 1-2-2 + 1-3 are unclear since it depends on the detail framework. Thus, we should further study the details of these options.

Figure 1: Example of option 1-1

Figure 2: Example of option 1-2-1 configured together with option 1-1

Figure 3: Example of option 1-2-2

Figure 4: Example of option 1-3

Figure 5: Example of unified option 1-2-2 and option 1-3

Proposal 1: The following options can be merged as a unified solution:

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

- Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

Proposal 2: Further study the following options in Rel-19

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

  - This option may replace DCP functionality

- Merged Option 1-2-2 + 1-3

  - Option 1-2-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- Conclusion

Proposal 1: The following options can be merged as a unified solution:

- Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

- Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

Proposal 2: Further study the following options in Rel-19

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

  - This option may replace DCP functionality

- Merged Option 1-2-2 + 1-3

  - Option 1-2-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- References

[1] RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

5. Annex - RAN1 agreements

In RAN1#117-bis meeting, following agreements has been made,

Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for Mos - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

In RAN1#116-bis meeting, following agreements has been made,

Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

In RAN1#116 meeting, following agreements has been made,

R1-2401698 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded. Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one R1-2401836 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.