3GPP TSG-RAN WG2 Meeting #127bis	R2-2407977

Hefei China, Oct. 14th – 18th, 2024

Agenda Item:	8.3.2

Source:	OPPO

Title:	Discussion on simulation result of RRM measurement

Document for:	Discussion, Decision

Introduction

In RAN2#126, the following cases are identified for RRM measurement prediction with 3 cases of high priority for evaluation.

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In RAN2#127, we have agreed that

Agreements 1 To keep two filtering options on the table and up to company to report.

In post-meeting email,

[POST127][030][AI mobility] RRM simulation assumptions (Oppo)

Intended outcome: agree to simulation assumptions

Deadline:  short

We agree to capture both filtering options in TP.

Figure 1.1 Sliding L1/L3 filtering

Figure 1.2: non-sliding L1/L3 filtering

Although companies have the freedom to report their chosen filtering options, the impact of these different options remains a subject of interest. In this paper, we will attempt to analyze this impact.

During the email discussion, efforts were made to select a fixed skipping pattern for the FR1 intra-frequency temporal domain case B, but these attempts were unsuccessful. Typically, there are two patterns under consideration, as depicted in Figures 1.3 and 1.4.

Figure 1.3 temporal domain case B example 1

Figure 1.4 temporal domain case B example 2

We have reached a consensus in the post-meeting email discussion that reporting the detailed pattern is optional. The question of whether different patterns affect the RRM prediction accuracy is worthy of further investigation. Therefore, we are keen to delve deeper into this matter.

In RAN2#127, we also agreed that

Cluster approach => it is up to companies to select the number of cells for input and output (companies should clarify what they are using for cluster-based approach). Cluster-based approach evaluation is optional and lower priority for now.

The cluster approach has been a topic of debate over several meetings, and yet its simulation results are well understood. Nonetheless, we are also interested in examining the impact of the cluster approach.

In summary, this contribution intends to show our evaluations of the three high-priority RRM measurement prediction cases. It also seeks to explore the effects of sliding versus non-sliding filtering, the skipping pattern in the temporal domain case B, and the cluster approach.

Discussion

Let us start from the most disputing case in the post-meeting email, i.e., intra-frequency temporal domain case B.

FR1 to FR1 intra-frequency temporal domain case B

The common simulation assumptions and information are listed in Table 1.

Table 2.1.1 FR1 to FR1 intra-frequency temporal domain case B prediction assumptions

| Report parameters |  | OPPO | FR1 to FR1 intra-frequency temporal domain case B |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |
|  | UE speed (30,60,90,120 Km/h) | 30 without explicit statement |  |
|  | Whether LOSsoft is modeled or not | No |  |
|  | spatial consistency option (A or B) | A |  |
|  | Number of TX beams | 1 |  |
|  | Number of RX beams | 1 |  |
| Data Size (Number of Samples) | Training/validity | ~0.20M |  |
|  | Testing | ~0.02M |  |
| AI/ML model input/output | Model input (Note 5) | L3-RSRP |  |
|  | Model output(Note 6) | L3-RSRP |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |
|  | Model complexity in a number of parameters(M) | ~0.17M |  |
|  | Model complexity in model size (e.g. Mbyte) | ~0.67Mbyte |  |
|  | Computational complexity [FLOPs] | ~0.41M |  |

To compare the impact of sliding/non-sliding filtering, we have tested four scenarios as shown in Figure 2.1.1.

Figure 2.1.1. Different scenarios of FR1 temporal domain case B with MRRT=50%.

The detailed differences between each scenario can be seen from Table 2.1.2.

Table 2.1.2. temporal domain case B results with different methods of data processing

| MRRT=50% | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- |
| Filtering option | Non-sliding | Non-sliding | Sliding | Sliding |
| Detailed skipping pattern | example 2 | example 1 | example 2 | example 2 |
| OW | 600ms (2 samples) | 600ms (3 samples) | 120ms (2 samples) | 600ms (10 samples) |
| PW | 200ms (1 sample) | 600ms (3 samples) | 40ms (1 sample) | 200ms (5 samples) |
| Average L3 cell-level RSRP difference (dB) | 0.26 | 0.46 | 0.10 | 0.17 |

Note: The samples within OW represent the measured samples, i.e., the number of yellow blocks in Figure 2.1.1

Scenarios 1 and 2 share the same OW and MRRT, but differ in detailed skipping pattern. Our analysis reveals that:

Observation 1: The detailed skipping pattern will influence the prediction accuracy of FR1 intra-frequency temporal domain case B, For non-sliding filtering with the same MMRT, the average L3-RSRP difference of skipping pattern example 1 is 0.2dB larger than that of pattern example 2.

The reason lies that the PW of example 2 is shorter than that of example 1 under the same MRRT. The bigger the distance between the measured instance and the predicted instances, the harder the prediction will be. Besides, example 1 in temporal domain case B resembles what we discussed in temporal domain case A. To differentiate the two cases, it is preferable to adopt example 2 for case B analysis.

Scenarios 1 and 4 both employ the example 2 pattern and utilize a 600ms OW and a 200ms PW. The key difference is that scenario 1 only collects 1 sample every other measurement period, whereas scenario 4 has 5 samples with a 40ms sampling interval.

Observation 2: With the same MRRT and equal lengths of OW and PW, sliding filtering exhibits slightly better prediction accuracy (0.09dB) than non-sliding filtering. The distinction between the two filtering methods is not pronounced.

From observation 2, we do not discern a distinct difference between the two filtering options. Besides, to some extent, non-sliding filtering is a special case of sliding filtering. If we take samples from sliding filtering every measurement period (i.e., taking the first time instance of each measurement period as the sample for that measurement period), we achieve results identical to non-sliding filtering.

Upon examining scenario 3, we identify the smallest L3-RSRP difference among the four discussed scenarios. This is attributed to sliding filtering's capability to predict on a small time scale, where RSRP variations are typically minimal.

Observation 3: Sliding filtering can predict RSRPs on a small time scale and achieve high prediction accuracy.

Table 2.1.3. temporal domain case B results with different MMRT and UE speed

| UE speed | 30km/h |  |  | 60km/h |
| --- | --- | --- | --- | --- |
| MRRT | 50% | 67% | 75% | 50% |
| OW | 600ms (2 samples) | 800ms (2 samples) | 1000ms (2 samples) | 600ms (2 samples) |
| PW | 200ms (1 sample) | 400ms (2 samples) | 600ms (3 samples) | 200ms (1 sample) |
| Average L3 cell-level RSRP difference (dB) | 0.26 | 0.41 | 0.64 | 0.51 |

Note: The samples within OW represent the measured samples, i.e., the number of yellow blocks in Figure 2.1.1

In our previous meeting, we observed that for sub-case 3, an increase in User Equipment (UE) speed correlates with a decrease in prediction accuracy [1]. A similar trend is evident in Table 2.1.3 for sub-case 2, where the same phenomenon occurs. Reports from various companies presented at the last meeting also corroborate this observation [2, 3]. Additionally, in [4], a company explored the impact of speed on sub-case 1 and noted similar findings. Based on these findings, we can conclude:

Observation 4: Across sub-cases 1, 2, and 3, higher UE speeds result in larger prediction errors in the FR1 intra-frequency temporal domain case B.

As MRRT increases, the model gets less information for analysis, making the prediction more challenging. Consequently, we observe that:

Observation 5: When MRRT increases from 50% to 75%, the average L3-RSRP difference gradually increases. The RSRP difference at MRRT=75% is 0.38 dB higher than that at MRRT=50%, yet it remains within 0.7dB.

FR1 to FR1 inter-frequency (frequency domain)

In accordance with the RAN2#126 agreements, for inter-frequency inter-cell prediction, the measurements of cells must not be reduced in either the temporal or spatial domains. As depicted in Figure 2.2.1, we utilize both current and historical RSRPs at the 2GHz frequency to forecast the current RSRP at the 4GHz frequency. Non-sliding filtering is employed, which means the time interval between two consecutive samples is 200ms.

Figure 2.2.1. Example for inter-frequency prediction

The detailed common simulation assumptions are listed in Table 1.

Table 2.2.1. FR1 to FR1 inter-frequency prediction assumptions

| Report parameters |  | OPPO | FR1 to FR1 inter-frequency |
| --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |
|  | UE speed (30,60,90,120 Km/h) | 30 |  |
|  | Inter-frequency correlation assumption in general (yes or no)(Note 1) | yes |  |
|  | inter-frequency shadow fading correction (e.g. full, partial, no)(Note 1) | Full |  |
|  | Whether LOSsoft is modeled or not | No |  |
|  | spatial consistency option (A or B) | A |  |
|  | Number of TX beams | 1 |  |
|  | Number of RX beams | 1 |  |
|  | Filtering option | Non-sliding filtering |  |
| AI/ML model input/output | Model input (Note 5) | L3-RSRP of Cell A |  |
|  | Model output(Note 6) | L3-RSRP of Cell B |  |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |

We have tested three scenarios, each containing two sub-scenarios with different OW. The three scenarios are:

- Scenario 1: Cell-based approach. The model input is data from the serving cell. The model output is the corresponding prediction of the co-sector cell in another frequency layer.

- Scenario 2: Cluster-based approach. The model input is data from the serving cell and its two co-site cells. The model output is the corresponding prediction of the three co-site cells in another frequency layer.

- Scenario 3: Cluster-based approach. The model input and output are data from all 21 cells. The model output is the corresponding prediction of the 21 cells in another frequency layer.

Table 2.2.2 shows the prediction results of these three scenarios. It should be noted that a cell-based model can be used for prediction of different cells. The cell-based model is not cell-specific (i.e., only applicable for one cell).

Table 2.2.2. cell-based versus cluster-based approach in FR1 inter-frequency prediction

| Scenario |  | Scenario 1 |  | Scenario 2 |  | Scenario 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AI/ML model input/output | Model input | One cell |  | Cluster with 3 cells |  | Cluster with 21 cells |  |
|  | Model output | One cell |  | Cluster with 3 cells |  | Cluster with 21 cells |  |
|  | OW | 400ms (2 samples) | 800ms (4 samples) | 400ms (2 samples) | 800ms (4 samples) | 400ms (2 samples) | 800ms (4 samples) |
|  | PW | 200ms (1 sample) | 200ms (1 sample) | 200ms (1 sample) | 200ms (1 sample) | 200ms (1 sample) | 200ms (1 sample) |
| Data Size (Number of Samples) | Training | ~52K | ~26K | ~52K | ~26K | ~52K | ~26K |
|  | Testing | ~12K | ~6K | ~12K | ~6K | ~12K | ~6K |
| AI/ML model description | Model complexity in a number of parameters(M) | 0.17 | 0.18. | 0.18 | 0.18 | 0.19 | 0.19 |
|  | Model complexity in model size (e.g. Mbyte) | 0.67 | 0.67 | 0.67 | 0.67 | 0.71 | 0.91 |
|  | Computational complexity [FLOPs] | 0.40 | 0.41 | 0.41 | 0.41 | 0.42 | 0.42 |
| Metrics | Average L3 cell-level RSRP difference (dB) | 0.28 | 0.80 | 0.43 | 1.49 | 0.78 | 2.78 |

Under the same OW, as the number of input and output cells increases, model complexity goes up in terms of the number of parameters, model size, and FLOPs.

Observation 6: With a similar model structure, cluster-based approach has higher model complexity than the cell-based approach. The more cells a cluster has, the more complexity the AI/ML model will have.

This reflects that model becomes larger and more complex when the size of the input and output layers increases. What is interesting is that a more complicated model does not necessarily equate to better prediction accuracy. It can be seen that the cell-based approach outperforms the cluster-based approach under the same set for OW and PW. The superiority becomes more pronounced when OW changes from 400ms to 800ms.

Observation 7: With a similar model structure, cluster-based approach has a larger RSRP difference than the cell-based approach under different observation windows. The gap becomes more evident (from 0.5dB to 1.98dB ) when OW changes from 400ms to 800ms. As the number of cells within a cluster grows, the average L3-RSRP difference tends to increase.

Given that the cell-based approach demonstrates superior performance over the cluster-based approach in terms of model complexity and prediction accuracy, it is proposed to confirm that

Proposal 1: Confirm that the cluster-based approach is optional and of low priority in the study and capture the following observation into TP: The cell-based approach outperforms the cluster-based approach in terms of model complexity, computational complexity, and prediction accuracy.

From Table 2.2.2, we can also see that

Observation 8: The average L3-RSRP increases from 0.28dB to 0.8dB when the OW of the serving cell doubles. In FR1 inter-frequency prediction, a larger OW does not necessarily equate to better prediction accuracy.

This is attributed to the fact that a significant temporal gap between the previous and current instances reduces the correlation between the two frequencies. The additional input data may act as interference rather than providing valuable information for the prediction. Therefore, it is proposed that:

Proposal 2: For the FR1 inter-frequency case, a small OW length, e.g., 400ms, is sufficient for making prediction. There is no need to introduce a longer OW length.

FR2 to FR2 intra-frequency temporal domain case A

Figure 4. Example for intra-frequency temporal domain case A

In [1], we have simulated the intra-frequency temporal domain case A under sliding filtering. In this section, we will have a look at performance under non-sliding filtering. The basic assumptions are listed in Table 2.3.1.

Table 2.3.1. FR2 to FR2 intra-frequency temporal domain case A prediction assumption

| Report parameters |  | OPPO | FR2 to FR2 intra-frequency temporal domain case A |  |  |
| --- | --- | --- | --- | --- | --- |
| Reported simulation assumptions | UE trajectory option (option 1,2,3 in[4]) | Option 1 |  |  |  |
|  | UE trajectory boundary processing option (option 1,2,3 in[4]) | Option 3 |  |  |  |
|  | UE speed (30,60,90,120 Km/h) | 60, 90, and 120 |  |  |  |
|  | Whether LOSsoft is modeled or not | No |  |  |  |
|  | spatial consistency option (A or B) | A |  |  |  |
|  | Number of TX beams | 32 |  |  |  |
|  | Number of RX beams | 4 |  |  |  |
|  | Filtering option | Non-sliding filtering |  |  |  |
| Data Size (Number of Samples) | Training/validity | ~7K |  |  |  |
|  | Testing | ~1.9K |  |  |  |
| AI/ML model input/output | Model input (Note 5) | filtered L1-RSRP |  | L3-RSRP | filtered L1-RSRP |
|  | Model output(Note 6) | L3-RSRP |  | L3-RSRP | L3-RSRP |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer …) | 2 LSTM layer + 1 dense layer |  |  |  |
|  | Model complexity in a number of parameters(M) | ~0.17M |  |  |  |
|  | Model complexity in model size (e.g. Mbyte) | ~0.67MB |  |  |  |
|  | Computational complexity [FLOPs] | ~0.41M |  |  |  |

Here are the simulation results.

Table 2.3.2. FR2 intra-frequency temporal domain case A prediction results under different OW and PW

| OW | 800ms (2 samples) | 1200ms (3 samples) | 1600ms (4 samples) | 800ms (2 samples) | 800ms (2 samples) |
| --- | --- | --- | --- | --- | --- |
| PW | 400ms (1 sample) | 400ms (1 sample) | 400ms (1 sample) | 800ms (2 samples) | 1600ms (4 samples) |
| Average L3 cell-level RSRP difference (dB) | 1.21 | 1.12 | 1.23 | 1.93 | 2.58 |

Table 2.3.2 examines the performance under sub case 2 (i.e., L3-RSRP as model input and output) with varying OW and PW. It is clear that

Observation 9: When OW extends from 800ms to 1600ms, the L3-RSRP difference of different lengths of OW is almost the same under the same PW. While increasing the length of PW from 400ms to 1600ms tends to deteriorate prediction accuracy under the same OW.

Recalling what we found from [1]: when OW changes from 200ms to 400ms, the average L3-RSRP becomes larger for both sub cases 2 and 3. This leads us to:

Observation 10: Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold (e.g., 800ms), further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.

This observation is instrumental in establishing an upper limit for OW, thereby reducing simulation workload. It is also a noteworthy finding that should be documented in our TP.

Proposal 3: Capture the following observation into TP:

- For FR2 intra-frequency temporal domain case A, increasing the OW length can improve prediction accuracy when OW is short. However, beyond a certain threshold (e.g., 800ms), additional increases do not provide benefits. For PW, longer durations lead to worse predictions.

In [1], we have found that, for sliding filtering, increasing UE speed will impair the prediction accuracy. A similar observation is noted in Table 2.3.3, leading to

Observation 11: For both filtering options, the average RSRP difference increases as UE speed increases from 60km/h to 120km/h in FR2 to FR2 intra-frequency temporal domain case A.

Table 2.3.3. Comparison of impact of sub cases and UE speed for FR2 intra-frequency temporal domain case A

| Sub case | Case 1 | Case 3 | Case 2 |  |  |
| --- | --- | --- | --- | --- | --- |
| UE speed | 60km/h | 60km/h | 60km/h | 90km/h | 120km/h |
| Model complexity in a number of parameters(M) | 0.19 | 0.19. | 0.17 |  |  |
| Model complexity in model size (e.g. Mbyte) | 0.74 | 0.72 | 0.67 |  |  |
| Computational complexity [FLOPs] | 0.42 | 0.42 | 0.41 |  |  |
| Average L3 cell-level RSRP difference (dB) | 2.29 | 3.22 | 1.21 | 1.49 | 1.50 |

Sub cases 1 and 3 yield different results compared to sub case 2.

Observation 12: Among sub cases 1, 2, and 3, sub case 2 demonstrates the highest prediction accuracy, with sub case 1 being the second best. The prediction error of sub cases 1 and 3 is more than 1.08 dB larger than that of case 2 at 60km/h.

The reason for this discrepancy is that sub-cases 1 and 3 have a larger number of inputs and outputs compared to sub-case 2. With a similar AI/ML structure, this means there is more data to process and predict, which increases the difficulty of prediction for sub-cases 1 and 3.

Following this assumption, one might expect sub-case 1 to be more challenging to predict than sub-case 3, but the observed prediction accuracy is reversed. This is attributed to the L3 filtering that occurs after the prediction of L1-RSRP. The filtering process smooths the L1 prediction results, as it utilizes previously measured L3-RSRP values for calculation.

In the context of measurement event prediction, the accuracy of the predictions is crucial for making correct handover decisions. Therefore, sub-case 2 appears to be a more suitable choice compared to sub-cases 1 and 3. We can prioritize sub-case 2 in indirect measurement event prediction simulations.

Proposal 4: Focus on sub case 2 for indirect measurement event prediction in simulations to reduce the workload for companies.

Conclusion

In this contribution, we have shared our simulation results on RRM measurement prediction and have the following observations:

Observation 1: The detailed skipping pattern will influence the prediction accuracy of FR1 intra-frequency temporal domain case B, For non-sliding filtering with the same MMRT, the average L3-RSRP difference of skipping pattern example 1 is 0.2dB larger than that of pattern example 2.

Observation 2: With the same MRRT and equal lengths of OW and PW, sliding filtering exhibits slightly better prediction accuracy (0.09dB) than non-sliding filtering. The distinction between the two filtering methods is not pronounced.

Observation 3: Sliding filtering can predict RSRPs on a small time scale and achieve high prediction accuracy.

Observation 4: Across sub-cases 1, 2, and 3, higher UE speeds result in larger prediction errors in the FR1 intra-frequency temporal domain case B.

Observation 5: When MRRT increases from 50% to 75%, the average L3-RSRP difference gradually increases. The RSRP difference at MRRT=75% is 0.38 dB higher than that at MRRT=50%, yet it remains within 0.7dB.

Observation 6: With a similar model structure, cluster-based approach has higher model complexity than the cell-based approach. The more cells a cluster has, the more complexity the AI/ML model will have.

Observation 7: With a similar model structure, cluster-based approach has a larger RSRP difference than the cell-based approach under different observation windows. The gap becomes more evident (from 0.5dB to 1.98dB ) when OW changes from 400ms to 800ms. As the number of cells within a cluster grows, the average L3-RSRP difference tends to increase.

Observation 8: The average L3-RSRP increases from 0.28dB to 0.8dB when the OW of the serving cell doubles. In FR1 inter-frequency prediction, a larger OW does not necessarily equate to better prediction accuracy.

Observation 9: When OW extends from 800ms to 1600ms, the L3-RSRP difference of different lengths of OW is almost the same under the same PW. While increasing the length of PW from 400ms to 1600ms tends to deteriorate prediction accuracy under the same OW.

Observation 10: Initially, increasing the OW length can enhance prediction accuracy in the temporal domain case A, especially when the OW is relatively short. However, once the OW exceeds a certain threshold (e.g., 800ms), further increases do not yield significant benefits. Conversely, for PW, longer durations correlate with decreased prediction accuracy.

Observation 11: For both filtering options, the average RSRP difference increases as UE speed increases from 60km/h to 120km/h in FR2 to FR2 intra-frequency temporal domain case A.

Observation 12: Among sub cases 1, 2, and 3, sub case 2 demonstrates the highest prediction accuracy, with sub case 1 being the second best. The prediction error of sub cases 1 and 3 is more than 1.08 dB larger than that of case 2 at 60km/h.

Based on these observations, it is proposed that

Proposal 1: Confirm that the cluster-based approach is optional and of low priority in the study and capture the following observation into TP: The cell-based approach outperforms the cluster-based approach in terms of model complexity, computational complexity, and prediction accuracy.

Proposal 2: For the FR1 inter-frequency case, a small OW length, e.g., 400ms, is sufficient for making prediction. There is no need to introduce a longer OW length.

Proposal 3: Capture the following observation into TP:

- For FR2 intra-frequency temporal domain case A, increasing the OW length can improve prediction accuracy when OW is short. However, beyond a certain threshold (e.g., 800ms), additional increases do not provide benefits. For PW, longer durations lead to worse predictions.

Proposal 4: Focus on sub case 2 for indirect measurement event prediction in simulations to reduce the workload for companies.

Reference

R2-2406310	, Discussion on simulation result of RRM measurement, OPPO

R2-2406422, Evaluation on RRM measurement prediction, ZTE

R2-2406579, Evaluation and Simulation Results for AIML RRM Prediction, MediaTek Inc.

R2-2407376, Initial simulation results for RRM measurement predictions, InterDigital Inc.