3GPP TSG-RAN WG2 Meeting #127bis                                                               R2-2407992

Hefei, China, Oct 14th – 18th, 2024

Source:	CATT

Title:	Adaptation of Common signal channel transmissions

Agenda Item:	8.5.4

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting, agreements on paging adaptation were:

Agreements on paging adaptation 1. Option-a) is about the bundling of PF for R19 NES UEs. 2. R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

And a long post email discussion [POST127][109][NES] was assigned. In this contribution, further considerations on adaptation of common signal/channel transmission are provided in section 2. Our proposals are summarized in Section 3.

Discussion

Adaptation of Paging

- Solutions

- Option a) Bundle paging frames

- Option b) Extend the values of N and Ns

With option a), new PF formula is introduced. Option b) introduces new values for N, Ns and firstPDCCH-MonitoringOccasionOfPO without changing PF formula. The specification impact of option b) is less than option a).

According to the email discussion [1], majority view is that no major difference between option a) and option b) in terms of system performance as option-b) can reach same time position for POs via appropriate configurations of firstPDCCH-MonitoringOccasionOfPO.

With option b), Ns and firstPDCCH-MonitoringOccasionOfPO values are extended. The signaling overhead of option b) is more than that of option a). But it is acceptable with appropriate extended Ns value and firstPDCCH-MonitoringOccasionOfPO value. For example: With option a), assume Ns=1 while the number of bits for firstPDCCH-MonitoringOccasionOfPO is 8bit (1 radio frame =140symbol). The overhead for paging monitoring occasion is 8bit. While with option b), Ns=8 while the number of bits for firstPDCCH-MonitoringOccasionOfPO is 11bit (8 radio frame =1120symbol). The overhead for PO monitoring occasion is 88bit (8*11). With appropriate signal overhead, flexible paging monitoring occasion and network energy saving can be achieved.

Table 1 Pros and cons of paging adaptation

|  | Option a) Bundle paging frames | Option b) Extend the values of N and Ns |
| --- | --- | --- |
| Spec impact | More. New PF formula with new parameter(s) is introduced. | Less. New values for N, Ns and firstPDCCH-MonitoringOccasionOfPO are introduced without changing PF formula. |
| System performance | Same | Same |
| Signalling overhead | Less | More but acceptable which achieves flexible paging monitoring occasion and network energy saving. |

According to pros and cons in table 1, we prefer option b).

Proposal 1: For paging adaptation, Option b is adopted.

- Legacy UE compatibility

In RAN2#125 meeting, we agreed to discuss two options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UEs.

3. For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE: - Option 1: Prevent the access of legacy UE via barring; - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

For adaptation of common signal/channel transmission, there is a note in WID [2] that there shall be no negative impact to legacy UEs, unless significant benefits are shown. Therefore, option 2 should be supported at least.

But it is a bit confusion of option 2. In our understanding, additional parameters for paging adaptation need to be introduced. This may lead some PF(s)/PO(s) for paging adaptation are different from legacy paging mechanism while some PF(s)/PO(s) for paging adaptation maybe overlapped with legacy paging. And it is recommended from network energy saving aspect to achieve overlap PF(s)/PO(s) between paging adaptation and legacy paging mechanism as much as possible. Therefore, we want to clarify that for option 2 separate paging parameters, not separate paging resources, for Rel-19 NES UEs are introduced if there are legacy UEs in the cell. Overlapping between legacy PF(s)/PO(s) and Rel-19 PF(s)/PO(s) for paging adaptation are allowed.

Proposal 2: For paging adaptation, Option 2(i.e. both paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) are broadcast in a cell) is adopted.

Proposal 3: Overlapping between legacy PF(s)/PO(s) and Rel-19 PF(s)/PO(s) is allowed.

- Mechanism to activate/deactivate the paging adaptation resources

Assume there are legacy paging mechanism and Rel-19 paging mechanism (e.g. option b)) in a cell. There are two possible solutions for to activate/deactivate the paging adaptation resources

- Solution 1:  Via system information update.

With the solution, the paging adaptation parameters are provided in system information if NW wants to activate the paging adaptation. The Rel-19 NES UE applies Rel-19 paging mechanism immediately upon receiving corresponding parameters in system information. The paging adaptation parameters are removed from system information if NW wants to deactivate the paging adaptation. The UE is notified of this via the legacy system information modification mechanism. And when paging adaptation parameters are absent in system information, the Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters.

- Solution 2: Via Short Message in paging DCI

With the solution, parameters for Rel-19 paging mechanism (e.g. option b)) are broadcasted in SI but not activated immediately. The Rel-19 NES UE applies legacy paging mechanism with received legacy paging parameters if Rel-19 paging mechanism is not activated. If the network wants to activate Rel-19 paging mechanism, Short Messages with paging adaptation indication is sent in one modification period. Then the UE can apply Rel-19 paging mechanism in the next modification period without acquiring system information for parameters of Rel-19 paging mechanism.

Both solution 1 and solution 2 are adapted Rel-19 paging mechanism in a semi-static way. But solution 2 needs to use the reserved bits in Short message although it may reduce the UE power saving without acquiring system information for parameters of Rel-19 paging mechanism. There are only 4 bits not used in Short message. Considering the limited reserved bits in Short message and the limit benefit of solution 2, solution 1 is preferred.

Proposal 4a: The legacy System information update procedure is used to activate/deactivate paging adaptation mechanism.

Proposal 4b: When parameters for paging adaptation is absent in the R19 NES cell, R19 NES UE follows legacy paging mechanism.

Adaptation of PRACH in time domain

RAN1 had already achieved many agreements on adaptation of PRACH in time domain, including:

- The adaptation mechanism for additional PRACH resources is applied to both CONNECTED mode UEs and IDLE/INACTIVE mode UEs.

- At least DCI based adaptation is supported. No introduction of new DCI format.

- SSB-RO mapping rules for additional PRACH resources support:

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

Based on RAN1 progress, RAN2 can start discuss scenarios from RAN2 point of view.

- CBRA or CFRA

The adaptation mechanism for additional PRACH resources is applied to both CONNECTED mode UEs and IDLE/INACTIVE mode UEs. It is straightforward to support CBRA for adaptation mechanism for additional PRACH resources. And CFRA is dedicated resource, the benefit of PRACH adaptation is not quite clear.

Proposal 5: Support CBRA for adaptation mechanism for additional PRACH resources.

- RA type

RAN1 discussions mainly focus on PRACH resource. If 2-step RA type is supported for PRACH adaptation, it is unclear whether additional PUSCH resources are also considered. Hence, RAN2 can focus on 4-step RA at first. And RAN2 can wait for RAN1 discussion whether 2-step RA can be also supported.

Proposal 6: RAN2 focus on 4-step RA for adaptation mechanism for additional PRACH resources.

- Whether to support RACH adaptation to RACH resources of R17 RACH partitions

In Rel-17, RACH partitioning was introduced. A list of RACH partitions (i.e. AdditionalRACH-Config) can be configured in SIB1, and the network can configure different RACH partitions (i.e. AdditionalRACH-Config) associated with different features (e.g., SDT) or feature combination (e.g., SDT+redcap). We need to clarify whether the RACH resource adaptation is supported for R15 RACH resources configuration only or for both R15 RACH resources configuration and R17 RACH partition resources configuration. In our understanding, it would be simpler if RACH resource adaptation is supported for R15 RACH resources configuration only on R19 NES cell. And when RACH adaptation is configured in the NES cell, there is no need to configure R17 RACH partitions in the NES cell for the sake of energy saving.

Therefore, it is proposed that,

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it only for one or more features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition.

Adaptation of SSB in time domain

In RAN1#118, agreements on scenarios for SSB adaptation included:

Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

For adaptation of SSB on SCell, RRM measurement may be impacted. But scenarios for SSB adaptation are still under discussion in RAN1. RAN2 can wait for RAN1 further conclusion on the topic.

Proposal 8: For adaptation of SSB, RAN2 can wait for RAN1's further conclusion.

Conclusion

According to the analysis in section 2, we propose:

Adaptation of paging

Proposal 1: For paging adaptation, Option b is adopted.

Proposal 2: For paging adaptation, Option 2(i.e. both paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs) are broadcast in a cell) is adopted.

Proposal 3: Overlapping between legacy PF(s)/PO(s) and Rel-19 PF(s)/PO(s) is allowed.

Proposal 4a: The legacy System information update procedure is used to activate/deactivate paging adaptation mechanism.

Proposal 4b: When parameters for paging adaptation is absent in the R19 NES cell, R19 NES UE follows legacy paging mechanism.

Adaptation of PRACH in time domain

Proposal 5: Support CBRA for adaptation mechanism for additional PRACH resources.

Proposal 6: RAN2 focus on 4-step RA for adaptation mechanism for additional PRACH resources.

Proposal 7: At least support the case when RACH resource adaptation is not restricted to any certain features. FFS whether to support it only for one or more features including (e)RedCap and/or Slicing and/or SDT and/or MSG3 repetition and/or MSG1 repetition.

Adaptation of SSB in time domain

Proposal 8: For adaptation of SSB, RAN2 can wait for RAN1's further conclusion.

Reference

- R2-2407974, Summary of [POST127][109], OPPO

- RP-242354, Revised WID: Enhancements of network energy savings for NR