3GPP TSG-RAN WG2 Meeting #127											    R2-2406707

Maastricht, Netherlands, Aug 19th – 23rd, 2024

Source: 	Xiaomi

Title:  	Discussion on the measurement event evaluation for LTM

Agenda Item:	8.6.3

Document for:	Discussion and Decision

# Introduction

According to the discussion on the event-triggered L1 measurement in the RAN2#126 and RAN2#125bis, RAN2 made the following agreements on the details of the measurement event evaluation behaviours:

Agreements on measurements:

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In this contribution, we provide our understandings on the measurement event evaluation for LTM.

# Discussion

## Measurement used for event evaluation

As RAN2 already agreed that “use of beam level measurement result for event evaluation is baseline”. We think that using cell level measurement for L1 measurement event evaluation is not needed, since the intention of the L1 measurement event report is to find a better beam serving the UE, and the same level of measurement (i.e. beam level measurement for L1 measurement event, and cell level measurement for L3 measurement event) should be used for a measurement event. If the network wants to know the cell level measurement results, the network can still use L3 measurement event, or have the cell level measurement piggy backed with the beam level measurement results reported for the L1 measurement event.

Proposal 1: The cell level measurement is not used for the L1 LTM measurement event evaluation.

Regarding the beam(s) of the serving cell and neighboring cell used for the event evaluations, we think that at least the indicated TCI state for either the serving cell or the neighbouring cell should be supported, so that the network can have the freedom to change its serving beam to any candidate beams.

Proposal 2: The indicated TCI state of the serving cell and neighbouring cell is used for event evaluation.

In addition, we think that the update of the activated TCI state is also important before the cell change, as the latency of switching the indicated TCI state is lower than the latency of switching the serving cell. Then when the UE is configured with more than one activated TCI states, the worst activated TCI state should be used to trigger the cell change and the beam change for LTM. Then, when/whether to use the indicated TCI state or the worst activated TCI state can be configurable via RRC message.

Proposal 3: The worst activated TCI state of the serving cell is used for event evaluation.

Proposal 4: Whether to use the indicated TCI state or the worst activated TCI state for event evaluation is be configurable via RRC configuration.

## Event LTM1

Regarding the “Event LTM 1 Beam of serving cell becomes better than absolute threshold” (same as “Event-3: Quality of a new beam is better than a certain threshold” discussed in the Rel-19 MIMO), we think that since the Rel-19 MIMO item have already defined some events to change the beam of serving cell according to the RAN1 MIMO agreements as quoted in the Annex, the Rel-19 LTM work item does not need to repeat the same discussion.

Proposal 5: Event LTM 1 “Beam of serving cell becomes better than absolute threshold” is not needed.

## Offset

3GPP TS 38.331: Event 2: Ms + Hys < Thresh Event 3: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off Event 4: Mn + Ofn + Ocn – Hys > Thresh Event 5: Mp + Hys < Thresh1 Mn + Ofn + Ocn – Hys > Thresh2

According to the legacy measurement events based on the cell-level measurement results as the example of Event A3 given above, one event could have the following offset:

- Frequency/measurement object specific offset (e.g. Ofn/ Ofp)

- Cell specific offset (e.g. Ocn/ Ofp)

- Event specific offset (e.g. Off)

According to the RAN2#126 meeting discussion, RAN2 agreed to introduce the beam specific offset, and it is still FFS for cell specific offset. According to our analysis given for the measurement used for event evaluation, the cell level measurement should not be used for the L1 LTM measurement event. Then we think that the cell specific offset is not needed, as the cell specific offset should not be added to the beam level measurement result. Since LTM is to support both intra-frequency and inter-frequency cell change, we think that frequency/measurement object specific offset can be added as the legacy L3 measurement event.

Proposal 6: For LTM event evaluation, cell specific offset is not needed.

Proposal 7: For LTM event evaluation, frequency/ measurement object specific offset is needed.

## Measurement configuration

38.331: LTM-CSI-ReportConfig-r18 ::= SEQUENCE { ltm-CSI-ReportConfigId-r18 LTM-CSI-ReportConfigId-r18, ltm-ResourcesForChannelMeasurement-r18 LTM-CSI-ResourceConfigId-r18, ltm-ReportConfigType-r18 CHOICE { periodic-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), p0alpha P0-PUSCH-AlphaSetId }, aperiodic-r18 SEQUENCE { reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128) }, ... }, ltm-ReportContent-r18 LTM-ReportContent-r18, ... }

According to the RAN2 agreement that L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signalling. To design the signalling association between the measurement event and the measurement resource, RAN2 should firstly decide where/how to config the measurement report in the RRC configuration. Here we consider that we could have the following two options to configure the measurement report:

- Option 1 (as the legacy CSI report (e.g. CSI-MeasConfig)): The LTM report configuration is provided per cell.

- Option 2 (as the L3 measurement report (e.g. MeasConfig)): The LTM report configuration is provided per UE.

From our understanding, the one difference between Option 1 and Option 2 is that Option 1 needs to provide the PUCCH resource used for the UCI reporting of the L1 measurement (if RAN2 decides to use UCI for reporting), and Option 2 does not need to provide reporting resources (if RAN2 decides to use MAC CE for reporting).

Proposal 8: RAN2 is kindly requested to consider the following options for configuring the measurement report of LTM event:

- Option 1: If UCI reporting is agreed for LTM event, the report configuration including the event configuration and the association between the event and the measurement resource is configured in CSI-MeasConfig.

- Option 2: If MAC CE reporting is agreed for LTM event, the report configuration including the event configuration and the association between the event and the measurement resource is configured in MeasConfig.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: The cell level measurement is not used for the L1 LTM measurement event evaluation.

Proposal 2: The indicated TCI state of the serving cell and neighbouring cell is used for event evaluation.

Proposal 3: The worst activated TCI state of the serving cell is used for event evaluation.

Proposal 4: Whether to use the indicated TCI state or the worst activated TCI state for event evaluation is be configurable via RRC configuration.

Proposal 5: Event LTM 1 “Beam of serving cell becomes better than absolute threshold” is not needed.

Proposal 6: For LTM event evaluation, cell specific offset is not needed.

Proposal 7: For LTM event evaluation, frequency/ measurement object specific offset is needed.

Proposal 8: RAN2 is kindly requested to consider the following options for configuring the measurement report of LTM event:

- Option 1: If UCI reporting is agreed for LTM event, the report configuration including the event configuration and the association between the event and the measurement resource is configured in CSI-MeasConfig.

- Option 2: If MAC CE reporting is agreed for LTM event, the report configuration including the event configuration and the association between the event and the measurement resource is configured in MeasConfig.

# Reference

[1] RAN2#125 meeting minutes

[1] RAN2#126 meeting minutes

# Annex

## RAN1 MIMO agreements for event-triggered L1 measurement

### RAN1#116 meeting agreement

Whole picture Agreement On UE-initiated/event-driven beam report, at least of following aspects should be included: - Trigger-event detection for beam reporting by UE - UE monitors RS to assess if a beam-reporting trigger condition has been met - FFS: Trigger condition for declaring beam-reporting event - Beam-report transmission by UE - Signaling contents in the beam report - Down-selection one or more options (strive for one) between the following options as signaling medium/container for beam report transmission - MAC-CE - UCI - Others are not precluded. On UE-initiated/event-driven beam report, the following aspects may be included: - UE requesting UL resource(s) for the beam report - UE notifying transmission of beam report - gNB preconfigured resources Other procedure(s) as required

Events Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, RAN1 further study at least the following aspects: quality metrics, event-definition and threshold. - Further study trigger events, including the following example as a starting point - Event-1: Quality of the current beam is worse than a certain threshold. - Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - Event-3: Quality of a new beam is better than a certain threshold. - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2. - Others are not precluded. - Note: Companies are encouraged to provide details on procedure (e.g. how it is used) related to their preferred event

Measurement resource Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP.

Signalling contents Agreement On UE-initiated/event-driven beam reporting, regarding signaling content(s), at least support DL RS resource indicator and L1-RSRP - FFS: Study and decide whether additional contents can be supported. - FFS: L1-RSRP format, e.g., absolute and/or differential value. - Note: Above does not imply to preclude discussion on L1-RSRP filtering. - The actual reported content depends on the triggering event - Support of one or multiple events will be discussed separately

### RAN1#116bis meeting agreement

Signalling Medium Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. - FFS: Details.

Event 2 Agreement On UE-initiated/event-driven beam reporting, regarding trigger-event detection for beam reporting, at least support Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. - At least L1-RSRP is supported as quality metrics used for Event-2 - FFS: How the L1-RSRP is used to determine the triggering event (e.g. timer, counter, filter coefficient) - FFS: Whether the network controls how the L1-RSRP is used to determine the triggering event - Regarding RS measurement for the new beam for Event-2, down-select one or more of the following: - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured by RRC (e.g., reusing legacy configuration of RS measurement or in TCI-State) or MAC-CE - Option-3b (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c (implicit manner): The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of configured TCI state(s). - Note-1: ‘New/current beam’ is for discussion purpose. - Note-2: Other trigger events/quality metrics (e.g., L1-SINR) are not precluded. - Note-3: For above implicit manner(s), if there are two QCL RSs in a TCI state, the measurement RS is derived from RS w.r.t. QCL-TypeD, if applicable. Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, ‘current beam’ is a beam corresponding to the indicated TCI state. - Regarding RS measurement for the current beam for Event-2, Option-2a is supported: - Option-2a (implicit manner): The RS for current beam is implicitly derived from a QCL RS of indicated TCI state. - FFS: The RS for current beam can be either the QCL RS in the indicated TCI state or the SSB which is QCLed with the QCL RS in the indicated TCI state. - FFS: Option-2c (explicit manner): The RS for current beam is explicitly configured by RRC or MAC-CE. - Note: SSB or CSI-RS can be configured Agreement On UE-initiated/event-driven beam reporting, regarding Event-2, the threshold value is RRC configured

Other event Agreement On UE-initiated/event-driven beam reporting, further study the following trigger events: - Event-1: Quality of the current beam is worse than a certain threshold. - Event-3: Quality of a new beam is better than a certain threshold. - Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2. - Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold. - Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting). - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality. - Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam. - Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Report content Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, the following options are provided for down-selection (other options are not precluded) in RAN1#117 - Option-1 (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - Option-1a (variable size): N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - At least one of N reported beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - FFS: Whether the indication of payload size should be provided additionally. - FFS: Details on how value of N is determined by the UE - Option-1b: N beam(s) are reported in the report instance, where N ∈ {1, 2, ..., Nmax} - The N beam(s) should satisfy the condition of Event-2 - Nmax is configured by gNB - Payload size does not vary as a function of N - FFS: Zero-padding can be provided if N is less than Nmax. - Option-2: Only N=1 beam is reported in the report instance - The reported beam should satisfy the condition of Event-2 - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - Other options are not precluded. - FFS: Whether the measurement results for current beam is always reported or can be enabled by RRC. - FFS: When current beam is reported, whether the current beam is counted in the N reported beams. - The selected option shall satisfy Event-2.

### RAN1#117 meeting agreement

Agreement On UE-initiated/event-driven beam reporting, regarding UL signaling content(s) of L1-RSRP report depending on Event-2, in a report instance, at least Option-3 is supported - Option-3: N ≥ 1 beam(s) are reported in the report instance, - At least one of N reported beam(s) should satisfy the condition of Event-2 - N is configured by gNB - FFS: candidate value of ‘N’. - FFS: RRC can enable or disable whether current beam is always reported in addition to the N beams - FFS: Option-1/1a/1b/2. - Above applies at least for the single CC case

Working Assumption On beam report transmission procedure for UE-initiated/event-driven beam reporting - For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report. - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling. - FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved. - FFS: details on the dedicated RRC signaling - Above applies at least for the single CC case.

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, support the both schemes as follows. - Scheme-1: RS for current beam is the QCL RS in the indicated TCI state - FFS: Whether/How to handle the case if only one TRS is configured in the indicated TCI state. - Scheme-2: the RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state. - Enabling one of either Scheme-1 or Scheme-2 is selected by NW. - FFS: The above selection is via an explicit RRC parameter or an implicit manner, e.g., if the RS(s) for new beam are CSI-RS, Scheme-1 is enabled; otherwise, Scheme-2 is enabled. - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. - The above QCL RS is the RS w.r.t. QCL-TypeD, if there are two QCL RSs in the indicated TCI state.

Agreement Regarding RS measurement for the new beam for Event 2, at least Option-3a is supported - Option-3a (explicit manner): The RS(s) for new beam(s) are explicitly configured - FFS: Option-3b/3c - Option-3b: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of activated TCI state(s). - Option-3c: The RS(s) for new beam(s) are implicitly derived from QCL RS(s) of TCI state(s) in a configured subset of the legacy RRC-configured TCI state list

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, for a report instance where N ≥ 1 beam(s) are reported, the following is supported. - RRC can enable or disable whether current beam is always reported - When enabled by RRC, the current beam + N beams from the measurement RSs for new beam(s) are reported - Note: The reported current beam is NOT counted in the N reported beams. - When disabled by RRC, N beams are reported.

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, the DCI format in Step-2 comprises UL-grant DCI format, and the second channel in Step-3 is at least PUSCH. - The UL-grant DCI format at least comprises DCI format 0_1/0_2. - FFS: DCI format 0_3 - FFS: How to trigger the UEI beam report by the UL-grant DCI format - FFS: the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH. - 1-bit field in the DL-grant DCI format is introduced to indicate the transmission of the UEI beam report - The PUCCH resource for HARQ-ACK transmission can be reused to carry both the HARQ-ACK and UEI beam report. - The DL-grant DCI format at least comprises DCI format 1_1/1_2. - FFS: DCI format 1_3

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Agreement On UE-initiated/event-driven beam reporting, regarding trigger events, the following Event-1 and 7a/7b, are provided for down-selection or combination in RAN1#118 (possible outcome is that no new event is supported) - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality. - Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

Agreement Regarding explicit RS configuration for new beam measurement for Event 2, down-select the following options in the RAN1#118: - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration; - FFS: The RS in the RS resource set can be updated by MAC-CE. - Option-2: A list of RS(s) for new beam measurement can be configured by RRC, and a subset can be activated for new beam measurement by MAC-CE. - FFS: If a list size is small, MAC-CE activation is not needed - Option-3: A list of RS resource (s) for new beam measurement can be configured by RRC, and a subset of RS resource(s) in the list can be provided for new beam measurement by indicated TCI state. - Others are not precluded. - FFS: Each RS for new beam measurement should be associated with a configured joint/DL TCI state which can be used as the indicated TCI state

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, besides for scheme-1 and scheme-2, further study the following for handling the case that only one TRS is configured in the indicated TCI state. - Option-1: Introducing additional scheme: the RS for current beam can be a CSI-RS for beam management derived from the QCL RS in the indicated TCI state; - Option-2: Further support TRS as measurement RS of current beam for determining L1-RSRP - Option-3: Introducing additional scheme: The RS for current beam is explicitly configured by RRC or MAC-CE (Option-2C in RAN1 116b agreement). - Option-4: No further enhancement (i.e., in such case, Scheme-2 is used) - Others are not precluded.