3GPP TSG-RAN WG2 Meeting #127bis										    R2-2408022

Hefei, China, Oct 14th – 18th, 2024

Source: 	Xiaomi

Title:  	Discussion on the pathloss offset update via MAC CE and RRC

Agenda Item:	8.12.2

Document for:	Discussion and Decision

# Introduction

According to the LS from RAN1 in [1], RAN1 asks RAN2 to design the pathloss offset configuration via RRC and update via MAC CE.

Agreement For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). “MAC-CE impact: MAC-CE updates the PL offset value(s) configured in one or more joint TCI states or UL TCI states”

In this contribution, we provide our understandings on how the association between PL offset and joint/UL TCI state is configured via RRC, and updated via MAC CE.

# Discussion

## RRC configuration for pathloss offset

According to the RRC parameter list provided in the RAN1 LS in [1], RAN1 made the following agreements for the pathloss offset configuration:

Agreement in RAN1#118: For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB • FFS: Extending the range to lower than -10dB • Step size is 4dB

The example of the RRC configuration of the association between PL offset and joint/UL TCI state can be as follows:

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]], [[ pathlossOffset-r18 ENUMERATED {dB-10, dB-6, dB-2, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R ]] }

The example of the RRC configuration of the association between PL offset and UL TCI state can be as follows:

TCI-UL-State-r17 ::= SEQUENCE { tci-UL-StateId-r17 TCI-UL-StateId-r17, servingCellId-r17 ServCellIndex OPTIONAL, -- Need R bwp-Id-r17 BWP-Id OPTIONAL, -- Cond CSI-RSorSRS-Indicated referenceSignal-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId, srs-r17 SRS-ResourceId }, additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond Mandatory ..., [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]], [[ pathlossOffset-r18 ENUMERATED {dB-10, dB-6, dB-2, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R ]] }

Although RAN1 agreed the value range and step size for pathloss offset, the parameter list from the RAN1 LS in [1] also indicated that the configuration of the value range for the pathloss offset is “up to RAN2”. We think that RAN2 can firstly confirm that the value range of the pathloss offset is [-10, 60] dB, with step size of 4 dB. Here we consider that the value 0 dB can be included, so that the value range is separated in [-10, 0] dB and [0, 60] dB, with the difference between each value in [-10, 0] or [0, 60] as 4 dB.

Proposal 1: The pathloss offset is an ENUMERATED field of a value range [-10, 60] dB, with the step size of 4 dB. Other values are pending for RAN1 decision.

Proposal 2: The pathloss offset includes the value of 0 dB.

## MAC CE for pathloss offset

Figure 1: Example of Pathloss Offset Update MAC CE

Regarding the MAC CE design, we could have the following options:

- Option 1 (fixed size): One MAC CE updates the pathloss offset value for one joint/UL TCI state.

- Option 2 (variable size): One MAC CE updates the pathloss offset value(s) for one or more joint/UL TCI state(s).

From our understanding, both Options can work. For Option 1, the gNB can provide multiple MAC CEs to update the pathloss offset values for more than one joint/UL TCI states. Since Option 2 saves more signalling overheads when more than one pathloss offset values need to be updated, we think that Option 2 should be adopted.

Proposal 3: One MAC CE can update the pathloss offset value(s) for one or more joint/UL TCI-state(s).

According to the RAN1 agreement quoted above, the MAC CE should be used to update the pathloss offset value, when the pathloss offset has already been configured for a TCI state. This means that when the pathloss offset is not configured for a TCI state by RRC configuration, the MAC CE should not be provided to indicate a pathloss offset value for the TCI state without the RRC-configured association to pathloss offset.

Proposal 4: RAN2 is kindly requested to confirm the understanding that Pathloss Offset MAC CE can only provide the pathloss offset value for a joint/UL TCI state, which has the association between the pathloss offset and the joint/UL TCI state as configured by RRC.

# Conclusion

According to the above analysis, we have the following proposals:

Proposal 1: The pathloss offset is an ENUMERATED field of a value range [-10, 60] dB, with the step size of 4 dB. Other values are pending for RAN1 decision.

Proposal 2: The pathloss offset includes the value of 0 dB.

Proposal 3: One MAC CE can update the pathloss offset value(s) for one or more joint/UL TCI-state(s).

Proposal 4: RAN2 is kindly requested to confirm the understanding that Pathloss Offset MAC CE can only provide the pathloss offset value for a joint/UL TCI state, which has the association between the pathloss offset and the joint/UL TCI state as configured by RRC.

# Reference

[1] R1-2407285, LSin, from RAN1, “LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5”