3GPP TSG RAN WG2 Meeting #127bis	R2-2408068

Hefei, Anhui Province, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.3

Source:	CMCC

Title:	Discussion on A-IoT paging

Document for:	Discussion

Introduction

In RAN2#127 meeting, The following progress are made for A-IoT paging, shown as:

Agreements 1 Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

Then in this contribution, we would like to provide some views on remaining issues for A-IoT paging between reader (gNB or intermediate UE) and A-IoT device(s).

Discussion

### 2.1 Message content

2.1.1  Identifier of target A-IoT device(s)

In RAN2#126 meeting, it has been agreed that [2]:

1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion.

It has been agreed that single device ID, group ID and no device ID can be contained in A-IoT paging message, while whether multiple device IDs can be contained need to be confirmed by SA2. For the case that multiple device IDs are contained in A-IoT paging, we have the following observations on the aspects other than use case. If an A-IoT paging containing multiple device IDs is supported, the maximum number of device IDs that can be included is influenced by the length of the device ID and the maximum amount of data that can be transmitted at once in the physical layer. Besides, the case that multiple deive IDs contained in an A-IoT paging may increase the length of A-IoT paging message and thus, increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it. While, the case that multiple device IDs contained in an A-IoT paging may slao bring some benefits. For example, compared with transmitting multiple A-IoT paging each with single device ID, transmitting an A-IoT paging with multiple device IDs may reduce the signaling overhead to some extent, especially when performing the same operation on them.

Observation 1: Containing multiple device IDs in an A-IoT paging message may increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it.

Observation 2: Containing multiple device IDs in an A-IoT paging message may reduce the signaling overhead, especially when performing the same operation on those devices.

2.1.2  Identifier of trigger cause

In RAN2#126 meeting, it is agreed that:

Agreements 1 As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS 2 As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. Clarify in TR that inventory and command doesn’t mean that AIoT paging includes both Inventory and Command in the same message. This doesn’t mean that inventory and command are received by the reader at the same time from upper layer. 3 From RAN2 point of view we will study “Command only” use case. FFS the options on how to support it : Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. Use baseline procedure for “inventory and command”(i.e. first triggers inventory procedure and then sends command)

It is agreed that the baseline procedures of “inventory only” and “inventory and command” case are both initiated by A-IoT paging transmission. For the procedure of “Command only” case, it can also be initiated by A-IoT paging transmission. From our point of view, the A-IoT paging can be not only one message but also a set of messages. For example, during the procedure of “command only” case, an A-IoT paging message whose content similar to A-IoT paging message for “inventory only” and “inventory and command” procedure is transmitted to A-IoT devices to identify target devices and indicate D2R resources for response at first. Then a subsequent A-IoT paging message which contains the command (Write, Read, etc.) is transmitted. In this way, it is beneficial for device monitoring A-IoT paging because the first A-IoT paging message are always in similar formats even for different procedures. Apart form this, even for the “inventory only” and “inventory and command” procedure initiated by A-IoT paging, it may also include a set of messages for device selection and resource allocation. For example, there may be two type of messages among A-IoT paging. One message contains only the identifier of target device(s) which is similar to “Select” command in RFID. The other message allocates D2R resources for response, which is similar to “Query” command in RFID. With that, the flexibility for both selecting target devices and allocating resources can be increased. However, compared to sending only one A-IoT paging message, sending a set of A-IoT paging messages may slightly increase signaling overhead, considering that the physical layer may add preamble, CRC, etc. separately.

With the consideration that A-IoT devices may conduct different follow-up processes according to different trigger cause of A-IoT paging. For example, if the A-IoT paging is used to inventory, the target device(s) perform random access. While, if a read command is sent as an A-IoT paging, the target device(s) may transmit the respond of read command directly. Therefore, an identifier should be contained in A-IoT paging for target A-IoT device(s) identifying different trigger causes (e.g. inventory, read, write, etc.).

Observation 3: For the procedure of “command only” case, transmitting an A-IoT paging message whose content similar to “inventory only” and “inventory and command” procedure at first may be beneficial for device monitoring.

Proposal 1: A-IoT paging procedure may include one message or a set of messages for device selection and resource allocation.

Proposal 2: In A-IoT paging, an identifier should be contained for target A-IoT devices identifying the trigger cause (e.g. inventory, read, write, etc.).

2.1.3  Identifier of multiple access method

In RAN1#118 meeting, it has been agreed that [2]:

Agreement Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following • How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 • How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1 Agreement For D2R scheduling, the following information potentially can be explicitly/implicitly indicated to the device via corresponding PRDCH: - Time domain resources - Frequency domain resources - MCS-like information - Chip duration - ID associated with device(s) - Repetitions FFS: other information FFS: For each information, whether higher-layer signaling and/or L1 R2D control signaling is used

It can be seen that both TDMA and FDMA of D2R transmissions for Msg1 are agreed to be studied by RAN1. To this end, one identifier which marks which multiple access methods (such as: TDMA, FDMA, TDMA and FDMA etc.) is used, should be contained in A-IoT paging. However, whether the identifier is presented explicitly or implicitly in A-IoT paging needs further discussion.

Proposal 3: A-IoT paging can indicate target devices about the multiple access method at least for Msg1 (e.g. TDMA, FDMA, TDMA and FDMA, etc.). FFS through explicit or implicit parameter

2.1.4  Identifier of D2R transmission resources

In RAN2#126 meeting, it has been agreed that:

Agreement - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/pre-configured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

Based on the study in RAN1, the D2R transmission resources may include at least time-domain (such as access occasion) and/or frequency-domain (such as the frequency-shift). From our point of view, the D2R transmission resources can be shared D2R resources or dedicated D2R resources, in which dedicated D2R resources represents the access resources assigned to specific Ambient IoT device, while shared D2R resources denote the common resources which should be shared and competed by all paged Ambient IoT devices. In other words, Ambient IoT devices with dedicated D2R resources can access/respond without competition, while Ambient IoT devices with shared D2R resources will compete with each other and may collide when responding. Note that in order to allocate dedicated D2R resources, one device ID or a list of device IDs (if multiple device IDs containing in an A-IoT paging is supported) should be contained in A-IoT paging. While, for the A-IoT paging targeted to one A-IoT device, it is not necessary to contain explicit indication of D2R transmission resources. For either FDMA or TDMA based access method, the number of paged devices is really helpful for gNB allocated related resources in A-IoT paging. For example, considering TDMA based D2R transmission, if ten devices are targeted, allocating at least ten access occasions is suitable to avoid collisions as soon as possible. Therefore the number of target A-IoT devices should be visible to gNB if available. For other case which CN can not provide the number of target A-IoT devices (such as paging all devices in coverage of a reader), the gNB should estimates the device number, whose method is given in our another papaer [3]. With the number of target A-IoT devices even if it is not very precise, gNB can allocated D2R resources for A-IoT radio more efficiently. Thus, RAN2 is asked to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

Proposal 4: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 5: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 6: RAN2 is suggested to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

2.1.5 Identifier of random access type

In RAN2, 2-step CBRA, 3-step CBRA, CFRA are being studied. For 2-step CBRA, it brings benefits only when the collision does not occur frequently. Considering that the Msg1 which contains both random number and device ID in 2-step CBRA is longer than that of 3-step CBRA, the reader have to wait for longer time when it detects collision during the reception of Msg1. Therefore, if collision occurs frequently in 2-step CBRA, then more time will be wasted. Besides, some companies thinks that there may be some devices which can only support 2-step CBRA. However based on the agreement made in RAN2, 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA. Thus, we do not think it is difficult for A-IoT devices supporting 3-step CBRA. Based on above analysis, 3-step CBRA should be a mandatory support for all A-IoT devices, 2-step CBRA is optional.

While, if different types of random access procedure (e.g. 2-step CBRA, 3-step CBRA, CFRA, etc.) are supported by each Ambient IoT device, an identifier which indicates the type of random access should be contained in A-IoT paging. Note that CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated. However, for random access procedures that cannot rely on implicit indications in A-IoT paging for differentiation, such as 2-step CBRA and 3-step CBRA, an identifier may be necessary for A-IoT devices determining the RA procedure.

Observation 4: For 2-step CBRA, it brings benefits only when the collision does not occur frequently.

Observation 5: 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA.

Observation 6: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Proposal 7: For CBRA, 3-step CBRA should be a mandatory support for all A-IoT devices, 2-step CBRA is optional.

Proposal 8: When both 2-step CBRA and 3-step CBRA are supported, an identifier indicating the type of random access may be contained in A-IoT paging.

2.1.6 Other identifier

- Identifier of the reader or A-IoT paging area

What’s more, in NR, the paging triggered by CN is broadcasted among all cells within the corresponding TAI list, while the paging triggered by RAN is broadcasted among all cells within the RAN Notification Area (RNA). For Ambient IoT, it has been approved that RAN2 assumes that the device will not support tracking/RAN area update procedure. To improve the efficiency of A-IoT paging, it is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated reader/area. For example, a command or inventory message can be sent to the reader associated with the target device which sends an A-IoT paging to target device as an initial attempt. If the response of target device is not received, that is, A-IoT paging failure occurs, more readers can be selected to send A-IoT paging. With the consideration of support the association, re-association, etc. efficiently, there may also be the identifier of the reader or something like A-IoT paging area contained in A-IoT paging. For example, when a reader ID is contained in A-IoT paging, the A-IoT device can identify its association relationship with reader by recording reader ID. Then when an A-IoT device finds the reader ID contained in A-IoT paging is different from the reader ID saved, it can perceive its association relationship with reader changes. With that, the reader can inventory the A-IoT device(s) whose association relationship with reader changes by transmitting specific A-IoT paging instead of always inventorying all A-IoT devices. Because inventorying all devices within a reader’s coverage each time wastes too much resources.

Observation 7: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Observation 8: If an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging and saved by devices,devices can perceive their association relationship changes. Otherwise, reader have to inventory all devices each time for gNB or CN updating the association relationship.

Proposal 9: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

### 2.2 Initiation

From our point of view, an A-IoT paging can be initiated by core network to inventory A-IoT device(s) or send read or write command to the A-IoT device(s). As for reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system saved in reader side which is not in RAN2 scope. For example, if the association relationship is saved in gNB-reader, then it may trigger an A-IoT paging to update the association relationship between reader and A-IoT device(s). For the cases that reader triggers multiple/subsequent A-IoT paging messages associated with same request from CN, they should essentially be classified as CN-initiated A-IoT paging as well. Besides, for the periodical A-IoT paging which is discussed by RAN3, it also belongs to CN-initiated A-IoT paging, because the period for transmitting A-IoT paging is configured by CN. Based on above analysis, RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Observation 9: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system (such as the association relationship) saved in reader side.

Observation 10: Subsequent A-IoT paging associated with same CN request and periodical A-IoT paging should also be considered as CN-initiated A-IoT paging .

Proposal 10: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

### 2.3 Transmitting behavior

In RAN1#117, it has been agreed as following [4]:

Agreement For R2D, the only physical channel is PRDCH. - PRDCH carries any higher-layer payload - PRDCH carries L1 R2D control information (if defined) - FFS details of device behaviour(s) for receiving PRDCH

It is clear that PRDCH is the only physical channel for R2D, thus, the A-IoT paging is transmitted on PRDCH.

Proposal 11: The A-IoT paging is transmitted on PRDCH by a reader to A-IoT devices.

In RAN2#125-bis meeting, it has been approved that:

- RAN2 assumes that RRC layer is not necessary between the reader and the device. RAN2 will continue to study the functionalities required and later discuss whether we will have: 1) a new AS protocol on top of A-IoT MAC layer; or 2) A-IoT MAC

From our point of view, the introduction of a new AS protocol on top of A-IoT MAC layer is not necessary but increasing the complexity and power consumption of A-IoT devices. To this end, A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 12: A-IoT paging is transmitted in A-IoT MAC layer.

### 2.4 Device unavailability due to energy harvesting

In RAN1#118 meeting, the following agreements has been made [2]:

For the study of the potential impact of device unavailability due to charging by energy harvesting, the following directions are captured in TR 38.769: - Direction 1: Reader does not provide information to a device regarding when the device may become available/unavailable. - Direction 2: Reader can provide information to a device based on which the device may become available/unavailable. - Note: The applicability of Direction 1 and/or 2 to different device types 1/2a/2b may be further discussed. - Note: Direction 1 and Direction 2 are not for down-selection.

From RNA1’s perspective, there are two directions for determining device unavailability. For direction 1, we do not see any RNA2’s impact since reader does not need to provide any information for determining device availability/unavailability. For direction 2, some information which indicates device become available or unavailable may need to be contained in A-IoT paging. From our point of view, direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1. To this end, RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy before discussing whether some related information needs to be contained in A-IoT paging.

Observation 11: For direction 1, there seems to be no RAN2’s impact. Direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1.

Proposal 13: RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy before discussing whether some related information needs to be contained in A-IoT paging.

### 2.5 Monitoring behavior and determination of transmission/access occasion

In RAN2#126 meeting, it is agreed that:

- From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

While, based on the RAN1’s study and RAN2’s discussion on device unavailability, the above agreement does not seem to make sense. From our point of view, device unavailability may also need to be take into consideration for devices’ monitoring behavior. Thus, we suggest to revise above agreement as following:

From RAN2 perspective, we assume the device can receive as long as it is available. We will wait for RAN1 further progress on device unavailability.

Note that device is available means that it is in “ON” state, not in “Sleep” or “OFF” state.

Proposal 14: RAN2 assume the device can receive as long as it is available, i.e. it is in “ON” mode, not in “Sleep” or “OFF” state. Wait for RAN1 further progress on device unavailability.

For determining the corresponding D2R transmission/access occasion after A-IoT paging, it has been discussed in RAN2#127 meeting. The result is waiting for further RAN1 progress on indication of the start of access occasion, since it is closely related to the timing accuracy of device 1, 2a and 2b. Considering the fact that the initial clock accuracy of device 1 is about 10^4 ~ 10^5 ppm which is the evaluation assumption in RAN1, it should be really difficult for device 1 determining the transmission/access occasion efficiently. Thus, in order to have a harmonized design, a R2D clock message, which is similar to QueryRep in RFID, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging. For the case that A-IoT paging provides all subsequent scheduling information without any R2D clock message, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously. From RAN2’s perspective, we can assume there is a R2D clock message which helps devices determining their transmission/access occasions as a baseline and follow RAN1 further progress.

Observation 12: With the consideration of device 1’s timing accuracy, a R2D clock message, which is similar to QueryRep in RFID, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 13: For the case that A-IoT paging provides all subsequent scheduling information without any R2D clock message, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Proposal 15: RAN2 can assume there is a R2D clock message which helps devices determining their transmission/access occasions as a baseline and follow RAN1 further progress.

### 2.6 Subsequent transmission

In RNA2#127 meeting, it has been approved as following:

- Agreements: - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

Considering the device unavailability caused by energy status, triggering multiple A-IoT paging messages which are associated with the same request from CN should be supported. However, each paged device is expected to respond only once for one request from CN, otherwise, duplicated response may case more collisions and greatly decrease the efficiency of the A-IoT system. To avoid the duplicated response from devices, there are following two possible options:

- Option 1: Maintaining a flag and a timer in device side.

- For option 1, a flag and a timer should be maintained by A-IoT device so that it can record whether it has accessed successfully or not. For the flag, there should be at least two status, such as “accessed” and “failed”. To be specific, if one device has accessed successfully, it should change its flag to “accessed” and maintain it for a period of time, e.g. T1 based on the timer. Then when the timer is not expired, the device is not expected to respond an other A-IoT paging message. After the timer is expired, the flag transits to “failed” state. If option 1 is selected, there is no impact on A-IoT paging message. However, the flexibility for reader transmitting A-IoT paging may decrease, because the latest accessed device can only monitor A-IoT paging at least after T1. Apart from this, the design of timer length which seems difficult to determine and timing accuracy of devices will affect Option 1’s performance greatly.

- Option 2: Containing an identifier representing the CN request in A-IoT paging message.

For option 2, an identifier representing the CN request, such as transaction ID, can be contained in A-IoT paging message. For the same request from CN, the same transaction ID should be contained in the A-IoT paging messages which are associated the mentioned CN request. With that, if a device is accessed successfully after being paged by an A-IoT paging, it should save the transaction ID contained in the A-IoT paging message. After that if another A-IoT paging message which contains the same transaction ID as the saved one, the device will not respond to it. If option 2 is selected, the A-IoT device need to save the transaction ID in its RAM or ROM which increases devices’ complexity. Apart from this, considering that the transaction ID should also be contained in A-IoT paging message, the signaling overhead also increases. However, option 2 offers more flexibility for reader transmitting the subsequent A-IoT paging message than option 1.

Based on above analysis, Option 2 is preferred since it is more flexible than Option 1 and the overhead of Option 2 can be mitigated by reducing the length of transaction ID.

Observation 14: For Option 1 mentioned above, there is no impact on A-IoT paging message. However, the flexibility for reader transmitting A-IoT paging may decrease and its performance will be affected by the design of timer length and timing accuracy of devices.

Observation 15: For Option 2 mentioned above, an identifier representing the CN request is contained in A-IoT paging message and devices should save it. However, its overhead of Option 2 can be mitigated by reducing the length of transaction ID.

Proposal 16: To avoid the duplicated response for multiple/subsequent A-IoT paging from devices, there are following two options:

- Option 1: Maintaining a flag and a timer in device side.

- Option 2: Containing an identifier representing the CN request in A-IoT paging message.

Option 2 is preferred.

Conclusion

Based on the discussions mentioned above, in this contribution we provide some discussions on paging for Ambient IoT:

Observation 1: Containing multiple device IDs in an A-IoT paging message may increase the energy consumption for an A-IoT device receiving the whole A-IoT paging message before determining whether the A-IoT paging is target to it.

Observation 2: Containing multiple device IDs in an A-IoT paging message may reduce the signaling overhead, especially when performing the same operation on those devices.

Observation 3: For the procedure of “command only” case, transmitting an A-IoT paging message whose content similar to “inventory only” and “inventory and command” procedure at first may be beneficial for device monitoring.

Observation 4: For 2-step CBRA, it brings benefits only when the collision does not occur frequently.

Observation 5: 3-step CBRA do not need extra capability except for transmitting random ID and device ID separately compared to 2-step CBRA.

Observation 6: CBRA and CFRA may be distinguished by whether dedicated D2R transmission resource is allocated or only shared D2R transmission resource is indicated in an A-IoT paging.

Observation 7: It is beneficial for gNB or CN to maintain an association relationship between A-IoT devices and associated area to improve the efficiency of A-IoT paging.

Observation 8: If an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging and saved by devices,devices can perceive their association relationship changes. Otherwise, reader have to inventory all devices each time for gNB or CN updating the association relationship.

Observation 9: For reader-initiated A-IoT, whether it exists depends on whether there is some information of A-IoT system (such as the association relationship) saved in reader side.

Observation 10: Subsequent A-IoT paging associated with same CN request and periodical A-IoT paging should also be considered as CN-initiated A-IoT paging .

Observation 11: For direction 1, there seems to be no RAN2’s impact. Direction 2 may only be feasible for device 2a and 2b, whether it is also feasible for device 1 is closely related to the timing accuracy which is still being studied in RAN1.

Observation 12: With the consideration of device 1’s timing accuracy, a R2D clock message, which is similar to QueryRep in RFID, may be necessary especially when a large number of transmission/access occasions indicated by an A-IoT paging.

Observation 13: For the case that A-IoT paging provides all subsequent scheduling information without any R2D clock message, it may cause higher signaling overhead and its performance is affected by the timing accuracy of devices seriously.

Observation 14: For Option 1 mentioned above, there is no impact on A-IoT paging message. However, the flexibility for reader transmitting A-IoT paging may decrease and its performance will be affected by the design of timer length and timing accuracy of devices.

Observation 15: For Option 2 mentioned above, an identifier representing the CN request is contained in A-IoT paging message and devices should save it. However, its overhead of Option 2 can be mitigated by reducing the length of transaction ID.

Proposal 1: A-IoT paging procedure may include one message or a set of messages for device selection and resource allocation.

Proposal 2: In A-IoT paging, an identifier should be contained for target A-IoT devices identifying the trigger cause (e.g. inventory, read, write, etc.).

Proposal 3: A-IoT paging can indicate target devices about the multiple access method at least for Msg1 (e.g. TDMA, FDMA, TDMA and FDMA, etc.). FFS through explicit or implicit parameter

Proposal 4: The dedicated or shared D2R resources can be indicated in an A-IoT paging.

Proposal 5: There may be no explicit indication of D2R transmission resources when paging a single device.

Proposal 6: RAN2 is suggested to study the method of estimating the number of paged devices for reader in some cases where CN does not provide the number.

Proposal 7: For CBRA, 3-step CBRA should be a mandatory support for all A-IoT devices, 2-step CBRA is optional.

Proposal 8: When both 2-step CBRA and 3-step CBRA are supported, an identifier indicating the type of random access may be contained in A-IoT paging.

Proposal 9: To improve the efficiency of A-IoT paging, an identifier of the reader or something like A-IoT paging area can be contained in A-IoT paging.

Proposal 10: RAN2 can assume there is no reader-initiated A-IoT paging unless more process is made or at least one typical use case of reader-initiated A-IoT paging are given by other WGs.

Proposal 11: The A-IoT paging is transmitted on PRDCH by a reader to A-IoT devices.

Proposal 12: A-IoT paging is transmitted in A-IoT MAC layer.

Proposal 13: RAN2 can wait for RAN1’s further progress on device unavailability and timing accuracy before discussing whether some related information needs to be contained in A-IoT paging.

Proposal 14: RAN2 assume the device can receive as long as it is available, i.e. it is in “ON” mode, not in “Sleep” or “OFF” state. Wait for RAN1 further progress on device unavailability.

Proposal 15: RAN2 can assume there is a R2D clock message which helps devices determining their transmission/access occasions as a baseline and follow RAN1 further progress.

Proposal 16: To avoid the duplicated response for multiple/subsequent A-IoT paging from devices, there are following two options:

- Option 1: Maintaining a flag and a timer in device side.

- Option 2: Containing an identifier representing the CN request in A-IoT paging message.

Option 2 is preferred.

Reference

- EPC Radio-Frequency Identity Protocols Generation-2 UHF RFID, v2.0.1.

- “RAN1 Chair’s Notes”, RAN1#118, Maastricht, NL.

- R2-2408093, “Further consideration on random access for ambient IoT”, RAN2#127bis, Hefei, China, CMCC.

- “RAN1 Chair’s Notes”, RAN1#117, Fukuoka City, Fukuoka, Japan.