3GPP TSG-RAN WG2 Meeting #127bis		R2-2408117

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127 meeting, the following agreements were made for inter-CU LTM [1]:

Clarification of DC

Reference configuration:

- Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported.

CSI resource and report configuration:

- The Rel-18 signaling structure for LTM CSI resource and report configuration is reused for inter-CU LTM, i.e. a common CSI resource configuration and cell-specific CSI report configuration.

- The source CU is responsible to generate the common CSI resource configuration.

Early DL sync:

- For inter-CU LTM, the R18 candidate TCI State activation/deactivation design (including MAC CE and related UE handling) is reused.

Inter-CU LTM switch:

- For inter-CU LTM cell switch, it’s the source DU that triggers the MAC CE and informs the source CU about the target LTM cell.

RLC and PDCP re-establishment in inter-CU LTM:

- If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

Handling of candidate configuration after inter-CU LTM cell switch:

- As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

DRB/PDU session mismatch in subsequent inter-CU LTM:

- RAN2 understand NW implementation can handle the concern raised in P6, R2-2407421.

LTM and L3 HO:

- L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM):

- When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network.

- The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- For the execution order between CHO and LTM, Rel-18 principle is applied.

Inter-CU SCG LTM:

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

In this paper, we will first discuss the remaining open issues for inter-CU LTM in the SA scenario. Then, we will discuss the scenarios and specification impacts of inter-CU LTM with DC.

- Discussion

  - Remaining issues for Inter-CU LTM in SA

L2 reset for mixture of intra-CU LTM and inter-CU LTM

In the last meeting, it was agreed to introduce a new Rel-19 ID in RRC for UE to decide whether to perform RLC and PDCP re-establishment when performing LTM cell switch.

RLC and PDCP re-establishment in inter-CU LTM:

6.    If the security key update is required, the UE shall perform MAC reset, RLC re-establishment and PDCP re-establishment. As baseline introduce a new Rel-19 ID in RRC: if the Rel-19 ID is different for the source cell and the target cell, the UE performs PDCP re-establishment, including security key update, however dependent on SA3 response, we can revisit it.

In this way, when both inter-CU LTM and intra-CU LTM candidate cells are configured, the UE will first determine whether PDCP re-establishment should be performed by comparing the new Rel-19 IDs of the serving cell and target cell when performing LTM cell switch. If the Rel-19 IDs differ between the serving and the target cell, the UE will perform PDCP re-establishment, including security key update. Otherwise, the UE will proceed to check if RLC re-establishment and PDCP data recovery should be performed by comparing the ltm-ServingCellNoResetID and ltm-NoResetID as defined in Rel-18.

Proposal 1: If the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

UE-based TA measurement

In the last meeting, whether UE-based TA measurement can be supported for inter-CU LTM was discussed. Some companies believe that RAN4 has already defined the UE requirement for UE-based TA measurement and it applies to both intra-CU and inter-CU scenarios. From our understanding, UE-based TA measurement is supported in synchronized cases in Rel-18 according to the reply LS by RAN4 [2]. Therefore, when the serving cell and candidate cells in Rel-19 from different CUs are synchronized, UE-based TA measurement is supported obviously and the Rel-18 mechanism can be reused.

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 2: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

However, for Rel-19 inter-CU LTM, as the serving cell and the candidate cells may belong to different CUs and are more likely to be unsynchronized, it seems highly beneficial to support UE-based TA measurement for unsynchronized scenarios. The decision on whether UE-based TA measurement for unsynchronized cases can be supported and how to support it should be made by RAN1 and RAN4. Therefore, an LS should be sent to RAN1 and RAN4 to inquire about the feasibility and approach for supporting UE-based TA when the serving cell and the candidate cell are unsynchronized.

Proposal 3: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 4: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

CFRA resource in LTM Cell Switch Command MAC CE

For RACH-based LTM, reserving specific CFRA resources for each UE via pre-configured RRC configuration will result in a huge waste of CFRA resources for the candidate cells. To resolve the issue in Rel-18, CFRA resources are supported to be indicated in the LTM cell switch command, which means the CFRA resource can be allocated when the S-DU determines to trigger LTM and prevent the waste of RACH resources.

To address the potential waste of CFRA resources caused by inter-CU LTM, we propose supporting the inclusion of CFRA resources in the LTM Cell Switch Command MAC CE for inter-CU LTM.

Proposal 5:  CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

  - Discussion on inter-CU LTM with DC

2.2.1 Inter-CU SCG LTM

In the last meeting, RAN2 made the following agreements about inter-CU SCG LTM:

RAN2#127 Agreements

- Inter-CU SCG LTM preparation can be initiated by source SN.

- The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message.

- Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19.

- RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works.

Compared with inter-CU MCG LTM, the procedure of inter-CU SCG LTM will involve much network signalling interaction among the MN, the source SN and the candidate SN(s). The network signalling interaction is similar to that in S-CPAC in Rel-18, but there are also some differences. The detailed signalling procedure is provided below, and the highlighted part includes the issues that need to be discussed for inter-CU SCG LTM.

Figure 1. Inter-CU SCG LTM signalling procedure

LTM preparation

1.   The source SN initiates the inter-SN LTM procedure by sending the SN Change Required message, which contains an LTM indication. The message also contains candidate node ID(s) and may include an SCG reference configuration (to support delta configuration), and contains the measurement results related to the candidate SN(s), and the number of PSCells that can be prepared by each candidate SN.

2/3  The MN requests each candidate SN(s) to allocate resources for the UE by means of the SN Addition procedure(s), indicating the request is for LTM, and provides the measurement results related to the candidate SN received from the source SN. The message also contains the target candidate cell ID, the LTM configuration ID of the candidate PSCells, the LTM configuration ID mapping list, and the CSI resource configuration. The MN may request PRACH resources from the candidate SN(s). The MN also provides the upper limit for the number of PSCells that can be prepared by each candidate SN and provides a list of KSN and associated sk-Counter values for each candidate SN. The candidate SN decides the list of PSCell(s) to prepare (considering the maximum number indicated by the MN) and, for each prepared PSCell, the candidate SN decides other SCG SCells and provides the new corresponding SCG radio resource configuration to the MN in an NR RRCReconfiguration** message contained in the SN Addition Request Acknowledge message with the prepared PSCell ID(s). If data forwarding is needed, the candidate SN provides data forwarding addresses to the MN. The candidate SN may also propose data forwarding to the MN or other candidate SN(s) for inter-SN LTM. The candidate SN may include an indication that the SCG radio resource configuration of a prepared PSCell is a complete configuration, i.e. that it is not a delta configuration with respect to the SCG reference configuration.

3a.   For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in

the Xn-U Address Indication message.

4/5. The MN sends a SN Modification Request message to the source SN and candidate SN(s) including the information related to early sync and the LTM configuration IDs for the prepared target candidate cell(s) in other SN(s). The MN may send the updated CSI resource configuration to the source SN and candidate SN(s). The source SN and candidate SN(s) respond with a SN Modification Response message including the updated candidate PSCell configuration(e.g., the updated CSI report configuration).

6.     The MN sends to the UE an RRCReconfiguration message including the LTM candidate configuration, i.e. a list of RRCReconfiguration* messages for the execution of subsequent LTM, in which each RRCReconfiguration* message contains the SCG configuration in the RRCReconfiguration** message received from one of the candidate SN(s) in steps 2 and 3, and possibly an MCG configuration. Besides, the RRCReconfiguration message can also include an updated MCG configuration, as well as the NR RRCReconfiguration*** message generated by the source SN. The RRCReconfiguration message also includes a security update configuration and may also include a reference configuration.

7.   The UE applies the RRCReconfiguration message received in step 6, stores the LTM configuration, and replies to the MN with an RRCReconfigurationComplete message, which can include an NR RRCReconfigurationComplete*** message. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

8/9. If an SN RRC response message is included, the MN informs the source SN with the SN RRCReconfigurationComplete*** message via the SN Change Confirm message.

The MN sends the SN Change Confirm message towards the source SN to indicate that LTM is prepared, and in such case the source SN continues providing user data to the UE. If early data forwarding is applied, the MN informs the source SN the data forwarding address(es), the source SN, if applicable, together with the Early Status Transfer procedure, starts early data forwarding. The PDCP SDU forwarding may take place during early data forwarding. In case multiple candidate SNs are prepared, the MN includes a list of Target SN ID and list of data forwarding addresses to the source SN.

In the LTM preparation procedure above, the S-SN sends the measurement results of candidate PSCell(s) to the MN and the MN forwards these measurement results to the candidate SN(s). Subsequently, the candidate SN(s) decides on its LTM candidate cells. This procedure is more like the legacy SN-initiated inter-SN change, wherein the T-SN determines the target PSCell, rather than inter-SN CPAC, where the S-SN propose candidate PSCells to the T-SN and the T-SN can only choose to accept or reject these candidates. Unlike CPAC, there are no execution conditions related to each LTM candidate cell for LTM candidate cell. Therefore, the source SN does not need to decide on the LTM candidate cells and can just send the measurement results to the T-SN and for the T-SN to make the decision.

Proposal 6: Source SN sends measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN.

In Step 2~5, MN collects the L1 measurement configuration and early synchronization configuration of each candidate PSCell and generates the common CSI resource configuration. Then the MN sends the common CSI resource configuration to the S-SN and each candidate SN and gets the updated L1 measurement report configuration of each candidate PSCell. This process is similar to the procedure in inter-CU MCG LTM.

Proposal 7: MN is responsible to collect the configurations and information of candidate PSCells from multiple candidate SN(s) and generates the common CSI resource configuration for L1 measurement on candidate PSCells.

Proposal 8: The MN needs to inform the candidate SN(s) about the common CSI resource configuration and the collected information (e.g., the early-RACH configuration, the TCI state configuration, etc.) of candidate PSCells from candidate SN(s). The candidate SN(s) responds with the updated candidate configuration to the MN accordingly (if needed).

Similar to S-CPAC, although the LTM candidate configuration is intended for inter-CU SCG LTM, it is possible that the corresponding MCG configuration may also need to be updated when applying the LTM candidate configuration. For instance, the allocation of radio bearers between the MN and SN could change after the inter-CU SCG LTM. If this occurs, the MCG configuration may require reconfiguration as well.

Proposal 9: The LTM candidate configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration, similar to S-CPAC.

Early sync

10. The UE performs DL synchronization with the LTM candidate cell(s) before receiving the cell switch command.

11. Early UL synchronization to the target candidate cell(s) may be performed as indicated by the source SN.

12. The candidate SN sends the TA value, the associated CFRA resource information, the candidate cell ID, the source gNB-CU ID, and the source gNB-DU ID in the SN-MN TA INFORMATION TRANSFER message to the MN.

13. The MN forwards the TA value, and the associated CFRA resource information to the source SN in the MN-SN TA INFORMATION TRANSFER message.

For the early sync procedure, the source SN triggers the UE to perform early RACH to the candidate SN by PDCCH order. After the candidate SN acquires the TA value, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

Proposal 10: After acquiring the TA value via the CFRA triggered by the PDCCH order of the source SN, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

LTM execution

14. The UE sends the L1 measurement results to the source SN.

15. The source SN sends the Cell Switch command to the UE.

16/17. The source SN sends the SN-MN CELL SWITCH NOTIFICATION message to the MN to indicate the initiation of the Cell Switch command to the UE, for which the message includes the target cell ID and the TCI state ID.

18.  If applicable, the MN triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the target PSCell, to start late data forwarding.

19/20.  If PDCP termination point is changed for bearers using RLC AM, the source SN sends the SN Status Transfer message to MN, which the MN sends then to the SN of the target PSCell if needed.

21.  If applicable, data forwarding from the source SN takes place. It may be initiated as early as the the source SN receives the early data forwarding address in step 9.

22.  The UE applies RRCReconfiguration* message corresponding to the target candidate PSCell, and sends an MN RRCReconfigurationComplete* message, including an RRCReconfigurationComplete** message for the target candidate PSCell, and information enabling the MN to identify the SN of the target candidate PSCell. The RRCReconfigurationComplete* message may also include the sk-Counter value associated with the selected candidate PSCell if a new sk-Counter value is selected.

23.  The MN informs the SN of the selected candidate PSCell (i.e. the selected candidate SN) that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the RRCReconfigurationComplete** message. If the sk-Counter value is received by the RRCReconfigurationComplete* message, the MN also indicates the received sk-Counter value to the SN.

24.   The UE performs the random access procedure towards the target cell if UE does not have valid TA of the target cell.

For inter-CU SCG LTM, to help the target SN schedule the UE, the cell switch notification should be sent from the source SN to the target SN via MN after LTM cell switch execution is triggered.

Proposal 11: Source SN notifies the MN after LTM cell switch is triggered, and the MN forwards the cell switch notification to the target SN.

LTM cell switch completion

25.  If the UE has performed a RA procedure in step 7, the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

26.  If applicable, the MN triggers the Xn-U Address Indication procedure to inform the source SN the address of the SN of the selected candidate PSCell, to start late data forwarding.

27.  If applicable, a PDU Session path update procedure is triggered by the MN.

Proposal 12: RAN2 takes the procedure above as baseline for inter-CU SCG LTM signaling procedure.

2.2.2 Inter-CU MCG LTM with SN released or unchanged

In the last meeting, the following agreements were made for the case of inter-CU MCG LTM with SN released or unchanged:

RAN2#127 Agreements

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

- Upon execution of inter-CU MN LTM with DC, the UE is required to perform refresh of security key, re-establishment of RLC and PDCP, and MAC reset at both MN and SN side (i.e. Rel-15 principle is applied).

- For the SN key update in inter-CU MN LTM with DC, the UE applies legacy R15 RRC reconfiguration with sync procedure.

First, what does “SN unchanged” means should be clarified. In our view, the following options can be considered:

- Option 1: The CU of SCG is unchanged (e.g., inter-CU MCG LTM with intra-CU PSCell change is supported)

- Option 2: The SCG is unchanged (e.g., the PSCell and SCell(s) of the current SCG are unchanged)

For option 1, the CU of current SCG remains unchanged when performing inter-CU MCG LTM, which means that intra-SN PSCell change can be performed together with the inter-CU MCG LTM.

For option 2, the SCG itself remains unchanged, meaning that both the PSCell and the SCell(s) are unchanged during the inter-CU MCG LTM. From our perspective, whether PSCell should be changed together with the inter-CU MCG LTM can be determined by the S-SN (also is the T-SN). It is understood that SCG change does not introduce additional specification impact as compared to the scenario where SCG remains unchanged.

Proposal 13: For the case of inter-CU MCG LTM with SN unchanged, intra-CU PSCell change is supported when performing inter-CU MCG LTM.

As we agreed, how to handle the SCG part (e.g. release or reconfiguration) during the inter-CU MCG LTM is up to network implementation. However, different network implementations will result in different UE behaviors and specification impacts. The following two kinds of network implementation are possible from our view:

- Option 1: NW implementation ensures that SN is always released or always unchanged when performing inter-CU MCG LTM with DC.

- Option 2: Each candidate MN decides whether to release the SN or keep the SN unchanged independently, and the UE may perform inter-CU MCG LTM with SN released followed by an inter-CU MCG LTM with SCG unchanged.

Option 1 ensures that inter-CU MCG LTM with SN addition will not happen, but it brings additional restrictions for network implementation. For instance, the MN should be responsible for ensuring that all the prepared LTM candidate cells maintain consistent configurations regarding SN—either all having the SN released or all leaving it unchanged.

For option 2, LTM candidate cells might be configured differently; some may trigger an SN release while others may not. In such scenarios, inter-CU MCG LTM with SCG released followed by an inter-CU MCG LTM involving SN reconfiguration can take place. This implies that inter-CU MCG LTM with SN addition is performed. From our standpoint, including SN addition in inter-CU MCG LTM does not introduce additional specification impact and thus should be supported in Rel-19 inter-CU LTM.

Proposal 14: In cases of inter-CU MCG LTM with SN released or unchanged, each candidate MN should independently decide whether to release the SN or keep it unchanged.

In addition, similar to Rel-17 CHO with SCG, inter-CU MCG LTM with SN unchanged also requires the signaling enhancement between MN and SN to make the SN recognize that the Addition/Modification is related to an LTM. For instance, an indicator should be added in the SN Addition Request and SN Modification Request message. Furthermore, the MN and UE IDs should also be included in the SN Addition Request to help the SN to optimize resource allocation.

Proposal 15: Network signaling enhancements between MN and SN (similar to Rel-17 CHO with SCG) are needed for inter-CU MCG LTM with SN unchanged. Send an LS to inform RAN3.

2.2.3 Coexistence of inter-CU MCG LTM and intra-CU SCG LTM

Coexistence of Intra-CU MCG LTM and inter-CU SCG LTM

In scenarios where the SCG is configured with Rel-18 intra-CU LTM and the MCG is configured with Rel-19 inter-CU LTM, different procedures for processing the LTM candidate configuration be applied, depending on whether the MCG LTM is executed first or the SCG LTM is executed first.

If intra-CU SCG LTM is executed first, the inter-CU MCG LTM candidate configuration can be kept, as the MCG is not expected to be affected by the intra-CU SCG LTM.

Proposal 16: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, UE shall keep the inter-CU MCG LTM configuration when intra-CU SCG LTM is performed.

If inter-CU MCG LTM with SN released is performed, the SCG LTM configuration will be released as part of the SN release procedure in Rel-18 specification. When inter-CU MCG LTM with SN unchanged is performed, the decision to keep or release the SCG LTM configurations can be made by the network and indicated in the SCG part of the MCG LTM candidate configuration.

Observation 2: For the coexistence of inter-CU MCG LTM with SN released and intra-CU SCG LTM scenario, the intra-CU SCG LTM configuration will be released as part of the SN release procedure when inter-CU MCG LTM with SN released is performed.

Proposal 17: For the coexistence of inter-CU MCG LTM with SN unchanged and intra-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU SCG LTM candidate configurations by explicit RRC signalling when performing the inter-CU MCG LTM with SN unchanged.

From the above discussion for the maintenance of LTM candidate configurations when  inter-CU MCG LTM and intra-CU SCG LTM are configured simultaneously, no additional specification impacts are required to support the coexistence; therefore, this scenario ought to be supported in Rel-19.

Proposal 18: UE can be configured with inter-CU MCG LTM and intra-CU SCG LTM simultaneously.

2.2.4 Coexistence of intra-CU MCG LTM and inter-CU SCG LTM

In this scenario, MCG is configured with Rel-18 intra-CU LTM while the SCG is configured with Rel-19 inter-CU LTM.

If the intra-CU MCG LTM is executed first, then according to Rel-18 specifications [3], the SN will be released.

TS 38.331 v18.2.0

mrdc-SecondaryCellGroupConfig

This field is used to configure and release an SCG in NR-DC and NE-DC. In case the RRCReconfiguration message is part of an LTM-Candidate IE associated with the MCG, if this field is present its value can only be set to release.

After SN is released, the inter-CU SCG LTM candidate configurations will be released as well.

Observation 3: In the case of intra-CU MCG LTM and inter-CU SCG LTM coexists, the SCG LTM candidate configurations will be released after the intra-CU MCG LTM is performed, according to the current Rel-18 specification.

As discussed in section 2.2.1, the inter-CU SCG LTM candidate configuration may also include the MCG part configuration. Therefore, if the inter-CU SCG LTM is performed first, the network can indicate whether the intra-CU MCG LTM candidate configuration should be kept or released within the inter-CU SCG LTM candidate configurations.

Proposal 19: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU MCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU SCG LTM.

From the above discussion, no additional specifications are required for the coexistence of inter-CU SCG LTM and intra-CU MCG LTM; therefore, this scenario ought to be supported in Rel-19.

Proposal 20: UE can be configured with intra-CU MCG LTM and inter-CU SCG LTM simultaneously.

- Conclusion

In this paper, we discuss the inter-CU LTM remaining open issues and also the specification impact of inter-CU LTM in DC scenarios. We have the following observations and proposals:

Remaining issues for Inter-CU LTM in SA

Observation 1: If the serving cell and candidate cells are from different CUs and are synchronized, UE-based TA measurement can be supported by reusing the Rel-18 mechanism.

Proposal 1: If the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

Proposal 2: RAN2 confirms that UE-based TA measurement is supported in the synchronized inter-CU LTM scenario, and no specification impact is expected.

Proposal 3: RAN2 confirms that supporting UE-based TA measurement in the unsynchronized case is beneficial for inter-CU LTM from the RAN2 perspective.

Proposal 4: Send an LS to RAN1/4 to ask whether it is feasible to support UE-based TA measurement in unsynchronized cases.

Proposal 5:  CFRA resources can be included in the LTM Cell Switch Command MAC CE for inter-CU LTM. Send an LS to inform RAN3.

Inter-CU SCG LTM

Proposal 6: Source SN sends measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN.

Proposal 7: MN is responsible to collect the configurations and information of candidate PSCells from multiple candidate SN(s) and generates the common CSI resource configuration for L1 measurement on candidate PSCells.

Proposal 8: The MN needs to inform the candidate SN(s) about the common CSI resource configuration and the collected information (e.g., the early-RACH configuration, the TCI state configuration, etc.) of candidate PSCells from candidate SN(s). The candidate SN(s) responds with the updated candidate configuration to the MN accordingly (if needed).

Proposal 9: The LTM candidate configurations of inter-CU SCG LTM can include both SCG configuration and possible corresponding MCG configuration, similar to S-CPAC.

Proposal 10: After acquiring the TA value via the CFRA triggered by the PDCCH order of the source SN, the candidate SN sends the TA value to the MN, and the MN then forwards the TA value to the source SN.

Proposal 11: Source SN notifies the MN after LTM cell switch is triggered, and the MN forwards the cell switch notification to the target SN.

Proposal 12: RAN2 takes the procedure above as baseline for inter-CU SCG LTM signaling procedure.

Inter-CU MCG LTM with SN released or unchanged

Proposal 13: For the case of inter-CU MCG LTM with SN unchanged, intra-CU PSCell change is supported when performing inter-CU MCG LTM.

Proposal 14: In cases of inter-CU MCG LTM with SN released or unchanged, each candidate MN should independently decide whether to release the SN or keep it unchanged.

Proposal 15: Network signaling enhancements between MN and SN (similar to Rel-17 CHO with SCG) are needed for inter-CU MCG LTM with SN unchanged. Send an LS to inform RAN3.

Coexistence of inter-CU MCG LTM and intra-CU SCG LTM

Observation 2: For the coexistence of inter-CU MCG LTM with SN released and intra-CU SCG LTM scenario, the intra-CU SCG LTM configuration will be released as part of the SN release procedure when inter-CU MCG LTM with SN released is performed.

Proposal 16: For the coexistence of inter-CU MCG LTM and intra-CU SCG LTM scenario, UE shall keep the inter-CU MCG LTM configuration when intra-CU SCG LTM is performed.

Proposal 17: For the coexistence of inter-CU MCG LTM with SN unchanged and intra-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU SCG LTM candidate configurations by explicit RRC signalling when performing the inter-CU MCG LTM with SN unchanged.

Proposal 18: UE can be configured with inter-CU MCG LTM and intra-CU SCG LTM simultaneously.

Coexistence of intra-CU MCG LTM and inter-CU SCG LTM

Observation 3: In the case of intra-CU MCG LTM and inter-CU SCG LTM coexists, the SCG LTM candidate configurations will be released after the intra-CU MCG LTM is performed, according to the current Rel-18 specification.

Proposal 19: For the coexistence of intra-CU MCG LTM and inter-CU SCG LTM scenario, network indicates whether to keep or release the intra-CU MCG LTM candidate configurations by explicit RRC signaling when performing the inter-CU SCG LTM.

Proposal 20: UE can be configured with intra-CU MCG LTM and inter-CU SCG LTM simultaneously.

- References

- RAN2#127_ChairNotes.

- R4-2314455, Reply LS beam application time and UE based TA measurement for LTM. Ericsson.

- TS 38.331 v18.2.0, Radio Resource Control (RRC) protocol specification (Release 18).