3GPP TSG-RAN WG2 Meeting #127bis		R2-2408120

Hefei, China, 14th – 18th Oct. 2024

Source:	vivo

Title:	Discussion on LS from SA2 on multi-modality

Agenda Item:	8.7.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis meeting, a replied LS from SA2 is received on multi-modality awareness at RAN [1]:

SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following: Question: In this meeting, as part of this study, RAN2 has agreed that multi-modality awareness at RAN is supported for both UL and DL. To continue the discussion during the study phase, RAN2 needs to understand whether/what information can be provided from SA2 (e.g., MMSID and/or synchronization thresholds in order to allow e.g., coordinated handling of the flows at RAN). Answer: SA2 specified policy control enhancements to support multi-modal services in Rel-18, with the use of Multi-modal Service ID (MMSID) in AF and PCF, please see TS 23.501 clause 5.37.2 for details. Forwarding MMSID to RAN via control plane is in principle technically feasible, but needs confirmation from RAN3 on the NG-RAN protocols impacts. Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work. Such feedback from RAN2 needs to also be assessed by RAN3 and SA4 to confirm the applicability and usage of this information.

In this contribution, we will discuss the RAN usage of multi-modality awareness information mentioned in the reply LS and provide the draft response from RAN2 point of view.

Besides, we also discuss the LS to SA2/SA4 on dependency and synchronization based on RAN2 progress in RAN2#127 meeting.

- Discussion

  - Multi-Modal service information awareness in RAN

In Rel-18, SA2 has supported the multi-modal service via policy control enhancement. Specifically, the AF provides for each data flow that belongs to the multi-modal service a Multi-modal service ID to PCF. Based on the explicit indication of Multi-modal service ID, PCF can identify the associated flows and generate the suitable QoS policy for each data flow. However, the association of the data flows is not visible to RAN by now. The multi-modality association information can be per QoS flow as Multi-modal Service ID (MMSID) currently defined in TS 23.501. Furthermore, such association information could also be per PDU set or per packet. In SA2 R19, potential solution on PDU Set correlation information which identifies how this PDU set is related or depended with other PDU Set within the same QoS flow has also been proposed. For MM services, this implies such correlation information could extend to different QoS flows. In case RAN can be aware of above association information, RAN may perform proper admission control in case of initial PDU session setup and handover scenarios. Besides, integrated handling like scheduling and useless SDU discard can be performed in order to save radio resources and avoid introducing delay to the following services transmission. So that more precise correlation information is needed.

Except Multi-Modal service association information, synchronization information was also discussed in SA2 in R18 study. Similarly, such synchronization information can also be per flow basis or per packet/PDU set basis. Once RAN obtain such synchronization information, it could optimize the scheduling algorithm to consider the relative delay requirement so that to satisfy the synchronization threshold as defined in TS 22.261 for better UE experience. Especially in case PDB/PSDB for each flow cannot directly reflect the requirement cross different QoS flows.

Observation: With multi-modal association and synchronization awareness at RAN, RAN can provide proper admission control and coordination transmission to satisfy the synchronization requirement as in TS 22.261, which has benefit in term of user experience and system capacity.

Technically, RAN could get the above discussed Multi-Modal service information either from core network or UE. Since in Rel-18, SA2 has already specified policy control enhancements to support multi-modal services. It is straight forward to send the MMSID from PCF/AMF to RAN together with other QoS parameters via NGAP messages. For further PDU set level association information and synchronization information per flow or per packet/PDU set, UE could get the information from its APP layer similar as UL PDU sets, Data Bursts and PSI identification in Rel-18, which also need further SA4 confirmation. And then UE could report that information to RAN via UAI message.

Proposal 1: From RAN2 perspective, MMSID could be sent from CN to RAN and PDU set level association information or synchronization threshold could be sent from UE to RAN, which are beneficial for capacity performance and UE experience. Send a LS to SA2/SA4 to further clarify whether there is technical concern on it.

  - Usage of Multi-Modality services information

Admission control

In lack of Multi-Modal services association information, not all QoS flows within a multi-modal service could be setup successfully during PDU session setup procedure due to RAN control strategy, e.g., reduce load in serving cell, radio resource is not available/enough, etc. This will significantly impact the user experience of immersive multi-modal XR application due to lack of some multi-modal flow(s). And it may further lead to all the other correlative QoS flows useless. Thus, in PDU session resource setup, handover and SN node addition/modification scenarios, NG RAN need to use MM association information to accept or reject the corresponding QoS flows accordingly. In detail, the multi-Modality services association information (e.g., MMSID) could be included in the IE “QoS Flow Level QoS Parameters” in NGAP and XnAP messages.

Scheduling implementation at gNB side

Moreover, without the Multi-Modal services information, RAN cannot always guarantee that the transmission of the QoS flows to satisfy the synchronization threshold as defined in TS 22.261. The reason is that the current RAN scheduling does not consider the association relationship of multiple LCHs/flows and relative delay between flows or packets. Given multi-modal association awareness at gNB, gNB could perform co-ordinated scheduling for the associated QoS flows in the downlink transmission. For UL transmission, gNB could configure the related QoS flows mapping to one DRB or more than one DRBs with the same LCH priority and LCP restrictions if the QoS profiles for those related QoS flows are similar to each other. In addition, if synchronization threshold can be further obtained at gNB, it can schedule the data from the related QoS flows more precisely to meet the synchronization requirements.

SDU discard

In addition, if gNB or UE could get the Multi-Modal service associated information, it can also perform SDU discard in case the related data from the other flow has been discarded already. So that the radio resource could be saved and the following data will not be delayed, especially in case of radio congestion.  Besides, SDU discard could be performed base on synchronization requirement. If one flow is considered as not meeting the sync requirement, some kind of low importance data could be discarded similar as PSI based SDU discard. Furthermore, SDU discard could also operate based on packet level sync information if such information could be obtained. More details would be discussed in the following section for questions to SA4.

DSR report

If one of the associated flows is considered as not meeting the sync requirement for UL, UE could also report to gNB via a MAC CE similar like DSR report. Furthermore, if UE could get per packet or per PDU set sync information, it could send the DSR with detailed data volume and remaining time to help RAN to perform more precise scheduling.

Proposal 2: Send a LS to SA2/SA4 to clarify MM awareness information could be used for admission control, scheduling enhancement, SDU discard and DSR report at RAN side.

  - Further questions on SDU discard for Multi-Modal services

From RAN2#127 meeting, there was an agreement on SDU discard:

- RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4.

And there is a note in the updated WID [2]: “RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE.”

Before RAN2 discuss the detailed SDU discard mechanism in PDCP, several issues need to be confirmed with SA4/SA2. Firstly, whether UE can obtain correlation and synchronization threshold information between PDU Sets or packets from two different QoS flows within one MM service from its APP layer. If above questions can be confirmed by SA4/SA2, the following question would be SDUs in one of the correlated PDU sets have been discarded/failed, whether the SDUs in the other PDU set are still useful to the decoder of the APP layer or not or whether the SDUs could be discarded as well. Since if such data packets are not useful any more, UE or RAN could also discard them to avoid unnecessary radio resource waste which is similar as PDU set discard indicated in PSIHI. Secondly, if synchronization threshold can be obtained, and SDUs in the transmitter that cannot meet such sync requirement, whether such SDUs are still useful to the decoder of the APP layer or not. This is similar as PDB/PSDB used for single QoS flow. If such out-of-date packets can be discarded, further radio resources can be saved.

In conclusion, the following questions are raised to be confirmed with SA4/SA2:

- Whether UE could obtain the correlation information between PDU sets or SDUs from different QoS flows within one MM service.

- Whether UE could obtain the synchronization threshold information of QoS flows or PDU sets or SDUs from different QoS flows within one MM service.

- In case of inter PDU set correlation information can be obtained and SDUs in one of the PDU sets have been discarded, whether the SDUs in the other PDU set are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- If synchronization threshold can be obtained, and SDUs in the transmitter that cannot meet such sync requirement, whether such SDUs are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

Proposal 3: RAN2 to discuss and draft a LS to SA4/SA2 on the following questions:

- Whether UE could obtain the correlation information between PDU sets or SDUs from different QoS flows within one MM service.

- Whether UE could obtain the synchronization threshold information of QoS flows or PDU sets or SDUs from different QoS flows within one MM service.

- In case of inter PDU set correlation information can be obtained and SDUs in one of the PDU sets have been discarded, whether the SDUs in the other PDU set are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- If synchronization threshold can be obtained, and SDUs in the transmitter that cannot meet such sync requirement, whether such SDUs are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- Conclusion

In this contribution, we discuss the issues in the LS from SA2 on multi-modality [1] and the LS to SA2/SA4 on dependency and synchronization. Based on the discussion, we have the following observations and proposals:

Observation: With multi-modal association and synchronization awareness at RAN, RAN can provide proper admission control and coordination transmission to satisfy the synchronization requirement as in TS 22.261, which has benefit in term of user experience and system capacity.

Proposal 1: From RAN2 perspective, MMSID could be sent from CN to RAN and PDU set level association information or synchronization threshold could be sent from UE to RAN, which are beneficial for capacity performance and UE experience. Send a LS to SA2/SA4 to further clarify whether there is technical concern on it.

Proposal 2: Send a LS to SA2/SA4 to clarify MM awareness information could be used for admission control, scheduling enhancement, SDU discard and DSR report at RAN side.

Proposal 3: RAN2 to discuss and draft a LS to SA4/SA2 on the following questions:

- Whether UE could obtain the correlation information between PDU sets or SDUs from different QoS flows within one MM service.

- Whether UE could obtain the synchronization threshold information of QoS flows or PDU sets or SDUs from different QoS flows within one MM service.

- In case of inter PDU set correlation information can be obtained and SDUs in one of the PDU sets have been discarded, whether the SDUs in the other PDU set are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- If synchronization threshold can be obtained, and SDUs in the transmitter that cannot meet such sync requirement, whether such SDUs are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- References

- R2-2407936/S2-2409444, LS reply on multi-modality awareness at RAN, SA2

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, RAN#105

- Annex2 - draft reply LS to SA2 on multi-modality

Title:	[Draft] reply LS on multi-modality awareness at RAN

Response to:	R2-2407936/S2-2409444

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	TBD [to be RAN2]

To:	SA2

Cc: 	RAN3, SA4

# Contact Person:

# Name:	              TBD

E-mail Address:	 TBD

# 1. Overall Description:

RAN2 would like to thank SA2 for the LS on multi-modality awareness at RAN in [R2-2407936/S2-2409444].

RAN2 has discussed the awareness and the usage of multi-modality, and concluded the following usages and benefit for multi-modality awareness in RAN [To be updated based on the discussion conclusion]:

Also, SA2 would like to first understand the usage RAN2 has envisioned with multi-modality awareness (i.e., the foreseen NG-RAN behaviour for "coordinated handling of the flows at NG-RAN") before SA2 can decide whether to provide the MMSID to RAN as part of Rel-19 alignment work.

RAN2 responses:

- RAN could perform joint admission control for multiple flows in case of PDU session setup and handover if multi-modality awareness is available.

- RAN could perform DRB mapping, parameter configuration and radio resource allocation jointly for multiple flows within one MM services if multi-modality awareness is available.

- RAN could perform unnecessary SDU discard across multiple QoS flows due to their correlation information if multi-modality awareness is available.

# 2. Actions:

To SA WG 2:

# RAN2 kindly request SA2 to take the above information into account and discuss to specify or provide the MMSID to RAN.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #128		18th Nov. – 22nd Nov. 2024		Orlando, USA

TSG RAN WG2 Meeting #129		17th Feb. – 21st Feb. 2025		Athens, Greece

- Annex2 - draft LS to SA4/SA2 on dependency and synchronization

Title:	[Draft] LS on SDU discard for multi-modality services

Response to:

Release:	Rel-19

Work Item:	NR_XR_Ph3-Core

Source:	vivo [to be RAN2]

To:	SA4

Cc: 	SA2, RAN3

# Contact Person:

# Name:	              Chenli

E-mail Address:	 Chenli5g@vivo.com

# 1. Overall Description:

In the previous RAN2 meeting, the SDU discard operation across different QoS flows within one Multi-Modal service has been discussed. And the below conclusion has been made from RAN2 point of view:

RAN2 thinks PDU Set discard across QoS flows of the same multi-modal service based on the dependency information between the multi-modal flows can only be achieved in case the synchronization information can be available at the UE which is up to SA2/SA4

In RAN#105, there are also some discussions on this and the updated WID on R19 XR has a note as: “RAN#106 to check handling of uplink discard based on SA2/SA4 outputs on whether the corresponding information is available at the UE.”

In RAN2#127bis, there are also some detailed questions been discussed and raised to SA4:

- Whether UE could obtain the correlation information between PDU sets or SDUs from different QoS flows within one MM service.

- Whether UE could obtain the synchronization threshold information of QoS flows or PDU sets or SDUs from different QoS flows within one MM service.

- In case of inter PDU set correlation information can be obtained and SDUs in one of the PDU sets have been discarded, whether the SDUs in the other PDU set are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

- If synchronization threshold can be obtained, and SDUs in the transmitter that cannot meet such sync requirement, whether such SDUs are still useful to the decoder of the APP layer or whether they could be discarded to save radio resources.

# 2. Actions:

To SA WG4:

RAN2 kindly asks SA4 to take the above information into account and provide answers to the questions above.

# 3. Date of Next RAN2 Meetings:

TSG RAN WG2 Meeting #128		18th Nov. – 22nd Nov. 2024		Orlando, USA

TSG RAN WG2 Meeting #129		17th Feb. – 21st Feb. 2025		Athens, Greece