3GPP TSG-RAN WG2 Meeting #127bis                                 R2-2408143

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.1.3

Source: 	Fujitsu

Title: 	Discussion on NW side Data Collection for Beam Management

WID/SID:	NR_AIML_air

Document for:	Discussion and Decision

Introduction

In last RAN2#127 meeting [1], there were discussions on the NW side data collection of beam management use cases and several agreements were achieved. In this contribution, we would like to discuss the remaining issues of the detailed solutions for NW side data collection of beam management use cases for model training.

Discussion

Logging Data Collection and Reporting

In RAN2 #127 meeting, the following agreements were made for data collection logging and reporting:

Agreements

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements. The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started. FFS if multiple configurations can be provided to the UE. FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size. This is across all use cases.

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE. It is up to UE implementation how the UE determines power state. FFS whether the UE stops autonomously or if it reports to the network.

- FFS whether AS buffer event-based reporting is supported. FFS if we send availability indication or full report if it is supported.

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signaling.

We would like to give our views for the remaining FFSs listed above.

Configurations for Data Logging and Reporting:

First, the interpretations of “configurations” should be clarified, it can be the “measurement configuration” which indicates the UE what to measure based on CSI-RS/SSB, it can also be the “data logging/reporting configuration” which indicates the UE to start/stop/suspend/resume the data logging/reporting procedure. These two “configurations” should be decoupled during the data collection procedure.

Concerning the “measurement configuration”, it refers to the set A/set B configuration for beam management, there may be multiple Set A/B combinations for multiple AI/ML-enabled features/FGs/models, from NW’s point of view, it may at least configure UE to perform the measurement for multiple Set A/B in parallel.

Proposal 1 For the measurement configuration of AI/ML data collection, multiple configurations can be supported to allow measurements for multiple Set A/B combinations at UE side.

And for the data logging and reporting, there is no need for multiple configurations, NW can configure its own preference to UE for what contents to be captured in the data reporting. NW only needs to dynamically control the procedures of the data collection, e.g., start/stop/suspend/resume of the data logging and/or reporting. For data reporting, NW may need to configure the UE to report measured data for different configuration separately, or with explicit/implicit indicator so that NW can distinguish collected data for different AI/ML-enable features/FGs/models at NW side.

Proposal 2 Data reporting for different measurement configurations should be separated and indicated so that NW can distinguish collected data for different AI/ML-enabled features/FGs/models.

As for the activation/deactivation, they are optional for the data collection procedure. UE will start/stop the data collection procedure upon configuration, and multiple events/conditions can be discussed to guarantee the flexibility to dynamically control the data collection procedure. Therefore, additional dynamic activation/deactivation signaling may not be necessary.

Proposal 3 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

AS Buffer Based Logging and Reporting:

For AI/ML data collection, it is significant to make sure the reported data contains all information of one AI/ML data sample (including at least model input and ground truth label). According to the LS reply from RAN1 [2], the most required data sample size is for positioning which may up to around 10KB for each sample, for beam management, the required size is relatively smaller, therefore, the minimum required AS buffer size should be at least 10KB to convey one data sample for AI/ML training purpose.

It is noticed that in existing specifications [3], the minimum memory size of MDT logged measurements is 64KB. For data collection of beam management, it is natural to follow MDT-based mechanism as baseline for both gNB and OAM-centric data collection. Therefore, it is suggested that the minimum size defined in logged MDT to be baseline for AS buffer-based data reporting for beam management.

Proposal 4 The minimum memory size of logged MDT in existing specification can be re-used as baseline to support AS buffer-based data reporting for AI/ML beam management.

The UE will stop the data logging if the AS buffer is full, and a natural further action is that UE reports the cached data to NW immediately under configuration, however, congestions may occur if UE reports the data when the buffer is full to NW without request. It is noted that in current MDT mechanism, the data reporting is triggered by NW request via UEInformationRequest, this request signaling can be re-used as a baseline to support AI/ML data reporting for beam management. Similarly, UE may respond the NW via UEInformationReponse as in MDT.

An availability indicator can be reported prior to the full data reporting from UE on NW request to indicate if the AS buffer is full or not. The details of this NW request signaling, e.g., if it can be coupled with the request for data reporting, can be further studied.

Proposal 5 If the AS buffer is full, UE can report the buffered data only if it receives NW request.

Proposal 6 Before reporting the buffered data, UE can report an availability indicator on NW request to indicate if the buffer is full or not.

UE actions upon low power

If the UE detects the state of low power, it needs to stop data logging to save power, automatic stopping is considered as baseline since the AI/ML data collection for training is considered as low priority and latency relaxed, the mechanism should follow the principles of MDT design that all data logging is optional to the UE, therefore, additional feedback or request to NW is unnecessary.

Proposal 7 UE can stop data logging automatically if low power state is detected.

On-demand Reporting

Similar to MDT mechanism, RRC signaling can be used for NW configure on-demand request for UE to report collected data for AI/ML purpose. UEInformatonRequest and UEInformatonResponse can be regarded as baseline for the information exchange between gNB and UE.

Proposal 8 RRC signaling (e.g., UEInformatonRequest and UEInformatonResponse) can be taken as baseline for the on-demand data reporting.

Data Reporting Parameters

Since L1 measurement result and other data related parameters for data collection are determined by RAN1, therefore, it is suggested that RAN2 wait for RAN1 further input at this stage.

Proposal 9 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

Assistance Information

To assist the network to categorize the collected data, it is important for the network to be aware of the related assistance information (e.g., additional conditions) when the data is generated and stored. There are UE-side and NW-side additional conditions. To better differentiate the characteristics of data, both additional conditions should be identified.

Observation 1 To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

In RAN1#116bis meeting, one solution was agreed for UE-side model training with the transmission of NW-sided additional conditions implicitly in term of NW associated ID(s) from network to UE. The AI/ML models will be developed at UE side based on the collected data corresponding to the NW-sided additional conditions/associated ID(s), or both UE-side and NW-side additional conditions/associated ID(s). The details can refer to the following agreements.

Agreement From RAN1 perspective, for UE-sided model(s) developed (e.g., trained, updated) at UE side, following procedure is an example (noted as AI-Example1) of MI-Option1 for further study (including the feasibility/necessity) - A: For data collection, NW signals the data collection related configuration(s) and it/their associated ID(s) - Associated IDs for each sub use case in relation with NW-sided additional conditions - B: UE(s) collects the data corresponding to the associated ID(s) - C: AI/ML models are developed (e.g., trained, updated) at UE side based on the collected data corresponding to the associated ID(s). - D: UE reports information of its AI/ML models corresponding to associated IDs to the NW. Model ID is determined/assigned for each AI/ML model - relationship between model ID(s) and the associated ID(s) - How model ID(s) is determined/assigned, e.g., - Alt.1: NW assigns Model ID - Alt.2: UE assigns/reports Model ID - Alt.3: Associated ID(s) is assumed as model ID(s) - “Model ID is determined/assigned for each AI/ML model” in D is not needed. - Alt.4: Model ID is determined by pre-defined rule(s) in the specification. - FFS: how to report - Note: D is to facilitate AI/ML model inference - Note: Step A/B/C and additional interaction of associated IDs between UE and NW can be considered as a different solution for resolving the consistency without model identification.

Similar principle can be applied to NW-side data collection for model training. With the UE-side and NW-side assistance information (e.g., additional conditions/associated ID(s)), the network will develop the AI model specific for these additional conditions/associated ID(s). One possible solution is that the network can send the NW-side assistance information to the UE. When the UE stores the collected data, it also includes the related NW-side assistance information. Additionally, the UE can also provide the corresponding UE-side assistance information with the collected data. The UE reports the collected data, NW-side assistance information and NW-side assistance information as the integrated data statistics.

Figure 1 UE- based solution

In option 2, the UE can only report UE-side assistance information together with the reported data. When receiving the data from the UE, the network can complement the corresponding NW-side assistance information.

Figure 2 UE- and NW-based solution

In option 3, it is also possible for the UE to provide UE-side assistance information to the network in advance. The UE can report the data as without any UE-side or NW-side assistance information. When receiving the data from UE, the network complements both the UE-side and NW-side assistance information.

Fig 3 NW-based solution

It is NW-side data collection, the NW needs to filter the data according to both NW and UE-side assistance information, however, the NW has no prior knowledge of UE-side assistance information, therefore in order to facilitate the aforementioned 3 options, the key aspect needs to be specified is the transmission of UE-side assistance information (e.g., additional conditions) from UE to NW. There are basically two solutions: 1) UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting; 2) The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE. Both solutions are reasonable, so we support to have both of the two proposals below:

Proposal 10 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 11 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Conclusion

In this contribution, we discuss the potential solutions for the NW-side data collection for model training of beam management. The following observation and proposals are considered:

Proposal 1 For the measurement configuration of AI/ML data collection, multiple configurations can be supported to allow measurements for multiple Set A/B combinations at UE side.

Proposal 2 Data reporting for different measurement configurations should be separated and indicated so that NW can distinguish collected data for different AI/ML-enabled features/FGs/models.

Proposal 3 Additional dynamic activation/deactivation may not be necessary since events/conditions can be designed to dynamically control the data collection procedure.

Proposal 4 The minimum memory size of logged MDT in existing specification can be re-used as baseline to support AS buffer-based data reporting for AI/ML beam management.

Proposal 5 If the AS buffer is full, UE can report the buffered data only if it receives NW request.

Proposal 6 Before reporting the buffered data, UE can report an availability indicator on NW request to indicate if the buffer is full or not.

Proposal 7 UE can stop data logging automatically if low power state is detected.

Proposal 8 RRC signaling (e.g., UEInformatonRequest and UEInformatonResponse) can be taken as baseline for the on-demand data reporting.

Proposal 9 For the detailed parameters of L1 data collection, RAN2 waits for the RAN1 inputs.

Observation 1 To categorize the data, it is necessary to consider both UE-side and NW-side assistance information.

Proposal 10 For NW-side data collection of beam management, UE provides to the NW the UE-side assistance information (e.g., additional conditions) together with the data collection reporting.

Proposal 11 For NW-side data collection of beam management, The NW sends the UE-side assistance information (e.g., additional conditions) filtering configuration to the UE.

Reference

[1] R2_127_ChairNotes_Final, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[2] R1-2310681, Reply LS on Data Collection Requirements and Assumptions, RAN1.

[3] TS38.306.