3GPP TSG-RAN WG2 Meeting #127bis	R2-2408153

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.7.4.2

Source: 	Fujitsu

Title: 	Discussions on DSR enhancements

Document for:	Discussion and decision

Introduction

On DSR retransmission enhancement in XR, RAN2 has made the following agreements in RAN2#127 meeting [1]:

Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In this contribution, we would like to discuss further details on the potential DSR enhancements for XR including the FFS issues discussed last meeting.

Discussion

Constraints on threshold configuration

In Rel-18, a threshold, i.e., remainingTimeThreshold, is defined for triggering a DSR for an LCH within an LCG. This threshold is configured for an LCG and is also used in the definition of delay-critical data. That is to say, a single threshold is used for both DSR triggering and delay-critical data definition, as shown in the following [2][3] respectively.

DSR triggering condition in the Rel-18 MAC specification:

5.4.9 Delay status reporting The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs that are buffered for the LCG but have not been transmitted in any MAC PDU as specified in clause 7.3 in TS 38.323 [4], and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively. RRC controls the DSR procedure by configuring the following parameter: - remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG. If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

Delay-critical PDCP SDU definition in the Rel-18 PDCP specification:

Delay-critical PDCP SDU: if pdu-SetDiscard is not configured, a PDCP SDU for which the remaining time till discardTimer expiry is less than the remainingTimeThreshold. If pdu-SetDiscard is configured, a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till discardTimer expiry less than the remainingTimeThreshold.

In Rel-19, based on the RAN2 agreements on the enhanced DSR, there is a single DSR triggering threshold, but there can be multiple remaining time thresholds for reporting for an LCG. There is an FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds.

In our understanding, on the need of configuration constraints on DSR triggering threshold and reporting thresholds, there can be following options:

- Option 1: The DSR triggering threshold is constrained to be configured as the maximum value of configured reporting thresholds.

- Option 2: The DSR triggering threshold is constrained to be configured as the minimum value of configured reporting thresholds.

- Option 3: No constraints are needed on the configuration of DSR triggering threshold and reporting thresholds for an LCG.

In option 1, the DSR triggering threshold is constrained to be configured as the maximum value of configured reporting thresholds. In this option, when a DSR is triggered based on the triggering threshold, all the data with remaining time below the reporting thresholds would be considered as delay-critical data for the LCG. Moreover, the DSR trigger threshold remainingTimeThreshold can still be used to determine the delay-critical PDCP SDU, and the definition of delay-critical data is not impacted. Therefore, the standard impact is relatively small. On the other hand, DSR may be more easily triggered since the triggering threshold is relatively high, so the signalling overhead due to DSR transmission could be a little higher.

In option 2, the DSR triggering threshold is constrained to be configured as the minimum value of configured reporting thresholds. In this option, when a DSR is triggered based on the triggering threshold, the data with remaining time higher than the triggering threshold but lower than the reporting threshold(s) will also be reported in the enhanced DSR MAC CE. If the definition of delay-critical data is not modified, non-delay critical data can also be reported in the enhanced DSR MAC CE and how to handle the reported non-delay critical data may need to be specified in the standard; otherwise, the definition of delay-critical data should be modified to keep the legacy UE behaviors on the handling of delay-critical data. From this perspective, the standard impact is big. On the other hand, DSR is not so easily triggered and the signalling overhead from DSR transmission is lower since the triggering threshold is relatively low.

In option 3, the DSR triggering threshold and the reporting thresholds are configured independently. In case that the DSR triggering threshold is configured higher than all the reporting thresholds, a DSR may be triggered for an LCG but there is no data with the remaining time below the reporting thresholds in the LCG. This situation is not preferred, so smart network implementation should ensure this situation would not happen. In addition, similar to option 2, the definition of delay-critical data may be impacted or the handling of non-delay critical data reported in DSR may have standard impact, depending on the configured values of the DSR triggering threshold and the multiple reporting thresholds.

The comparison of the above 3 options and the impact on delay-critical data definition is shown in the following table:

|  | Advantages | Disadvantages | Need to change delay-critical data definition | Inclusion of non-delay critical data |
| --- | --- | --- | --- | --- |
| Option 1: triggering threshold is configured as the maximum reporting threshold | Simple. Small standard impact. | Signalling overhead with more easily triggered DSR. | No | No (*) |
| Option 2: triggering threshold is configured as the minimum reporting threshold | Simple. Low signalling overhead. | Bigger standard impact. | Yes | No (*) |
|  |  |  | No | Yes |
| Option 3: No constraints | Flexible configuration. | Unnecessary DSR trigger with no data | Maybe yes | Maybe yes |

* The non-delay-critical data ahead of delay-critical data in the buffer is not considered.

From the comparison, we slightly prefer option 2 due to its simplicity and smaller impact to the standard. Moreover, the definition of delay-critical data is not impacted. Therefore, we have the following proposals:

Proposal 1: DSR triggering threshold is configured as the maximum reporting threshold.

Proposal 2: The definition of the delay-critical data is not modified.

Inclusion of non-delay critical data ahead of delay-critical data

In the last RAN2 meeting, there is an FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR. The scenario raised by the proponent company is that the UE buffer is not ordered according to the discard timer and the UE may send non-delay critical data before delay-critical data. In our opinion, although RAN2 has agreed not to do intra-LCH prioritization, UE implementation can differentiate the delay-critical data from non-delay critical data and can transmit delay-critical data properly with the LCP enhancements. Therefore, there is no need to include non-delay critical data ahead of delay-critical data in the buffer size calculation for DSR.

Proposal 3: Non-delay critical data ahead of delay-critical data is not included in the buffer size calculation for DSR.

Need of PSI

In RAN2#126 meeting, it is discussed whether PDU Set Importance (PSI) needs to be included in DSR. In Rel-18, in case of uplink congestion, PSI-based discard is activated and then the PDCP SDUs with low importance can be discarded. Currently the UE does not differentiate data with different PSI during LCP procedure, so the benefits of including PSI in the enhanced DSR MAC CE are not clear.

Proposal 4: PSI is not included in the enhanced DSR.

Conclusion

In this contribution, we have discussed the further details of the DSR enhancements. We have the following proposals:

Proposal 1: DSR triggering threshold is configured as the maximum reporting threshold.

Proposal 2: The definition of the delay-critical data is not modified.

Proposal 3: Non-delay critical data ahead of delay-critical data is not included in the buffer size calculation for DSR.

Proposal 4: PSI is not included in the enhanced DSR.

References

- R2- 2407575, RAN2#127 Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)

- TS38.321 v18.3.0

- TS38.323 v18.3.0