3GPP TSG-RAN WG2 Meeting #127bis	R2-2408186

Hefei, China, 14th Oct – 18th Oct, 2024

Agenda Item		:	8.2.3 (FS_Ambient_IoT_solutions)

Source		:	LG Electronics Inc.

Title		:	Discussion on ambient IoT paging

Document for	:	Discussion and Decision

1.	Introduction

In the RAN2#127 meeting [1], the following agreements were achieved on ambient IoT (A-IoT or AIoT) paging.

Agreements – RAN2#127 - Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

In the RAN2#126 meeting [2], the following agreements were reached regarding the A-IoT paging.

Agreements – RAN2#126 - RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. - AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. - From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

During the RAN2#127 meeting, the following issues regarding A-IoT paging were captured, but were either addressed briefly or not at all due to time constraints [1].

- Paging Triggers/Retransmission

- Definition of the access occasion

- Determination of the access occasion in CBRA case

- Determination of the access occasion in CFRA case (single device and multiple devices)

- Access type/Service type

- Additional contents/indications in the paging message

In this contribution, we further discuss the issues related to A-IoT paging, including the agreements and concerns raised during the RAN2#127 meeting.

2.	Discussion

2.1.	Multiple/subsequent paging and re-access

In RAN2#127, RAN2 agreed to support multiple/subsequent A-IoT paging messages associated with the same request from the CN and to study how to avoid duplicated response from devices. One motivation for this multiple/subsequent paging is to facilitate re-access for A-IoT devices that fail to complete A-IoT procedure related to a specific A-IoT service due to the various reasons, such as energy shortages, collisions, or interference. One use case for this multiple/subsequent paging is reader-triggered periodic A-IoT paging. Here we discuss this issue further.

Multiple/subsequent paging to enable re-access:

In RAN2#126 meeting [2], RAN2 agreed on various A-IoT procedures (i.e., “inventory”, “inventory and command”, and “command only”), stating that A-IoT paging serves as the initial trigger of all types of A-IoT procedures. Specifically, A-IoT paging triggers the target devices for the corresponding A-IoT service. The recipients of the A-IoT paging, who are targeted for the A-IoT service, then proceed with subsequent A-IoT procedures, such as the random access (RA) procedure and data transmission.

Agreements – RAN2#126 - As baseline, the “inventory only” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - As baseline, the “inventory and command” case is supported by the procedure: - Step A: A-IoT paging; - Step B: Device ID transmission (via Random Access or without using RA). Details are FFS - Step C: reader to device data transmission (e.g. the R2D command), and - Step D: corresponding device to reader data transmission (e.g. the feedback). FFS whether this is optional, pending other WG discussions. - From RAN2 point of view we will study “Command only” use case. - FFS the options on how to support it: - Initial trigger message from the reader contains the command. Final feasibility depends on SA2 and SA3 work/conclusions. - Use baseline procedure for “inventory and command” (i.e. first triggers inventory procedure and then sends command)

As abovementioned, one motivation for the multiple/subsequent paging is to trigger re-access to address A-IoT device failures during A-IoT procedures. One example of such a failure is inability to receive the A-IoT paging message. Various reasons (e.g., energy shortage, channel unreliability, interference, etc.) could lead to some A-IoT devices failing to receive the A-IoT paging message. Even if they receive a reader-to-device (R2D) message after the paging message, they cannot determine if they are the intended target of the ongoing A-IoT procedure (service) because they missed the paging message, and therefore cannot respond to the R2D messages. In this situation, to ensure A-IoT services function properly for devices that have failed to receive the A-IoT paging message, re-access is necessary. This means the device needs to receive A-IoT paging message, making multiple/subsequent A-IoT paging messages prerequisite.

Meanwhile, failure in the A-IoT procedure could occur even after the target device successfully receives the A-IoT paging message, such as during subsequent procedures like RA procedure or data transmission. In this case, re-access is still necessary to ensure proper operation of A-IoT services for the affected devices. Unlike the previous failure scenario, the A-IoT device can identify itself as the target of the ongoing A-IoT procedure through the received paging message. This implies that, in this scenario, other R2D messages, in addition to the A-IoT paging message, could also be considered as a re-access trigger. However, we note that one of the most important considerations in designing A-IoT protocol is the limited performance and capability of the A-IoT devices. Therefore, simple protocol designs should be preferred. It is important to remind that multiple/subsequent A-IoT paging messages can still be utilized for re-access in the second failure scenario. Given this, we propose multiple/subsequent A-IoT paging message as a unified trigger for re-access to handle failures during the A-IoT procedure (e.g., failure to receive an A-IoT paging message, failure in subsequent procedures).

Proposal 1: To handle failures during the A-IoT procedure, re-access should be supported, and the multiple/subsequent A-IoT paging messages should be a unified trigger for such re-access to address any failure during A-IoT procedure, such as not receiving A-IoT paging message or failures in subsequent procedures.

Reader-triggered periodic A-IoT paging:

In the RAN2#127 meeting, some companies proposed to discuss reader-triggered periodic A-IoT paging according to the configuration from CN as a use case of the multiple/subsequent A-IoT paging messages. One motivation for this approach is that it could reduce CN requests and corresponding overhead by allowing the reader to perform A-IoT paging periodically without ongoing requests from the CN, except for the initial one. However, the potential gains appear marginal.

First, even in the case of CN-triggered paging, the paging request from CN to the reader would be sent via non-A-IoT interface (e.g., NG interface in topology 1). Therefore, the additional overhead from these requests would be marginal and would not add significant burden to the A-IoT interface. Furthermore, reader-triggered periodic paging would require a ‘stop request’ from the CN to quit paging, and any reconfiguration would necessitate additional requests. Thus, allowing reader-triggered periodic paging could introduce extra control and increase system complexity. Given the importance of simplicity in designing the A-IoT protocol and the limited impact on overhead reduction, we propose not to allow reader-triggered periodic paging and instead letting CN request A-IoT paging on-demand as needs.

Proposal 2: Reader-triggered periodic A-IoT paging should not be supported. Instead, when periodic paging is needed, letting CN request A-IoT paging from reader periodically is enough.

How to avoid duplicated response from devices:

To ensure efficient operation of multiple/subsequent paging and re-access, A-IoT devices that have already responded to an A-IoT paging message for a specific A-IoT procedure (service) should refrain from responding again to another paging message for the same A-IoT procedure (i.e., avoid duplicated response). However, the devices should still be able to respond to paging messages for new A-IoT procedures. Therefore, it should be distinguishable to the A-IoT devices whether an A-IoT paging message pertains to a procedure they have already performed.

To support this, we propose that each A-IoT paging message include information related to the specific A-IoT procedure it triggers. This information will enable A-IoT devices to identify whether the received paging message is one they should respond to. An A-IoT device will only respond to a paging message if the information indicates that the procedure related to the message has not yet been performed. How the information is defined and managed is FFS.

Proposal 3: A-IoT paging message should include information about the specific A-IoT procedure it pertains to. How the information is defined and managed is FFS.

Proposal 4: A-IoT device should only respond to a paging message if the related A-IoT procedure has not yet been performed.

2.2.	Reader identity and multiple-reader scenario

In the RAN3#124 meeting [3], RAN3 reached an agreement regarding the A-IoT device location information as follows. RAN3 set “whether and how to utilize more than one reader” as FFS. The corresponding text proposal was subsequently agreed in RAN3#125 meeting [4][5].

Agreements – RAN3#124 AIoT device location information may be used for the following purposes: (a) improving the AIoT operation itself, e.g. by sending a Command to one or more readers (e.g., the last reader(s)) associated to the device rather than sending it blindly. (b) providing location information to the consumer of the AIoT service. Locating an Ambient IoT device at “reader ID granularity” is useful for both purposes. How to know the “reader” location is FFS. Whether and how to utilize more than one “readers” for location purposes is FFS depending on involved complexity. Signalling details on how to provide the location information to the A-IoT CN node.

Given that RAN3 is considering the use of multiple readers for location purposes, RAN2 needs to study the implications of this potential change, particularly for location and possibly other applications. Since readers initiate A-IoT procedure through A-IoT paging messages, discussions on A-IoT paging should also consider this multiple-reader scenario. For instance, the same command (e.g., for location purposes) could be requested from multiple readers, leading an A-IoT device to receive A-IoT paging messages for the same command multiple times from different readers.

Here, we briefly analyse the issues that could arise from the multiple-reader scenario. A situation may occur where the A-IoT device should decide whether to respond to only one paging message (e.g., the first) or to all paging messages from different readers. For example, during an inventory service, if a target device has never been inventoried, its location relative to the readers is unknown to the CN. In this case, to quickly locate the target device, the CN may request inventory service from all readers, prompting them to initiate the A-IoT paging procedure. In this scenario, it is sufficient for the A-IoT device to respond to the first received A-IoT paging message.

Conversely, the A-IoT device may need to respond to all A-IoT paging messages from different readers. As RAN3 considers, for accurate location purposes, it may be necessary to inventory a specific A-IoT device using multiple readers. In this scenario, the A-IoT device should respond to each A-IoT paging message from different readers. However, it remains beneficial to avoid duplicate responses to the same reader (see Section 2.1) while allowing multiple responses to different readers. To facilitate this, the A-IoT device should be able to distinguish the source of each A-IoT paging message. Therefore, we propose that the A-IoT paging message include the reader identity (ID) of the sender.

Observation 1: In A-IoT systems, there could be scenarios where multiple readers transmit A-IoT paging messages for the same A-IoT service. In some cases, an A-IoT device may be required to respond to each of these paging messages, while in other cases, the device may only need to respond to one of the messages (e.g., the first one received).

Proposal 5: For A-IoT devices to operate properly within the multiple-reader scenario, A-IoT paging message should include the reader ID of the sender of the A-IoT paging message.

We believe there are more issues related to the multiple-reader scenario, and we propose that RAN2 further investigates this multiple-reader scenario.

2.3.	Other issues related to A-IoT paging and paging message content

In addition to the information/indications related to the abovementioned proposals to be included in the A-IoT paging message, here we further discuss the additional contents needed to be incorporated into the A-IoT paging message.

Definition of the access occasion:

During the RAN2#127 meeting, companies discussed the definition of the access occasion. There were differing opinions on what indicates the start of the access occasion. Some believe it is marked by A-IoT paging message, while others suggest a different R2D message. Eventually, RAN2 decided to wait for further RAN1 progress on this matter. We agree that further RAN1 progress is needed to define the exact access occasion. However, we believe RAN2 could at least discuss the baseline procedure to be used to indicate the start of the access occasion.

Considering that A-IoT paging message serves as the initial trigger for the entire A-IoT procedure, as agreed in RAN2#126 meeting (see Section 2.1), we propose that A-IoT paging message should not be used for indicating the start of the access occasion. To fulfill its role as a trigger, the paging message should include various pieces of information, such as details about the target device and control information (e.g., the number of access occasions) needed for the procedure. Given the potential length of A-IoT paging message, transmitting it every time to indicate each access occasion is inefficient, especially considering the energy and performance constraints of the A-IoT devices and the preference for a streamlined protocol design.

Instead, we propose to introduce a different R2D message for this purpose. We would like to highlight that each access occasion is not a separate A-IoT procedure, and they compromise the A-IoT procedure. We also note that whether the A-IoT paging message could directly indicate the start of the first access occasion is FFS.

Proposal 6: A-IoT paging message should not be used to indicate the start of the random-access occasion within the A-IoT random access procedure; instead, a different R2D message should be used for that purpose. It is FFS whether A-IoT paging message could directly indicate the start of the first access occasion.

In RAN1#118 meeting [6], RAN1 reached an agreement on the frequency-domain multiple access for A-IoT as follows.

Agreement – RAN1#118 Study FDMA of D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access, including following • How the frequency domain resources are allocated for the FDMA of D2R transmissions for Msg.1 • How a device determines the frequency domain resource for the D2R transmissions for Msg.1 Note: this does not preclude discussion on TDMA for D2R transmissions for Msg.1

According to this RAN1 agreement, frequency-domain multiple access of device-to-reader (D2R) transmissions is under study. This implies that A-IoT procedures can be performed across multiple different frequencies. Therefore, it is necessary for RAN2 to explore ways to incorporate frequency-domain multiple access into the A-IoT procedure. Considering that the A-IoT procedure, especially A-IoT RA procedure (including D2R transmissions for Msg. 1), is initiated by the A-IoT paging, A-IoT paging message should include frequency information for performing the RA procedure.

Proposal 7: A-IoT paging message should include frequency information for performing the A-IoT RA procedure (including D2R transmissions for Msg. 1).

Determination of the access occasion in CBRA case:

As agreed in RAN2#126 (see Section 2.1), all the A-IoT procedures begin with A-IoT paging. Following this, CBRA procedure could be performed. We note that, for A-IoT devices to choose a random access occasion properly, the number of access occasions should be known. Otherwise, the A-IoT devices would not know within which range to choose the access occasion. Given that the A-IoT paging comes before and triggers CBRA procedure, the appropriate method to indicate the number of access occasion before CBRA procedure begins is through A-IoT paging. Therefore, we derive Proposal 8. We also note that whether different R2D message should also include the number of the access occasion is FFS.

Proposal 8: A-IoT paging message should include the number of access occasions for the subsequent CBRA procedure, so that A-IoT devices are aware of the number of access occasions.

Determination of the access occasion in CFRA case (single device and multiple devices):

As mentioned in Section 1, in the RAN2#126 meeting, RAN2 agreed to study a case where A-IoT paging message contains multiple IDs of A-IoT devices. The need for this use case remains unconfirmed, pending the SA2 discussion. At the same time, RAN2 agreed that A-IoT paging message should indicate information that allows the device to determine resources to be used for response.

We believe that the case of A-IoT paging message including multiple IDs of devices could indicate information for devices’ resource determination for CFRA procedure. For example, A-IoT devices can choose their access occasion based on the order of the IDs listed in the A-IoT paging message (e.g., the device matched with the first ID selects the first access occasion, and the device matched with the third ID selects the third access occasion). Given this, we think that RAN2 could confirm the need for this A-IoT paging case without waiting for the SA2 discussion.

Additionally, one SA2 draft reply LS from SA2#164 meeting [7] states that “the case to send message(s) to multiple AIoT devices simultaneously exists, and it is up to RAN2 to decide ‘a message containing multiple IDs of A-IoT devices’ or multiple messages with each message containing one AIoT device ID.” Based on this, we derive Proposal 9.

Proposal 9: Given that A-IoT paging message including multiple IDs of A-IoT devices could indicate information for resource determination in the CFRA procedure, RAN2 should confirm the need for A-IoT paging message to include multiple IDs of A-IoT devices without waiting for the SA2 discussion.

Access type/Service type:

As mentioned in Section 2.1, RAN2 agreed on the different identified A-IoT procedures, such as “inventory only”, “inventory and command”, and “command only”. In the “inventory and command” case, the A-IoT device may not know whether the command procedure will follow the inventory procedure unless explicitly indicated. During the A-IoT procedure, transmission failures can occur due to reasons like contention resolution failure, channel unreliability, or CRC errors. If a transmission failure occurs during the command procedure, the reader may detect it, but the A-IoT device may not, potentially leading the A-IoT device to incorrectly assume that the overall procedure completed successfully. A more detailed discussion of this transmission failure issue is provided in our companion paper [8].

Observation 2. Without knowing the type of A-IoT procedure (e.g., “inventory only”, “inventory and command”, and “command only”), the A-IoT device may not detect a failure in the command procedure following the inventory procedure, and therefore cannot handle the failure effectively.

To help A-IoT devices effectively manage such failure issues, we propose a simple solution: include the A-IoT procedure type (or at least the number of messages for the procedure) in the A-IoT paging message. This information could enable the A-IoT device to determine whether the command procedure has been completed successfully and thus handle failures appropriately.

Besides, RAN2 is discussing energy status reporting for A-IoT devices [9], which indicates whether they have sufficient energy to complete their procedures. To correctly assess their energy status, A-IoT devices should consider the type of procedure (e.g., “inventory only”, “inventory and command”, and “command only”). At least, the number of expected messages for the procedure should be considered.

To enable A-IoT devices to obtain this information, the type of ongoing procedure or at least the number of messages involved should be included in the A-IoT paging message.

Proposal 10: A-IoT paging message should include information indicating the type of ongoing procedure (e.g., “inventory only”, “inventory and command”, and “command only”), or at least the number of messages to be involved in that procedure.

3.	Conclusion

This contribution makes the following observations and proposals:

Proposal 1: To handle failures during the A-IoT procedure, re-access should be supported, and the multiple/subsequent A-IoT paging messages should be a unified trigger for such re-access to address any failure during A-IoT procedure, such as not receiving A-IoT paging message or failures in subsequent procedures.

Proposal 2: Reader-triggered periodic A-IoT paging should not be supported. Instead, when periodic paging is needed, letting CN request A-IoT paging from reader periodically is enough.

Proposal 3: A-IoT paging message should include information about the specific A-IoT procedure it pertains to. How the information is defined and managed is FFS.

Proposal 4: A-IoT device should only respond to a paging message if the related A-IoT procedure has not yet been performed.

Observation 1: In A-IoT systems, there could be scenarios where multiple readers transmit A-IoT paging messages for the same A-IoT service. In some cases, an A-IoT device may be required to respond to each of these paging messages, while in other cases, the device may only need to respond to one of the messages (e.g., the first one received).

Proposal 5: For A-IoT devices to operate properly within the multiple-reader scenario, A-IoT paging message should include the reader ID of the sender of the A-IoT paging message.

Proposal 6: A-IoT paging message should not be used to indicate the start of the random-access occasion within the A-IoT random access procedure; instead, a different R2D message should be used for that purpose. It is FFS whether A-IoT paging message could directly indicate the start of the first access occasion.

Proposal 7: A-IoT paging message should include frequency information for performing the A-IoT RA procedure (including D2R transmissions for Msg. 1).

Proposal 8: A-IoT paging message should include the number of access occasions for the subsequent CBRA procedure, so that A-IoT devices are aware of the number of access occasions.

Proposal 9: Given that A-IoT paging message including multiple IDs of A-IoT devices could indicate information for resource determination in the CFRA procedure, RAN2 should confirm the need for A-IoT paging message to include multiple IDs of A-IoT devices without waiting for the SA2 discussion.

Observation 2. Without knowing the type of A-IoT procedure (e.g., “inventory only”, “inventory and command”, and “command only”), the A-IoT device may not detect a failure in the command procedure following the inventory procedure, and therefore cannot handle the failure effectively.

Proposal 10: A-IoT paging message should include information indicating the type of ongoing procedure (e.g., “inventory only”, “inventory and command”, and “command only”), or at least the number of messages to be involved in that procedure.

4.	References

- Draft Report of 3GPP TSG RAN WG2 meeting #127

- R2-2406202, Report of 3GPP TSG RAN WG2 meeting #126

- (draft) RAN3 #124 Meeting Report

- (draft) RAN3 #125 Meeting Report

- R3-244840, (TP for TR 38.769) Locating Ambient-IoT device

- Draft Report of 3GPP TSG RAN WG1 #118 v0.3.0

- S2-2407729, [Draft] Reply LS on RAN2 agreements and assumptions for Ambient IoT, NEC

- R2-2407265, Discussion on random access aspects for Ambient IoT, LGE

- R2-2409029, Discussion on functionality aspects of ambient IoT, LGE