3GPP TSG-RAN WG2 Meeting #127-bis	R2- 2408261

Hefei, China, Oct 14th - Oct 18th, 2024

Agenda Item:	8.6.4

Source:	MediaTek Inc.

Title:	Initial thinking on conditional LTM

Document for:	Discussion, Decision

Introduction

The latest objective for conditional LTM in Rel-19 Mobility Enhancement is formulated as follows:

- • Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

This contribution provides initial thinking on different aspect to support conditional LTM design.

Discussion

Motivation and general design principle

Conditional Handover

In Rel-16, Conditional Handover was introduced for UE initiated handover for multiple candidate cells. Compared with legacy handover, two main benefits can be found in Conditional Handover:

- Improve Reliability: UE decides to perform handover when condition is met to react quickly to deteriorating conditions to prevent condition degrade to a point where handover might fail. Besides, the handover command is omitted to prevent HOF caused by this signalling failure.

- UE autonomy and flexibility: Allows the UE to proactively decide which candidate cell to perform handover by certain predefined conditions and pre-configured candidate cells configurations. The decision made by UE may be more accurate and reasonable due to more detailed measurement context of its radio environment, which allows UE to detect more changes that might not be included in the measurement report.

Observation 1: Two main benefit can be found in conditional handover compared with legacy handover:

- The reliability is increased by UE’s quick response and omitting signalling from failure.

- The accuracy is increased by UE to proactively decide which candidate cell to perform handover based on its finer measurement context.

Rel-18 LTM

In Rel-18, LTM was introduced for handover based on beam level measurement report. Compare with legacy handover, two main benefits can be found in LTM to optimize the performance for mobility:

- Interruption reduction. The early sync procedure is introduced to allow UE to establish synchronization toward candidate cells before handover execution, which shortens the interruption for each handover process. This is more important in the scenario when cell switch is performed frequently (e.g., FR2, high speed or ping-pong scenarios)

- Latency reduction: LTM decision is made from L1 beam level measurement. This approach allows for a finer and faster report of the radio environment to let network to make a quick decision to get a timely and detailed cell switch indication. MAC-CE is used instead of RRC message to further fasten the transmission of CSC to prevent channel deterioration.

Observation 2: Two main benefit can be found in LTM compared with legacy handover:

- The interruption for each cell switch is reduced to realize higher data throughput.

- The latency is reduced by L1 measurement report as well as introducing CSC MAC-CE for finer and faster measurement report and cell switch indication.

Rel-19 conditional LTM

The benefits of Conditional Handover are not yet integrated into LTM. As such, Rel-19 aims to incorporate these advantages into Conditional LTM. However, the current LTM synchronization process is network-initiated. To enhance CHO within LTM, several level of improvements can be proposed:

Reliability. LTM currently uses MAC-CE for cell switch commands, which lacks RLC-AM protection, leading to potential signal loss especially at cell edges where channel condition is poor. Therefore, the 1st objective of Rel-19 Conditional LTM is to enable UE initiated LTM cell switch execution without network indication. It can also reduce the latency of signalling interaction when condition is met to react quickly towards deteriorating channel conditions.

UE autonomy: To improve flexibility and accuracy, the UE can be allowed to proactively select a candidate cell based on its detailed measurements context, but it also needs UE initiated early synchronization and then to inform the network. Considering the complexity of different mechanisms used in Conditional LTM, the specific implementation of which mechanism are introduced will be discussed in the following sections.

Considering different expectation of the scope, it is recommended that RAN2 first clarify the preferred level of implementation of condition LTM.

Proposal 1: RAN2 to discuss which level of conditional LTM is performed:

- Option 1: Early synchronization is network-initiated. UE only decides the LTM execution time and initiate the cell switch procedure.

- Option 2: The UE decides the candidate cell to perform early synchronization and informs the network of its decision (FFS how to indicate network in each step). Subsequently, UE decides the LTM execution time and initiate the cell switch procedure.

Discussion by the lack of LTM CSC MAC-CE

No matter which level of conditional LTM is chosen, the UE initiated LTM execution is a must and the details should be further discussed. As similar as conditional handover, the LTM CSC MAC-CE should be replaced by UE initiated LTM cell switch procedure, which should be a baseline for conditional LTM.

Proposal 2: For conditional LTM, the LTM CSC MAC-CE is replaced by UE initiated LTM cell switch procedure.

In Rel-18 LTM, the LTM CSC MAC-CE has several information:

- LTM candidate cell ID. To indicate the target cell to UE among the LTM candidate cells.

- TA value. To indicate the TA value used for the UL synchronization toward target cell.

- TCI state ID. To indicate and activate the (DL and/or UL) TCI state for the LTM target cell.

- CFRA related information. To provide CFRA information to UE if RACH-less LTM is not performed.

For conditional LTM, the target cell is decided by UE, so candidate cell ID is not needed. However, whether and how does UE acquire the other information (TA, TCI state ID, CFRA related information) may still need discussion.

Timing advance and early UL synchronization

The TA value is used for the synchronization of uplink transmission to adjust the timing from UE so that the signalling arrives at the gNB at the correct instance. UE must acquire the TA before sending the first UL message. In legacy RACH procedure, TA is included in the RAR message for both CBRA and CFRA. In RACH-less LTM, network omits the RAR after UE sends a preamble and instead indicates the TA in the LTM CSC MAC-CE to reduce latency and avoid redundant signalling interactions. As the LTM CSC MAC-CE is no longer used in conditional LTM, reintroducing the RAR to indicate the TA value seems a valid choice.

However, for early UL synchronization, UE remains connected to the serving cell while sending RACH preamble to target cell, and it brings extra complexity for UE to receive RAR from target cell while maintaining the connection with serving cell. Therefore, transferring the RAR from the serving cell to the UE may be necessary for early UL sync in conditional LTM.

Even so, the TA value could be outdated if there is a significant gap between early UL sync and LTM execution. UE may still need to perform RACH-based LTM in this case.

On the other hand, another simple solution is that early UL sync is not performed for conditional LTM, and conditional LTM is always RACH-based LTM.

Proposal 3: RAN2 to discuss the early UL sync and the acquisition of TA for conditional LTM:

- Option1:  For RACH-less conditional LTM, RAR is introduced to indicate TA value, which is transferred from serving cell to UE. If the TA is outdated during LTM execution, UE perform RACH-based LTM.

- Option 2: Conditional LTM is always RACH-based LTM, and early UL sync is not performed.

TCI states acquisition

TCI state is used to indicate the antenna QCL relationship of the target cell. In legacy handover, the UE determine the default antenna QCL assumption by RACH procedure, then receives a finer TCI state activation indication after the cell switch. In Rel-18 LTM, the TCI states are pre-configured to UE and can be activated before LTM cell switch. UE tracks the QCL RS of this candidate TCI state until receiving the LTM cell switch command MAC CE. If the same TCI state is indicated in LTM CSC, the interruption can be reduced.

For conditional LTM, UE initiate the LTM cell switch without network indication. UE may not be able to acquire TCI state when LTM cell switch is triggered. Several possible solutions for TCI state acquisition are:

- Option 1: Early TCI state activation is supported. UE chooses one of TCI states indicated in early TCI state activation for LTM cell switch and assume network is ready for any decision made by UE.

- Option 2: No early TCI state activation or TCI indication at LTM cell switch. UE determine the default antenna QCL assumption through RACH procedure as in legacy handover. UE receives a TCI state activation indication after the LTM cell switch.

- Option 3: Early TCI state activation is supported. UE informs source cell for the decision of LTM cell switch (e.g., by event triggered L1 MR) in advance, and network indicates the TCI states used for LTM cell switch.

In option 2, the early DL synchronization is not performed in conditional LTM, and UE use the default TCI state as in legacy handover. For LTM, this may result in the loss of benefits of fine beam reporting by L1 MR and an increase in the duration of data interruption. It become more significant in Rel-19 CSI-RS based measurement, if there is any possibility that it can be combined with conditional LTM.

In option 3, the UE needs to notify the source cell before making the LTM cell switch decision, which will increase the handover latency and lose the benefit of "UE initiated" for conditional LTM.

Based on the above analysis, we prefer option1.

Proposal 4: For the TCI state acquisition, early TCI state activation is supported. UE chooses one of TCI states indicated in early TCI state activation for LTM cell switch and assume network is ready for any decision made by UE.

No matter which option is used for TCI state acquisition, RAN1 should be informed.

CFRA related information

In Rel-18 LTM, both CFRA and CBRA-based LTM are supported. In Conditional LTM, without LTM CSC MAC-CE, it should be acceptable to use CBRA for RACH-based conditional LTM, or pre-configure CFRA resources during LTM preparation. Introducing additional signalling to inform the network of the LTM cell switch decision and requiring CFRA resources is not a good choice as it complicates the process.

Therefore, the following proposal is made:

Proposal 5: RAN2 to discuss the following options for RACH-based conditional LTM:

- Option 1: The CFRA resources for candidate cells are pre-configured in LTM config, as conditional handover.

- Option 2: For RACH-based conditional LTM, only CBRA is used.

Triggering condition and event triggered L1 MR

In Rel-18 LTM, there is no triggering configuration as the L1 measurement report is sent periodically, semi-persistently, or aperiodically. For conditional LTM, a triggering condition is necessary. Considering the parallel agenda for event-triggered L1 MR, we can use the triggering condition for event-triggered L1 MR as the baseline (i.e., LTM2 to LTM5).

Proposal 6: The triggering condition for event triggered L1 MR (I.e., LTM2 to LTM5) can be used for the baseline for the triggering config of conditional LTM.

Furthermore, event-triggered L1 MR is designed to notify the network only when necessary and has been designed to used a new MAC-CE as the container, This design aligns well with conditional LTM and may assist in informing UE decisions to the network. Therefore, a general proposal is made to integrate event-triggered L1 MR with conditional LTM.

Proposal 7: Event triggered L1 MR can be used for conditional LTM. FFS if it can be used to inform UE decision to network.

Conclusion

In this contribution, the following observations and proposals are made:

Motivation and general design principle

Observation 1: Two main benefit can be found in conditional handover compared with legacy handover:

- The reliability is increased by UE’s quick response and omitting signalling from failure.

- The accuracy is increased by UE to proactively decide which candidate cell to perform handover based on its finer measurement context.

Observation 2: Two main benefit can be found in conditional handover compared with legacy handover:

- The reliability is increased by UE’s quick response and omitting signalling from failure.

- The accuracy is increased by UE to proactively decide which candidate cell to perform handover based on its finer measurement context.

Proposal 1: RAN2 to discuss which level of conditional LTM is performed:

- Option 1: Early synchronization is network-initiated. UE only decides the LTM execution time and initiate the cell switch procedure.

- Option 2: The UE decides the candidate cell to perform early synchronization and informs the network of its decision (FFS how to indicate network in each step). Subsequently, UE decides the LTM execution time and initiate the cell switch procedure.

Discussion by the lack of LTM CSC MAC-CE

Proposal 2: For conditional LTM, the LTM CSC MAC-CE is replaced by UE initiated LTM cell switch procedure.

Proposal 3: RAN2 to discuss the early UL sync and the acquisition of TA for conditional LTM:

-	Option1:  For RACH-less conditional LTM, RAR is introduced to indicate TA value, which is transferred from serving cell to UE. If the TA is outdated during LTM execution, UE perform RACH-based LTM.

-	Option 2: Conditional LTM is always RACH-based LTM, and early UL sync is not performed.

Proposal 4: For the TCI state acquisition, early TCI state activation is supported. UE chooses one of TCI states indicated in early TCI state activation for LTM cell switch and assume network is ready for any decision made by UE.

Proposal 5: RAN2 to discuss the following options for RACH-based conditional LTM:

-	Option 1: The CFRA resources for candidate cells are pre-configured in LTM config, as conditional handover.

-	Option 2: For RACH-based conditional LTM, only CBRA is used.

Triggering condition and event triggered L1 MR

Proposal 6: The triggering condition for event triggered L1 MR (I.e., LTM2 to LTM5) can be used for the baseline for the triggering config of conditional LTM.

Proposal 7: Event triggered L1 MR can be used for conditional LTM. FFS if it can be used to inform UE decision to network.

References

- RP-242356-Revised-WID-NR-Mob-Enh

Annex: Simulation Assumptions

| Simulation Assumptions |  |
| --- | --- |
| Scenario | Uma@30GHz (Bandwidth: 50 MHz) FR2 |
| Topology | 7 wrapped-around cells with ISD = 200m |
| Distance-dependent path loss | 38.901 UMa pathloss model |
| Antenna Setting (M,N,P,Mg,Ng,Mp,Np) | BS (4,8,2,1,1,1,1) MS (1,2,2,1,2,1,1) |
| Sub-carrier spacing | 60KHz |
| SSB periodicity | 20ms |
| BS Total TX power | 46 dBm |
| TimeToTrigger (for legacy HO) | 160ms |
| a3_offset (for legacy HO) | 2dB |
| Handover preparation delay (for legacy HO) | 60ms |
| LTM Preparation NW delay | 0ms/10ms |
| LTM execution NW delay | 0ms /10ms |
| HO execution time (for legacy HO) | 75ms |
| LTM execution time | 10ms |
| RRC Re-establishment time | 250ms |
| RRC Set-up time | 200ms |
| T310 | 1000ms |
| N310 | 1 |
| n311 | 1 |
| RRC Re-establishment time | 250ms |
| qIn | -6dB |
| qOut | -8dB |
| RRC Reselection Timer | 256ms |
| # of samples for L1 average | 1 sample |