3GPP TSG-RAN WG2 Meeting #127bis	R2-2408303

Hefei, China, Oct 14th – 18th, 2024

Agenda item:		8.9.2

Source:		Lenovo

Title:		Access control and information exchange for Store and Forward operation

Document for:		Discussion

- Introduction

The latest RAN2#126 meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

- RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In this contribution we discuss issues and potential solutions in access control (cell barring) and operation information exchange for S&F operations in IoT NTN.

- Discussion

S&F operation is an operation mode of a 5G system with satellite-access, where the 5G system can provide some level of service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g., to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

## NTN cell access control for Rel-19 NTN UEs

The S&F operation is different from normal operation in that the user data is temporarily stored in the satellite eNB without connection to the core network and forwarded when the connection is available, which needs to be supported by new designed schemes of QoS handling and UE awareness. Therefore, the potential impacts were identified in RAN2#127 meeting focusing on barring legacy UEs and still allowing Rel-19 S&F capable UEs. If an NTN cell operates in S&F mode, legacy Rel-17 and Rel-18 NTN UEs as well as Rel-19 NTN UEs incapable of such capability may not be able to initiate such operation and access control may be necessary.

Based on current RAN2 discussion, cellBarred-NTN introduced in Rel-17 IoT NTN can be reused to restrict Rel-17 & Rel-18 NTN UEs from accessing an NTN cell operating in S&F. However, the access control strategy for Rel-19 NTN UEs need further discussion, i.e., how to allow/restrict access of NTN UE capable/incapable of S&F to an NTN cell operating in S&F mode. It is obvious that the cellBarred-NTN alone cannot facilitate the expected access control for Rel-19 NTN UEs. For example, if cellBarred-NTN is used to prevent access of Rel-17 & Rel-18 NTN UEs to an NTN cell operating in S&F mode, it has to be set to barred, which also prevents access of all Rel-19 NTN UEs.

Observation 1: Using cellBarred-NTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs for S&F operation.

Following this assumption, Rel-19 NTN UEs capable of S&F needs a new scheme to avoid from being barred by cellBarred-NTN set to barred because of operating in S&F (while it may still need to be barred for other reasons when cellBarred-NTN is set to barred). Generally, either explicit indication (e.g., new cell barring bit) or implicit indication (e.g., S&F configuration) can be considered.

Observation 2: Explicit or implicit indication beyond cellBarred-NTN can be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in S&F mode.

For the option of explicit indication, a new barring bit (e.g., cellBarred-NTN_SF) can be introduced, while for the option of implicit indication, an NTN S&F operation indication or configuration can be used. In general, the following alternatives of NTN cell barring logic for NTN-capable UEs can be considered:

- If an NTN-capable UE is capable of S&F (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of the NTN cell:

- Alt. A1: controlled by cellBarred-NTN_SF value. UE ignores the cellBarred-NTN in SIB1 set as barred, and considers the cell as not barred if cellBarred-NTN_SF is set as not barred, or considers the cell as barred if cellBarred-NTN_SF is set as barred.

- Alt. A2: controlled by cellBarred-NTN_SF presence. UE considers the cell as not barred if cellBarred-NTN_SF is present (ignoring cellBarred-NTN in SIB1 set as barred), or considers the cell as barred if cellBarred-NTN_SF is absent (following cellBarred-NTN in SIB1 set as barred).

- Alt. A3: controlled by S&F operation indication or configuration presence. UE considers the cell as not barred if a S&F operation indication is indicated or a configuration is configured, or considers the cell as barred if a S&F operation indication or configuration is absent (following cellBarred-NTN in SIB1 set as barred).

- If an NTN-capable UE is incapable of S&F (e.g., SSB periodicity extension larger than 160ms), it determines the barring status of the NTN cell:

- Alt. B1: controlled by cellBarred-NTN value (same as Rel-17 and Rel-18 UEs). UE considers the cell as not barred if cellBarred-NTN is set as not barred, or considers the cell as barred if cellBarred-NTN is set as barred.

- Alt. B2: controlled by cellBarred-NTN_SF presence. UE considers the cell as barred if cellBarred-NTN_SF is present (following cellBarred-NTN in SIB1 set as barred).

- Alt. B3: controlled by S&F operation indication or configuration presence. UE considers the cell as barred if a S&F operation indication is indicated or a configuration is configured.

Proposal 1: RAN2 to discuss the following options for cell barring regarding NTN S&F.

For UE capable of S&F:

- Alt. A1: controlled by cellBarred-NTN_SF value.

- Alt. A2: controlled by cellBarred-NTN_SF presence.

- Alt. A3: controlled by S&F operation indication or configuration presence.

For UE incapable of S&F:

- Alt. B1: controlled by cellBarred-NTN value (same as Rel-17 and Rel-18 UEs).

- Alt. B2: controlled by cellBarred-NTN_SF presence.

- Alt. B3: controlled by S&F operation indication or configuration presence.

## Validation of access control status

The legacy cell barring schemes including cellBarred-NTN are solely based on the corresponding bit status due to that the network type (e.g., NTN or TN) is fixed, and for cell reselection the barring status is fixed to 300s. UE can only re-acquire the SIB1 to understand if the cell barring status has changed.

Meanwhile, in Rel-19 NTN the access control could be time sensitive as the NTN cell may change between modes enabling and disabling S&F (SSB periodicity extension). And it is possible that some prior information can be determined by network and even be indicated to UE to implement optimized access control e.g., based on flexible time duration in coordination with NTN S&F operations.

Observation 3: The cell barring status regarding NTN S&F is associated to the operation duration or cycle which can be flexible and pre-determined.

Therefore, if UE can be informed about the NTN S&F operation duration or cycle, it may also determine the validity duration of a cell barring status determination, which can prevent UE from unnecessary SIB1 acquiring for cell barring status or allow UE to acquire SIB1 for cell barring status in time.

Proposal 2: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN S&F.

## Uu information exchange for S&F operation

Referring to SA2 works as in TR22.865, at least the following scenarios or use cases can be considered for S&F operations: MO, MT, data transfer for IoT devices in remote areas, and emergency report. The main different between S&F operation and normal operation in NTN is whether there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network i.e., whether it requires the service and feeder links to be active simultaneously. Such differences have brought in some new requirements as consolidated by SA2:

- support S&F operation for authorized UEs;

- support to inform a UE whether S&F Satellite operation is applied;

- support to allow the operator or a trusted 3rd party to apply an S&F data retention period;

- support to allow the operator or a trusted 3rd party to apply an S&F data storage quota;

- support a mechanism to configure and provision specific required QoS and policies for UE’s data;

- support to provide related information (e.g. estimated delivery time to the authorised 3rd party) to an authorized UE.

- support to inform an authorised 3rd party whether S&F Satellite operation is applied for communication with a UE and to provide related information (e.g. estimated delivery time to the authorised UE).

- support forwarding of the stored data from one satellite to another satellite (e.g., which has an available feeder link to the ground network), through ISLs.

- support suitable means to resume communication between the satellite and the ground station once the feeder link becomes available.

- support mechanisms for a UE to register with the network when the network is in S&F Satellite operation.

- support mechanisms to authorize subscribers for receiving services when the network is in S&F Satellite operation.

Observation 4: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

According to the listed cases and requirements, the services that can leverage S&F operations have some specific characteristics including delay-tolerant (could be hours) and remote (source or target could be reached only be limited satellite coverage), and the UE may need some availability information about the S&F operation supported by the network, e.g., for which area or for how long the data can be delivered, or some functional information about the S&F operation supported by the network, e.g., support of providing delivery status, support of cast types for delivery (unicast, multicast and broadcast), support of eNB/gNB-only S&F (no CN entity is involved during the S&F operation).

Observation 5: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

On the other hand, network may also need to be aware of UE requirements for S&F operation, e.g., to configure UE for S&F data transmission/reception or to understand for which area or for how long the data shall be delivered, or whether providing delivery status is needed, or which cast types for delivery is expected.

Observation 6: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

When both the network and UE supports S&F operation, the UE needs to determine when an S&F operation service can be initiated e.g., whether a service can leverage S&F operation. From protocol design perspective, the AS-NAS functional spilt in UE for S&F shall be specified, e.g., which stratum is responsible for the initiation and the other stratum can provide assistances. Moreover, AS-NAS interaction could be needed to ensure appropriate S&F initiation and data delivery, e.g., start/suspend NAS->AS S&F data delivery when satellite coverage is valid/invalid to avoid congestion.

Observation 7: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

Based on the above analysis, the following aspects can also be considered for RAN2’s study on supporting S&F operations:

Proposal 3: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).

- Conclusion

In this contribution we discuss issues and potential solutions in access control (cell barring) and operation information exchange for S&F operations in IoT NTN. The following observations are given:

Observation 1: Using cellBarred-NTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs for S&F operation.

Observation 2: Explicit or implicit indication beyond cellBarred-NTN can be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in S&F mode.

Observation 3: The cell barring status regarding NTN S&F is associated to the operation duration or cycle which can be flexible and pre-determined.

Observation 4: At least from RAN2 perspective, solutions may also be developed to fulfil some requirements of S&F operation as consolidated by SA2.

Observation 5: For S&F operations, it may also be necessary to indicate UE some availability information about the S&F operation.

Observation 6: For S&F operations, the network may need to know UE’s service requirements for S&F operation.

Observation 7: Metrics and interactions to initiate an S&F operation at UE may need to be specified.

And we proposed:

Proposal 1: RAN2 to discuss the following options for cell barring regarding NTN S&F.

For UE capable of S&F:

- Alt. A1: controlled by cellBarred-NTN_SF value.

- Alt. A2: controlled by cellBarred-NTN_SF presence.

- Alt. A3: controlled by S&F operation indication or configuration presence.

For UE incapable of S&F:

- Alt. B1: controlled by cellBarred-NTN value (same as Rel-17 and Rel-18 UEs).

- Alt. B2: controlled by cellBarred-NTN_SF presence.

- Alt. B3: controlled by S&F operation indication or configuration presence.

Proposal 2: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN S&F.

Proposal 3: To support S&F operation and fulfill requirements from SA2’s work, RAN2 to study the following:

- how to indicate UE the availability information about the S&F operation (e.g., estimated delivery time);

- how to indicate network the UE’s service requirements for S&F operation;

- AS-NAS functional split and interactions to support S&F operation (may pending on SA2 future work).