3GPP TSG-RAN WG2 Meeting#127bis	R2-2408316

Hefei, China, 10 - 14 October 2024

Agenda item:	 8.10.2

Source:		Lenovo

Title:		Discussion on MRO for R18 mobility

Document for:		Discussion and Decision

1	Introduction

It was agreed in RAN2#125bis meeting that:

For LTM MRO, RAN2 considers the following three connection failure cases:

- -	Too late LTM

- -	Too early LTM

- -	LTM to wrong cell

For too late LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 1a: the UE detects RLF in source cell after receiving LTM candidate configurations and performs reestablishment procedure.

- -	Case 1b: the UE detects RLF in source cell after receiving LTM candidate configurations, selects an LTM candidate cell, detects HOF with the selected LTM cell.

- -	Case 1c: the UE detects RLF in source cell after receiving LTM candidate configurations, and successfully completes LTM execution with the selected LTM candidate cell.

For too early LTM, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 2a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 2b: the UE detects HOF/RLF in the LTM target cell, selects the source cell which is also an LTM candidate cell, detects HOF with the source cell, and performs reestablishment procedure.

- -	Case 2c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected source cell which is also an LTM candidate cell.

LTM to wrong cell, the following sub-cases are considered but we may down prioritize later (not limiting):

- -	Case 3a: the UE detects HOF/RLF in the LTM target cell and performs reestablishment procedure with the source cell.

- -	Case 3b: the UE detects HOF/RLF in the LTM target cell, selects an LTM candidate cell which is different from the source or target one, detects HOF with the selected LTM candidate cell, and performs reestablishment procedure.

- -	Case 3c: the UE detects HOF/RLF in the LTM target cell, and successfully completes LTM execution with the selected LTM candidate cell which is different from the source or target one.

RAN2 considers SHR, RA report and RLF for MCG LTM SON.

RAN2 will start work on MCG LTM.

RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

It was further agreed in RAN2#126 meeting that:

If available, log the L1 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure.

Reuse the existing approach of using timeConnFailure and the reconnectCellId in RLF-report also for LTM failures, details TBD.

Log the LTM cell ID upon performing recovery an LTM candidate cell, details TBD e.g. which field.

Extend lastHO-Type in RLF-Report to indicate the LTM cell switch as last executed mobility procedure.

RAN2 to enhance the RLF report with additional information regarding the state of the two execution conditions. We see later if we also can enhance the SCGFailureInformation report.

RAN2 will look into failure and near failure scenarios for subsequent execution of SCPAC and see if/what enhancements are needed.

RAN2#127 meeting further agreed that:

Only the field description associated to the timeConnFailure IE needs to be updated accordingly. Otherwise, we don’t expect any further specification impact for timeConnFailure and reconnectCellId is foreseen, TBC.

introduce a new field in RLF report to indicate the LTM recovery cell id.

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

We do not include in the RLF report, the time since the last reception of PDCCH order towards the target cell till reception of cell switch command.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report

Reuse the existing approach of using timeUntilReconnection in RLF-report also for LTM failure scenarios.

We will not define and log timeSinceLTM-Reconfig (like timeSinceCHO-Reconfig) within RLF-report and SHR in LTM failure and near failure cases.

We will not log interruption time in SON reports (e.g. SHR) for LTM.

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

Log L3 measurements for serving cell, target cell and other LTM candidate cells in RLF report, upon RLF or mobility failure. RAN2 assumes this is already possible with existing spec.

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

In this paper, we would discuss the details of MRO for R18 mobility features, including LTM, CHO with candidate SCG(s) and subsequent CPAC.

2	Discussion

  - 2.1 MRO for LTM

We would start work with MCG LTM to mainly discuss RLF report, SHR and RA report for MCG LTM.

To reduce interruption time during PSCell change procedure, SCG LTM is introduced in R18 with same priority as MCG LTM. SCG LTM failure case and even near failure successful SCG LTM case should be avoided to improve network performance, we can further discuss MRO for SCG LTM when the discussion of MRO for MCG LTM is stable.

    - 2.1.1 MRO for connection failure in a LTM procedure

Since LTM is a L1/L2 triggered mobility which is totally different from legacy L3 mobility procedure, when connection failure happens in LTM, in order to modify LTM related configurations properly, it is necessary to introduce a new type for last mobility e.g. LTM cell switch in the RLF report, especially for the case that only legacy IEs in RLF report are reused for LTM (e.g. in case that LTM failure recovery is not configured, thus LTM recovery cell id is not reported in RLF report).

Proposal 1: Introduce a new type for last mobility e.g. LTM cell switch in the RLF report.

Last RAN2#127 meeting agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Similar as near failure case, for connection failure case in LTM, an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not may be included in the RLF report especially for the case that UE context related with LTM configurations (e.g. the list of LTM candidate cells) is released in network.

Proposal 2: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the RLF report.

    - 2.1.2 MRO for near failure in a LTM procedure

In legacy, SHR is introduced to detect sub-optimal successful handover. For one LTM cell switch execution in a MCG LTM procedure, it would be a near-failure successful LTM PCell switch. The UE may store near-failure successful MCG LTM related information to assist network to perform MRO analysis and optimisation. Since a LTM procedure is triggered based on L1 measurement report, RAN2#126 meeting agreed to log L1 measurements for serving cell, target cell and other LTM candidate cells if available in RLF report upon RLF or mobility failure, similar as failure case due to wrong selection of LTM candidate cell, it is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when the LTM cell switch execution is successful.

Proposal 3: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

Last RAN2#127 meeting agreed to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit. Considering the case that network does not have UE context related with LTM configurations (e.g. the list of LTM candidate cells) when receiving the SHR, an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not may be included in the SHR.

Proposal 4: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR.

    - 2.1.3 RA report for a LTM procedure

In a LTM procedure, depending on the availability of a valid TA value, the UE performs either a RACH-less LTM or RACH-based LTM cell switch:

- the UE performs RACH-less LTM cell switch upon receiving the cell switch command:

  - if the TA value is provided in the cell switch command, the UE applies the TA value as instructed by the network. For this case, before cell switch command, early TA acquisition/UL synchronization procedure is triggered by PDCCH order to an LTM candidate cell, or the UE performs RACH for early UL synchronization or RACH for early TA acquisition;

  - if UE-based TA measurement is configured, but no TA value is provided in the cell switch command, the UE applies the TA value by itself if available.

- if no valid TA value is available, the UE performs RACH-based LTM cell switch upon receiving the cell switch command:

  - the UE performs CFRA to the target cell, if contention-free Random Access Resources are provided in LTM Cell Switch Command MAC CE;

  - or, the UE performs CFRA to the target cell, if no contention-free Random Access Resource is provided in LTM Cell Switch Command MAC CE, but contention-free Random Access Resources are provided in the RRC Reconfiguration message for the LTM procedure;

  - or, the UE performs CBRA to the target cell, if no contention-free Random Access Resource is provided.

To optimize RA resources for LTM procedure, the UE may report L1 measurement results (of source cell, or LTM candidate cell, or target cell) during LTM procedure in the RA report. Additionally, for optimization of early UL synchronization or early TA acquisition, a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early UL synchronization or early TA acquisition may be included in the RA report.

Proposal 5: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

  - 2.2 MRO for CHO with candidate SCG(s)

    - 2.2.1 MRO for connection failure case

RAN3 summarized the failure cases as below table in [1]:

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |

It was agreed in RAN3#125 meeting that MRO for case 4/5/6 is in the scope and it was FFS whether there is any specification impact for case 4/5/6. In case of 4/5/6, configuration of CHO only is configured to the UE together with configuration of CHO with candidate SCG(s) configuration, the UE performs CHO only execution due to CHO with candidate SCG(s) can’t be triggered (e.g. CHO execution condition of CHO with candidate SCG(s) is fulfilled but CPAC execution condition of CHO with candidate SCG(s) is not fulfilled, CPAC execution condition of CHO with candidate SCG(s) is fulfilled but CHO execution condition of CHO with candidate SCG(s) is not fulfilled, neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled). To improve mobility performance, it is beneficial for network to know the reason about why CHO with candidate SCG(s) together with CHO only is configured but CHO only is performed instead of CHO with candidate SCG(s), e.g. only CHO execution condition or only CPAC execution condition of CHO with candidate SCG(s) is fulfilled, or neither CHO execution condition nor CPAC execution condition of CHO with candidate SCG(s) is fulfilled. RAN3#125 meeting agreed the RLF report includes the type of the first fulfilled execution condition e.g. CPAC or CHO, fulfilment means all events of the type are met. This agreement applies to the case 4/5, for case 6, SCG failure information message needs to include the type of the first fulfilled execution condition of CHO with candidate SCG(s) if any e.g. CPAC or CHO.

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Last RAN3#125 meeting also agreed that MRO for case 1a, 1b, 2a, 2b, 3a, 3b, 7a, 7b, 8a, 8b, 9 will be addressed. For case 3a, 3b, 4, 5, 7a, 7b, 8a, 8b and 9, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions before the failure is encountered in the RLF report. For case 2a, 2b, 7a, 7b, 8a, 8b and 9, it is needed to include identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the RLF report. For case 6, it is needed to include identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before the failure is encountered in the SCG failure information message.

Proposal 7: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in RLF report or SCG failure information message.

For case 7 and case 8, connection failure happens after CHO with candidate SCG(s) is triggered. Last RAN2#127 meeting agreed that the UE includes time information regarding condition fulfilment for CHO with candidate SCGs in RLF report, the details are FFS. To analyse whether the failure is caused due to inappropriate CHO execution condition and/or CPAC execution condition, it is beneficial to store and report time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition in RLF report or SCG failure information message.

Proposal 8: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in RLF report or SCG failure information message.

    - 2.2.2 MRO for near failure successful case

When CHO with candidate SCG(s) is executed, a near-failure successful PCell change and/or a near-failure successful PSCell addition or PSCell change may happen.

For legacy SHR or SPR, T310/T312/T304 related trigger threshold is configured by network to trigger SHR or SPR. It seems that there is no issue to reuse legacy SHR or SPR trigger threshold for the CHO with candidate SCG(s) procedure.

When CHO with candidate SCG(s) is performed, both CHO execution condition of the target PCell and corresponding CPAC execution condition of the associated target PSCell are fulfilled, there is a possibility that the time duration between the fulfilment of CHO execution condition and the fulfilment of CPAC execution condition is higher than a threshold, which may cause a near-failure successful CHO with candidate SCG(s). Therefore, besides legacy T310/T312/T304 related trigger threshold, RAN2 can consider new trigger condition(s) for SHR or SPR for the CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell. When the time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell is higher than the threshold, SHR and SPR may be generated by the UE.

Proposal 9: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

No matter what is the trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), for sub-optimal successful CHO execution, to help network know how to modify the configuration of the CHO with candidate SCG(s) procedure, R17 SHR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SHR.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Similarly, for any sub-optimal successful CPAC, R18 SPR can be enhanced, e.g. the type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) and the time duration between two fulfilled execution conditions (e.g. the time elapsed between CHO execution condition is fulfilled and the associated CPAC execution condition is fulfilled) can be stored and reported by the UE in SPR.

Proposal 11: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

  - 2.3 MRO for subsequent CPAC

In R18, intra-SN subsequent CPAC initiated by the SN, inter-SN subsequent CPAC initiated by either MN or SN are supported. Subsequent CPAC is only supported for NR-DC. The subsequent CPAC mechanism is defined as a conditional PSCell addition or change procedure that is executed after a PSCell addition, a PSCell change, a PCell change or an SCG release based on pre-configured subsequent CPAC configuration of candidate PSCell(s) without reconfiguration and re-initiation of CPC/CPA. The UE keeps the configured subsequent CPAC configuration (unless the network indicates to release it) and evaluates the execution conditions of candidate PSCells after completion of a PSCell addition, a PSCell change, a PCell change or an SCG release.

    - 2.3.1 MRO for connection failure in a subsequent CPAC procedure

The following failure may happen in initial CPAC execution phase or following subsequent CPC execution phase in a subsequent CPAC procedure:

- an SCG failure occurs before initial CPC execution or following subsequent CPC execution is executed;

- an SCG failure occurs during initial CPAC execution phase or following subsequent CPC execution phase, i.e. the UE fails to access the target PSCell of initial CPAC execution or following subsequent CPC execution;

- an SCG failure occurs shortly after successful initial CPAC execution or following subsequent CPC execution.

In legacy PSCell change or CPAC, SCG Failure Information message can be used by the UE to report SCG failure related information. Similarily, when a SCG failure happens in initial CPAC execution phase or following subsequent CPC execution phase, the SCG Failure Information message in Uu can betaken as the baseline. Since configuration for initial CPAC or following subsequent CPC is different, to enable NW better modify corresponding configuration for MRO, it is benecifical to enable NW know whether the failure happens in initial CPAC or following subsequent CPC. For this purpose, an indication concerning the SCG Failure Information message is for a initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message‎.

Proposal 12: An indication concerning the SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message.

    - 2.3.2 MRO for near failure in a subsequent CPAC procedure

A near-failure successful CPAC execution may occur during initial CPAC execution phase or at least one following subsequent CPC execution phase. In R18, SPR is introduced to detect sub-optimal successful PSCell addition or change, the SPR is not sent immediately after a successful PSCell addition or change, and only the latest successful PSCell addition or change is stored or reported by the UE. For subsequent CPAC, we should discuss whether SPR can be reused or whether to introduce a new successful report, especially considering that multiple CPAC executions are performed in a subsequent CPAC procedure. If SPR is reused, we should further discuss whether only the latest successful CPAC execution should be included in the SPR as legacy, or whether SPR should be enhanced to include multiple successful CPAC executions.

Proposal 13: RAN2 to discuss whether SPR can be reused for subsequent CPAC.

3	Conclusion

In this paper, the details of MRO for R18 mobility features are discussed. We have the following proposals:

MRO for LTM

Proposal 1: Introduce a new type for last mobility e.g. LTM cell switch in the RLF report.

Proposal 2: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the RLF report.

Proposal 3: It is beneficial for the UE to report L1 measurement results for source cell, target cell and other LTM candidate cells if available in SHR when LTM cell switch execution is successful.

Proposal 4: Include an explicit indication concerning whether a neighbour cell is an LTM candidate cell or not in the SHR.

Proposal 5: The UE can report L1 measurement results during LTM procedure, and a new RA purpose to indicate that the random access procedure was triggered by a PDCCH order for early TA acquisition, in the RA report.

MRO for CHO with candidate SCG(s)

Proposal 6: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition) if any, can be stored and reported by the UE in SCG failure information message.

Proposal 7: Identifier of candidate PCell(s) which met the configured CHO execution conditions and/or identifier of candidate PSCell(s) which met the configured CPAC execution conditions before connection failure is encountered can be stored and reported by the UE in RLF report or SCG failure information message.

Proposal 8: Time duration between the fulfilled CHO execution condition and the fulfilled CPAC execution condition can be stored and reported by the UE in RLF report or SCG failure information message.

Proposal 9: Introduce new trigger condition(s) for SHR or SPR for a CHO with candidate SCG(s), e.g. a threshold related to time duration between fulfilment of CHO execution condition configured for the target PCell and fulfilment of CPAC execution condition configured for the associated target PSCell.

Proposal 10: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SHR.

Proposal 11: The type of the first fulfilled execution condition (e.g. CHO execution condition or CPAC execution condition), and time duration between two fulfilled execution conditions, can be stored and reported by the UE in SPR.

MRO for subsequent CPAC

Proposal 12: An indication concerning the SCG Failure Information message is for an initial CPAC execution or a following subsequent CPC execution may be included in the SCG Failure Information message.

Proposal 13: RAN2 to discuss whether SPR can be reused for subsequent CPAC.

Reference

[1] R3-244223, Failure scenarios on MRO for CHO with candidate SCGs, Samsung, Cybercore, ZTE