3GPP TSG-RAN WG2 Meeting #127bis	R2-2408376

Hefei, China, Oct 14th – 18th, 2024

Agenda item:	8.1.2.2

Title:	Discussion on LCM for UE-sided model for Beam Management

Source:	ZTE Corporation

Document for:	Discussion and Decision

Introduction

As to the applicability reporting aspects, following agreement were reached in RAN2#125bis[2].

Agreements for positioning and beam management

- Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message.

- Support reactive reporting of UE-sided applicable functionality.  The network configures AI/ML functionalities via RRC/LPP message.  FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality

3	FFS how the two approaches will be specified and whether we can combine them into one procedure.    FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed)

In RAN2#126[3], following agreements related to UE side LCM were reached:

Agreements

1	RAN2 will support functionality activation/deactivation after inference configuration.   FFS initial state of configuration and how activation/deactivation is achieved.   FFS what Deactivation refers to:  examples discussed: 1) fallback to legacy 2) switching, etc.

2	We will work offline on the definitions for functionality types and define what is availability.

3	The UE will indicate the gNB/LMF whether the AI/ML functionality is available/applicable.   For a functionality to be applicable at least there should at least one model available within it.   FFS other details on what is applicability/non-applicability.

4	For network-side additional conditions, RAN2 assumes that RRC signaling from gNB to UE can be designed for consistency between inference and training. RAN2 will wait for RAN1 input for further details. FFS if the same applies to positioning

5	For BM use case, As a baseline the UE determines whether a functionality is applicable. Existing UAI framework is used at least for proactive reporting of applicable functionality.  FFS reactive

In RAN2#127[4], following agreement were reached:

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from network to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide network-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on network-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if network-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to network-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc - Step 5: 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. - The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality - We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

This contribution provides considerations on the issues that following issue which are not dependent on RAN1 input:

- Applicability reporting genenral aspects

- Activation/deactivation of functionality aspects

- Applicability reporting and inference configuration during mobility

- UE request for resource for data collection for UE-side model training aspects.

Discussion

Applicability Reporting Aspect

In last RAN2 meeting, the applicability reporting procedure was discussed based on following procedure.

There are some FFS issues related to applicability reporting that can be discussed in RAN2 without further RAN1 input.

- “Step 3”: Following configurations are provided from network to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide network-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on network-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if network-side additional condition is not provided in step 3.

The detail definition of network-side additional condition is pending in RAN2 and waiting for RAN1 input. However RAN2 can still discuss the FFS on RRC signaling used to provide network-side additional condition and whether it is mandatory or optional.

In our understanding, for a specific functionality, network knows the network-side additional condition of the current serving cell, and what it intends to configures to UE. So the network of course can include the network-side additional conditions for which network want to request applicability. And in order to save signaling size, network should be able to filter the applicability reporting by indicating network-side additional conditions to UE. UE reports applicability only for those requested by network. Since the functionality configuration is decided by network, network should have full control-ability on what UE needs to report.

- The gNB should provide information of the functionality and associated ID to UE for applicability reporting. The UE performs applicability reporting only for the requested functionality and associated ID

- “Step 4”: UE reports applicable functionality in the following scenarios: 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI 2) As response to network-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration), FFS via UAI or RRCReconfigurationComplete, etc

As to the signaling used for applicability reporting in step 4, we think it is preferred to have an unified solution.

Several options were proposed on the signaling used for reactive reporting and proactive reporting in previous meeting. In our understanding, the main difference between reactive reporting and proactive reporting is the time point when the UE perform reporting. For reactive reporting, the UE can response/report immediately upon reception of the applicability request. While for proactive reporting, the reporting may be triggered a while after reception of request, e.g. after model is available or the applicability changes after network-side requesting.

Some companies may think the difference between reactive reporting and proactive reporting is whether full configuration is provided before applicability reporting. But we cannot agree with this. We think for both reactive and proactive reporting, full configuration is not needed. So, it is possible to combine reactive and proactive reporting as an unified solution.

Currently there are two main options on signaling use for applicability reporting. One is to use RRCReconfigurationComplete/RRCResumeComplete message, another is to use UAI. Considering that applicability may change dynamically according to UE side additional conditions/network side additional conditions, applicability reporting may be triggered at any time. So UAI is more preferred to be adopted as base line.

- To adopt an unified signaling solution for reactive and proactive reporting, e.g. information for applicability request is conveyed in RRCReconfiguration/RRCResume (step 3), applicability reporting is transmitted via UAI as baseline (step 4).

Activation/deactivation aspect

On activation and deactivation, it is still FFS on the initial state of a functionality upon inference configuration is provided to UE before applicability reporting and after applicability reporting. And FFS on the additional L1/L2 signaling for activation/deactivation[4]:

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality

First of all, we think whether full interference configuration is provided in step 3 is up to network implementation. No matter whether full inference configuration is needed in step 3 for applicability decision, the network can provide it anyway to the UE as implementation.

However, when network provides full inference configuration to UE in step 3, network has no idea whether the functionality is applicable or not. So naturally the initial state of the functionality should be deactivated. Otherwise, the network has to assume the configured functionality is applicable and has to prepare network side corresponding resource for the functionalities, e.g. starting to transmitting corresponding beams in step 3. It seems unreasonable for network to have such implementation. Further, the time when the UE activates the functionality will be uncertain from network side of view, and may result in misalignment between UE and network.

- To allow UE automatically activating AI/ML functionality when UE thinks the functionality is applicable, e.g. in step 3, network has to assume all configured functionalities applicable and always prepare corresponding resources. This is not a reasonable implementation for network and will result in misalignment between UE and network.

Some companies proposed to provide full inference configuration in step 3 and UE activate the applicable functionality automatically to achieve fast activation of the functionality. However, we don’t think it is so necessary to have fast activation for AI/ML functionality because it is a substitute for legacy functionality. In other words, before the functionality is activated, the corresponding legacy function always works. So we cannot see why fast activation is critical.

- To achieve fast activation by allowing UE automatic activating functionality is not necessary for AI/ML functionality.

Based on above discussion, we prefer the AI/ML functionality is initially deactivated upon configuration. The functionality can only be activated by network explicit indication. For a legacy configuration, it is configured only if the UE support it. The legacy function can be activated upon configuration without additional activation indication. While for AIML functionality, supporting of a AIML functionality does not mean the functionality can be activated. That’s why applicability reporting is needed. So activation upon configuration logic is not suitable for AI/ML functionality.

- Different to legacy features, supporting an AI/ML functionality does not mean it is applicable or can be activated. Thus it is improper to define the initial state of AIML functionality upon configuration as activated by default.

As to the signaling used to activating functionality. We think the baseline can be an activation/deactivation flag included in a RRC message, e.g. the RRC message that is used to configure inference configuration. Whether additional L1/L2 signaling is needed depends on whether there is such use case that fast activation is needed. And it may be decided by RAN1.

- AI/ML functionality is by default deactivated upon configuration in both step 3 and step 5. AI/ML functionality can only be activated by explicit indication. The RRC signaling is used as baseline for activation and deactivation.

As to deactivation of functionality, the network may want to deactivate a functionality in case the functionality becomes inapplicable, or the performance of the functionality becomes worse than the threshold. So the network should be able to explicitly deactivate a functionality or implicitly deactivate it by remove the inference configuration.

On the other hand, it is unclear whether UE can deactivate a functionality automatically in case of exceptional event. For example, the beam failure is detected when beam management functionality is running. It may be preferred to deactivate the functionality and fallback to legacy beam measurement. Or in case the performance of a functionality is degrade below a threshold, UE may need to fall back to legacy beam management as soon as possible.

- Network can deactivate a functionality explicitly by indicating “deactivation” or implicitly by deleting inference configuration. FFS whether and when UE can deactivate functionality automatically.

Applicability during mobility aspect

There are 3 options regarding how to handle applicability reporting and inference configuration during mobility.

Option 1: Target cell requests applicability reporting during or after handover.

Figure 1: mobility scheme option 1

As illustrated in figure 1, target cell includes information for applicability reporting, e.g. functionality information, and network-side additional condition of the target cell in the RRCReconfiguration message prepared for the UE. The UE reports applicability of the functionality either via RRCReconfigurationComplete or UAI message after it accesses the target cell. Then the target cell can configure/activate the functionality accordingly.

This option is straightforward without additional impact on specification. So it can be regarded as a baseline solution.

Option 2: Source cell transfers source cell’s applicability information to target cell to assist configuration in target cell.

Figure 2: mobility scheme option 2

The source cell transfers the applicability information gathered for source cell to the target cell via handover preparation procedure. The applicability information includes applicable network-side additional conditions of the source cell.

Based on the source cell’s applicability information, the target cell deduces whether the functionality is also applicable in target cell by comparing the network-side additional condition of source cell and the target cell. Although network-side additional conditions, i.e. Associated ID, is unique only within serving cell level, this option is still feasible if by implementation source cell and neighbor cell have the knowledge on whether network-side additional condition represented by source cell’s Associated ID and the target cell’s Associated ID are the same.

The advantage of this option is that the target cell can configure and activate functionality in the handover command. And the applicability reporting procedure in the target cell may be saved.

Note the option 2 is also applicable to the mobility schemes that source cell prepares multiple candidate configurations in advance, e.g. LTM and CHO. The source can transfer source cell’s applicable network-side additional conditions to candidate cells, and candidate cell may prepare candidate configuration including inference configuration. Thus when UE is switched to target cell, the corresponding inference configuration for the functionality can be activated without additional signaling procedure between UE and target cell.

Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

Figure 3: mobility scheme option 3

In this option, the source cell requests functionality applicability for the target cell before handover occurs.

Althougth it is still FFS in RAN1 on whether the assumption on Associated ID can be applicable for multiple cells, i.e. multiple cell level Associated ID, the network side additional of source cell and target cell may still be different. So it is meaningful to discuss this option no matter the Associated ID is cell level or multiple cell level.

Before applicability reporting, RAN3 signaling is needed to exchange network-side additional conditions between neighbor cells.

The source cell request the applicability with target cell’s network-side additional condition(s), in the same way as request applicability for source cell’s network-side additional condition.

Then the source can transfer the gathered applicability information to target cell during handover request procedure.

The target cell prepares the configuration based on the applicability information and sends the configuration to UE via RRCReconfiguration message.

Note that this solution is also useful for CHO and LTM where candidate configuration is prepared in advance. And the same benefit as in option 2 can be achieved.

- RAN2 to consider following options regarding the applicability reporting and inference configuration during mobility:

- Option 1: Target cell requests applicability reporting during or after handover.

- Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

- Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

If option 3 is adopted, RAN2 needs to decides whether UAI message can be used to convey applicability reporting for a functionality in neigbor cell. In option 3, the funtionality information, neighbor cell’s network side additional conditions, and part or full inference configuration may be provided to UE in step 3. Applicability reporting for the neighbor cell will be conveyed in UAI message. We don’t think it is a problem to use UAI for applicability reporting for neighbor cell.

- If option 3 is adopted, RAN2 to decide whether UAI message can be used to convey applicability reporting for neigbor cell.

UE request for RS for training aspect

Some companies proposed that UE can be allowed to request resource for data collection for UE-side training. The UE can inform the network about the need to start/stop a data collection. We understand this is only useful when AI/ML functionality BM use case is deployed for a period of time. E.g. only beams included in set B are transmitted while beam included in set A are not. In this case, if UE needs to perform data collection for model re-training , it is meaningful to allow UE to request resources needed for data collection.

- RAN2 to discuss whether UE is allowed to request resource for data collection for UE-side model training.

If it is allowed, what UE can request should be limited by network configuration. In our understanding, although the model is UE-sided, it still depends on network configuration and intention. In other words, it is meaningless to train UE side model which will not be configured by network. So it is reasonable for the network to be able to configure what UE can request for data collection. FFS on the how to signal it.

- Network configures what UE can request for data collection for UE side training. UE can only request what the network allows to. FFS how to signal it.

Further, the network should be able to decide whether to transmit the requested resource for data collection. Because what UE request will increase overhead in network side, the network can transmit what UE request if the network load is allowed. Otherwise, the network should be able reject UE’s request.

- Network should be able to reject UE’s request for data collection for UE-side training.

Conclusion

On applicability reporting aspect:

- The gNB should provide information of the functionality and associated ID to UE for applicability reporting. The UE performs applicability reporting only for the requested functionality and associated ID

- To adopt an unified signaling solution for reactive and proactive reporting, e.g. information for applicability request is conveyed in RRCReconfiguration/RRCResume (step 3), applicability reporting is transmitted via UAI as baseline (step 4).

On functionality activation / deactivation:

- To allow UE automatically activating AI/ML functionality when UE thinks the functionality is applicable, e.g. in step 3, network has to assume all configured functionalities applicable and always prepare corresponding resources. This is not a reasonable implementation for network and will result in misalignment between UE and network.

- To achieve fast activation by allowing UE automatically activating functionality is not necessary for AI/ML functionality

- Different to legacy features, supporting an AI/ML functionality does not mean it is applicable or can be activated. Thus it is improper to define the initial state of AIML functionality upon configuration as activated by default.

- AI/ML functionality is by default deactivated upon configuration in both step 3 and step 5. AI/ML functionality can only be activated by explicit indication. The RRC signaling is used as baseline for activation and deactivation.

- Network can deactivate a functionality explicitly by indicating “deactivation” or implicitly by deleting inference configuration. FFS whether and when UE can deactivate functionality automatically.

On functionality applicability in mobility:

- RAN2 to consider following options regarding the applicability reporting and inference configuration during mobility:

- Option 1: Target cell requests applicability reporting during or after handover.

- Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

- Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

- If option 3 is adopted, RAN2 to decide whether UAI message can be used to convey applicability reporting for neigbor cell.

On UE request resource for data collection:

- RAN2 to discuss whether UE is allowed to request resource for data collection for UE-side model training.

- Network configures what UE can request for data collection for UE side training. UE can only request what the network allows to. FFS how to signal it.

- Network should be able to reject UE’s request for data collection for UE-side training.

References

- TR 38.843

- RAN2#125bis chairman notes

- RAN2#126 chairman notes

- RAN2#127 chairman notes