3GPP TSG-RAN WG2 Meeting #127bis	R2-2408399

Hefei, China, Oct 14th – 18th, 2024

Source:         ZTE Corporation

Title:            Consideration on Supporting 3M Channel Bandwidth

Agenda item:    7.25.1

Document for:   Discussion and Decision

- Introduction

About 3M channel bandwidth, some per band capabilities have been defined for both the symmetric and asymmetric case. However during some offline discussion, companies still have question on whether there is a need to extended them to the per FSPC level, e.g. include the 3M in per FSPC capability “supportedBandwidthDL/UL”.

Furthermore, in the R19, the 3M capability would also be supported for the CA/DC case, and the related capabilities may also be discussed in RAN4 this meeting.

In this paper, we’d like to share our further consideration on supporting 3M channel bandwidth.

- Discussion

According to the RAN2 previous discussion, even for the single CC case, the correponding capability would be reported with a BC.

To determine the channel bandwidth for each CC of the BC, the Note was added in the field description of the channelBWs-DL/UL, in which:

- For the normal bandwidth: the supportedBandwidthDL/UL, the channelBWs-DL/UL and the supportedBandwidthCombinationSet would be considered together.

- For the special cases (e.g. 90M), some exceptional description would be added.

channelBWs-DL ….. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with other bandwidth combination set than BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 90 MHz for the band combination with BCS5, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. To determine whether the UE supports a channel bandwidth of 400 MHz, the network may ignore this capability and validate the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the supportedBandwidthDL, and supportedBandwidthCombinationSetIntraENDC-v1790. For serving cell(s) with other channel bandwidths: - If supportedAggBW-FR1-r17 is reported, the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL-v1780, supportedMinBandwidthDL, supportedAggBW-FR1-r17, and supportedBandwidthCombinationSetIntraENDC-v1790. - Otherwise, the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL/supportedBandwidthDL-v1710, supportedMinBandwidthDL, supportedAggBW-FR2-r17, and supportedBandwidthCombinationSetIntraENDC-v1790.

However, for the 3MHz, only some per band level capabilities are indicated in the current spec. Without the exceptional description in the Note of the channelBWs-DL/UL, the NW would determine the 3M as not supported even though the capability support3MHz-ChannelBW-Symmetric-r18 and/or support3MHz-ChannelBW-Asymmetric-r18 was included.

To solve this issue, there are 2 options:

- Option 1: Only consider the single CC case and add the exceptional description to Note of the field description of the channelBWs-DL/UL;

- Option 2: Introduce new per FSPC level capability or extend the supportedMinBandwidthDL/UL to include 3M.

The advantage of the option 1 is that it has no impact to the ASN.1 coding, but the drawback is that it may have no forward compatibility for the R19 CA/DC case (if the per BC level capability is required). With the option 2, it’s compatible for both the non-CA and CA/DC case (if the per BC level capability is required), however it would introduce some ASN.1 work, and there is no conclusion in RAN4 on how to support CA/DC case. Considering that it may need several meetings to discuss the UE capability structure for the R19 3M for the CA/DC case, we slightly prefer to go to the option 1 at this stage, a CR based on the option 1 was also provided in [1].

Proposal 1: For Single CC with 3M channel bandwidth case, add the exceptional description to Note of the field description of the channelBWs-DL/UL as in [1].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For Single CC with 3M channel bandwidth case, add the similar exceptional description to Note of the field description of the channelBWs-DL/UL as in [1].

- Reference

[1] R2-2408400 Clarification on the Channel Bandwidth for the 3M  38306  1169  CR   ZTE Corporation