3GPP TSG-RAN WG2 Meeting #127bis											R2-2408424

Hefei, China, October 14th – 18th, 2024

Agenda Item:	8.7.5

Source: 	Sharp

Title:  	Discussion on Timely Retransmission

Document for: 	Discussion

# Introduction

In RAN2#126 meeting, RAN2 made the following agreements on timely retransmission:

To achieve timely retransmissions on RLC layer for XR traffic, RAN2 will consider the following options:

    - Autonomous retransmission (i.e. without status report) of PDUs based on some triggers (existing or new triggers can be considered)

    - Retransmission based on enhanced status report

    - Retransmission based on enhanced polling

- FFS whether any enhancements are needed or this can be solved with proper configuration and current mechanism

Impact on capacity should be considered

RAN2 focuses on the enhancements for UL traffic

In RAN2#127, there was no discussion due to the lack of online time. This document analyzes possible options.

# Discussion:

According to the discussion in RAN2#126, there are three solutions on the table:

- Autonomous retransmission without status report

- Retransmission based on enhanced status report

- Retransmission based on enhanced polling

All candidate solutions aim to accelerate retransmissions to guarantee the timely transmission. More specifically, all the solutions are targeted to enable early retransmissions under certain conditions such the remaining time until the required delivery time is less than a threshold value. Also, RAN2 agreed to focus on the enhancements for UL traffic.

Enhanced status report would mean that when the remaining time until the required delivery time is short (e.g., delay-critical data), the receiver triggers a status report early and asks the UE to perform retransmission early. In this solution, the gNB decides when to send the status report. In uplink, it is fully up to gNB implementations which mean that it is possible currently. Thus, no further spec impact is expected.

In uplink, retransmission based on enhanced status report is already possible and no further spec impact is expected.

Enhanced polling would mean that when the remaining time until the required delivery time is short (e.g., delay-critical data), the transmitter triggers a polling early to trigger the status report early and the UE performs retransmission early. This mechanism may need to modify only polling procedure. However, actual early retransmission will have delay from setting a poll until status report reception at the UE. It is not appropriate for XR traffic which has delay constraint. Moreover, if the status report experiences congestion in downlink, the early retransmission might not be possible. Also, the polled PDU may experience congestion or retransmissions. Thus, it is not considered as a good solution.

Retransmission based on enhanced polling has delay until the reception of the status report and may experience congestion.

Autonomous retransmission without status report is a more direct way than enhanced polling. Also, autonomous retransmission without status report is useful when either uplink or downlink is experiencing congestion. The only expected spec impact would be when the autonomous retransmission is triggered in RLC.

One may argue that the autonomous retransmission wastes the uplink resource. However, the autonomous retransmission is triggered only for urgent data to reduce performance degradation. Excessive resource waste does not occur for most of the time, and RLC ACK usually prohibits the unnecessary retransmission.

Autonomous retransmission without status report is useful for congestion scenario and potential resource waste can be under network control.

Based on the observations above, we see that RAN2 should introduce an RLC autonomous retransmission mechanism without status report.

RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

Retransmission based enhanced status report and retransmission based on enhanced polling are excluded.

# Conclusions

Based on the discussion given above, RAN2 is requested to agree the following proposals:

- RAN2 to introduce an RLC autonomous retransmission mechanism without status report.

Retransmission based enhanced status report and retransmission based on enhanced polling are excluded.