3GPP TSG-RAN WG2 Meeting #127-bis	R2-2408437

Hefei, China, October 14th – 18th, 2024

Agenda item:	8.10.5

Source:	Samsung

Title:	Reporting failure cause for SDT

Document for:	Discussion and Decision

# Introduction

In RAN2#127, RAN2 made the following agreements:

UE logs and reports the failure cause for SDT to the network. FFS the details, e.g. if we down select some of the failure causes.

In this contribution we discuss further on the details of reporting failure causes.

# Discussion

The possible failure for SDT according to TS 38.331 are highlighted:

5.3.13.5 Handling of failure to resume RRC Connection The UE shall: 1> if timer T319 expires: (skipped) 1> else if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing; or 1> if random access problem indication is received from MCG MAC while SDT procedure is ongoing; or 1> if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; or 1> if integrity check failure indication is received from lower layers while SDT procedure is ongoing; or 1> if T319a expires: 2> consider SDT procedure is not ongoing; 2> perform the actions upon going to RRC_IDLE as specified in 5.3.11 with release cause 'RRC Resume failure'.

Based on the above highlighted section, UE may log the following causes for SDT failure.

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

  - if network send UE to RRC_IDLE in response to the resume procedure initiated for SDT.

  - if network send UE to RRC_INACTIVE in response to the resume procedure initiated for SDT.

Proposal 1: UE logs the following failure causes of SDT to the network:

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

It is also possible that after initiating SDT procedure, UE perform SDT as the response for the RRC Resume request for SDT is RRC Release or RRCSetup or RRCReject.  It could be beneficial for the network to know about these cases and RAN2 may discuss them.

For example,

From TS 38.331, the possible responses are the following:

In response to a request to resume the RRC connection or in response to a resume procedure initiated for SDT, the network may resume the suspended RRC connection and send UE to RRC_CONNECTED, or reject the request to resume and send UE to RRC_INACTIVE (with a wait timer), or directly re-suspend the RRC connection and send UE to RRC_INACTIVE, or directly release the RRC connection and send UE to RRC_IDLE, or instruct the UE to initiate NAS level recovery (in this case the network sends an RRC setup message).

Among the above mentioned possible responses, the case where the network resumed the suspended connection and send UE to RRC_CONNECTED can be identified either by the network internally or based on UE reporting. Other three cases may need UE reporting, as the network may not be able to identify the UE at this point.

Proposal 2: RAN2 to discuss whether UE reports the following to the network:

  - the network rejected the request to resume and send UE to RRC_INACTIVE with a wait timer.

  - the network released the RRC connection and send UE to RRC_IDLE.

  - the network instructed the UE to initiate NAS level recovery through RRCSetup.

# Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: UE logs the following failure causes of SDT to the network:

  - if indication from the MCG RLC that the maximum number of retransmissions has been reached is received while SDT procedure is ongoing.

  - if random access problem indication is received from MCG MAC while SDT procedure is ongoing.

  - if the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing.

  - if integrity check failure indication is received from lower layers while SDT procedure is ongoing.

  - if T319a expires.

Proposal 2: RAN2 to discuss whether UE reports the following to the network:

  - the network rejected the request to resume and send UE to RRC_INACTIVE with a wait timer.

  - the network released the RRC connection and send UE to RRC_IDLE.

  - the network instructed the UE to initiate NAS level recovery through RRCSetup.

# References

[1] RP-234038: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4

[2] RAN2#127 meeting report.