3GPP TSG-RAN2 Meeting #127bis	R2-2408445

Hefei, China, Oct 14th – 18th, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on on-demand SIB1 operation for NES

Agenda Item:	8.5.3

Document for:	Discussion and decision

# Introduction

In the RAN#105 meeting, the revised Work Item for network energy saving enhancements has been approved in RP-242354 including an objective to specify the Case 2 based procedures and signaling for on-demand SIB1 operation [1]:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In the last meeting, RAN2 continued the discussion on on-demand SIB1 operation and several agreements have been achieved [2]:

Agreements on OD-SIB1

Scenarios:

1.	No consensus for RAN1 case 3 in RAN2.

OD-SIB1 Validity:

2.	We can rely on legacy UE behaviour to ensure UE has valid SIB1 for the cell (i.e. UE reacquire SIB1 whenever (re)selecting cell).

3.	Further UE keeps SIB1 updated while on cell via regular SI modification procedure (confirmation of earlier RAN2 agreement).

WUS configuration of NES cell from NES cell:

4.	Once Rel-19 NES UE camps on the NES cell, the UE expects to receive UL WUS configuration updates from the NES Cell, e.g., via legacy SI modification procedures.

MSG3 based OD-SIB1 REQ:

5.	Msg 3 based OD-SI procedure is not supported for on-demand SIB1 request in case 2 (for requesting to the NES Cell).

Access restriction for legacy UEs:

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Impacts to RRC connected UEs:

7.	RAN2 understands the NW can avoid impact on legacy RRC connected UE and R19 RRC connected UE due to on-demand SIB1 (e.g. NES cell with OD-SIB1 is measured by legacy RRC_CONNECTED UE and can be configured as its PSCell/SCells/target cell).

OD-SIB1 SI conclusion:

8.	RAN2 conclude that on-demand SIB1 is feasible from RAN2 perspective and recommend normative work of case 2 for on-demand SIB1.

In this contribution, we provide our further considerations on on-demand SIB1 operation.

# Discussion

## 2.1 How to indicate NES Cell state

In RAN1’s discussion, the identification of NES Cell was studied with the following agreement [3]:

Agreement RAN1 to further study UE identification of NES cell with on-demand SIB1 based on one, both, or combination of the following options: - Option 1: By WUS configuration - Option 2: By PBCH payload of NES cell

For the case 2 where the WUS configuration provided from Cell A, the WUS configuration can naturally be the identification of the NES Cell. When Cell A has provided the WUS configuration, it should be clear for NES UEs which cell(s) is/are the NES Cell and that the NES Cell is in on-demand SIB1 operation. As for the indication from PBCH payload of the NES Cell, it is challenging to put the indication in the PBCH of the NES Cell and RAN1 is studying several methods of doing so.

Since the WUS configuration from Cell A is essential for the NES UE and contains information to identify a NES cell, it should be a baseline to indicate the NES Cell’s status, i.e. that such cell is in on-demand SIB1 operation via the WUS configuration.

Proposal 1: The NES Cell identification and status is implicitly derived at least from the WUS configuration received from Cell A.

## 2.2 The transmission behavior of OD-SIB1

After the UE has sent the WUS to trigger the OD-SIB1 and has received the RAR from the NES Cell to indicate that the NES Cell will broadcast OD-SIB1, the next issue is how to broadcast the OD-SIB1. To save the NES Cell’s energy, the periodical broadcast behaviour of OD-SIB1 needs to be avoided. The triggered OD-SIB1 broadcast should last for a specific time. In the legacy OD-SI procedure, the si-WindowLength is provided in SIB1 to indicate the transmission of OD-SI. Following such mechanism, the si-Window based design can be reused for OD-SIB1. The SI window for legacy is broadcasted in SIB1. Since we don’t have SIB1 we would need to broadcast it e.g. in msg2 in the NES cell.

Proposal 2: Reuse the SI-window mechanism for the OD-SIB1 procedure.

To make sure the SI-window is always up to date for the requesting UE it should be conveyed in RAR. If the NW does not intend to send OD-SIB1 in a specific timeframe, it should be also possible to indicate that SIB1 will not be broadcasted, so that the UE will not repeat the OD-SIB1 request. For that purpose, the RAR should also include status indication of SIB1 transmission. If the NES cell receives the UE request and decides to response negatively, i.e., not ready to send OD-SIB1, at least not in the near future as shown in Figure 2, then the UE will benefit from this response to stop its WUS request procedure (power ramp up with retransmissions) creating less interference in the uplink and saving the UE power. If the cell response is positive, i.e., the OD-SIB1 provision will start, as shown in Figure ‎1, then the UE could start monitoring the type 0 PDCCH monitoring occasions during the window indicated in the WUS response.

Figure 1: Illustration of a WUS response with sib1-BroadcastStatus = broadcast and a time window for type 0 PDCCH monitoring occasions to receive OD-SIB1.

Figure 2: Illustration of a WUS response with sib1-BroadcastStatus = not broadcast and a time window for no type 0 PDCCH monitoring occasions and no OD-SIB1 reception nor WUS request (re)transmission.

Proposal 3: The RAR responding to OD-SIB1 request should contain at least a time window and OD-SIB1 broadcast status.

Another issue is what happens when RAR is not received. The UE needs to be allowed to resend the OD-SIB1 request when no response from the NW was received. If the RAR reception has failed the UE will re-transmit the UL-WUS with the power ramping up. The time and number of retransmissions of the WUS request procedure (power ramp up with retransmissions) should be restricted to create less interference in the uplink and save the UEs power. After the maximum retransmission number has been reached, and the RAR was not received, the UE will treat this cell as barred. An illustration of this process is presented on Figure 3.

Figure 3. the WUS retransmission behaviour procedure

Proposal 4: For OD-SIB1 request retransmission, the UE is allowed to retransmit WUS signal with retransmission timer limitation and maximum retransmission number restriction.

## 2.3 WUS config transmission in Cell A and NES cell

As stated in the WID, the UE obtains UL WUS configuration from Cell A. Furthermore, it was agreed in RAN2 that a single cell A can provide WUS configurations for several NES cells. As discussed in section 2.4 the exact contents of WUS configuration are not yet agreed but since there can possibly be several configurations to broadcast they should not be included in cell A’s SIB1 as it would significantly increase the overhead for Cell A. The optimal solution is to include all WUS configurations in a new SIB in Cell A.

Proposal 5: The WUS configurations are provided in a new on-demand SIB of Cell A.

For the NES Cell itself, we have agreed that the updated WUS configuration can be provided by the NES Cell. Considering that the WUS configuration is more related to the parameters in SIB1, using several additional bits in NES cell’s SIB1 is sufficient to indicate the parameters of an updated WUS configuration.

Observation 1: The majority of WUS configuration parameters are already included in NES cell’s MIB and SIB1.

Another question is whether a NES Cell could be the Cell A of another NES Cell. In the WID, it is classified that Cell A is the cell that is periodically transmitting at least its own SIB1. Due to the fact that the NES Cell does not broadcast its own SIB1 periodically, it is not preferred that NES Cell could be another NES Cell’s Cell A. Since minimising power consumption of NES cells is the main objective of this WI allowing a NES cell to broadcast WUS configurations of other cells would be contradictory to that objective. We believe cell A is meant to be the anchor cell to provide WUS configurations for all neighbouring NES cells. Moreover idle/inactive UEs will not be able to find WUS configurations of other neighbouring cells until they camp on that cell and acquire SIB1.

Proposal 6: For UEs camping on the NES cell, if the WUS configuration is updated, the new parameters are provided in SIB1 of the NES Cell via legacy SI modification procedures.

## 2.4 Details of the UL WUS configuration

RAN1 has discussed UL WUS configuration contents during RAN1#117 and reached the following agreement.

Agreement For further study of on-demand SIB1 in idle/inactive mode, use the following Table I (from R1-2405106, Ericsson) as a starting point to discuss the required parameters/contents inside the UL WUS configuration. Table I. Purpose; Parameters. To which cell does the config applies; NES-CellId; PhysCellId. ARFCN-ValueNR. WUS transmission; SIB1-RequestConfig; ss-PBCH-BlockPower. rach-OccasionsSIB1; Prach-ConfigurationIndex. msg1-FDM. msg1-FrequencyStart. zeroCorrelationZoneConfig. preambleReceivedTargetPower. preambleTransMax. powerRampingStep. ra-ResponseWindow. ssb-perRACH-Occasion. sib1-RequestPeriod. sib1-RequestResource; ra-PreambleStartIndex. ra-AssociationPeriodIndex. ra-ssb-OccasionMaskIndex. rsrp-ThresholdSSB. prach-RootSequenceIndex. msg1-SubcarrierSpacing. restrictedSetConfig. tdd-UL-DL-ConfigurationCommon. frequencyInfoUL; frequencyBandList. absoluteFrequencyPointA. offsetToCarrier. p-Max. frequencyShift7p5khz. SIB1 reception; pdcch-ConfigSIB1; ssb-SubcarrierOffset. controlResourceSetZero. searchSpaceZero. RAR Reception; pdcch-ConfigOD-SIB1-RAR; controlResourceSet. monitoringSlotPeriodicityAndOffset. Duration. monitoringSymbolsWithinSlot. aggregationLevels.

Since several parameters are L1 dependent and further discussion is expected during RAN1#118-bis, RAN2 can postpone the discussion on UL WUS content until more progress in RAN1 is made.

Proposal 7: Postpone discussion on parameters/contents inside the UL WUS configuration until more progress in RAN1 is achieved.

## 2.5 Legacy UE impact

For the legacy UE impact, two different scenarios should be considered. One is cell reselection from Cell A to NES Cell and another is initial cell selection on the NES Cell. Some options were listed during the last meeting:

Access restriction for legacy UEs:

6.	Following example options to handle legacy UEs (i.e. UEs not supporting OD-SIB1) can be considered in normative work. Details and further analysis need to be further discussed in normative work. Other existing options are not excluded.

- Option 1: Legacy UEs bar the OD-SIB1 cell based on cellBarred bit set to barred in MIB.

- Option 2: Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication via ssb-SubcarrierOffset in MIB.

- Option 3: Network includes cells supporting OD-SIB1 to list of excluded cells.

Cell reselection

For the former case, the intention of avoiding/deprioritizing legacy UEs from camping on NES Cells is to avoid legacy UEs initiating the procedure of searching for the NES Cell without SIB1 transmission to reduce the overhead of finding a suitable cell. As mentioned in the discussion, there are several options to avoid legacy UEs to search for the NES Cell.

Option 1: Set lower priority level to the NES Cell frequency for legacy UEs. The NES UEs would need a new priority to be allowed to camp on the NES Cell. For this option we would like to note that the OD-SIB1 feature is controlled per cell (not per carrier) therefore any modifications to cell reselection need to be per cell. Since the priority is controlled per carrier, the priority based solution is not suitable.

Option 2: Use excluded cell list for the legacy UEs not to consider the NES Cell as candidate. At the same time a new excluded cell list without the NES Cells included in it should be provided to the NES UEs to allow the NES UEs to consider camping on the NES Cells. For this purpose, a new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs in SIB3 can be added, in addition to the legacy IntraFreqExcludedCellList / InterFreqExcludedCellList, to give the NW a possibility to configure NES cell camping restrictions separately.

Proposal 8: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Initial cell selection/cell barring

As a current principle if a legacy UE cannot obtain SIB1 from a given cell it considers it as barred. We can identify several legacy options to indicate to legacy UEs that this cell should not be camped on. In our view any option that is chosen should not be dependent on setting the cellBarred in MIB to barred. If the cellBarred in MIB is set to barred, the R19 NES UE needs to check other bits to determine whether the cell is barred for NES UEs or not. Since SIB1 reception in OD-SIB1 operation will be delayed and the UE needs to know the barring status beforehand “option 1” from the RAN2#127 agreement is not feasible.

Proposal 9: Legacy UE barring will not be based on cellBarred bit set to barred in MIB.

Another option would be to use the ssb-SubcarrierOffset, if the field indicates that the cell does not provide SIB1 and there is no CORESET#0 configured, the legacy UE will treat the cell as barred. This option also means that the cell operates on NCD-SSB. Currently the cell selection and reselection rely on the CD-SSB on the sync raster. If the NES Cell does not operate on the CD-SSB, legacy UEs will not consider it as a candidate cell for cell selection and cell reselection. Relevant text from the specifications is highlighted below.

### --Cited from TS38.300 v18.2.0 ### 9.2.1 Mobility in RRC_IDLE #### 9.2.1.1 Cell Selection The principles of PLMN selection in NR are based on the 3GPP PLMN selection principles. Cell selection is required on transition from RM-DEREGISTERED to RM-REGISTERED, from CM-IDLE to CM-CONNECTED and from CM-CONNECTED to CM-IDLE and is based on the following principles: - The UE NAS layer identifies a selected PLMN and equivalent PLMNs; - Cell selection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4): >>Omit some unrelated part<< #### 9.2.1.2 Cell Reselection A UE in RRC_IDLE performs cell reselection. The principles of the procedure are the following: - Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

Cited from TS 38.331 v18.2.0 pdcch-ConfigSIB1 Determines a common ControlResourceSet (CORESET), a common search space and necessary PDCCH parameters. If the field ssb-SubcarrierOffset indicates that SIB1 is absent, the field pdcch-ConfigSIB1 indicates the frequency positions where the UE may find SS/PBCH block with SIB1 or the frequency range where the network does not provide SS/PBCH block with SIB1 (see TS 38.213 [13], clause 13). ssb-SubcarrierOffset Corresponds to kSSB (see TS 38.213 [13]), which is the frequency domain offset between SSB and the overall resource block grid in number of subcarriers. (See TS 38.211 [16], clause 7.4.3.1). For operation with shared spectrum channel access in FR1 (see 37.213 [48]), this field corresponds to kSSB, and kSSB is obtained from (see TS 38.211 [16], clause 7.4.3.1); the LSB of this field is used also for deriving the QCL relation between SS/PBCH blocks as specified in TS 38.213 [13], clause 4.1. The value range of this field may be extended by an additional most significant bit encoded within PBCH as specified in TS 38.213 [13]. This field may indicate that this cell does not provide SIB1 and that there is hence no CORESET#0 configured in MIB (see TS 38.213 [13], clause 13). In this case, the field pdcch-ConfigSIB1 may indicate the frequency positions where the UE may (not) find a SS/PBCH with a control resource set and search space for SIB1 (see TS 38.213 [13], clause 13).

Cited from TS 38.331 v18.2.0 #### 5.2.2.5 Essential system information missing The UE shall: 1> if in RRC_IDLE or in RRC_INACTIVE or in RRC_CONNECTED while T311 is running: 2> if the UE is unable to acquire the MIB: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> perform barring as if intraFreqReselection, or intraFreqReselectionRedCap for RedCap UEs, or intraFreqReselection-eRedCap for eRedCap UEs or intraFreqReselection2RxXR for 2Rx XR UEs, is set to allowed; 2> else if the UE is unable to acquire the SIB1: 3> consider the cell as barred in accordance with TS 38.304 [20]; 3> if the UE is a RedCap UE: 4> perform barring as if intraFreqReselectionRedCap is set to allowed; 3> else if the UE is an eRedCap UE: 4> perform barring as if intraFreqReselection-eRedCap is set to allowed; 3> else if the UE is a 2Rx XR UE: 4> perform barring as if intraFreqReselection2RxXR is set to allowed; 3> else: 4> perform cell re-selection to other cells on the same frequency as the barred cell as specified in TS 38.304 [20]. NOTE 1: The SIB19 is essential for NTN access. If UE is unable to acquire the SIB19 for NTN access, the action is up to UE implementation (e.g., cell re-selection to other cells). NOTE 2: The SIB22 is essential for ATG access. If UE is unable to acquire the SIB22 for ATG access, the action is up to UE implementation (e.g., cell re-selection to other cells).

Observation 2: Legacy idle/inactive UEs bar the NES cell if they are unable to acquire the MIB e.g., no CD-SSBs located on the synchronization raster or if they are unable to acquire the SIB1 i.e., NCD-SSB exist on the global synchronization raster but kSSB>23 for FR1 or if kSSB>11 for FR2.

As can be seen from the RAN2 spec excerpt above, the contents of ssb-SubcarrierOffset/ kSSB and pdcch-ConfigSIB1 are defined in TS 38.213.

Cited from TS 38.213, clause 4.1

Upon detection of a SS/PBCH block, the UE determines from MIB that a CORESET for Type0-PDCCH CSS set, as described in clause 13, is present if kSSB<24 [4, TS 38.211] for FR1 or if kSSB<12 for FR2. The UE determines from MIB that a CORESET for Type0-PDCCH CSS set is not present if kSSB>23 for FR1 or if kSSB>11 for FR2; the CORESET for Type0-PDCCH CSS set may be provided by PDCCH-ConfigCommon.

Proposal 10: Technical details of cell barring using no CD-SSB located on the global synchronization raster from NES cell or using ssb-SubcarrierOffset/ kSSB and pdcch-ConfigSIB1 for SSBs on the global synchronization raster are left to RAN1.

# Conclusion

In this contribution, we discussed the on-demand SIB1 operation, and made the following observations and proposals:

Proposal 1: The NES Cell identification and status is implicitly derived at least from the WUS configuration received from Cell A.

Proposal 2: Reuse the SI-window mechanism for the OD-SIB1 procedure.

Proposal 3: The RAR responding to OD-SIB1 request should contain at least a time window and OD-SIB1 broadcast status.

Proposal 4: For OD-SIB1 request retransmission, the UE is allowed to retransmit WUS signal with retransmission timer limitation and maximum retransmission number restriction.

Proposal 5: The WUS configurations are provided in a new on-demand SIB of Cell A.

Observation 1: The majority of WUS configuration parameters are already included in NES cell’s MIB and SIB1.

Proposal 6: For UEs camping on the NES cell, if the WUS configuration is updated, the new parameters are provided in SIB1 of the NES Cell via legacy SI modification procedures.

Proposal 7: Postpone discussion on parameters/contents inside the UL WUS configuration until more progress in RAN1 is achieved.

Proposal 8: Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

Proposal 9: Legacy UE barring will not be based on cellBarred bit set to barred in MIB.

Observation 2: Legacy idle/inactive UEs bar the NES cell if they are unable to acquire the MIB e.g., no CD-SSBs located on the synchronization raster or if they are unable to acquire the SIB1 i.e., NCD-SSB exist on the global synchronization raster but kSSB>23 for FR1 or if kSSB>11 for FR2.

Proposal 10: Technical details of cell barring using no CD-SSB located on the global synchronization raster from NES cell or using ssb-SubcarrierOffset/ kSSB and pdcch-ConfigSIB1 for SSBs on the global synchronization raster are left to RAN1.

# References

- RP-242354, Revised WID: Enhancements of network energy savings for NR, Ericsson

- R2-2407571 Report from session on V2X/SL, R18/19 MOB, and R19 NES

- Chair notes RAN1#116bis eom2.doc