3GPP TSG-RAN WG2 Meeting #127bis 	R2-2408458

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.2

Source:	Panasonic

Title:	Discussion on Ambient IoT device AS ID and device energy status report

Document for:	Discussion

# Introduction

In this paper, we discuss two functionality aspects for A-IoT operation: device AS ID and device energy status report.

# Discussion

## AS ID

In the previous meeting, there were several proposals from multiple companies related to temporary device ID or AS ID captured in the chair notes [1], although they were not treated due to lack of time.

R2-2406737 Proposal 4: Whether to introduce temporary ID to identify A-IoT devices needs further discussion. R2-2406769 Proposal 4 A device identifier is used to identify a device during communication with the reader. Proposal 5 For contention-based random access, the random ID transmitted in Msg1 can be used as the device identifier after the devices receives Msg2 which completes contention resolution. Proposal 6 For contention-free access, device identifier is allocated by the reader, e.g. in AIoT paging message. Proposal 7 Device identifier can be updated, e.g. based on the reader’s request. R2-2406656 Proposal 2: Data can be transferred between the reader and device in the R2D direction as long as the device’s AS identifier (random number or value assigned by the reader) remains valid. R2-2406818 Proposal 14 Study whether a requirement on how long the AS ID can be kept in memory is feasible and reliable

Furthermore, the following has been agreed (in agenda 8.2.4 A-IoT random access):

Agreement: FFS(for further study): if reader assigns the AS ID for scheduling purposes

In general, we support the view to introduce a temporary AS ID for scheduling purposes. Although the exact naming can be further discussed, the temporary AS ID can be used to address a targeted device by a reader in air interface. One counter argument is that permanent device ID or some upper layer device ID can be directly used without introducing another AS ID for addressing purposes. However, we see such alternative less appealing for the reasons below.

First of all, permanent device ID is expected to have very large size (using one example of Electronic Product Code, the typical size could be around 100bits). Using such permanent device ID every time when reader wants to communicate with the device would cause unnecessary signaling overhead and energy usage. Furthermore, it may risk exposure of the permanent A-IoT device ID and thus authorized device tracking. Therefore, an AS temporary ID with shorter length could be a more reasonable option. And since it is temporary and can be re-assigned as needed, it can also protect device from unwanted tracking.

Second, it may be true that upper layer can also assign a kind of temporary ID to the device and this can be used for both NAS and AS layer communication. However, such aspects might be better to discuss at SA2 rather than RAN2. Nevertheless, RAN2 can progress independently on the AS-level temporary ID discussion. Depending on the progress of SA2, RAN2 can later decide on whether to make sense to have two levels of temporary IDs or merged them into one temporary ID.

Proposal 1. Temporary AS ID should be supported to address a targeted device in A-IoT air interface.

Proposal 2. RAN2 to discuss on temporary ID usage in AS level without waiting for SA2. Whether to merge it to SA2 level discussion or not can be discussed in future.

In the following, we further discuss the temporary AS ID assigning procedure and the life time of this ID.

AS ID assignment

For CBRA (contention-based random access), since there is a random ID generated by device and send to reader in Msg1 for contention resolution, and if contention is resolved by reader echoing this ID in Msg2, the natural question is whether this random ID can be used as AS ID for later usage.

Although we acknowledge that reusing the random ID in Msg1 as AS ID can save some signaling overhead, this cannot guarantee the uniqueness of the AS ID in the coverage of a reader. A more robust solution is to let reader (i.e. UE or gNB) to decide whether to use the random ID as AS ID or assign a new AS ID to the device if needed. The AS ID can be assigned to device after Msg3. This can be illustrated by the following Fig.1.

Fig.1. Device obtains AS ID via CBRA

For CFRA(contention-free random access) case, there is no need for device to generate a random ID for contention resolution. The device would skip contention resolution Msg1 and Msg2 in the Fig.1 and directly sends the upper layer data (e.g. permanent device ID) in its very first D2R message after being triggered (by A-IoT paging). Following the same procedure as CBRA, reader can then generate an AS ID for the device and send it to the device via the next R2D transmission.

Proposal 3. Reader can assign an AS ID to an A-IoT device by the A-IoT random access procedure, e.g. a R2D transmission after Msg3.

How long AS ID can be kept in device memory

Generally speaking, it is preferable to keep the AS ID as long as possible to avoid re-assigning the AS ID, which requires power consumption and signaling overhead. However, we foresee that the life time of AS ID may also depend on how the device store the AS ID, which can be one of the following:

- Type-A: AS ID is kept in device as long as “sustainable operation time” (in volatile memory).

- Type-B: AS ID can be kept in device once assigned (in non-volatile memory) even after OFF.

The sustainable operation time refers to the time duration where the volatile memory can be kept. Note that the relation with device 1/2a/2b needs further discussed later. Further, it may depend on the energy states that are being discussed in RAN1 currently. The related discussion in RAN1#118 is captured below (see R1-2407202).

FL1 High Priority Proposal 4.1-1b RAN1 studies energy harvesting impacts in the following way: - Case 1: Device is assumed to have two states: ON, OFF - Case 2: Device is assumed to have three states: ON, OFF, SLEEP - Identify function(s) of the device that can be assumed supported and assumed not supported in each of the above device states, subject to: - ON state supports at least: transmission, reception for communication - OFF state does not support at least: transmission, reception for communication, running a clock - OFF state supports at least: energy harvesting - SLEEP state supports at least: - Maintaining a memory content from ON state - Running a clock or maintaining a timer (RAN1 to discuss purpose(s) of clock) - Energy harvesting - FFS: monitoring some reception - SLEEP state does not support at least: transmission, reception for communication - Identify reader knowledge / control of the above states for case 2 - RAN1 is not required to establish a consensus model for the duration of each of the states

Our understanding is that the volatile memory content can be kept in ON and SLEEP states, but it could be lost in OFF state. Whether and how reader should know the device state is under discussion in RAN1. From the life time management of AS ID point of view, all the above-mentioned 2 types of devices can benefit from supporting AS ID. It is obvious that type-B can make most use of the assigned AS ID since it will not be affected by device energy state. On the other hand, for type-A devices, as long as the device can alternate between ON and SLEEP states without falling into OFF, the assigned AS ID can be kept, which can be used for potentially multiple rounds of transactions between reader and device.

Proposal 4. The AS ID can be supported for following types of devices:

- Type-A: AS ID is kept in device as long as “sustainable operation time” (in volatile memory)

- Type-B: AS ID can be always kept in device once assigned (in non-volatile memory)

FFS whether and how the network should differentiate the above types of devices in terms of AS ID operation.

## Energy status report

In the previous meeting, the following proposal on energy status report has been discussed without consensus [1].

R2-2406590 Proposal 1: It is proposed to include energy status report to be delivered from the UE to the reader in case there the device does not have anergy for the follow up messages Proposal 2: It is proposed to discuss if one bit of information is sufficient for any use case highlighted above or there is a need to represent the energy status in a different way (e.g. sufficient energy for sending of a particular number of bits or an expected time when the device will be ready for subsequent operations or any others)

In principle, we support the view that it would be beneficial for the reader to know when the device is available for communication and how long the device can remain available. Otherwise, communication between reader and device would be very inefficient (e.g. reader is wasting resources to page a device multiple time while device is charging and cannot respond, or in another example, device has responded to paging but fails to complete the rest of necessary transmissions and receptions for this transaction).

To achieve the above-mentioned benefits, supporting energy status report from device to reader can be one way to consider. However, we are not convinced that one-bit information is sufficient. Basically, one-bit indication would be to inform reader that it does not have energy for the follow-up messages. In such case, a wiser operation is to just skip responding because by sending the indication will also consume energy of the device. Therefore, we think a more important aspect is that the A-IoT device should be allowed to skip responding if the device does not have sufficient energy to complete the remaining access procedure triggered by the paging. Whether the energy is sufficient can be judged by device implementation. This procedure can be illustrated using Fig.2. below.

Fig.2. Device skips responding based on judgement of its energy

Proposal 5. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

By contrast, the energy status report becomes useful when it can carry more information. One example of the information is the remaining available time for communication. Once reader knows such information, reader can consider not to deliver further commands to device, and instead wait for some time duration.

As another example, the energy status report can indicate when the device will be expected to become available again. This may be related to the discussion of energy states in RAN1 as mentioned in previous section. For example, assuming device goes alternating between ON and SLEEP by timer. For example, device is ON for 10ms for communication and SLEEP for 40ms for harvesting energy. After being paged in ON state, device can report the remaining time for ON state and the SLEEP time duration. Based on this information, the reader can know when device is expected to become ON again. However, this can wait for progress of RAN1 discussion.

Proposal 6. RAN2 to revisit discussion on energy status report after RAN1 has progressed on energy state discussion.

# Conclusion

In this paper, we discuss device AS ID and device energy status report. We have the following proposals.

Proposal 1. Temporary AS ID should be supported to address a targeted device in A-IoT air interface.

Proposal 2. RAN2 to discuss on temporary ID usage in AS level without waiting for SA2. Whether to merge it to SA2 level discussion or not can be discussed in future.

Proposal 3. Reader can assign an AS ID to an A-IoT device by the A-IoT random access procedure, e.g. a R2D transmission after Msg3.

Proposal 4. The AS ID can be supported for following types of devices:

- Type-A: AS ID is kept in device as long as “sustainable operation time” (in volatile memory)..

- Type-B: AS ID can be always kept in device once assigned (in non-volatile memory)

FFS whether and how the network should differentiate the above types of devices in terms of AS ID operation.

Proposal 5. Device is allowed to skip responding if device judges its remaining energy cannot sustain to complete whole access procedure triggered by this A-IoT paging.

Proposal 6. RAN2 to revisit discussion on energy status report after RAN1 has progressed on energy state discussion.

# Reference

[1] R2_127_ChairNotes_24-08-23_17-00_eom, 3GPP

[2] R1-2407364, Reply LS on Clarification of requirements for Ambient IoT, RAN1

[3] TR 38.769 V0.1.0, Study on solutions for ambient IoT

[4] TR 23.700-13 V1.0.0, Study on Architecture support of Ambient power-enabled Internet of Things