3GPP TSG-RAN WG2 Meeting #127bis	R2-2408512

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.2.4

Source:	Panasonic

Title: 	Considerations for D2R failure/success indication in A-IoT

Document for:	Discussion

# Introduction

In the last meeting, RAN2 has discussed some of the aspects for A-IoT random access in 3-step CBRA and agreed the following aspects.

Agreements

1	For 3-step CBRA Support fixed random ID size is 16 bit.   The ID is randomly generated.

2	Failure/success indication of D2R will be studied.   FFS if it would be implicit or explicit and for which use case it is needed.  FFS whether it is applied only to some cases.

This document provides more details on the failure scenarios of D2R transmissions including Msg3 transmission failure during 3-step contention based random access (CBRA) and discusses possibilities to solve some of the open issues.

# Discussion

RAN2 has agreed upon a baseline procedure for A-IoT as seen below, the data flow and the steps involved in R2D/ D2R depends on the type of service triggered for an A-IoT device, ‘inventory only’ or ‘inventory and command’

| Inventory only | Step A | Initial trigger message or A-IoT paging |
| --- | --- | --- |
|  | Step B | Random access and transmission of device ID (FFS on details) |
| Command followed by inventory (for inventory + command) | Step C | Reader to device transmission (e.g. command in R2D) |
|  | Step D | Corresponding device to reader data transmission (e.g. the feedback) |

The random access-like procedure in step B is further studied in separate procedures, a longer 3-step format consisting of Msg1 and Msg3 which are sent on D2R and a shorter 2-step format consisting of only Msg1 in D2R. For either of these formats Msg2 is sent in R2D in response to Msg1 (though the full requirement of Msg2 in 2-step RA is still under study). After completing random access procedure in step B, based on the triggered service type, more D2R messages can follow in corresponding R2D messages. The D2R messages transmitted after random access as step D can be considered as ‘generic D2R messages’ since the R2D messages and the corresponding D2R transmission is always addressed to a single device (which also includes for the case when contention free random access is triggered for a single device in step-A and step-B).

The failure scenarios for contention based random access-like D2R transmission (i.e. Msg3) during inventory operation and the generic D2R messages following the inventory operation (or the inventory operation of a CFRA) is analysed, which are described in the next sections.

## Msg3 failure handling for contention based random access

Considering the scenario where multiple devices are triggered to start 3-step random access procedure and to transmit Msg1, upon receiving an initial trigger message or A-IoT paging sent from the A-IoT reader. It is assumed that the random ID in Msg1 is uniquely generated by each device, since the corresponding Msg2 consisting of the echoed random ID is expected to resolve contention among these devices before moving ahead with the follow-up procedures, but even though the size of the random ID (16-bits as decided in the last meeting) may have a larger code space for multiple devices to choose a unique value, it cannot be ruled out completely that two (or more) devices may (even though rarely occurred) choose the same random ID when transmitting Msg1. The following figure illustrates one such example.

Figure 1: Msg3 failure during 3-step CBRA

For the above case, there are multiple possibilities at the reader to occur as a consequence of receiving contention among same random ID, one of possible cases is where the reader may detect the random ID (for example, since the transmitted information is same from multiple devices so when the signal has traversed the same path and encountered less interference). But it cannot detect the number of devices transmitted on the same occasion when one of the random ID is detected. Therefore, the reader would send one Msg2 response upon detecting this random ID.

Further when Msg3 is sent in D2R from each of these devices it is likely to undergo contention again. At this stage, if the reader detects one of the Msg3 contents while the remaining be lost (for example, one device transmission power is received stronger than the others), then those devices which did not receive the follow-up R2D message (e.g. step-C), could wait for Msg3 confirmation indefinitely since it is not aware of the Msg3 failure. Also, here the reader will not be aware of the failed device.

Therefore, it is essential that the Msg3 failure detection is implicitly detected at the device, for example using a timer, to address all the corner conditions of RACH procedure failures.

Proposal 1:  Msg3 failure indication for 3-step CBRA to be defined and handled implicitly at the device using a timer (with time information like TD2Rmax) to address any failure scenarios due to contention, since it cannot be detected by the reader for every scenario.

Further, if the reader detects Msg3 from one device during a 3-step CBRA, then that device can be identified using the device ID that is included within this D2R message. Hence the success indication can be indicated using the follow-up R2D message received after Msg3.

Proposal 2: Success of Msg3 transmission in a 3-step CBRA can be indicated using follow-up R2D message.

When subsequent R2D message is not intended after Msg3, for example during ‘inventory only’ procedure, then at least the explicit success indication is sent to the device in R2D to stop implicit Msg3 failure detection mechanism as in proposal-1.

Proposal 2a: When subsequent R2D message is not intended to be transmitted for a device after receiving Msg3 successfully for a 3-step CBRA, the reader sends success indication in R2D to stop implicit failure detection at the device.

## Generic D2R failure handling

For the case of generic D2R messages sent from A-IoT device, a similar failure scenario is seen in the example illustrated from the figure below. Here the reader after detecting that the D2R transmission from one device is not received, should trigger R2D message again (since the reader cannot detect if the R2D has failed or the corresponding D2R has failed, so the command procedure is repeated to ensure the service is completed) which may fail once again as shown in the example below. Then for such scenario the [TD2Rmin, TD2Rmax] time which is maintained at the device can be used to indicate D2R transmission failure implicitly.

Figure 2: D2R failure illustration

Proposal 3: D2R failure indication to be defined and handled implicitly at the device using a timer (with timer information like TD2Rmax) since the D2R transmission failure can occur either due to the D2R not received at the reader or due to the subsequent R2D (if any) not received at the device.

Further, similar to Msg3 success indication, the success indication of generic D2R transmissions also depends on whether the reader intends to schedule (or not) a subsequent R2D message after receiving the D2R message. In either case the explicit success indication is required according to the following proposals.

Proposal 4: Success indication for D2R transmissions to be explicitly indicated to the device using R2D message.

For the case when subsequent R2D scheduling is intended or not (for example, for intended for another command operation) then the following applies.

Proposal 4a: Subsequent R2D messages received at the device following a D2R transmission can be used for success indication of the last transmitted D2R.

Proposal 4b: When subsequent R2D message is not intended to be transmitted for a device after the current D2R transmission, then the reader sends success indication in R2D to stop implicit failure detection at the device.

# Conclusions

Msg3 failure/success handling

Proposal 1:  Msg3 failure indication for 3-step CBRA to be defined and handled implicitly at the device using a timer (with time information like TD2Rmax) to address any failure scenarios due to contention, since it cannot be detected by the reader for every scenario.

Proposal 2: Success of Msg3 transmission in a 3-step CBRA can be indicated using follow-up R2D message.

Proposal 2a: When subsequent R2D message is not intended to be transmitted for a device after receiving Msg3 successfully for a 3-step CBRA, the reader sends success indication in R2D to stop implicit failure detection at the device.

Generic D2R failure/success handling

Proposal 3: D2R failure indication to be defined and handled implicitly at the device using a timer (with timer information like TD2Rmax) since the D2R transmission failure can occur either due to the D2R not received at the reader or due to the subsequent R2D (if any) not received at the device.

Proposal 4: Success indication for D2R transmissions to be explicitly indicated to the device using R2D message.

Proposal 4a: Subsequent R2D messages received at the device following a D2R transmission can be used for success indication of the last transmitted D2R.

Proposal 4b: When subsequent R2D message is not intended to be transmitted for a device after the current D2R transmission, then the reader sends success indication in R2D to stop implicit failure detection at the device.