3GPP TSG-RAN WG2 Meeting #127bis                            R2-2408523

Hefei, China, Oct 14th – 18th, 2024

Source:       	ZTE Corporation, CATT, Ericsson

Title:          Further discussion on RAN3 LS on intra-SN SCPAC in MN format

Agenda item:	7.4.2

Document for:	Discussion and decision

Introduction

In previous meeting, a RAN3 reply LS [1] on inter-node RRC message for intra-SN SCPAC in MN format was sent to RAN2 as follows:

1. Overall Description: RAN3 thanks RAN2 for the LS on inter-node RRC message for intra-SN SCPAC in MN format. Regarding the following agreement made by RAN2, RAN3 has discussed and would like to further clarify. - RAN2 understands both CG-Config message associated with the source SCG and CG-CandidateList message associated with candidate PSCell(s) can be included in one SN Modification Required message for intra-SN subsequent CPAC in MN format. From RAN3’s point of view, currently, one SN Modification Required message can only include either CG-Config message or CG-CandidateList message. In addition, RAN3 would like to understand the following two questions: Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message? Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message? 2. Actions: To RAN2 group. ACTION: RAN3 kindly asks RAN2 to provide answers to the questions above.

In the LS, RAN3 asked two questions about inter-node RRC message transfer for intra-SN SCPAC in MN format. At last meeting, RAN2 discussed how to handle these two questions but no consensus was reached [2].

Response to RAN3 LS: R2-2407562 Discussion on RAN3 LS on intra-SN SCPAC in MN format ZTE Corporation discussion Rel-18 NR_Mob_enh2-Core Proposal 1: From RAN2 perspective, CG-Config and CG-CandidateList are related to different configuration of intra-SN subsequent CPAC in MN format, and should be sent together in the same SN Modification Required XnAP message. [CATT]: Think there is no problem if two are sent in separate. Source node can combine them. Proposal 2: From RAN2 perspective, CG-Config message and CG-CandidateList message can not be included in a single existing inter-node RRC message. RAN2 sends the reply LS to RAN3 to kindly ask RAN3 to include them in a single SN Modification Required XnAP message. Proposal 3: RAN2 to approve the reply LS in the Annex. [Session chair]: Revisit it next meeting since there is no consensus. - Revisit it next meeting.

In this contribution, we further discussed the questions in the RAN3 LS, and provided the draft reply LS.

Discussions

Discussion on Q1

In RAN3 LS, the following question was raised:

Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message?

According to the previous RAN2 discussion in [3], the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format (i.e. via candidateCellInfoListCPC), which is associated with the current serving SCG configuration. The CG-CandidateList message is used to transfer the information related to candidate PSCell/SCG(s), including the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format (i.e. via candidateCellInfoListSubsequentCPC). Thus, these two messages are related to different configuration for intra-SN SCPAC in MN format.

- Currently, the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configuration for intra-SN SCPAC in MN format.

The detailed description for these two messages in the current RRC spec [4] is shown as below:

TS 38.331 v18.3.0 – CG-CandidateList This message is used to transfer the SCG radio configuration for one or more candidate cells for Conditional PSCell Addition (CPA), Conditional PSCell Change (CPC), inter-SN subsequent CPAC, or CHO with candidate SCG(s) as generated by the candidate target SgNB, or the serving SgNB in case of intra-SN subsequent CPAC in MN format. Direction: Secondary gNB to master gNB or eNB. CG-CandidateList message -- ASN1START -- TAG-CG-CANDIDATELIST-START CG-CandidateList ::= SEQUENCE { criticalExtensions CHOICE { c1 CHOICE{ cg-CandidateList-r17 CG-CandidateList-r17-IEs, spare3 NULL, spare2 NULL, spare1 NULL }, criticalExtensionsFuture SEQUENCE {} } } CG-CandidateList-r17-IEs ::= SEQUENCE { cg-CandidateToAddModList-r17 SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CG-CandidateInfo-r17 OPTIONAL, cg-CandidateToReleaseList-r17 SEQUENCE (SIZE (1..maxNrofCondCells-r16)) OF CG-CandidateInfoId-r17 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } CG-CandidateInfo-r17 ::= SEQUENCE { cg-CandidateInfoId-r17 CG-CandidateInfoId-r17, candidateCG-Config-r17 OCTET STRING (CONTAINING CG-Config) } CG-CandidateInfoId-r17::= SEQUENCE { ssbFrequency-r17 ARFCN-ValueNR, physCellId-r17 PhysCellId } -- TAG-CG-CANDIDATELIST-STOP -- ASN1STOP – CG-Config This message is used to transfer the SCG radio configuration as generated by the SgNB or SeNB. It can also be used by a CU to request a DU to perform certain actions, e.g. to request the DU to perform a new lower layer configuration. Direction: Secondary gNB or eNB to master gNB or eNB, alternatively CU to DU. ... candidateCellInfoListCPC Contains information regarding candidate target cells for Conditional PSCell Change (CPC) or inter-SN subsequent CPAC that the source secondary gNB suggests the target secondary gNB to consider configuring for CPC or subsequent CPAC, and/or that the source secondary gNB prepares for intra-SN subsequent CPAC in MN format. This field is only used in SN initiated CPC and SN initiated subsequent CPAC.. candidateCellInfoListSubsequentCPC Contains information regarding candidate target cells for subsequent CPAC that candidate secondary gNB (or the serving secondary gNB in case of intra-SN subsequent CPAC in MN format) suggests the master gNB to consider configuring for subsequent CPAC. This field is only used in MN initiated and SN initiated subsequent CPAC. This field is only included in a CG-Config message which is contained within a CG-CandidateList message..

Since these two messages are related to different parts of configuration for intra-SN SCPAC in MN format, the MN needs to collect all related configurations from the serving SN before generating the final RRC message for inra-SN SCPAC in MN format and sending to the UE via one RRCReconfiguration message. Regarding how to transfer these two messages from the SN to the MN, there are several options discussed in the RAN3#124 meeting discussion [5]:

- Option 1: define new XnAP IE in the SN MODIFICATION REQUIRED message to include both containers referring to different RRC messages, while the presence are optional. The procedural text needs to be updated.

- Option 2: define new RRC IE to include both CG-Config and CG-CandidateList, thus no change is foreseen for the single container in XnAP. The semantics description needs to be updated by adding new reference.

- Option 3: allow the SN to send multiple SN MODIFICATION REQUIRED message to transfer the configurations.

In option 1, a new XnAP IE is required to be introduced in the SN MODIFICATION REQUIRED message, which is not preferred by some RAN3 companies in the previous RAN3 discussion. In option 2, a new inter-node RRC message needs to be introduced to include both CG-Config and CG-CandidateList messages. Considering that ASN.1 was frozen at RAN2, it’s also undesired to introduce a new inter-node RRC message to contain these two messages together. As a compromise, option 3 can be considered to avoid large spec impact at RAN2 and RAN3.

Thus, we proposed that:

- From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

Discussion on Q2

In RAN3 LS, the second question was mentioned as below:

Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message?

If the proposal above is agreeable, CG-Config message and CG-CandidateList messages can be send via multiple XnAP messages, e.g. separate SN Modification Required XnAP messages. Thus, there is no need to include them in a single RRC message.

However, it should be noted that only one SN Modification Required XnAP message is sent from the SN to the MN to trigger the preparation procedure for intra-SN SCPAC with MN involvement in the current TS 37.340 [6].

- There is only one SN Modification Required XnAP message sent from the SN to the MN to trigger the preparation procedure for intra-SN SCPAC with MN involvement in the current TS 37.340.

In order to ensure the MN can collect both CG-Config message and CG-CandidateList messages from the SN, the SN needs to send at least two XnAP messages to the MN upon initiation the preparation of intra-SN SCPAC with MN involvement procedure. Companies proposed two options at last meeting:

- Option 1: SN sends two SN Modification Required XnAP messages to the MN, i.e. one includes CG-Config message and the other includes CG-CandidateList message.

- Option 2: SN sends CG-Config message and CG-CandidateList message separately via SN Modification Required XnAP message and SN Modification Request Acknowledge XnAP message.

In option 1, two SN Modification Required XnAP messages are always required to initiate the intra-SN SCPAC with MN involvement procedure in the current flow chart. In option 2, currently, the MN initiated SN modification procedure is optionally performed. If we want to use the SN Modification Request Acknowledge XnAP message to transfer CG-Config message or CG-CandidateList message, this procedure should become mandatory. In our understanding, both options can work and require some modification or clarification in the current stage-2 procedure. Considering that the overall procedure for intra-SN SCPAC with MN involvement is introduced at RAN3, we suggest to leave the detailed message/procedure design to RAN3 discussion.

- RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion.

Based on the proposals above, a draft reply LS is provided in the Annex.

- RAN2 to approve the reply LS in the Annex.

Conclusion and Proposals

In this contribution, we discussed the RAN3 LS on intra-SN SCPAC in MN format with the following observations and proposals:

- Currently, the CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configuration for intra-SN SCPAC in MN format.

- There is only one SN Modification Required XnAP message sent from the SN to the MN to trigger the preparation procedure for intra-SN SCPAC with MN involvement in the current TS 37.340.

- From RAN2 perspective, CG-Config and CG-CandidateList are related to different configurations of intra-SN subsequent CPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

- RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion.

- RAN2 to approve the reply LS in the Annex.

References

R2-2406217	  Reply LS on intra-SN SCPAC in MN format (R3-243775; contact: ZTE)	  RAN3

RAN2#127 meeting chair notes

R2-2402745	  Discussion on inter-node RRC message for intra-SN SCPAC in MN format	 ZTE Corporation

TS 38.331 v18.3.0

RAN3#124 meeting chair notes

TS 37.340 v18.3.0

- Annex

Title:	Reply LS on intra-SN SCPAC in MN format

Response to:	R2-2406217/R3-243775

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN2

To:	RAN3

Cc:	-

Contact Person:

Name:	Xiaohui Song

E-mail Address:	song.xiaohui@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, 3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks RAN3 for the reply LS on inter-node RRC message for intra-SN SCPAC in MN format. Regarding the following questions, RAN2 has discussed and would like to provide the responses as below:

Q1: Are both CG-Config and CG-CandidateList related to same configuration of intra-SN subsequent CPAC in MN format, and must be sent together in the same SN Modification Required XnAP message?

RAN2 response: The CG-Config message is used to transfer the initial execution condition for intra-SN SCPAC in MN format, while CG-CandidateList message is used to transfer the candidate SCG configuration and the execution condition for the following execution of intra-SN SCPAC in MN format. These two messages are related to different configurations for intra-SN SCPAC in MN format, but they are not required to must be sent together in the same SN Modification Required XnAP message.

Q2: If both RRC messages must be sent together, is it feasible to include both CG-Config message and CG-CandidateList messages in a single RRC message (new or existing), which could then be transferred in a single SN Modification Required XnAP message?

RAN2 response: From RAN2 perspective, CG-Config message and CG-CandidateList message are allowed to be sent separately, so there is no need to include both CG-Config message and CG-CandidateList messages in a single RRC message.

Besides, 	RAN2 understands that two XnAP messages can be sent from the SN to the MN to transfer CG-Config message and CG-CandidateList messages separately. It requires some modification or clarification in the current stage-2 procedure. The detailed message and procedure can be up to RAN3 discussion.

2. Actions:

To RAN3 group.

ACTION: 	RAN2 kindly asks RAN3 to take the above feedback into consideration.

3. Date of Next TSG-RAN2 Meetings:

TSG-RAN2 Meeting #128	18 - 22 November 2024	Orlando, US

TSG-RAN2 Meeting #129	17 - 21 February 2025	Athens, GR