3GPP TSG-RAN WG2 Meeting #127bis	R2-2408576

Hefei, China, 14 – 18 October 2024

Agenda item:	8.7.4.1

Source:	Apple

Title:	Delay-based Logical Channel Priority Adjustment

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

Based on the study in Rel-19 XR Phase 3, the following objective has been added to the work item [1]:

-	Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]:

-	Specify additional Logical Channel priority handling using delay/deadline information of packets;

-	Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This paper focusses on the topic of Logical Channel (LCH) priority adjustment. It has been agreed that, RAN2 will explore the direction of LCH priority adjustment based on the presence of delay-critical data in the buffer [2, 3]:

RAN2 #126 Agreements: Delay-aware LCP enhancement to resolve the issue of data with low remaining time being delayed due to data from other LCHs with no delay critical data is supported in Rel-19 XR. The solution should consider impact on UE complexity (as already indicated in SI objective description) For delay-aware LCP enhancement, RAN2 considers the following option to override/adjust the priority of LCH based on delay/deadline information as a baseline: - - Use additional priority configured to LCHs in case of these LCHs with delay-critical data FFS whether the priority only applies to delay-critical data within the LCH or for the whole LCH We try to avoid RAN1 impacts. RAN2 assumes no dynamic indications are needed for triggering the delay-aware LCP mechanism. RAN2 assumes this mechanism is configured in a semi-static way. For LCP restrictions based solutions, RAN2 will not discuss solutions requiring RAN1 work. FFS whether other LCP restrictions based approaches are needed/beneficial The solutions should not disallow non-delay critical data from using an UL grant. LCP prioritization within a logical channel will not be considered in RAN2 discussions FFS whether a separate remaining time threshold can be configured for delay aware LCP (i.e. different from the one used for DSR). RAN2 #127 Agreements: RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

# Discussions

## Operations of LCH Priority Adjustment

To enable LCH priority adjustment, we presume the LCH could be configured with at least two priority levels, namely a default LCH priority, and an additional LCH priority. Furthermore, at any time the LCH operates in one of the following modes:

- Normal Mode: The default LCH priority is used for this LCH in the LCP procedure.

- Delay-Critical Mode: The additional LCH priority is used for this LCH in the LCP procedure.

The LCH can switch between these two modes based on certain triggering and exiting conditions, as shown in Figure 1:

Figure 1 Illustration of mode switching for a Logical Channel (LCH)

In general, the triggering condition would be very similar to DSR. That is, as soon as the remaining time till expiry of the discard timer for a buffered PDCP SDU becomes lower than a threshold (this remaining time threshold may be the same or different to the DSR triggering threshold), the LCH may enter the “delay-critical mode” and use the additional LCH priority in subsequent or upcoming LCP procedures.

Proposal 1: A LCH triggers Delay-Critical Mode and applies the additional priority, when at least one PDCP SDU in its buffer has a remaining time till expiry of its discard timer becomes smaller than a threshold.

If PDU Set Discarding is configured for the DRB corresponding to this LCH, the UE should also consider the integrity handling of the PDU Set. Thus, instead of solely deciding the LCH priority based on the remaining time of each PDCP SDU individually, we think the exiting conditions of LCH priority adjustment should be further based on whether any buffered packet belong to a PDU Set that already have at least one delay-critical packet. For instance, even when all PDCP SDUs in the buffer have remaining time larger than a threshold, the LCH should continue to stay in Delay-Critical Mode, as long as any of the buffered PDCP SDU belongs to the same PDU Set as another transmitted PDCP SDU that has previously triggered LCH priority adjustment. In this way, we can ensure that all packets of a urgent PDU Set can be transmitted rapidly using the additional priority, in spite of the remaining time of each individual PDCP SDU of this PDU Set.

Proposal 2: A LCH fallback to Normal Mode and applies the default priority, when all of the PDCP SDUs in its buffer has a remaining time larger than a threshold, and (if pdu-SetDiscarding is configured) do not belong to a same PDU Set as another transmitted PDCP SDU that has previously triggered Delay-Critical Mode.

Based on the above, we can see that triggering/exiting of Delay-Critical Mode for a LCH is hinged on the factors including remaining time of PDCP discard timer and/or PDU Set association. Since we have agreed that the solution should consider the impacts to UE complexity in RAN2 #126, we think this is necessary for RAN2 to confirm that the conditions of changing the priority of a LCH should be checked by PDCP rather than MAC. It is worth highlighting that, there will be a lot of complications if MAC needs to compare the remaining time threshold configured for each LCH (or LCG) against the remaining value of discard timers running in PDCP – such cross-layer operation is highly undesirable for UE implementation due to high complexity. In contrast, it is much more straightforward for the PDCP to compare the remaining time of discard timer of each PDCP SDU with a remaining time threshold configured at PDCP, and to identify the PDCP SDUs that belong to a PDU Set. If the conditions for LCH priority change is fulfilled, PDCP can send an indication to notify MAC layer. Hence, upon the reception of such indication from PDCP, the MAC can switch the priority for the corresponding LCH accordingly.

Proposal 3: The conditions of LCH priority adjustment should be monitored by PDCP instead of MAC. When the conditions for LCH priority adjustment triggering/exiting is detected at PDCP layer, it sends an indication to the MAC layer, and MAC can switch the LCH priority accordingly.

Therefore, we think it is more reasonable if the remaining time threshold for LCH priority adjustment is configured in pdcp-config (which is different from remainingTimeThreshold configured per LCG in Rel-18), while the additional priority is configured in logicalChannelConfig.

Proposal 4: The logical channel priority adjustment scheme operates based on the following configurations:

- A remaining time threshold configured in the IE of pdcp-config, and

- An additional priority configured in the IE of logicalChannelConfig.

Since triggering of LCH priority adjustment is based on the remaining time till discarding, it is questionable if this is applicable to both discardTimer and discardTimerForLowImportance when PSI-based SDU discarding is active (i.e. when uplink congestion has been detected on the corresponding DRB). In particular, LCH priority adjustment may be triggered much more frequently as shorter discard timer values are used for the packets of low importance PDU Sets, which may impact the fairness to other LCHs especially when congestion is known to be present and radio resource is already scarce. In our views, the most straightforward approach is to refrain from triggering LCH priority adjustment by packets with low importance. That is, LCH priority adjustment is only triggered by the remaining time till expiry of discardTimer, but not discardTimerForLowImportance.

Proposal 5: LCH priority adjustment is not to be triggered by low importance packets when PSI-based SDU Discarding is activated for the corresponding DRB.

## LCH Priority Adjustment during LCP Procedure

A few companies have proposed that, the priority of a LCH can be changed during an LCP procedure. In particular, after the first round of data allocation, if all the urgent data from a LCH is already multiplexed into the MAC PDU, the LCH can directly fallback to the default priority before starting the second round. We would like to point out that, LCP is a real-time operation with very tight processing timeline, and this is very difficult for the MAC to further take the status of running PDCP discard timers into account for this procedure. Even if this is checked by PDCP (as suggested in Proposal 3), we cannot guarantee that MAC can react to the indication from PDCP in time. Thus, we think dynamically changing the priority during LCP brings a lot of complications without clear benefits, and in our views it clearly violates the RAN2 #126 agreement on the impacts to UE complexity.

Proposal 6: RAN2 does not pursue dynamic change of logical channel priority in the middle of an LCP procedure.

# Conclusions

In this paper, we have presented our views on LCH priority adjustment, and proposed the following:

Proposal 1: A LCH triggers Delay-Critical Mode and applies the additional priority, when at least one PDCP SDU in its buffer has a remaining time till expiry of its discard timer becomes smaller than a threshold.

Proposal 2: A LCH fallback to Normal Mode and applies the default priority, when all of the PDCP SDUs in its buffer has a remaining time larger than a threshold, and (if pdu-SetDiscarding is configured) do not belong to a same PDU Set as another transmitted PDCP SDU that has previously triggered Delay-Critical Mode.

Proposal 3: The conditions of LCH priority adjustment should be monitored by PDCP instead of MAC. When the conditions for LCH priority adjustment triggering/exiting is detected at PDCP layer, it sends an indication to the MAC layer, and MAC can switch the LCH priority accordingly.

Proposal 4: The logical channel priority adjustment scheme operates based on the following configurations:

- A remaining time threshold configured in the IE of pdcp-config, and

- An additional priority configured in the IE of logicalChannelConfig.

Proposal 5: LCH priority adjustment is not to be triggered by low importance packets when PSI-based SDU Discarding is activated for the corresponding DRB.

Proposal 6: RAN2 does not pursue dynamic change of logical channel priority in the middle of an LCP procedure.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.

- 3GPP RAN2 #127 Chair’s Notes, Maastricht, Netherlands, August 2024.