3GPP TSG-RAN WG2 Meeting #127-bis		         R2-2408684

Hefei, China, Oct 14th – 18th, 2024

Agenda Item:	8.5.4

Source: 	ITRI

Title:  	Discussion on the paging adaptation scheme for NES cell

Document for:	Discussion

Introduction

In this contribution, we present our preliminary thoughts on paging adaptation and propose methods for indicating such adaptations.

Discussion

The paging adaptation method proposed to improve the network energy-saving efficiency in this WI involves reconfiguring the time location of paging occasions. However, the specifics of how to adapt this occasion are still being discussed in RAN2 meetings.

Another aspect of this method is determining how to configure it. Given that the paging occasion is crucial for monitoring paging DCI by UE in various states such as RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, the configuration of the paging adaptation method should be included in the system information as legacy paging configuration.

Whether the configuration of the paging adaptation method can be included in a dedicated control messages may vary depending on the adaptation method used. If individual UE configuration has beneficial for the paging adaptation method to improve the network energy-saving efficiency, employing dedicated control messages for configuration could be considered.

Proposal 1: The adaptation of paging occasion configuration may be conveyed in a system information message, and whether the configuration could be conveyed by a dedicated control message is FFS.

The utilization of the paging adaptation method becomes pertinent when the network detects a reduction in paging instances.  Similar to the cell DTX/DRX scheme, this method requires an activation and deactivation protocol to optimize resource usage and enhance network energy efficiency. For instance, when paging instances decrease, the network may activate the paging adaptation method to improve network energy-saving efficiency. Conversely, when paging instances increase, the network may deactivate this method to evenly distribute paged UE access the network. During activation, UE should utilize the paging adaptation method to monitor the paging DCI, whereas during deactivation, the legacy paging method should be used for monitoring the paging DCI.

The activation and deactivation protocols for the paging adaptation method could be facilitated through the utilization of the paging DCI. The paging message may not be suitable for activating or deactivating the paging adaptation method, because when UEs in the RRC_CONNECTED state may not invariably require paging message reception.

Proposal 2: The adaptation of paging occasion should also support activation and deactivation schemes.

Proposal 2-1: The activation or deactivation of the paging adaptation method could utilize Paging DCI.

Conclusion

Proposal 1: The adaptation of paging occasion configuration may be conveyed in a system information message, and whether the configuration could be conveyed by a dedicated control message is FFS.

Proposal 2: The adaptation of paging occasion should also support activation and deactivation schemes.

Proposal 2-1: The activation or deactivation of the paging adaptation method could utilize Paging DCI.

References

[1]	RP-240170 Revised WID for Enhancements of network energy savings for NR; Ericsson;