3GPP TSG-RAN WG2 Meeting #127bis	R2-2408734

Hefei, China, 14 - 18 October, 2024

Agenda item :	8.0

Source: 		Huawei, HiSilicon, China Telecom

Title: 	Discussion on the LS from SA5 on the number of UEs in RRC_INACTIVE state with data transmission

Document for:	Discussion and Decision

# 1 Introduction

RAN2 received an LS from SA5 in [1] where the following is stated:

The intention of the SDT (Small Data Transmission) mechanism aims to allow UEs to transmit data in the RRC_INACTIVE state. SA5 understands the number of SDT UEs in RRC_INACTIVE state can help operators to evaluate the load status of a specific cell. SA5 would like to ask RAN2 to evaluate whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission.

In this contribution we evaluate the request from SA5 and propose further actions related to it.

# 2 Discussion

The LS is related to SA5’s “WID on 5G performance measurements and KPIs phase 4” where SA5 has the following objective:

WT-1: To define new 5G performance measurements and KPIs for the following features, including: WT-1.1 Mobility Enhancements: Layer 1/2 triggered mobility (LTM); WT-1.2 Architecture Enhancements for XR (Extended Reality) and media service; WT-1.3 Mobile Terminated-Small Data Transmission; WT-1.4 Enhancements of NR Multicast and Broadcast Services; WT-1.5 Access Traffic Steering, Switch and Splitting support in the 5G system architecture Phase 3.

Performance measurements and KPIs are defined by SA5, e.g. in TS 28.552, but they also often rely on the definitions of L2 measurements as defined by RAN2 in TS 38.314 [3]. This is why SA5 would like RAN2 to get involved in this task. In section 5.1.1.23 of TS 28.552 [2], the measurements related to number of active UEs are specified, e.g. mean/max number of Active UEs per cell or per UL/DL direction. These measurements in turn rely on the measurements defined in TS 38.314, section “4.2.1.3	Number of active UEs in RRC_CONNECTED” and its purpose is defined as follows:

#### 4.2.1.3 Number of active UEs in RRC_CONNECTED ##### 4.2.1.3.1 General The objective of the measurement is to measure the number of active UEs per QoS level for OAM performance observability or for SON functions e.g., mobility load balancing. It is intended to be part of a calculation to determine the bitrate UEs achieve when they are active, i.e. when applications are transmitting and receiving data. The measurements are applicable for both non-split gNB and split gNB deployment scenario.

However, as the name suggests, currently this metric considers only the UEs that have data available for transmission and/or reception and which are in RRC_CONNECTED state. Since Rel-17, the UEs are also able to transmit/receive data while in RRC_INACTIVE state which is not considered by the metric. On the other hand, such UEs contribute to the overall load status of a specific cell. In our understanding, SA5 goal is to fill this gap to be able to determine the overall cell load status.

Observation 1: The goal of SA5 is to be able to evaluate the overall load status of a specific cell by considering UEs in RRC_INACTIVE state with data transmission, in addition to the UEs having data transmission in RRC_CONNECTED state.

It should be noted that there is also another metric called “Number of stored inactive UE contexts” which is defined in section 4.2.1.4 of TS 38.314. However, it cannot be used for this purpose as its only used to estimate the impact on storage memory in RAN, as clarified in its description in section 4.2.1.4.1 of TS 38.314:

#### 4.2.1.4 Number of stored inactive UE contexts ##### 4.2.1.4.1 General The objective of the measurement is to measure number of stored inactive UE contexts for OAM performance observability. It is intended to be part of indication about the memory consumption in a RAN node. The measurement is obtained by sampling at a pre-defined interval, the number of inactive UE contexts for each gNB and then taking the arithmetic mean or maximum value over pre-defined time duration.

Observation 2: Existing Layer 2 metrics defined in TS 38.314 cannot be used to provide information about the number of UEs in RRC_INACTIVE with data transmission.

Therefore, RAN2 would have to define a new set of metrics in TS 38.314 to address the requirement from SA5. There are two aspects to consider from RAN2 point of view:

- Technical feasibility for the gNB to perform such measurement: from gNB perspective such measurement should be equally possible as the corresponding measurement performed for UEs in RRC_CONNECTED. The gNB is always aware that a UE performs SDT procedure and can identify whether the data is available for such UEs in UL and DL directions.

- Impact on RAN2 specifications: new metrics would have to be added to TS 38.314, but their definitions should be very similar to the ones existing already for RRC_CONNECTED UEs, as defined in section 4.2.1.3 of TS 38.314. It is however not entirely clear from the LS whether SA5 requires all these metrics (i.e. per UL/DL, per DRB etc.). We may ask for a clarification on this aspect. For the time being, we provide a potential TP in Annex A of this contribution, assuming all corresponding metrics as for RRC_CONNECTED UEs are needed.

Based on the above, we propose to reply to SA5, as follows:

Proposal 1: RAN2 should reply to SA5 that it is possible for RAN2 to define a set of metrics for the number of UEs in RRC_INACTIVE state with data transmission, similar to the ones defined for RRC_CONNECTED UEs in section 4.2.1.3 of TS 38.314.

Proposal 2: RAN2 should ask for a clarification from SA5 on whether all the metrics as defined for RRC_CONNECTED UEs are needed, e.g. per UL/DL, per DRB etc.

A draft of the LS is provided in Annex B where we also ask SA5 to confirm that the above is aligned with SA5 intention. Provided that SA5 confirms that RAN2 analysis and proposal are aligned with an intention from SA5, RAN2 would also need to decide how to handle the required CRs procedurally. We see two options:

- Agree the CR as part of TEI19 – even though the changes as captured in Annex A are lengthy, the exact contents and definitions of the new metrics follow to a great extent the ones already defined for UEs in RRC_CONNECTED. Therefore, the related work does not require a lot of discussions and can be handled within a single meeting and hence is appropriate for treatment as part of TEI19.

- Update Rel-19 SON/MDT WID to include the definition of these additional metrics requested by SA5. This WID is already rather busy, but since Layer 2 metrics are usually defined as part of SON/MDT work, it would be procedurally a cleanest way to include it there. As mentioned above, the additional work related to this task would not be high and an additional amount of, e.g. 0.25 TU for one meeting should be sufficient.

Proposal 3: RAN2 to discuss whether to handle the definition of the metrics requested by SA5 as part of TEI19 or as part of (updated) SON/MDT WID.

# 3 Conclusion

Based on the discussion in this paper, we observe and propose the following:

Observation 1: The goal of SA5 is to be able to evaluate the overall load status of a specific cell by considering UEs in RRC_INACTIVE state with data transmission, in addition to the UEs having data transmission in RRC_CONNECTED state.

Observation 2: Existing Layer 2 metrics defined in TS 38.314 cannot be used to provide information about the number of UEs in RRC_INACTIVE with data transmission.

Proposal 1: RAN2 should reply to SA5 that it is possible for RAN2 to define a set of metrics for the number of UEs in RRC_INACTIVE state with data transmission, similar to the ones defined for RRC_CONNECTED UEs in section 4.2.1.3 of TS 38.314.

Proposal 2: RAN2 should ask for a clarification from SA5 on whether all the metrics as defined for RRC_CONNECTED UEs are needed, e.g. per UL/DL, per DRB etc.

Proposal 3: RAN2 to discuss whether to handle the definition of the metrics requested by SA5 as part of TEI19 or as part of (updated) SON/MDT WID.

In Annex A, we provide a TP for TS 38.314, which assumes all corresponding metrics as for RRC_CONNECTED UEs are needed for reflecting the number of UEs in RRC_INACTIVE state with data transmission.

In Annex B, a draft of the LS is provided where we also ask SA5 to confirm RAN2 understanding and clarify whether all the metrics as defined for RRC_CONNECTED UEs are needed.

# 4 References

- S5-245138,  LS on Number of UEs in RRC_INACTIVE state with data transmission, Source: SA5, To: RAN2, Cc: RAN3

- 3GPP TS 28.552	Management and orchestration; 5G performance measurements

- 3GPP TS 38.314	NR; Layer 2 Measurements

# Annex A – TP for TS 38.314

#### 4.2.1.X	Number of active UEs in RRC_INACTIVE

##### 4.2.1.X.1	General

The objective of the measurement is to measure the number of active UEs in RRC_INACTIVE per QoS level for OAM performance observability or for SON functions e.g., mobility load balancing. Active UEs in RRC_INACTIVE are defined as the UEs which have an ongoing SDT procedure, i.e. are actively exchanging data with the network while remaining in RRC_INACTIVE state. It is intended to be part of a calculation to determine the bitrate UEs achieve during an ongoing SDT procedure, i.e. when applications are transmitting and receiving data while the UE remains in RRC_INACTIVE. The measurements are applicable for both non-split gNB and split gNB deployment scenario.

##### 4.2.1.X.2	Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.2-1: Definition for Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell

| Definition | Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the DL for SDT DRBs. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.2-2 below. |
| --- | --- |

Table 4.2.1.X.2-2: Parameter description for Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell

| M(T,drbid,p) | Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i. Data available for transmission includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

##### 4.2.1.X.3	Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.3-1: Definition for Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell

| Definition | Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the DL for SDT DRBs. Detailed Definition: M(T,drbid,p)=maxTNi,drbid, where explanations can be found in the table 4.2.1.3.3-2 below. |
| --- | --- |

Table 4.2.1.X.3-2: Parameter description for Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell

| M(T,drbid,p) | Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the DL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i. Data available for transmission includes data for which HARQ transmission has not yet terminated. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

##### 4.2.1.X.4	Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell

Protocol Layer: MAC

Table 4.2.1.X.4-1: Definition for Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell

| Definition | Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.4-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.4-2: Parameter description for Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell

| M(T,drbid,p) | Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

##### 4.2.1.X.5	Max number of active UEs in RRC_INACTIVE in the UL per DRB per cell

Protocol Layer: MAC

Table 4.2.1.X.5-1: Definition for Max number of active UEs in RRC_INACTIVE in the UL per DRB per cell

| Definition | Maximum number of active UEs in RRC_INACTIVE in the UL per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs. Detailed Definition: MT,drbid,p=maxTNi,drbid, where explanations can be found in the table 4.2.1.3.5-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.5-2: Parameter description for Max number of active UEs in RRC_INACTIVE in the UL per DRB per cell

| M(T,drbid,p) | Maximum number of active UEs in RRC_INACTIVE in the UL per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i This is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

##### 4.2.1.X.6	Mean number of active UEs in RRC_INACTIVE per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.6-1: Definition for Mean number of active UEs in RRC_INACTIVE per cell

| Definition | Mean number of active UEs in RRC_INACTIVE per cell. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs, or there is data available for transmission for the DL for SDT DRBs, or both. Detailed Definition: M(T,p)=∀iN(i)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.6-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.6-2: Parameter description for Mean number of active UEs in RRC_INACTIVE per cell

| M(T,p) | Mean number of active UEs in RRC_INACTIVE per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |

##### 4.2.1.X.7	Max number of active UEs in RRC_INACTIVE per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.7-1: Definition for Max number of active UEs in RRC_INACTIVE per cell

| Definition | Maximum number of active UEs in RRC_INACTIVE per cell. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs, or there is data available for transmission for the DL for SDT DRBs, or both. Detailed Definition: M(T,p)=maxTNi, where explanations can be found in the table 4.2.1.3.7-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.7-2: Parameter description for Max number of active UEs in RRC_INACTIVE per cell

| M(T,p) | Maximum number of active UEs in RRC_INACTIVE per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |

##### 4.2.1.X.8	Mean number of active UEs in RRC_INACTIVE per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.8-1: Definition for Mean number of active UEs in RRC_INACTIVE per DRB per cell

| Definition | Mean number of active UEs in RRC_INACTIVE per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs, or there is data available for transmission for the DL for SDT DRBs, or both. Detailed Definition: M(T,drbid,p)=∀iN(i,drbid)I(T,p)*1010, where explanations can be found in the table 4.2.1.3.8-2 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.8-2: Parameter description for Mean number of active UEs in RRC_INACTIVE per DRB per cell

| M(T,drbid,p) | Mean number of active UEs in RRC_INACTIVE per DRB per cell, averaged during time period T. Unit: 0.1. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| I(T,p) | Total number of sampling occasions during time period T. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

##### 4.2.1.X.9	Max number of active UEs in RRC_INACTIVE per DRB per cell

Protocol Layer: MAC, RLC

Table 4.2.1.X.9-1: Definition for Max number of active UEs in RRC_INACTIVE per DRB per cell

| Definition | Maximum number of active UEs in RRC_INACTIVE per DRB per cell. The DRBs are mapped with the same 5QI. This measurement refers to UEs for which there is data available for transmission for the UL for SDT DRBs, or there is data available for transmission for the DL for SDT DRBs, or both. Detailed Definition: M(T,drbid,p)=maxTNi,drbid, where explanations can be found in the table 4.2.1.3.9-1 below. |
| --- | --- |

NOTE:	For this measurement, the expected accuracy is dependent on application scenario, cell load, UE configuration and how DRBs are distributed over logical channel groups.

Table 4.2.1.X.9-2: Parameter description for Max number of active UEs in RRC_INACTIVE per DRB per cell

| M(T,drbid,p) | Maximum number of active UEs in RRC_INACTIVE per DRB per cell, averaged during time period T. Unit: Integer. |
| --- | --- |
| N(i,drbid) | Number of UEs for which there is data available for transmission for the UL or for the DL or for both in MAC or RLC protocol layers for a Data Radio Bearer of traffic class at sampling occasion i For UL, this is a gNB estimation that is expected to be based on Buffer Status Reporting, provided configured grants and progress of ongoing HARQ transmissions (by including data for which HARQ transmission has not yet terminated). In addition, the gNB can use the analysis of received data in the estimation. In such case, when DRB cannot be determined at the time of the sampling occasion, gNB can determine DRB after successful reception of data. |
| i | Sampling occasion during time period T. A sampling occasion shall occur once every p seconds. |
| p | Sampling period length. Unit: second. The sampling period shall be at most 0.1 s. |
| T | Time Period during which the measurement is performed, Unit: second. |
| drbid | The DRBs mapped with the same 5QI. |

# Annex B – Draft reply LS to SA5

3GPP TSG-RAN WG2 Meeting #127bis	   R2-24xxxxx

Hefei, China, 14 - 18 October, 2024

Title:	Reply to SA5 LS on Number of UEs in RRC_INACTIVE state with data transmission

Response to:	S5-245138

Release:	Rel-19

Work Item:	PM_KPI_5G_Ph4

Source:	RAN2

To:	SA5

Cc: 	RAN3

Contact Person:

Name:	Dawid Koziol

Email:		dawid.koziol AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN2 would like to thank SA5 for the LS on Number of UEs in RRC_INACTIVE state with data transmission. RAN2 evaluated whether there is a possibility to specify the number of UEs in RRC_INACTIVE state with data transmission, as requested by SA5. In RAN2 understanding it is feasible to define a set of metrics similar to the ones specified in TS 38.314, section 4.2.1.3, but referring to active UEs in RRC_INACTIVE, i.e. the UEs with an ongoing SDT procedure, i.e.:

- Mean number of active UEs in RRC_INACTIVE in the DL per DRB per cell.

- Max number of active UEs in RRC_INACTIVE in the DL per DRB per cell.

- Mean number of active UEs in RRC_INACTIVE in the UL per DRB per cell.

- Max number of active UEs in RRC_INACTIVE in the UL per DRB per cell.

- Mean number of active UEs in RRC_INACTIVE per cell.

- Max number of active UEs in RRC_INACTIVE per cell.

- Mean number of active UEs in RRC_INACTIVE per DRB per cell.

- Max number of active UEs in RRC_INACTIVE per DRB per cell.

Before moving forward with the necessary CR, RAN2 would like to ask SA5 whether the above is aligned with SA5’s intention, e.g. whether all the metrics as defined for RRC_CONNECTED UEs are required (e.g. per UL/DL, per DRB etc.).

2. Actions:

To SA5:

ACTION: RAN2 respectfully asks SA5 to confirm that the above understanding from RAN2 is aligned with SA5 intention, and clarify whether all the metrics as defined for RRC_CONNECTED UEs are needed, e.g. per UL/DL, per DRB etc.

3. Date of Next RAN2 Meetings:

RAN2#128	18th – 22nd November 2024			Orlando, US

RAN2#129	17th – 21st February 2025				Athens, Greece