3GPP TSG-RAN WG2 #127bis	R2-2408750

Hefei, China, Oct 14th – 18th, 2024

Source:		NEC

Agenda item:		8.10.2	MRO enhancements for Rel-18 mobility features

Title:	MRO for CHO with candidate SCG(s)

Document for:		Discussion and decision

- Introduction

During last RAN2 meeting, and achieved the following agreement was made for MRO enhancements for CHO with candidate SCG(s) [1]:

UE includes following information in RLF report:

- b.	Time information regarding condition fulfilment for CHO with candidate SCGs. Details are FFS. We consider both the case when both CHO condition and associated CPC condition are fulfilled, and the case when CHO (or CPC) is fulfilled but CPC (or CHO) conditions are not fulfilled.

- c.	Measurement results of PCells and PSCells.

In this paper, we provide our further consideration on MRO for the CHO with candidate SCG(s).

- Discussion

2.1 CHO with candidate SCG(s)

During last meeting, it was agreed to include time information regarding condition fulfilment for CHO with candidate SCG(s). And both the case when both CHO condition and associated CPC condition are fulfilled, and the case when only one of the CHO condition and CPC condition is fulfilled.

In case both CHO condition associated CPC conditions are fulfilled, the time of second fulfilled condition is the time of the execution of CHO with candidate SCG(s), which can be deduced by the network based on timeConnFailure and timeSinceFailure. And in order to report time information of the both fulfilled conditions, similar to Rel-17 MRO for CHO, the UE needs to include in RLF-report additional information indicating the condition (CHO or CPC) which fulfils first, and the time gap between the first fulfilled condition and second fulfilled condition.

Proposal 1: In case of both CHO condition and associated CPC condition fulfilled, the UE includes in RLF-report additional information indicating

- the condition (CHO condition or CPC condition) which fulfils first

- time gap between the first fulfilled condition and second fulfilled condition.

In case only one of the CHO condition and CPC condition is fulfilled, to report time information of the one fulfilled condition, the UE needs to include needs to include in RLF-report additional information indicating the condition which is fulfilled, and the time gap between the fulfilled condition until the connection failure.

Proposal 2: In case of only one of CHO condition and CPC condition fulfilled, the UE includes in RLF-report additional information indicating

- the condition which is fulfilled

- the time gap between the fulfilled condition until the connection failure

For execution condition of one candidate cell, at most two triggering conditions can be configuration. Currently, the UE can store information related to trigger condition(s) for candidate PCell in measurement results of RLF-report, i.e., the first fulfilled trigger condition, and the time gap between the fulfilment of the two trigger conditions for candidate PCell. In this way, the network can further optimize the two trigger conditions associated with the PCell. During last meeting, it was also agreed that measurement results of PCells and PSCells is included in the RLF-report. For CHO with candidate SCG(s), it is beneficial that the UE can also collect the information related to the two trigger conditions for the candidate PSCell, such that the network can optimize the trigger conditions of the candidate PSCell in case of CHO with candidate SCG(s).

Proposal 3. UE stores in the measurement results of PSCell of RLF-report, information related to the triggering conditions for the candidate PSCell, which comprises

- the first fulfilled triggering condition associate with the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for candidate PSCell

For near failure of CHO with candidate SCG(s), in previous meeting, whether to introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition for SHR or SPR. In our understanding, near failure means that the mobility procedure is successful, however the performance of the UE is not good during the mobility procedure, e.g., out of sync is triggered (corresponding to T310 trigger), the service interruption is too long (corresponding to T304 trigger). For the case “time gap between fulfilment of the execution conditions of PCell and PSCell being larger than a threshold”, the performance of the UE may be not impacted. Just like the legacy CHO/CPAC, the time gap between the fulfilment of the two execution conditions of the candidate cell being larger than a threshold is not one triggering condition for SHR and SPR. Therefore, we do not think “time gap between fulfilment of the execution conditions for PCell and PSCell being larger than a threshold” is a useful triggering condition for SHR or SPR.

Proposal 4. Do not introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition of SHR or SPR.

# 3. Conclusion

In this paper, we discuss on further consideration on the MRO for CHO with candidate SCG(s), and have the following proposals:

MRO for CHO with candidate SCG(s)

Proposal 1: In case of both CHO condition and associated CPC condition fulfilled, the UE includes in RLF-report additional information indicating

- the condition (CHO condition or CPC condition) which fulfils first

- time gap between the first fulfilled condition and second fulfilled condition.

Proposal 2: In case of only one of CHO condition and CPC condition fulfilled, the UE includes in RLF-report additional information indicating

- the condition which is fulfilled

- the time gap between the fulfilled condition until the connection failure

Proposal 3. UE stores in the measurement results of PSCell of RLF-report, information related to the triggering conditions for the candidate PSCell, which comprises

- the first fulfilled triggering condition associate with the candidate PSCell

- the time gap between the fulfilment of the two triggering conditions for candidate PSCell

Proposal 4. Do not introduce “time gap between fulfilment of the execution conditions being larger than a threshold” as an additional triggering condition of SHR or SPR.

# Reference

[1] Meeting report of RAN2 #127